3GPP TSG-SA WG6 Meeting #65	S6-250212

Athens, Greece, 17th – 21st February 2025	(revision of S6-250xyz)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.222 | CR | 0254 | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Solving ENs on finer granularity of access control for service API |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | CAPIF_Ph3 |  |  |  |  |  | Date: |  |  | 2025-01-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | S6-244696 agreed in SA6#63 extended clauses 8.3.2.1, 8.7.2.1 and 8.7.2.2 to include information about the supported operations and resources during API Discovery and API Publihsing. Clause 8.11.3 was extended to specify that the CCF Authorization server may include information about the service operation(s) and resource(s) the API invoker may access. In relation to this functionality, the following ENs remain in 8.11.3: Editor's Note: Authorization for finer level service API access and information intended to be provided by the API invoker at invocation, whilst accounting for the provided purpose for data processing, is to be detailed by SA3. This EN should be removed, since accounting for the purpose for data processing is described under clause 8.31. Consequently, the purpose IE needs to be removed as well from the step 1. and Editor's Note: Access authorization per service operation and, optionally, reesource information is to be detailed by SA3. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Step 1 is updated to remove the purpose for data processing parameter from the list of provided parameters. The ENs is removed. - Step 3 is updated, to indicate that the support of finer granularity for API invoker access to the exposed API is based on the OAuth 2 based authorization specified in TS 33.122 clause 6.5.2.3. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The ENs remain. In addition, the incorrect information will remain, mixing parameters related with RO authorization (purpose) with parameters to authorize the API invoker access. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.11.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 8.11.3	Procedure

Figure 8.11.3-1 illustrates the procedure for obtaining authorization to access the service API.

Pre-condition:

1.	The API invoker is onboarded and has received an API invoker identity.

Figure 8.11.3-1: Procedure for the API invoker obtaining authorization for service API access

1.	The API invoker sends an obtain service API authorization request to the CAPIF core function for obtaining permission to access the service API by including the API invoker information (including its own identity and optionally application service information such as application service provider and application identifier), optionally service API operation(s) and resource(s), and any information required for authentication of the API invoker. The request may include desired Network Slice Info of the service API. In addition to the information identifying the service API, the request may include finer level service API access requirements (e.g., access per service API operation or access per API service resource).

2.	The CAPIF core function validates the authentication of the API invoker (using authentication information) and checks whether the API invoker is permitted to access the requested service API considering service API operation(s) and resource(s), if available. The CAPIF core function may additionally verify the Network Slice Info, e.g., check that the desired Network Slice Info by the API invoker is included in the supported Network Slice Info for the indicated service API.

Editor's Note:		Authorization related to network slice are to be detailed by SA3.

NOTE 1:	The authorization of Network Slice Info is performed by the CAPIF core function within the PLMN trust domain.

NOTE 2:	The authentication process is specified in subclause 6.5.2 of 3GPP TS 33.122 [12].

3.	Based on the API invoker's subscription information the authorization information to access the service APIs, optionally including access authorization per service operation and, optionally, resource, is sent to the API invoker in the obtain service API authorization response.

NOTE 3:	API invoker authorization granularity mechanisms are specified in 3GPP TS 33.122 [12].

NOTE 4:	The mechanism for distribution of the authorization information for the API invoker to the API exposing function over CAPIF-3 reference point is specified in subclause 6.5.2 of 3GPP TS 33.122 [12].

* * * END of Change * * * *