h3GPP TSG-SA WG6 Meeting #65	S6-250227

Athens, Greece, 17th – 21st February 2025	(revision of S6-25xxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.482 | CR | 0024 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to AI/ML Task Transfer |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_App |  |  |  |  |  | Date: |  |  | 2025-02-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Errors exist in the IEs of the information flows for AI/ML task transfer services. In addition, there are ENs left for AI/ML Task Transfer: Editor’s Note: Whether there is need of additional IEs or reduction of IEs is FFS. This paper proposes to correct the errors, and remove the ENs as no additional IEs or reduction of IEs is needed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The changes include: - Correct errors in the IEs of the information flows for AI/ML task transfer services. - Removed the ENs under AI/ML task transfer assist request and Direct AI/ML task transfer request tables. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Errors exist and ENs left in the specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.6.3.2, 8.6.3.3, 8.6.3.4, 8.6.3.6, 8.6.3.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 8.6.3.2	AI/ML task transfer assist request

Table 8.6.3.2-1 describes information elements for the AI/ML task transfer assist request from the source AI/ML member to the AIMLE Server.

Table 8.6.3.2-1: AI/ML task transfer assist request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The identifier of the source AI/ML member (e.g. identifier of the source AIMLE client or the VAL UE). |
| VAL service ID | O | The identifier of the VAL service for which the assistance information is requested. |
| AI/ML Task Type | M | The type of the AI/ML operation (e.g. ML model training). |
| AI/ML Information Type | M | The type of the AI/ML information in the AI/ML task need be transferred (e.g. intermediate AI/ML operation status, intermediate AI/ML operation results). |
| > AI/ML model remaining training requirement (NOTE) | O | Indicate the requirement for AI/ML model training including, required remaining training resource, required remaining training number of iterations, |
| > AI/ML intermediate information (NOTE) | O | Indicate the AI/ML intermediate information for intermediate AI/ML operation, including AI/ML intermediate model, AI/ML intermediate model used training time, used training resource, used training number of iterations. |
| Time validity | O | The time validity of the request. |
| NOTE: The IE only present when AI/ML Task Type is ML model training. |  |  |

* * * Next Change * * * *

#### 8.6.3.3	AI/ML task transfer assist response

Table 8.6.3.3-1 describes information elements for the AI/ML task transfer assist response from the AIMLE Server to the source AI/ML member.

Table 8.6.3.3-1: AI/ML task transfer assist response

| Information element | Status | Description |
| --- | --- | --- |
| Transfer Mode | O | Indication the transfer mode (e.g. direct transfer). |
| Target AI/ML member(s) | M | The identifier of the target AI/ML member (e.g. identifier of the target AIMLE client or the VAL UE). |
| Assistance Information | M | Assistance information for the AI/ML task transfer (e.g. time window for the transfer). |

* * * Next Change * * * *

#### 8.6.3.4	AI/ML task transfer request

Table 8.6.3.4-1 describes information elements for the AI/ML task transfer request from the AIMLE Server to the target AI/ML member.

Table 8.6.3.4-1: AI/ML task transfer request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The identifier of the the AIMLE server. |
| Source AI/ML member | M | The identifier of the source AI/ML member (e.g. identifier of the source AIMLE client or the VAL UE). |
| AI/ML Task Type | M | The type of the AI/ML operation (e.g. ML model training). |
| AI/ML Information Type | M | The type of the AI/ML information in the AI/ML task need be transferred (e.g. intermediate AI/ML operation status, intermediate AI/ML operation results). |
| AI/ML Task Transfer Time | O | Information on time or time window for the AI/ML task transfer. |
| Time validity | O | The time validity of the request. |

* * * Next Change * * * *

#### 8.6.3.6	Direct AI/ML task transfer request

Table 8.6.3.6-1 describes information elements for the direct AI/ML task transfer request from the source AI/ML member to the target AI/ML member.

Table 8.6.3.6-1: Direct AI/ML task transfer request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The identifier of the source AI/ML member (e.g. identifier of the source AIMLE client or the VAL UE). |
| AI/ML Task Type | M | The type of the AI/ML operation (e.g. ML model training). |
| AI/ML Information Type | M | The type of the AI/ML information in the AI/ML task need be transferred (e.g. intermediate AI/ML operation status, intermediate AI/ML operation results). |
| AI/ML Task Transfer Time | O | Information on time or time window for the AI/ML task transfer. |
| Time validity | O | The time validity of the request. |

* * * Next Change * * * *

#### 8.6.3.8	AIMLE server-controlled AI/ML task transfer request

Table 8.6.3.8-1 describes information elements for the AIMLE server-controlled AI/ML task transfer request from the source AI/ML member to the AIMLE server.

Table 8.6.3.8-1: AIMLE server-controlled AI/ML task transfer request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The identifier of the source AI/ML member (e.g. identifier of the source AIMLE client or the VAL UE). |
| AI/ML Task Type | M | The type of the AI/ML operation (e.g. ML model training). |
| AI/ML Information Type | M | The type of the AI/ML information in the AI/ML task need be transferred (e.g. intermediate AI/ML operation status, intermediate AI/ML operation results). |
| AI/ML Task Transfer Time | O | Information on time or time window for the AI/ML task transfer. |
| Time validity | O | The time validity of the request. |

* * * END of Changes * * * *