3GPP TSG-SA WG6 Meeting #65	S6-250230

Athens, Greece, 17th – 21st February 2025	(revision of S6-25xxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.482 | CR | 0027 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding AIMLE Client to FL Member Registration Procedure and Informaiton Flows |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_App |  |  |  |  |  | Date: |  |  | 2025-02-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | AIMLE client can be selected as FL member involve a FL training process as in clauses 8.12, 8.17, and 8.18. However, AIMLE Client is missing in the FL member registration procedure and information flows. This paper proposes to add AIMLE client to FL member registration procedure and information flows. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The changes include: - Added AIMLE client to FL member registration procedure. - Added AIMLE client to FL member registration information flows. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification is incompleted, AIMLE Client is missing in the FL member registration procedure and information flows. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.4, 8.4.1, 8.4.2, 8.4.3, 8.4.3.2, 8.4.3.3, 8.4.3.4, 8.4.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

## 8.4	Support for FL member registration

### 8.4.1	General

This clause provides the procedures for the registration and registration update of candidate FL members in the ML repository which serves as service registry for the FL members undertaking a task related to the ML model lifecycle, such as ML model local training. Candidate FL members can be application layer entities at the server side (e.g., VAL server, AIMLE server), which can potentially be selected as FL clients or FL server for a particular ASP /vertical requirement, or at the client side (e.g. AIMLE client), which can potentially be selected as FL clients. If VAL server or the AIMLE client is the candidate FL member, it registers indirectly via AIMLE server to the ML repository.

* * * Next Change * * * *

### 8.4.2	Procedure on FL member registration

Figure 8.4.2-1 illustrates the procedure where the registration of a candidate FL member happens via the ML repository, serving as AIML service registry.

Figure 8.4.2-1 Procedure for registration on FL member registry

1.	The candidate FL member (e.g., VAL server via AIMLE server, AIMLE server, AIMLE client via AIMLE server) sends an FL member registration request to the ML repository for registering to the ML repository which acts as the AIML service registry.

2.	The ML repository validates the received request and generates the identity and other security related information for all the FL members listed in the registration request.

3.	The ML repository sends the generated information in the FL member registration response message to the candidate FL member.

* * * Next Change * * * *

### 8.4.3	Procedure on FL member registration update

Figure 8.4.3-1 illustrates the procedure where the registration update of a candidate FL member happens via the ML repository.

Figure 8.4.3-1: Procedure for registration update

1.	The candidate FL member (VAL server via AIMLE server, AIMLE Server, AIMLE client via AIMLE server) sends an FL member registration update request to the ML repository acting as AIML service registry for updating the registration of the FL member.

2.	The ML repository validates the received update request and generates the identity and other security related information for the FL member.

3.	The ML repository sends the generated information in the FL member registration update response message to the candidate FL member.

* * * Next Change * * * *

#### 8.4.4.2	FL member registration request

Table 8.4.4.2-1 describes information elements for the FL member registration request from the candidate FL member (VAL server via AIMLE server, AIMLE server) to the ML repository/registry.

Table 8.4.4.2-1: FL member registration request

| Information element | Status | Description |
| --- | --- | --- |
| Security information | M | Information for ML repository to validate the registration request. |
| FL member ID | M | The identifier of the candidate FL member. This can be the VAL server ID, the EAS ID, the AIMLE server ID, the AIMLE client ID, based on which entity is the candidate FL member. |
| FL member supported AI/ML role | M | The supported AI/ML role of FL member which can be used as FL server or FL client. |
| FL member capabilities | O (NOTE) | FL member capability information (e.g. ML application type, allowed resource usage level). |
| FL member velocity | O | Indicates the FL member velocity (e.g., mobile or static). |
| FL member Location information | O | Indicates the location information of the UE (e.g., Cell Identity, Tracking Area Identity, GPS Coordinates or civic addresses). |
| FL member Availability schedule | O (NOTE) | Availability schedule of the FL member for a certain FL member. |
| Supported ML model ID list | O | The list of ML model IDs for which the FL member can be used. |
| Area of Interest | O | The area of interest (i.e. location co-ordinates) for which the registration applies. |
| Time validity | O | The time validity for the registration of the FL member. |
| NOTE: At least one of these shall be present. |  |  |

* * * Next Change * * * *

#### 8.4.4.3	FL member registration response

Table 8.4.4.3-1 describes information elements for the FL member registration response to the candidate FL member (VAL server via AIMLE server, AIMLE server, AIMLE client via AIMLE server) from the ML repository/registry.

Table 8.4.4.3-1: FL member registration response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | The result of the registration request (positive or negative acknowledgement). |
| Registration ID | M | The generated ID for the registration. |

* * * Next Change * * * *

#### 8.4.4.4	FL member registration update request

Table 8.4.4.4-1 describes information elements for the FL member registration update request from the candidate FL member (VAL server via AIMLE server, AIMLE server, AIMLE client via AIMLE server) to the ML repository/registry.

Table 8.4.4.4-1: FL member registration update request

| Information element | Status | Description |
| --- | --- | --- |
| Registration ID | M | Identifier of the existing registration for which the update request applies. |
| Security information | M | Information for ML repository to validate the registration update request |
| FL member ID | M | The identifier of the candidate FL member for which the update applies. This can be the VAL server ID, the EAS ID, the AIMLE server ID, the AIMLE client ID, based on which entity is the candidate FL member. |
| Updated registration parameters | M | The updates of the registration parameters. |
| > FL member supported AI/ML role | O (see NOTE) | The updated supported AI/ML role of FL member which can be FL server, FL client. |
| > FL member Availability schedule | O (see NOTE) | Updated Availability schedule of the FL member. |
| > supported ML model ID list | O (see NOTE) | The updated list of ML model IDs for which the FL member can be used |
| > area of interest | O (see NOTE) | The updated area of interest for the registration update. |
| > time validity | O (see NOTE) | The update time validity for the FL member registration update. |
| NOTE: At least one of these information elements shall be present. |  |  |

* * * Next Change * * * *

#### 8.4.4.5	FL member registration update response

Table 8.4.4.5-1 describes information elements for the FL member registration update response to the candidate FL member (VAL server via AIMLE server, AIMLE server, AIMLE client via AIMLE server) from the ML repository/registry.

Table 8.4.4.5-1: FL member registration update response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | The result of the registration update request (positive or negative acknowledgement). |
| Registration ID | O | The generated ID for the new registration based on the update (if needed). |

* * * END of Changes * * * *