3GPP TSG-SA WG6 Meeting #65	S6-2500238

Athens, Greece 17th – 21st February 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.437 | CR | 0010 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | SMAS discovery procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | InterDigital |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | Metaverse_App |  |  |  |  |  | Date: |  |  | 2025-02-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | This contribution proposes to resolve the following EN for SMAS discovery: “Editor’s Note: Whether the SM service discovery procedure is defined in 3GPP TS 23.437 and whether CAPIF API discovery filters in 3GPP TS 23.222 [7] can be enhanced to support discovery of an API provider with specific spatial map coverage is FFS.” It is proposed to include SMAS discovery capabilities to the SEAL SM server for discovering VAL servers with SM capabilities based on the conclusions captured in 3GPP TR 23.700-21. |  |  |  |  |  |  |  |  |
| Summary of change: |  | SMAS discovery procedures, information flows and APIs. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The Metaverse_App feature is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.6.5, 9.7.1, 9.7.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 9.6.5	SMAS discovery

#### 9.6.5.1	General

SMAS discovery enables an authorized consumer (VAL server or SEAL SM client) to discover VAL servers with SM capabilities.

#### 9.6.5.2	Procedure

Figure 9.6.5.2-1 depicts the procedure for SMAS discovery.

Pre-conditions:

1.	The VAL server or SEAL SM client has received information (e.g. URI, IP address) related to the SEAL SM server;

2.	The VAL server or SEAL SM client has received security credentials authorizing it to communicate with the SEAL SM server.

3. The SEAL SM server does not have a spatial map available for the area of interest for which requestor is discovering a spatial map.

Figure 9.6.5.2-1: SMAS discovery procedure

1.	The VAL server or SEAL SM client sends a SMAS discovery request message to the SEAL SM server to discover VAL servers with SM capabilities. The request includes a requestor ID, security credentials, three-dimensional area of interest or localization information to discover VAL servers with SM capabilities and optionally discovery filters as described in clause 9.6.5.3.1.

2.	The SEAL SM server authorizes the requestor and validates the request. If the request is authorized, the SEAL SM server then determines if any VAL server(s) registered with the SEAL SM server can offer spatial map(s) according to the requested area of interest.

3.	The SEAL SM server sends a SMAS discovery response message to the requestor. If the SEAL SM server has determined at least one VAL server with SM capabilities registered with the SEAL SM server, the response includes an indication of success and SMAS profile(s). If the SEAL SM server cannot determine VAL server(s) with SM capabilities, the SEAL SM server sends a failure indication with a reason for failure. The SEAL SM client provides the SMAS profile to the VAL client that triggered the SMAS discovery request.

NOTE: 	A requestor (e.g., VAL server or VAL client) can discover the APIs associated with the VAL server with SM capabilities via CAPIF as specified in 3GPP TS 23.222 [7].

#### 9.6.5.3	Information flows

##### 9.6.5.3.1	SMAS discovery request

Table 9.6.5.3.1-1 describes information elements of the SMAS discovery request sent from the VAL server or SEAL SM client to the SEAL SM server.

Table 9.6.5.3.1-1: SMAS discovery request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor ID | M | The identifier of the requestor (e.g., SEAL SM client, UE, VAL client, VAL server) |
| Security credentials | M | The security credentials of the requestor |
| Discovery filters | M | Characteristics of the requested spatial map. |
| > Area of interest | M | Three-dimensional area of interest of the requested spatial map. |

##### 9.6.5.3.2	SMAS discovery response

Table 9.6.5.3.2-1 describes information elements of the SMAS discovery response sent from the SEAL SM server to the VAL server or SEAL SM client.

Table 9.6.5.3.2-1: SMAS discovery response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the discovery request was successful |
| > SMAS profiles | M | The list of SMAS profiles that can offer a spatial map as described in Table 9.6.2.3.1-2. |
| Failure response | O | Indicates that the request failed |
| > Cause | O | Indicates the cause of the failure |

* * * Next Change * * * *

### 9.7.1	General

Table 9.7.1-1 illustrates the SEAL APIs for spatial map management.

Table 9.7.1-1: List of SEAL APIs for spatial map management

| API Name | API Operations | Known Consumer(s) | Communication Type |
| --- | --- | --- | --- |
| SS_SmManagement | Create | VAL server, SEAL SM client | Request/Response |
|  | Get | VAL server, SEAL SM client |  |
|  | Update | VAL server, SEAL SM client |  |
|  | Delete | VAL server, SEAL SM client |  |
|  | Subscribe | VAL server, SEAL SM client | Subscribe/Notify |
|  | Notify | VAL server, SEAL SM client |  |
|  | Unsubscribe | VAL server, SEAL SM client |  |
| SS_SmDiscovery | Request | SEAL SM client, VAL server | Request/Response |
| SS_SmLocalization | Request | VAL server, SEAL SM client | Request/Response |
| SS_SmDataSourceRegistration | Request | SEAL SM client | Request/Response |
|  | Update | SEAL SM client |  |
|  | Deregister | SEAL SM client |  |
| SS_SmDataSourceDiscovery | Request | VAL server | Request/Response |
|  | Notify | SEAL SM client | Notify |
| SS_SmSmasRegistration | Request | VAL server | Request/Response |
|  | Update | VAL server |  |
|  | Deregister | VAL server |  |
| SS_SmSmasDiscovery | Request | VAL server, SEAL SM client | Request/Response |

* * * Next Change * * * *

### 9.7.x	SS_SmSmasDiscovery API

#### 9.7.x.1	General

Service description: This API enables a VAL server or SEAL SM client to discover VAL servers with SM capabilities registered in a SEAL SM server.

#### 9.7.x.2	Request operation

API operation name: SS_SmSmasDiscovery_Request

Description: Discover VAL servers with SM capabilities.

Known Consumers: VAL server, SEAL SM client.

Inputs: Refer Table 9.6.5.3.1-1

Outputs: Refer Table 9.6.5.3.2-1

See clause 9.6.5 for the details of usage of this API operation.

* * * End of Changes * * * *