3GPP TSG-SA WG6 Meeting #65	S6-250240

Athens, Greece 17th – 21st February 2025	(revision of S6-250xxx)

Source:	Huawei, HiSilicon

Title:	Pseudo-CR on information flows of the Application calling service API

Spec:	3GPP TS 23.392 v1.0.0

Agenda item:	9.13

Document for:	Approval

Contact:	wanghan28@huawei.com

1. Introduction

This pCR proposes to add the information flows of the Application calling service API.

2. Reason for Change

The information flows of the Application calling service API are missed.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TS 23.392 v1.0.0.

* * * First Change * * * *

#### 8.4.2.3	Information flows

##### 8.4.2.3.1	Application Call establishment request

Table 8.4.2.3.1-1: Application Call establishment request

| Information element | Status | Description |
| --- | --- | --- |
| Originating ID | M | The identifier of the caller. |
| Terminating ID | M | The identifier of the callee. |
| Media information | O | Identifier of one or more media type(s) which is expected to be used in the call, i.e. the media type(s) to be applied to the participants in the call session. In this IE, the media type(s) includes a) audio; b) video; c) audio and video. The detailed media information, e.g. the information included in the SDP of mmtel call session is negotiated by the underlying network. If the parameter is absent, the media type(s) and detailed media information are all negotiated by the underlying network. |
| DC media information | O | Identifier of whether DC media is expected to be used in this call |
| Application Profile requested | O | The DC application profile expected to be used in this call |
| Notification address | M | The address where the call related notification, e.g. whether the call between UE A and UE B is established successfully or not, is sent to. |

##### 8.4.2.3.2	Application Call establishment response

Table 8.4.2.3.2-1: Application Call establishment response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates the Application Call establishment request was successful |
| > Session ID | M | The identifier of the call session. This ID is as same as the Session ID in IMS. |
| Failure response | O | Indicates the Application Call establishment request was failed |
| > Cause | O | Indicates the cause of request failure |

* * * End of Changes * * * *