3GPP TSG-SA WG6 Meeting #65	S6-250245

Athens, Greece 17th – 21st February 2025	(revision of S6-250xxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.433 | CR | 0141 | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Client initiated data transmission |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3_App |  |  |  |  |  | Date: |  |  | 2025-02-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | SEALDD specification TS 23.433 in the clause for SEALDD services over Satellite Access, currently there is support only for SEALDD enabled data transmission for delay-tolerant satellite services and the support for client initiated data transmission for delay-tolerant satellite services is missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding procedure for client initiated data transmission for delay-tolerant satellite services based on the existing clause 9.2.2.7 Client initiated regular data transmission path establishment procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Service may be impacted when delivering via satellite access. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New{10.2.2.x, 10.2.3.x} |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 10.2.2.x	Client initiated data transmission for delay-tolerant satellite services

Figure 10.2.2.x-1 illustrates the procedure for establishing S&F data transmission connection, and the SEALDD facilitates the delay-tolerant satellite services to transmit data initiated by the VAL client to VAL server.

Pre-condition:

-	The VAL client is aware of the availability of delay-tolerant satellite services to transmit data.

Figure 10.2.2.x-1: Client initiated data transmission for delay-tolerant satellite services

1.	The VAL client decides to use SEALDD service for delay-tolerant traffic transfer.

2.	Same as steps 2-4 of clause 9.2.2.7.

3.	The SEALDD client sends the UL data to the SEALDD server via the 3GPP network.

4.	The SEALDD server subscribes the recipient UE S&F status events from the 3GPP CN and is notified about the recipient UE S&F status events information e.g., whether UE is registered in S&F Mode, the estimated delivery time indicating the DL estimated time to send data from the gateway to the recipient UE. The SEALDD server stores the received recipient UE S&F status events information for further control the downlink application data transmission.

5.	The SEALDD server controls the downlink application data delivery to the SEALDD client based on the UE S&F status events information, when the recipient UE is in S&F mode and receives the DL estimated delivery time is received from 3GPP CN.

6.	The SEALDD server sends the Sdd_S&FTransmission_notification to the SEALDD client including the DL estimated delivery time.

7.	The SEALDD client provides the DL estimated delivery time as a response for the request made in step 1.

NOTE:	Details of the VAL client service request in step 1 and the corresponding response in step 7 are out of scope of the current specification.

* * * Next Change * * * *

#### 10.2.3.x	Sdd_S&FTransmission_notification

Table 10.2.3.x-1 describes the information flow from the SEALDD server to the SEALDD Client to notify the DL estimated delivery time.

Table 10.2.3.x-1: Sdd_S&FTransmission_notification

| Information element | Status | Description |
| --- | --- | --- |
| VAL server ID | M | Identity of the VAL server |
| VAL service ID | O | Identity of the VAL service. |
| Identity | M | Identifier of specific UE or VAL user |
| Identity | M | Identifier of specific recipient UE or VAL user |
| > DL estimated delivery time | M | DL estimated delivery time for a recipient UE |

* * * End of Changes * * * *