3GPP TSG-SA WG6 Meeting #65	S6-250260

Athens, Greece 17th – 21st February 2025	(revision of S6-250xxx)

Source:	Nokia

Title:	Proposal for clause 8 digital asset media management

Spec:	3GPP TS 23.438 v1.0.0

Agenda item:	9.12

Document for:	Approval

Contact:	Niranth (niranth.amogh@nokia.com)

1. Introduction

This contribution provides a proposal for clause 8 digital asset media management

2. Reason for Change

The details of digital asset media management are missing.

3. Proposal

It is proposed to agree the following changes to 3GPP TS 23.438 v1.0.0.

* * * First Change * * * *

#### 8.2.3.1	Digital asset profile create request

Table 8.2.3.1-1 describes the information elements for a digital asset profile create request sent from a requestor (e.g. DA client) to a DA server.

Table 8.2.3.1-1: Digital asset profile create request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor. |
| Security credentials | M | Security credentials to authenticate and authorize the requestor. |
| Digital asset profile parameters | M | Parameters to create digital asset profile |

Table 8.2.3.1-2: Parameters for digital asset profile

| Information element | Status | Description |
| --- | --- | --- |
| Digital asset owner identifier(s) | M | A list of identifier(s) for the digital asset owner. |
| Digital asset type | M | The type of digital asset, e.g. avatar, wallet, etc. |
| List of Metaverse service provider identifiers | O | List of service providers for the digital asset. |
| access control list | O | List of tuples indicating the requestors allowed to perform operations with this asset and the allowed operation types. |
| > allowed user list | O | List of users allowed to access this asset |
| > allowed operations | O | List of operations allowed by this user. Possible operations are (one or more of) : Discovery, Get, Update, Delete |
| > list of predictive models | O | For each allowed user, list of one or more predictive model for the user, in order to predict user's behaviour. |
| spatial conditions | O | List of spatial conditions (e.g. locations) where this asset profile is allowed to be accessed |
| > current location | O | Current location where the profile is being used |
| allowed application list | O | List of application IDs which are allowed to use this asset profile |
| associated accessories | O | List of accessories (e.g. Hat, watch, shoes) which are purchased for this asset profile |
| expiry time | O | Time until this asset profile is valid |

Editor's Note: The association of a subscriber/user identifier for the digital asset is FFS.

Editor's Note: Additional parameters (communication parameters, experience requirements) are FFS,

* * * Next Change * * * *

#### 8.3.3.2	Digital asset discovery response

Table 8.3.3.2-1 describes the information elements for the digital asset discovery response sent from a DA server to a requestor (e.g. DA client or VAL server).

Table 8.3.3.2-1: Digital asset discovery response

| Information element | Status | Description |
| --- | --- | --- |
| Status | M | A status for the request: success, fail. If the status is fail, a failure code may be provided. |
| Discovery results | M | List of discovery results matching the discovery criteria. |
| >Asset identifier | O (NOTE) | Identifier of the asset matching the discovery criteria. |
| >Asset name | O (NOTE) | Name of the asset matching the discovery criteria. |
| >Discovered asset profile identifier | O (NOTE) | Profile identifier of the digital asset matching the discovery criteria. |
| >URL | O | A URL of the service provider the asset is associated with. |
| >Asset status | M | Status of the asset. |
| >Asset descriptions | O | Description of the asset. |
| NOTE: At least one of the information elements must be present. |  |  |

* * * Next Change * * * *

## 8.4	Digital asset media management

### 8.4.1	General

The following clauses specify procedures, information flows, and APIs for digital asset media management.

### 8.4.2	Procedures

#### 8.4.2.1	Upload digital asset media

Figure 8.4.2.1-1: Upload digital asset media

1.	A requestor (e.g. DA client) sends a upload digital asset media request to the DA server (at the Digital asset media upload endpoint). The request includes information listed in Table 8.4.3.1-1. If the digital asset identifier corresponds to a digital asset collection then a list of digital assets is included in the request.

2.	The DA server authenticates and authorizes the request and verifies the digital asset identifier. If authorized, the DA server receives the media object or media URI corresponding to the digital asset profile, stores the media or the information about the digital asset media.

3.	The DA server sends a upload digital asset media response to the requestor. The response includes information listed in Table 8.4.3.2-1.

#### 8.4.2.2	Download digital asset media

Figure 8.4.2.2-1: Download digital asset media

1.	A requestor (e.g. DA client) sends a download digital asset media request to the DA server. The request includes information listed in Table 8.4.3.3-1.

2.	The DA server authenticates and authorizes the request and verifies the digital asset identifier. If authorized, the DA server determines the digital asset media as per the media adaptation characteristics provided in step 1. If the digital asset identifier corresponds to a digital asset collection, multiple digital asset media objects are provided in the response.

3.	The DA server sends a download digital asset media response to the requestor. The response includes information listed in Table 8.4.3.4-1.

### 8.4.3	Information flows

#### 8.4.3.1	Upload digital asset media request

Table 8.4.3.1-1 describes the information elements for a upload digital asset media request sent from a requestor (e.g. DA client) to a DA server.

Table 8.4.3.1-1: Upload digital asset media request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor. |
| Security credentials | M | Security credentials to authenticate and authorize the requestor. |
| List of digital assets | M | List of digital assets to be uploaded |
| > Digital asset profile identifier | M | Identifier of the digital asset profile |
| > Media URI (NOTE) | O | URI where the media is stored |
| > Media object (NOTE) | O | Media object (e.g. JPG file) |
| NOTE: One of the IE shall be included |  |  |

#### 8.4.3.2	Upload digital asset media response

Table 8.4.3.2-1 describes the information elements for the upload digital asset media response sent from a DA server to a requestor (e.g. DA client).

Table 8.4.3.2-1: Upload digital asset media response

| Information element | Status | Description |
| --- | --- | --- |
| Status | M | A status for the request: success, fail. If the status is fail, a failure code may be provided. |
| Digital asset identifier | M | An identifier for the digital asset. |

#### 8.4.3.3	Download digital asset media request

Table 8.4.3.3-1 describes the information elements for the download digital asset media request sent from a requestor (e.g. DA client) to a DA server.

Table 8.4.3.3-1: Download digital asset media request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor. |
| Security credentials | M | Security credentials to authenticate and authorize the requestor. |
| Digital asset identifier | M | An identifier for the digital asset. |
| Adaptation requirements: Media adaptation related to UE: network QoS situation (QoS X: compression, resolution of media, QoS Y: compression, resolution of media, …). | M | Media adaptation requirements of the application |
| > Application | M | Information of the application. |
| > Media adaptation characteristics related to VAL server (NOTE) | O | If requestor is a VAL server, then the media adaptation characteristics requested like compression, resolution of media. |
| > Media adaptation characteristics related to UE (NOTE) | O | If requestor is a UE, then the media adaptation characteristics requested for different QoS supported like: QoS X: Compression, resolution of media. QoS Y: Compression, resolution of media. |
| NOTE: One of the IE shall be included |  |  |

#### 8.4.3.4	Download digital asset media response

Table 8.4.3.4-1 describes the information elements for the download digital asset media response sent from a DA server to a requestor (e.g. DA client).

Table 8.4.3.4-1: Download digital asset media response

| Information element | Status | Description |
| --- | --- | --- |
| Status | M | A status for the request: success, fail. If the status is fail, a failure code may be provided. |
| Digital asset identifier | O | If digital asset is a collection, then this identifies the digital asset collection. |
| List of digital assets | M | List of digital asset media objects included |
| > Digital asset profile identifier | M | An identifier for the digital asset. |
| > Media URI (NOTE 1) | O | URI where the media is stored |
| > Media object (NOTE 1) | O | Media object (e.g. JPG file) |
| > Media characteristics supported | M | Supported media characteristics for the digital asset media |
| >> Media adaptation characteristics related to VAL server (NOTE 2) | O | The media adaptation characteristics supported based on the request like compression, resolution of media. |
| >> Media adaptation characteristics related to UE (NOTE 2) | O | The media adaptation characteristics supported for different QoS based on the request like: QoS X: Compression, resolution of media. QoS Y: Compression, resolution of media. |
| NOTE 1: One of the IE shall be included NOTE 2: One of the IE shall be included |  |  |