3GPP TSG-SA WG6 Meeting #65	S6-250278

Athens, Greece 17th – 21st February 2025	(revision of S6-250xxx)

Source:	Ericsson

Title:	Pseudo-CR on MMTel_App_DCH_alignment_with_SA2_P2A_flow

Spec:	3GPP TS 23.392

Agenda item:	9.13

Document for:	Approval

Contact:	Cristina Badulescu, Ericsson; cristina.badulescu@ericsson.com

1. Introduction

P2A: SA2 has removed the use of OMA API in their solution 23.228 Rel-19, this CR updates the TS23.392 to align with the AS2 solution.  
Current solution using “OMA API” entity to interact with the MMTel AS does not work, as the OMA API session ID is the REST session ID and not the IMS session ID which is not passed over in the REST API. So the assumption that the IMS session ID is avaialble to the MMTel enabler server is faulted.

The architecture figure in Figure 6.2.1-1: MMTel Enabler architecture shows the MMTel enabler server interacting via N33, DC4, MDC2, MDC3 with EPC/5GC/IMS but sections 8.4.1 and 8.4.2 show a different interaction not based on N33, nor DC4 but rather interacting with an entity “OMA API”. Unclear which entity in the network is designated by the “OMA API” box (which AEF maps to in the figure 6.2.1-1 architecture)

2. Reason for Change

Update the flows to replace OMA API with exposed NEF capabilities o manage an IMS session for the P2A case. Removal of Editor’s Notes.3. Conclusions

4. Proposal

It is proposed to agree the following changes to 3GPP TS 23.392

* * * First Change * * * *

### 8.4.1	Application calling service API with Data Channel capability

#### 8.4.1.1	General

The MMTel Enabler Server exposes the Application calling service API with Data Channel capability to the Applications in order to support establishing a call between Application and DCMTSI client.

#### 8.4.1.2	Procedure

##### 8.4.1.2.1	Establishing an Application Call with Data Channel capability (A2P)

[..]

##### 8.4.1.2.2	Establishing an Application Call with Data Channel capability (P2A)

Figure 8.4.1.2.2-1 illustrates the operation to establish an Application Call with Data Channel capability between a user (on UE1) and a Data Chanel (DCH) Application via the MMTel Enabler Server.

Pre-conditions:

1.	The Application can use DCH (supports the media set information for DCH) and is authorized to use the service API provided by the MMTel Enabler Server.

2.	The MMTel Enabler Server is authorized to use IMS DC capability APIs as defined in SA2 [3].

3.	The MMTel Enabler Server is authorized to use the NEF exposed IMS session management capabilities as specified in [8].

4. The MMTel Enabler Server is subscribed to the AEF (NEF) for IMS session events information for the specific DCH Application (using the DCH Application Called party ID).

NOTE: how the IMS session ID is provided by the NEF exposure of IMS events [8] is out of scope of this specification.

5. The DCH Application Server is subscribed to the MMTel enabler server for events related to the DCH application.

6. An IMS MMTel session is ongoing for User (UE).

Figure 8.4.1.2.2-1: Establishing a Application Call with Data Channel capability

- The user (on UE1) sends a request, via the 5GC and IMS, to add DCH media for a specific DCH Application.

- The NEF notifies the MMTel Enabler Server of the new IMS session event for the DCH Application.

- The MMTel enabler server notifies the DCH Application Server of the new request for DCH media.

- The DCH Application Server accepts the request and provides the MDC2 media resource requested.

5. 	The MMTel Enabler Server utilizes the IMS session  update capability exposed by the NEF [8] to request addition of the DCH media to the existing IMS MMTel Call Session. The NEF further interacts with the IMS system to request the DCH media addition to the ongoing MMTel session and IMS further handles this as specified in Annex AC of 3GPP TS 23.228 [3].

6. The NEF responds to the IMS session management Update request.

Post-condition: The Application data channel is activated.

Editor's note 3:	The requirements to deploy the MMTel Enabler Server in a trusted 3rd party call service provider domain is FFS.

* * * Next Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.261: "Service requirements for the 5G system".

[3]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS)".

[4]	3GPP TS 26.114: "IP Multimedia Subsystem (IMS); Multimedia Telephony; Media handling and interaction"

[5]	GSMA NG.134: "IMS Data Channel, Version 1.0".

[6]	3GPP TS 23.434: "Service Enabler Architecture Layer for Verticals (SEAL)".

[7]

[8] 	3GPP TS 23.502: " Procedures for the 5G System (5GS); Stage 2".

…

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".