3GPP TSG SA WG6 Meeting #65	S6-250282

# Athens, GR, 17th – 21st February 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.222 | CR | 0264 | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | On mapping of active-inactive API states to SA5 solutions and aligning Service API Information descriptions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | CAPIF_Ph3 |  |  |  |  |  | Date: |  |  | 2025-01-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | This CR provides the mapping of “active/inactive” API states with SA5 defined management states (see TS 28.541, TS 28.532 and TS 28.538). More detailed explanation is provided in discussion paper S6-251abc Additional changes and justifications: - The Service API Information descriptions are copy-paste and disctributed across the specification with conflicting parts, e.g., in table 8.3.2.1-1, List of public IP ranges of UEs is optional; however, in table 8.25.2.1-1, List of public IP ranges of UEs is optional and applicable only on CAPIF-6 interface. This is proposed to align the Service API Information description via referecnce to table 8.3.2.1-1 to avoid conflicting and/or missing information elements. - Editorial corrections, e.g., unnecessary capitalization of “T” in table and missing hard spaces according to drafting rules. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR introduces the changes needed based on LS in S6-245008. Provides alignment of Service API Information description in the specification. Implemented editorial corrections. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistency in Service API information definition in 23.222 that may cause interoperability and maintainence issues (e.g., adding IEs to the Service API information). |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 8.1.2.2, 8.3.2.1, 8.3.2.2, 8.5.2.2, 8.6.2.1, 8.6.2.2, 8.7.2.2, 8.25.2.1, 8.25.2.4, 8.25.2.8, 8.25.2.9, 8.25.2.10 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | - This CR addresses LS in S6-245008. |  |  |  |  |  |  |  |  |
| () |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Additional discussion(if needed):

# Proposed changes:

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.682: "Architecture enhancements to facilitate communications
with packet data networks and applications".

[3]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[4]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[5]	3GPP TR 26.981: "MBMS Extensions for Provisioning and Content Ingestion".

[6]	3GPP TS 32.240: "Telecommunication management; Charging management; Charging architecture and principles".

[7]	ETSI GS MEC 011 (V1.1.1): "Mobile Edge Computing (MEC); Mobile Edge Platform Application Enablement".

[8]	ETSI GS MEC 009 (V1.1.1): "Mobile Edge Computing (MEC); General Principles for Mobile Edge Service APIs".

[9]	OMA-ER_Autho4API-V1_0-20141209-A: "Authorization Framework for Network APIs".

[10]	OMA-TS-REST_NetAPI_Common-V1_0-20180116-A: "Common definitions for RESTful Network APIs".

[11]	OMA-TS-NGSI_Registration_and_Discovery-V1_0-20120529-A: "NGSI Registration and Discovery".

[12]	3GPP TS 33.122: "Security Aspects of Common API Framework for 3GPP Northbound APIs".

[13]	3GPP TS 23.435: "Procedures for Network Slice Capability Exposure for Application Layer Enablement Service".

[14]	IETF RFC 6749 (October 2012): "The OAuth 2.0 Authorization Framework".

[15]	3GPP TS 23.434: "Service Enabler Architecture Layer for Verticals (SEAL); Functional architecture and information flows".

[16]	3GPP TS 33.501: "Security architecture and procedures for 5G System".

[17]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

[18]	3GPP TS 28.532: "Management and orchestration; Generic management services".

[19]	3GPP TS 28.538: "Management and orchestration; Edge Computing Management".

* * * Next change * * * *

#### 8.1.2.2	Onboard API invoker response

Table 8.1.2.2-1 describes the information flow onboard API invoker response from the CAPIF core function to the API invoker.

Table 8.1.2.2-1: Onboard API invoker response

| Information element | Status | Description |
| --- | --- | --- |
| Onboarding status | M | The result of onboarding request i.e., success indication is included if the API invoker is granted permission otherwise failure. |
| Enrolled information | O (see NOTE 1) | Information from the provisioned API invoker profile which may include information to allow the API invoker to be authenticated and to obtain authorization for service APIs. |
| Service API information | O (see NOTE 2) | The service API information as specified in table 8.7.2.2-1. |
| Reason | O (see NOTE 3) | This element indicates the reason when onboarding status is failure. |
| Expiration time | O | Indicates the expiration time of the onboarding. At expiration, CCF cancels the enrollment of the API invoker from CAPIF. If omitted, it indicates the onboarding does not expire. |
| NOTE 1: Information element shall be present when onboarding status is successful. NOTE 2: Information element may be present when onboarding status is successful. NOTE 3: Information element shall be present when onboarding status is failure. |  |  |

* * * Next change * * * *

#### 8.3.2.1	Service API publish request

Table 8.3.2.1-1 describes the information flow service API publish request from the API publishing function to the CAPIF core function.

Table 8.3.2.1-1: Service API publish request

| Information element | Status | Description |
| --- | --- | --- |
| API publisher information | M | The information of the API publisher may include identity, authentication and authorization information |
| Service API information | M (see NOTE 2) | The service API information includes the service API name, API provider name (optional), List of public IP ranges of UEs (optional), service API category (e.g. V2X, IoT), service API status (e.g. active, inactive), communication type, description, Serving Area Information (optional), AEF location (optional), interface details (e.g. IP address, port number, URI), protocols, version numbers, data format, Service KPIs (optional), service API operation(s) and resource(s) (optional), and Network Slice Info (optional), security methods. |
| Shareable information | O (see NOTE 1) | Indicates whether the service API information or the service API category can be published to other CCFs. And if sharing, a list of CAPIF provider domain information where the service API information or the service API category can be published is contained. |
| NOTE 1: If the shareable information is not present, the service API information is not allowed to be shared. NOTE 2: The active and inactive states of the service APIs are based on the respective combinations of the management states (administrative, operational) specified in the management domain in TS 28.541 [17], TS 28.532 [18] and TS 28.538 [19]. |  |  |

The Service KPIs is defined as below:

Table 8.3.2.1-2: Service KPIs

| Information element | Status | Description |
| --- | --- | --- |
| Maximum Request rate | O | Maximum request rate from the API Invoker supported by the server. |
| Maximum Response time | O | The maximum response time advertised for the API Invoker's service requests. |
| Availability | O | Advertised percentage of time the server is available for the API Invoker's use. |
| Available Compute | O | The maximum compute resource available for the API Invoker. |
| Available Graphical Compute | O | The maximum graphical compute resource available for the API Invoker. |
| Available Memory | O | The maximum memory resource available for the API Invoker. |
| Available Storage | O | The maximum storage resource available for the API Invoker. |
| Connection Bandwidth | O | The connection bandwidth in Kbit/s advertised for the API Invoker's use. |

* * * Next change * * * *

#### 8.3.2.2	Service API publish response

Table 8.3.2.2-1 describes the information flow service API publish response from the CAPIF core function to the API publishing function.

Table 8.3.2.2-1: Service API publish response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure of publishing the service API information |
| Service API published information reference | O (see NOTE) | The information which can be used for referencing the information (set) about the published service API by the API publishing function. |
| Service API information | O (see NOTE) | The authorized service API information, which may be a subset or the full set, of the Service API information as specified in table 8.3.2.1-1. |
| NOTE: This information element is included when the Result indicates success. |  |  |

* * * Next change * * * *

#### 8.5.2.2	Service API get response

Table 8.5.2.2-1 describes the information flow service API get response from the CAPIF core function to the API publishing function.

Table 8.5.2.2-1: Service API get response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure of retrieving the service API information |
| Service API information | O (see NOTE) | The service API information as specified in table 8.3.2.1-1. |
| NOTE: Shall be present if the Result information element indicates that the service API get request is successful. Otherwise service API information shall not be present. |  |  |

* * * Next change * * * *

#### 8.6.2.1	Service API update request

Table 8.6.2.1-1 describes the information flow service API update request from the API publishing function to the CAPIF core function.

Table 8.6.2.1-1: Service API update request

| Information element | Status | Description |
| --- | --- | --- |
| API publisher information | M | The information of the API publisher may include identity, authentication and authorization information |
| Service API published information reference | M | The information (set) provided by the CAPIF core function about the published service API which can be used for reference by the API publishing function. |
| Service API information | M | The service API information as specified in table 8.3.2.1-1 which is required to replace the existing service API information |
| Reason | O | The reason of the update (e.g. change log). |

NOTE:	How to monitor service API status when the APF is unable to update service API status is not specified in this release.

* * * Next change * * * *

#### 8.6.2.2	Service API update response

Table 8.6.2.2-1 describes the information flow service API update response from the CAPIF core function to the API publishing function.

Table 8.6.2.2-1: Service API update response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure of updating the service API information |
| Service API information | O | The authorized service API information during update, applicable when the update result is success. This can be a subset or the full set, of the Service API information as specified in table 8.3.2.1-1. |

* * * Next change * * * *

#### 8.7.2.2	Service API discover response

Table 8.7.2.2-1 describes the information flow service API discover response from the CAPIF core function to the API invoker.

Table 8.7.2.2-1: Service API discover response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure of the discovery of the service API information |
| Service API information (see NOTE 2) | O (see NOTE 1, NOTE 3) | The service API information as specified in table 8.3.2.1-1. |
| Interface information | O (NOTE 3) | The interface details (e.g. IP address, port number, URI) of the requested service API(s) |
| CAPIF core function identity information | O (see NOTE 1, NOTE 3) | Indicates the CAPIF core function serving the service API category provided in the query criteria |
| NOTE 1: The service API information or the CAPIF core function identity information or both shall be present if the Result information element indicates that the service API discover operation is successful. Otherwise, both shall not be present. NOTE 2: If topology hiding is enabled for the service API, the interface details shall be the interface details of AEF acting as service communication entry point for the service API. NOTE 3: "Interface information" IE shall not be present when "Service API information" or "CAPIF core function identity information" is present. |  |  |

* * * Next change * * * *

#### 8.25.2.1	Interconnection API publish request

Table 8.25.2.1-1 describes the information flow interconnection API publish request from CAPIF core function to CAPIF core function.

Table 8.25.2.1-1: Interconnection API publish request

| Information element | Status | Description |
| --- | --- | --- |
| CCF information | M | The information of the CAPIF core function which publishes APIs, may include identity, authentication and authorization information |
| Service API information | O (see NOTE 1) | The service API information as specified in table 8.3.2.1-1. |
| Service API category | O | Service API category |
| Shareable information | O (see NOTE 2) | Indicates whether the service API information or the service API category can be published to other CCFs. And if sharing, a list of CAPIF provider domain information where the service API information or the service API category can be published is contained. |
| NOTE 1: Either the Service API information or Service API category shall be present. NOTE 2: If the shareable information is not present, the service API information is not allowed to be shared. There is one and only one CAPIF provider domain information sharable via the CAPIF-6e interface. |  |  |

* * * Next change * * * *

#### 8.25.2.4	Interconnection service API discover response

Table 8.25.2.4-1 describes the information flow interconnection service API discover response from one CAPIF core function to another CAPIF core function.

Table 8.25.2.4-1: Interconnection service API discover response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure of the discovery of the service API information |
| Service API information | O (see NOTE) | List of service APIs corresponding to the request, including service API information as specified in table 8.3.2.1-1. |
| CAPIF core function identity information | O (see NOTE) | Indicates the CAPIF core function matching the query criteria |
| NOTE: The service API information or the CAPIF core function identity information or both shall be present, if the Result information element indicates that the interconnection service API discover operation is successful. Otherwise, both shall not be present. |  |  |

* * * Next change * * * *

#### 8.25.2.8	Interconnection get service API response

Table 8.25.2.8-1 describes the information flow interconnection get service API response from a CAPIF core function to another CAPIF core function.

Table 8.25.2.8-1: Interconnection get service API response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure of getting the service API information |
| Service API information | O (see NOTE) | The service API information as specified in Table 8.3.2.1-1. |
| NOTE: Shall be present if the Result information element indicates that the interconnection get service API request is successful. Otherwise, service API information shall not be present. |  |  |

* * * Next change * * * *

#### 8.25.2.9	Interconnection update service API request

Table 8.25.2.9-1 describes the information flow interconnection update service API request from a CAPIF core function to another CAPIF core function.

Table 8.25.2.9-1: Interconnection update service API request

| Information element | Status | Description |
| --- | --- | --- |
| CCF information | M | The information of the CCF requesting the update operation. It may include identity, authentication and authorization information |
| Service API information | O (see NOTE 1) | The service API information as specified in Table 8.3.2.1-1. |
| Service API category | O (see NOTE 1) | The category of the service APIs to be published, (e.g. V2X, IoT) |
| Shareable information | O (see NOTE 2) | Indicates whether the service API information or the service API category can be published to other CCFs. And if sharing, a list of CAPIF provider domain information where the service API information or the service API category can be published is contained. |
| NOTE 1: Either the Service API information or Service API category shall be present. NOTE 2: If the shareable information is not present, the service API information is not allowed to be shared. There is one and only one CAPIF provider domain information sharable via the CAPIF-6e interface. |  |  |

* * * Next change * * * *

#### 8.25.2.10	Interconnection update service API response

Table 8.25.2.10-1 describes the information flow interconnection update service API response from a CAPIF core function to another CAPIF core function.

Table 8.25.2.10-1: Interconnection update service API response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure of updating the service API information |
| Service API information | O | The authorized service API information during update, applicable when the update result is success. This can be a subset or the full set, of the Service API information as specified in table 8.3.2.1-1. |

* * * End of changes * * * *