3GPP TSG-SA WG6 Meeting #65	S6-250283

Athens, Greece 17th – 21st November 2025

# Title:	Discussion on answers from SA5 in LS (S6-245008) on LS reply to SA6 LS on API availability support

# Source:	Ericsson

# Document for:	Agreement

Agenda Item:	x

Abstract: It is proposed to discuss the answers received from SA5 and determine the next needed actions in SA6.

# 1. Background

Following the LS reply S6-245008 from SA5 on the availability of the SA5 Network Resource Model and APIs for management of 5GC including managed entities that in SA6 play the role of AEFs (e.g., NEF, EES/ECS, etc): 
We understand that in SA5 there are specifications that already provide sufficient support to enable SA6 CAPIF to control the states of the AEF fulfilling the requirements for the active/inactive states.

Based on answers to Questions 1 and 2, SA5 defined 2 states:

- An administrative state (administrativeState) which permits or prohibits to use a specific AEF service instance.

- An operational state (operationalState) which describes if the AEF (e.g., NEF/EES) service instance is operable or inoperable.

The combinations of the management defined states in the management domain in TS 28.541 [x], TS 28.532 [y] and TS 28.538 [z] are enough to derive whether an AEF service instance is functional (can receive traffic requests) or not. This shall translate to AEF service instance being:

- INACTIVE is when either one or both of the following is true:

  - the administrativeState is set to LOCKED 
- OR-

  - the operationalState is set to DISABLED.

- ACTIVE is when both of the following are true:

  - the administrativeState is set to UNLOCKED
 – AND-

  - the operationalState is set to ENABLED.

Based on answer to Question 3, the existing SA5 mechanisms (as described in answers to Questions #1 and #2) can perform service API activation at per API granularity i.e., one or more service APIs of API provider instance can be activated or deactivated.

Based on answer to Question 4, the service-based management architecture (SBMA) defined by SA5 enables a consumer-producer paradigm that allows consumption of the SA5 Management Services that expose the management of the administrative and operational states of 5GS NFs, including those that can provide AEF services.

2. Analysis of impacts to SA6 and CAPIF

Analysis:

Throughout the TS23.222 the “Service API information” IE is inconsistently described in the different info models.

The description of the “Service API information” IE in Table 8.7.2.2-1 is an almost copy/paste from the “Service API information” IE in Table 8.3.2.1-1 (below), with the exception that in Table 8.7.2.2-1  (also referenced in Table 8.1.2.2-1) in the onboading response and discovery response the information on the active/inactive states of the service API is not listed. 
The description in Table 8.25.2.9-1 is identical with the one in Table 8.3.2.1-1 (below), except for a conflicting limitation on the List of public IP ranges of UEs which in Table 8.25.2.9-1 is stated to be only applicable over CAPIF-6/6e, despite the fact that it is present in the IE in all other info models.

The active/inactive state of the Service API is only mentioned in:

- Service API publish request, get response, update request, Interconnection API publish request, Interconnection update service API request and Interconnection service API discover response.

The active/inactive state of the Service API is missing in:

- Onboard API invoker response, Service API discover response and Interconnection get service API response.

Table 8.1.2.2-1: Onboard API invoker response

| Information element | Status | Description |
| --- | --- | --- |
| Onboarding status | M | The result of onboarding request i.e., success indication is included if the API invoker is granted permission otherwise failure. |
| Enrolled information | O (see NOTE 1) | Information from the provisioned API invoker profile which may include information to allow the API invoker to be authenticated and to obtain authorization for service APIs |
| Service API information | O (see NOTE 2) | The service API information as specified in Table 8.7.2.2-1. |
| Reason | O (see NOTE 3) | This element indicates the reason when onboarding status is failure. |
| Expiration time | O | Indicates the expiration time of the onboarding. At expiration, CCF cancels the enrollment of the API invoker from CAPIF. If omitted, it indicates the onboarding does not expire. |
| NOTE 1: Information element shall be present when onboarding status is successful. NOTE 2: Information element may be present when onboarding status is successful. NOTE 3: Information element shall be present when onboarding status is failure. |  |  |

Table 8.3.2.1-1: Service API publish request

| Information element | Status | Description |
| --- | --- | --- |
| API publisher information | M | The information of the API publisher may include identity, authentication and authorization information |
| Service API information | M | The service API information includes the service API name, API provider name (optional), List of public IP ranges of UEs (optional), service API category (e.g. V2X, IoT), service API status (e.g. active, inactive), communication type, description, Serving Area Information (optional), AEF location (optional), interface details (e.g. IP address, port number, URI), protocols, version numbers, data format, Service KPIs (optional), and Network Slice Info (optional). |
| Shareable information | O (see NOTE) | Indicates whether the service API information or the service API category can be published to other CCFs. And if sharing, a list of CAPIF provider domain information where the service API information or the service API category can be published is contained. |
| NOTE: If the shareable information is not present, the service API information is not allowed to be shared. |  |  |

Table 8.3.2.2-1: Service API publish response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure of publishing the service API information |
| Service API published information reference | O (see NOTE) | The information which can be used for referencing the information (set) about the published service API by the API publishing function. |
| Service API information | O (see NOTE) | The authorized service API information. |
| NOTE: This information element is included when the Result indicates success. |  |  |

Table 8.5.2.2-1: Service API get response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure of retrieving the service API information |
| Service API information | O (see NOTE) | The service API information as specified in Table 8.3.2.1-1. |
| NOTE: Shall be present if the Result information element indicates that the service API get request is successful. Otherwise service API information shall not be present. |  |  |

Table 8.6.2.1-1: Service API update request

| Information element | Status | Description |
| --- | --- | --- |
| API publisher information | M | The information of the API publisher may include identity, authentication and authorization information |
| Service API published information reference | M | The information (set) provided by the CAPIF core function about the published service API which can be used for reference by the API publishing function. |
| Service API information | M | The service API information includes the service API name, service API type, service API status (e.g. active, inactive), communication type, description, List of public IP ranges of UEs (optional), Serving Area Information (optional), AEF location (optional), interface details (e.g. IP address, port number, URI), protocols, version numbers, data format , and Network Slice Info (optional) which is required to replace the existing service API information |
| Reason | O | The reason of the update (e.g. change log) |

Table 8.6.2.2-1: Service API update response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure of updating the service API information |
| Service API information | O | The authorized service API information during update, applicable when the update result is success. |

Table 8.7.2.2-1: Service API discover response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure of the discovery of the service API information |
| Service API information (see NOTE 2) | O (see NOTE 1) | List of service APIs corresponding to the request, including service API information such as service API name, API provider name (optional), service API category, communication type, description, Serving Area Information (optional), interface details (e.g. IP address, port number, URI), protocols, version, data format, Service KPIs (optional), and Network Slice Info (optional). |
| CAPIF core function identity information | O (see NOTE 1) | Indicates the CAPIF core function serving the service API category provided in the query criteria |
| NOTE 1: The service API information or the CAPIF core function identity information or both shall be present if the Result information element indicates that the service API discover operation is successful. Otherwise both shall not be present. NOTE 2: If topology hiding is enabled for the service API, the interface details shall be the interface details of AEF acting as service communication entry point for the service API. |  |  |

Table 8.25.2.1-1: Interconnection API publish request

| Information element | Status | Description |
| --- | --- | --- |
| CCF information | M | The information of the CAPIF core function which publishes APIs, may include identity, authentication and authorization information |
| Service API information | M | The service API information includes the service API name, API provider name (optional), List of public IP ranges of UEs (optional and applicable only on CAPIF-6 interface), service API category (e.g. V2X, IoT), service API status (e.g. active, inactive), communication type, description, Serving Area Information (optional), AEF location (optional), interface details (e.g. IP address, port number, URI), protocols, version numbers, data format, Service KPIs (optional), and Network Slice Info (optional). |
| Shareable information | O (see NOTE) | Indicates whether the service API information or the service API category can be published to other CCFs. And if sharing, a list of CAPIF provider domain information where the service API information or the service API category can be published is contained. |
| NOTE 2: If the shareable information is not present, the service API information is not allowed to be shared. There is one and only one CAPIF provider domain information sharable via the CAPIF-6e interface. |  |  |

Table 8.25.2.4-1: Interconnection service API discover response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure of the discovery of the service API information |
| Service API information | O (see NOTE) | List of service APIs corresponding to the request, including service API information as specified in Table 8.25.2.1-1. |
| CAPIF core function identity information | O (see NOTE) | Indicates the CAPIF core function matching the query criteria |
| NOTE: The service API information or the CAPIF core function identity information or both shall be present, if the Result information element indicates that the interconnection service API discover operation is successful. Otherwise, both shall not be present. |  |  |

Table 8.25.2.8-1: Interconnection get service API response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure of unpublishing the service API information |
| Service API information | O (see NOTE) | The service API information includes the service API name, service API type, communication type, description, Serving Area Information (optional), interface details (e.g. IP address, port number, URI), protocols, version numbers, and data format. |
| NOTE: Shall be present if the Result information element indicates that the interconnection get service API request is successful. Otherwise, service API information shall not be present. |  |  |

Table 8.25.2.9-1: Interconnection update service API request

| Information element | Status | Description |
| --- | --- | --- |
| CCF information | M | The information of the CCF requesting the update operation. It may include identity, authentication and authorization information |
| Service API information | O | The service API information includes the service API name, API provider name (optional), List of public IP ranges of UEs (optional and applicable only on CAPIF-6 interface), service API type, service API status (e.g. active, inactive), communication type, description, Serving Area Information (optional), AEF location (optional), interface details (e.g. IP address, port number, URI), protocols, version numbers, and data format, Service KPIs (optional). |
| Service API category | O | The category of the service APIs to be published, (e.g. V2X, IoT) |
| Shareable information | O | Indicates whether the service API or the service API category can be published to other CCFs. And if sharing, a list of CAPIF provider domain information where the service API or the service API category can be published is contained. |

Table 8.25.2.10-1: Interconnection update service API response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure of updating the service API information |
| Service API information | O | The authorized service API information during update, applicable when the update result is success. |

3. Proposals

Follow-up with a CR (add CR TDoc number) to TS 23.222 to:

- Specify the Service API information IE in one place, and reference it in the other information models.

- Add a NOTE to the IE stating that the active and inactive states of the service APIs are based on the combinations of the management states (administrative, operational) specified in the management domain in TS 28.541[x], TS 28.532[y] and TS 28.538.[z]

- Where some filtered information based on the API state is performed, such as excluding inactive service APIs in any of the operations, add an explicit statement about it.

The proposed changes are available in CR#: xxxx