3GPP TSG–T1/SIG Meeting #24	Tdoc T1S-020384

Yokohama, Japan, 29th- 31st July 2002

| CR-Form-v4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 34.123-1 |  |  |  | CR | CRNum |  |  |  rev |  |  |  |  |  |  Current version: |  |  |  |  | 5.0.1 |  |  |  |
| For HELP on using this form, see bottom of this page or look at the pop-up text over the  symbols. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed change affects:  |  |  |  |  | (U)SIM |  |  | ME/UE |  | X |  | Radio Access Network |  |  |  |  |  |  |  |  | Core Network |  |  |
| Title:  |  | Corrections to package 2 TCs in clause 8.4 of TS 34.123-1 as T1S-020359rev1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source:  |  | Panasonic |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Work item code:  |  | TEI |  |  |  |  |  |  |  |  |  |  |  |  |  | Date:  |  | 16 July 2002 |  |  |  |  |  |
| Category:  |  | F |  |  |  |  |  |  |  |  |  |  |  |  |  | Release:  |  | REL-5 |  |  |  |  |  |
|  |  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Use one of the following releases: 2 (GSM Phase 2) R96 (Release 1996) R97 (Release 1997) R98 (Release 1998) R99 (Release 1999) REL-4 (Release 4) REL-5 (Release 5) |  |  |  |  |  |  |
| Reason for change:  |  |  | - Update Conformance Requirement according to core specification 25.331. - Editorial changes. - Add generic procedure at the end of some test cases to verify the final state of the UE. - Add specific procedures, either before or after the actual testing, to verify that the UE continues with any ongoing processes and procedures as if the erroneous message has not been received. - After the modification of system information, the SS should send PAGING TYPE 1 or SYSTEM INFORMATION CHANGE INDICATION message to notify the UE. - It is more stable to perform measurement test cases when RABs are established in either CELL_FACH or CELL_DCH state, therefore the initial conditions of some of the test cases have to be revised or in some cases RABs are proposed to be set up before the measurement takes place. - In some test, RABs are established when UE is in CELL_FACH, such test cases are not applicable to CS only UE. - ICS/IXIT statement is applied to Inter frequency measurement test cases. Corrections to T1S-020359 - Editorial corrections. - In clause 8.4.1.7, as UE moves to cell 2, it reads the system information and therefore, the UE shall send measurement report for intra-frequency measurement as UE moves into CELL_DCH in step 25. - In clause 8.4.1.14, the description of the test in the test procedure mismatches the expected sequence. - In clause 8.4.1.18, if the measurement validity of the traffice volume measurement is not present in step 33, the UE shall not perform any traffic volume measurement after it transit to CELL_DCH in step 35. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Summary of change:  |  |  | In clause 8.4.1.2 - Conformance requirement and Test Purpose are updated. - In step 4, SS should execute procedure P11 (clause 7.4.2.5.2) or P13 (clause 7.4.2.6.2) specified in TS 34.108, and not P3 (clause 7.4.2.1.2) or P5 (clause 7.4.2.2.2) - SS called generic procedure C.3 to check that UE is in CELL_DCH at the end of test. - ICS/IXIT statement is added and accordingly the test procedure and message contents are revised. In clause 8.4.1.7 - After SIB 12 is modified, SS should send SYSTEM INFORMATION CHANGE INDICATION to UE. In clause 8.4.1.8 - Conformance requirement and Test Purpose are updated. - The Initial Condition of UE is PS-DCCH+DTCH_DCH (state 6-10). This test is not applicable to CS-DCCH+DTCH_DCH (state 6-9). - ICS/IXIT statement is added and accordingly the test procedure and message contents are revised. In clause 8.4.1.14 - Conformance requirement and Test Purpose are updated. - The test method are revised to meet the test purpose. In clause 8.4.1.16 - Conformance requirement and Test Purpose are updated. - Reference is added. - The Initial Condition of UE can only be “Registered idle mode in PS”. This test case is not applicable to “Registered idle mode in CS”. - After step 1, SS should executes procedure P6, P10 and P14. - SS called generic procedure C.2 to check that UE is in CELL_FACH at the end of test. In clause 8.4.1.17 - Conformance requirement and Test Purpose are updated. - Reference is added. - After step 1, SS should executes procedure P3, P7 and P11 (for CS), or P5, P9 and P13 (for PS). - SS called generic procedure C.3 to check that UE is in CELL_DCH at the end of test. In clause 8.4.1.18 - Test Purpose is updated. - In step 33, MIB with a different tag value should be broadcasted. - After SIB 11 is modified, SS should send SYSTEM INFORMATION CHANGE INDICATION to UE. In clause 8.4.1.19 - Test Purpose is updated. In clause 8.4.1.23 - The Initial Condition of UE should be “CS-DCCH+DTCH_DCH (State 6-9) or PS-DCCH+DTCH_DCH (State 6-10)”. - SS called generic procedure C.3 to check that UE is in CELL_DCH at the end of test. The modification is added in T1S-020359 as below with blue marker. In clause 8.4.1.2, 8.4.1.8 - Related ICS/IXIT statement : Compressed mode Supported  required - To add an extra blank line on the left hand size when proposed change on right on size takes more than one line in specific message contents. - It is replaced from “if flag X0 is set to TRUE” to “If UE requires compressed mode” and from “if flag X0 is set to FALSE” to “If UE do not require compressed mode”. The modification is added in T1S-020359 as below with yellow marker. In clause 8.4.1.7 - The direction of the arrows in step 24 and 25 is reversed. - Step 26 is revised so that MEASUREMENT REPORT message is expected. In clause 8.4.1.18 - The measurement validity of traffic volume measurement in step 33 is changed to “all states”. In clause 8.4.1.14 - The test procedure is revised to match the expected sequence. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  |  | If changes are not approved, UE might not be tested properly. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected:  |  |  | 8.4.1.2, 8.4.1.7, 8.4.1.8, 8.4.1.14, 8.4.1.16, 8.4.1.17, 8.4.1.18, 8.4.1.19, 8.4.1.23 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other specs  |  |  |  | Other core specifications  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| affected: |  |  |  | Test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | O&M Specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other comments:  |  |  | Affects R99, REL-4, REL-5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

How to create CRs using this form:

Comprehensive information and tips about how to create CRs can be found at: http://www.3gpp.org/3G_Specs/CRs.htm.  Below is a brief summary:

1)	Fill out the above form. The symbols above marked  contain pop-up help information about the field that they are closest to.

2)	Obtain the latest version for the release of the specification to which the change is proposed. Use the MS Word "revision marks"  feature (also known as "track changes") when making the changes. All 3GPP specifications can be downloaded from the 3GPP server under ftp://ftp.3gpp.org/specs/ For the latest version, look for the directory name with the latest date e.g. 2001-03 contains the specifications resulting from the March 2001 TSG meetings.

3)	With "track changes" disabled, paste the entire CR form (use CTRL-A to select it) into the specification just in front of the clause containing the first piece of changed text.  Delete those parts of the specification which are not relevant to the change request.

#### 8.4.1.2	Measurement Control and Report: Inter-frequency measurement for transition from idle mode to CELL_DCH state

8.4.1.2.1	Definition

8.4.1.2.2	Conformance requirement

Upon transition from idle mode to CELL_DCH state, the UE shall:

1>	stop monitoring the list of cells assigned in the IE "inter-frequency cell info list" in System Information Block type 12 (or System Information Block type 11).

Upon reception of a MEASUREMENT CONTROL message the UE shall:

1>	read the IE "Measurement command";

1>	if the IE "Measurement command" has the value "setup":

2>	store this measurement in the variable MEASUREMENT_IDENTITY according to the IE "measurement identity", first releasing any previously stored measurement with that identity if that exists;

2>	for measurement types "inter-frequency measurement":

3>	if, according to its measurement capabilities, the UE requires compressed mode to perform that measurement type and a compressed mode pattern sequence with an appropriate measurement purpose is simultaneously activated by the IE "DPCH compressed mode status info"; or

3>	if the IE "Inter-frequency cell info list" for that measurement identity is empty; or

3>	if, according to its measurement capabilities, the UE does not require compressed mode to perform the measurements:

4>	if the measurement is valid in the current RRC state of the UE:

5>	begin measurements according to the stored control information for this measurement identity.

If the IE "Reporting Cell Status" is not received for inter-frequency measurement, the UE shall:

1>	exclude the IE "Cell Measured Results" for any cell in MEASUREMENT REPORT.

Reference

3GPP TS 25.331 clauses 8.4.1.3, 8.4.1.8.2 and 8.6.7.9

8.4.1.2.3	Test Purpose

- To confirm that the UE stops monitoring the list of cells assigned in the IE "inter-frequency cell info" in System Information Block type 11 messages, after it enters CELL_DCH state from idle mode.

- To confirm that the UE starts to perform inter-frequency measurement and related reporting activities, when it receives a MEASUREMENT CONTROL message with the "DPCH compress mode status info" IE indicating that a stored compressed mode pattern sequence be simultaneously activated.

- To confirm that the UE excludes the IE "cell measured results" for any cells in the MEASUREMENT REPORT messages, after it receives a MEASUREMENT CONTROL message with "Reporting cell status" IE omitted.

8.4.1.2.4	Method of test

Initial Condition

System Simulator: 2 cells – Cell 1 and cell 4 are active..

UE: "Registered idle mode on CS" (state 2) or "Registered idle mode on PS" (state 3) in cell 1 as specified in clause 7.4 of TS 34.108, depending on the CN domain supported by the UE. If the UE supports both CS and PS domains, the initial UE state shall be "Registered idle mode on CS/PS" (state 7).

Related ICS/IXIT statements

-	Compressed mode required	yes/no

Test Procedure

Table 8.4.1.2-1 illustrates the downlink power to be applied for the 2 cells.

- Table 8.4.1.2-1

| Parameter | Unit | Cell 1 | Cell 4 |
| --- | --- | --- | --- |
| UTRA RF Channel Number |  | Ch. 1 | Ch. 2 |
| CPICH Ec | dBm/3.84 MHz | -60 | -75 |

The UE is initially at idle mode and has selected cell 1 for camping. The System Information Block type 11 messages are modified with respect to the default settings to prevent reporting of "Cell synchronisation information", and also to include cell 4 into "inter-frequency cells info list" IE.

SS prompts the operator to make an outgoing call for one of the traffic classes supported by the UE. SS and UE shall execute procedure P3 (for CS service) or P5 (for PS service). The RRC CONNECTION SETUP message used in procedure P3 or P5 should contain IE "DPCH compressed mode info", activating the transmission pattern gap sequence with TGPSI=1 only if UE requires compressed mode. Next SS and UE shall execute procedure P7 (for CS service) or P9 (for PS service). Then SS and UE shall execute procedure P11 (for CS service) or P13 (for PS service).The UE shall not transmit any MEASUREMENT REPORT messages, which pertain to measurement readings for cells listed in the IE "inter-frequency cell info list" in System Information Type 11.

If UE requires compressed mode, SS sends PHYSICAL CHANNEL RECONFIGURATION message on the downlink DCCH, specifying that compressed mode sequence pattern with TGPSI=1 be deactivated. The UE shall reply with PHYSICAL CHANNEL RECONFIGURATION COMPLETE message on the uplink DCCH if UE configures according to the PHYSICAL CHANNEL RECONFIGURATION message.

SS sends MEASUREMENT CONTROL message on the downlink DCCH. In this message, SS requests UE to perform inter-frequency measurement with periodic reporting of CPICH RSCP values for cell 4. If UE requires compressed mode, IE "DPCH compressed status info" IE to activate the transmission gap pattern sequence with TGPSI = 1 is included in this message.

The UE shall start inter-frequency measurement and reporting for cell 4's CPICH RSCP values. It shall report this measurement result by transmitting MEASUREMENT REPORT messages on uplink DCCH periodically at 16 seconds interval.

SS sends MEASUREMENT CONTROL message on the downlink DCCH omitting the IE "Reporting cell status". The UE shall send MEASUREMENT REPORT messages on the uplink DCCH, with the IE "Cell measured results" excluded in these messages. SS calls for generic procedure C.3 to check that UE is in CELL_DCH state.

# Expected Sequence

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 |  |  | System Information Block type 11 | The UE is idle mode and camped onto cell 1.System Information Block Type 11 to be transmitted is different from the default settings (see specific message contents) |
| 2 |  |  | SS executes procedure P3 (clause 7.4.2.1.2) or P5 (clause 7.4.2.2.2) specified in TS 34.108. | SS prompts the operator to make an outgoing call. |
| 3 |  |  | SS executes procedure P7 (clause 7.4.2.3.2) or P9 (clause 7.4.2.4.2) specified in TS 34.108. |  |
| 4 |  |  | SS executes procedure P11 (clause 7.4.2.5.2) or P13 (clause 7.4.2.6.2) specified in TS 34.108. |  |
| 5 |  |  | Void |  |
| 6 |  |  |  | SS checks to see that no MEASUREMENT REPORT messages are received. If compresed mode is not required ( refer ICS/IXIT ), then goto step 9. |
| 7 |  |  | PHYSICAL CHANNEL RECONFIGURATION | Existing compressed mode sequence pattern is de-activated in this message |
| 8 |  |  | PHYSICAL CHANNEL RECONFIGURATION COMPLETE | UE shall remain in CELL_DCH state. |
| 9 |  |  | MEASUREMENT CONTROL | SS requests UE to start inter-frequency measurement for cell 4, and performing periodic reporting for cell 4's CPICH RSCP. "See specific message content below. |
| 10 |  |  | MEASUREMENT REPORT | UE shall report cell 4's CPICH RSCP reading periodically. |
| 11 |  |  | MEASUREMENT CONTROL | SS changes the reporting criteria of cell 4 to 'event 2c'. "Reporting cell status" IE in this message is omitted. |
| 12 |  |  | MEASUREMENT REPORT | SS monitors the uplink DCCH to make sure that only 1 such message is received 32 seconds after step 11. This message shall not contain IE "Inter-frequency cell measured results" |
| 13 |  |  | CALL C.3 | If the test result of C.3 indicates that UE is in CELL_DCH state, the test passes, otherwise it fails. |

# Specific Message Content

All messages indicated below shall use the same content as described in default message content, with the following exceptions:

System Information Block type 11 (Step 1)

| Information Element | Value/Remark |
| --- | --- |
| SIB12 indicator FACH measurement occasion info Measurement control system information - Intra-frequency measurement system information - Inter-frequency measurement system information - Inter-frequency cell info list - CHOICE inter-frequency cell removal - New inter-frequency cells - Inter-frequency cell id - Frequency info - UARFCN uplink (Nu) - UARFCN downlink (Nd) - Cell info - Cell individual offset - Reference time difference to cell - Read SFN Indicator - CHOICE mode - Primary CPICH Info - Primary Scrambling Code - Primary CPICH TX power - TX Diversity Indicator - Cell selection and re-selection info - Qoffsets,n - Maximum allowed UL TX power - HCS neighbouring cell information - CHOICE Mode - Qqualmin - Qrxlevmin - Inter-RAT measurement system information - Traffic volume measurement system information - UE internal measurement system information | FALSE Not Present Not Present No inter-frequency cells removed 4 Set to the uplink UARFCN of cell 4 Set to the downlink UARFCN of cell 4 0 dB 0 chips FALSE FDD Set to same code as used for cell 4 Not Present FALSE 0 dB 0 dBm Not Present FDD -20 dB -115dBm Not Present Not Present Not Present |

RRC CONNECTION SETUP (Step 2)

If UE do not require compressed mode, use the message found in TS 34.108 clause 9.

If UE requires compressed mode, use the message found in TS 34.108 clause 9., with the following exceptions:

| Information Element | Value/Remarks |
| --- | --- |
| Downlink information common for all radio links - Downlink DPCH info common for all RL - Timing Indication - CFN-targetSFN frame offset - Downlink DPCH power control information - DPC mode - CHOICE Mode - Power offset PPilot-DPDCH - DL rate matching restriction information - Spreading factor - Fixed or flexible position - TFCI existence - Number of bits for Pilot bits (SF=128, 256) - DPCH compressed mode info - TGPSI - TGPS Status Flag - TGCFN - Transmission gap pattern sequence configuration parameters - TGMP - TGPRC - TGSN - TGL1 - TGL2 - TGD - TGPL1 - TGPL2 - RPP - ITP - CHOICE UL/DL Mode - Downlink compressed mode method - Uplink compressed mode method - Downlink frame type - DeltaSIR1 - DeltaSIRAfter1 - DeltaSIR2 - DeltaSIR2After2 - N identify abort - T Reconfirm abort - TX Diversity Mode - SSDT information - Default DPCH Offset Value | Initiaise Not Present Single TPC FDD 0 Not Present Refer to the parameter set in TS 34.108 Flexible FALSE Refer to the parameter set in TS 34.108 1 Activate (Current CFN + (256 – TTI/10msec))mod 256 FDD Measurement Infinity 4 7 Not Present 0 3 Not Present Mode 0 Mode 0 UL and DL or DL only depending on UE capability SF/2 SF/2 or Not present depending on UE capability B 2.0 1.0 Not Present Not Present Not Present Not Present None Not Present 0 |

PHYSICAL CHANNEL RECONFIGURATION (Step 7)

Use the same message sub-type in Annex A titled “Non speech in CS” or "Speech in CS" or “Packet to CELL_DCH from CELL_DCH in PS” , with the following exceptions:

| Information Element | Value/remark |
| --- | --- |
| CHOICE channel requirement | Uplink DPCH info |
| - Uplink DPCH power control info |  |
| - DPCCH power offset | -6dB |
| - PC Preamble | 1 frame |
| - SRB delay | 7 frames |
| - Power Control Algorithm | Algorithm1 |
| - TPC step size | 1dB |
| - Scrambling code type | Long |
| - Scrambling code number | 0 |
| - Number of DPDCH | Reference to TS34.108 clause 6.10 Parameter Set |
| - Spreading factor | SF is reference to TS34.108 clause 6.10 Parameter Set |
| - TFCI existence | Reference to TS34.108 clause 6.10 Parameter Set |
| - Number of FBI bit | SF is rReference to TS34.108 clause 6.10 Parameter Set |
| - Puncturing Limit | Reference to TS34.108 clause 6.10 Parameter Set |
| CHOICE Mode | FDD |
| - Downlink PDSCH information | Not Present |
| Downlink information common for all radio links |  |
| - Downlink DPCH info common for all RL |  |
| - Timing Indication | Maintain |
| - Downlink DPCH power control information |  |
| - DPC mode | 0 (single) |
| - CHOICE mode | FDD |
| - Power offset PPilot-DPDCH | 0 |
| - DL rate matching restriction information | Not Present |
| - Spreading factor | Reference to TS34.108 clause 6.10 Parameter Set |
| - Fixed or Flexible Position | Reference to TS34.108 clause 6.10 Parameter Set |
| - TFCI existence | Reference to TS34.108 clause 6.10 Parameter Set |
| - Number of bits for Pilot bits (SF=128,256) | Reference to TS34.108 clause 6.10 Parameter Set |
| - DPCH compressed mode info |  |
| - Transmission gap pattern sequence |  |
| - TGPSI | 1 |
| - TPGS status Flag | deactivate |
| - TGCFN - Transmission gap pattern sequence configuration parameters | Not Present Not Present |
| - TX Diversity mode | None |
| - SSDT information | Not Present |
| - Default DPCH Offset Value | 0 |
| Downlink information per radio link list | Not Present |

MEASUREMENT CONTROL (Step 9)

If UE requires compressed mode,

| Information Element | Value/Remark |
| --- | --- |
| Measurement Identity Measurement Command Measurement Reporting Mode - Measurement Reporting Transfer Mode - Periodic Reporting / Event Trigger Reporting Mode Additional measurements list CHOICE measurement type - Inter-frequency cell info list - CHOICE inter-frequency cell removal - New inter-frequency info list - Inter-frequency cell id - Frequency info - UARFCN uplink (Nu) - UARFCN downlink (Nd) - Cell info - Cell individual offset - Reference time difference to cell - Read SFN Indicator - CHOICE mode - Primary CPICH Info - Primary Scrambling Code - Primary CPICH TX power - TX Diversity Indicator - Cells for measurement - Inter-frequency cell id - Inter-frequency measurement quantity - CHOICE reporting criteria - Filter Coefficient - Measurement quantity for frequency quality estimate - Inter-frequency reporting quantity - UTRA Carrier RSSI - Frequency quality estimate - Non frequency related cell reporting quantities - SFN-SFN observed time difference reporting indicator - Cell synchronisation information reporting indicator - Cell Identity reporting indicator - CPICH Ec/No reporting indicator - CPICH RSCP reporting indicator - Pathloss reporting indicator - Reporting cell status - CHOICE reported cell - Maximum number of reported cells - Measurement validity - Inter-frequency set update - CHOICE report criteria - Amount of reporting - Reporting interval DPCH compressed mode status info TGPS reconfiguration CFN Transmission gap pattern sequence - TGPSI - TGPS Status Flag - TGCFN | 1 Setup Acknowledged Mode RLC Event Trigger Not Present Inter-frequency measurement No inter-frequency cells removed 4 UARFCN of the uplink frequency for cell 4 UARFCN of the downlink frequency for cell 4 0 dB 0 chips FALSE FDD Set to same code as used for cell 4 Not Present FALSE 4 Inter-frequency reporting criteria 0 CPICH RSCP FALSE FALSE No report FALSE FALSE FALSE TRUE FALSE Report cell within active and/or monitored set on used frequency or within active and/or monitored set on non-used frequency 2 Not present Not present Periodic reporting criteria Infinity 16 seconds (Current CFN + (256 – TTI/10msec))mod 256 1 Activate (Current CFN + (256 – TTI/10msec))mod 256 |

If UE do not require compressed mode,

| Information Element | Value/Remark |
| --- | --- |
| Measurement Identity Measurement Command Measurement Reporting Mode - Measurement Reporting Transfer Mode - Periodic Reporting / Event Trigger Reporting Mode Additional measurements list CHOICE measurement type - Inter-frequency cell info list - CHOICE inter-frequency cell removal - New inter-frequency info list - Inter-frequency cell id - Frequency info - UARFCN uplink (Nu) - UARFCN downlink (Nd) - Cell info - Cell individual offset - Reference time difference to cell - Read SFN Indicator - CHOICE mode - Primary CPICH Info - Primary Scrambling Code - Primary CPICH TX power - TX Diversity Indicator - Cells for measurement - Inter-frequency cell id - Inter-frequency measurement quantity - CHOICE reporting criteria - Filter Coefficient - Measurement quantity for frequency quality estimate - Inter-frequency reporting quantity - UTRA Carrier RSSI - Frequency quality estimate - Non frequency related cell reporting quantities - SFN-SFN observed time difference reporting indicator - Cell synchronisation information reporting indicator - Cell Identity reporting indicator - CPICH Ec/No reporting indicator - CPICH RSCP reporting indicator - Pathloss reporting indicator - Reporting cell status - CHOICE reported cell - Maximum number of reported cells - Measurement validity - Inter-frequency set update - CHOICE report criteria - Amount of reporting - Reporting interval DPCH compressed mode status info | 1 Setup Acknowledged Mode RLC Event Trigger Not Present Inter-frequency measurement No inter-frequency cells removed 4 UARFCN of the uplink frequency for cell 4 UARFCN of the downlink frequency for cell 4 0 dB 0 chips FALSE FDD Set to same code as used for cell 4 Not Present FALSE 4 Inter-frequency reporting criteria 0 CPICH RSCP FALSE FALSE No report FALSE FALSE FALSE TRUE FALSE Report cell within active and/or monitored set on used frequency or within active and/or monitored set on non-used frequency 2 Not present Not present Periodic reporting criteria Infinity 16 seconds Not Present |

MEASUREMENT REPORT (Step 10)

| Information Element | Value/Remarks |
| --- | --- |
| Measurement identity Measured Results - CHOICE measurement - Inter-frequency measurement results - Frequency info - UARFCN (uplink) - UARFCN (downlink) - UTRA carrier RSSI - Inter-frequency cell measurement results - Cell measured results - Cell Identity - SFN-SFN observed time difference - Cell synchronisation information - Primary CPICH Info - Primary Scrambling Code - CPICH Ec/No - CPICH RSCP - Pathloss Measured Results on RACH Additional Measured results Event Results | Check to see if set to 1 Check to see if set to "Inter-frequency measured results list" Check to see if set to the UARFCN of the uplink frequency for cell 4 Check to see if set to the UARFCN of the downlink frequency for cell 4 Check to see if it is absent Check to see if it is absent Check to see if it is absent Check to see if it is absent Check to see if set to the same code for cell 4 Check to see if it is absent Check to see if it is present Check to see if it is absent Check to see if it is absent Check to see if it is absent Check to see if it is absent |

MEASUREMENT CONTROL (Step 11)

| Information Element | Value/Remark |
| --- | --- |
| Measurement Identity Measurement Command Measurement Reporting Mode - Measurement Reporting Transfer Mode - Periodic Reporting / Event Trigger Reporting Mode Additional measurements list CHOICE measurement type - Inter-frequency cell info list - CHOICE inter-frequency cell removal - New inter-frequency info list - Inter-frequency cell id - Frequency info - UARFCN uplink (Nu) - UARFCN downlink (Nd) - Cell info - Cell individual offset - Reference time difference to cell - Read SFN Indicator - CHOICE mode - Primary CPICH Info - Primary Scrambling Code - Primary CPICH TX power - TX Diversity Indicator - Cells for measurement - Inter-frequency cell id - Inter-frequency measurement quantity - CHOICE reporting criteria - Filter Coefficient - Measurement quantity for frequency quality estimate - Inter-frequency reporting quantity - UTRA Carrier RSSI - Frequency quality estimate - Non frequency related cell reporting quantities - SFN-SFN observed time difference reporting indicator - Cell synchronisation information reporting indicator - Cell Identity reporting indicator - CPICH Ec/No reporting indicator - CPICH RSCP reporting indicator - Pathloss reporting indicator - Reporting cell status - Measurement validity - Inter-frequency set update - CHOICE report criteria - Parameters required for each event - Inter-frequency event identity - Threshold used frequency - W used frequency - Hysteresis - Time to trigger - Reporting cell status - Parameters required for each non-used frequency - Threshold non used frequency - W non used frequency DPCH compressed mode status info | 1 Modify Acknowledged Mode RLC Event Trigger Not Present Inter-frequency measurement No inter-frequency cells removed 4 UARFCN of the uplink frequency for cell 4 UARFCN of the downlink frequency for cell 4 0 dB 0 chips FALSE FDD Set to same code as used for cell 4 Not Present FALSE 4 Inter-frequency reporting criteria 0 CPICH RSCP FALSE FALSE No report FALSE FALSE FALSE TRUE FALSE Not Present Not present Not present Inter-frequency measurement reporting criteria 2c Not Present Not Present 0.5 dB 0 milliseconds Not Present -85 dBm 0 Not Present |

MEASUREMENT REPORT (Step 12)

| Information Element | Value/Remarks |
| --- | --- |
| Measurement identity Measured Results - CHOICE measurement - Inter-frequency measurement results - Frequency info - UARFCN (uplink) - UARFCN (downlink) - UTRA carrier RSSI - Inter-frequency cell measurement results Measured Results on RACH Additional Measured Results Event Results - CHOICE event result - Inter-frequency event identity - Inter-frequency cells - Frequency info - UARFCN (uplink) - UARFCN (downlink) - Non frequency related measurement event results - CHOICE Mode - Primary CPICH info - Primary Scrambling Code | Check to see if set to 1 Check to see if set to "Inter-frequency measured results list" Check to see if set to the UARFCN of the uplink frequency for cell 4 Check to see if set to the UARFCN of the downlink frequency for cell 4 Check to see if it is absent Check to see if it is absent Check to see if it is absent Check to see if it is absent Check to see if this IE is set to "Intra-frequency measurement event results" Check to see if this IE is set to "2c" Check to see if set to the UARFCN of the uplink frequency for cell 4 Check to see if set to the UARFCN of the downlink frequency for cell 4 Check to see if set to "FDD" Check to see if set to the same code as cell 4 |

8.4.1.2.5	Test Requirement

After step 5 the UE shall not transmit any MEASUREMENT REPORT messages pertaining to the measurement of CPICH RSCP of cell 4.

If UE requires compressed mode, after step 7, UE shall transmit PHYSICAL CHANNEL RECONFIGURATION COMPLETE message on uplink DCCH using AM RLC.

After step 9 the UE shall transmit MEASUREMENT REPORT messages on uplink DCCH, reporting cell 4's CPICH RSCP value at periodic time interval of 16 seconds in "inter-frequency cell measurement results" IE.

After step 11 the UE shall transmit only 1 MEASUREMENT REPORT message on the uplink DCCH. In this message, IE "inter-frequency cell measured results" shall be absent.

#### 8.4.1.7	Measurement Control and Report: Intra-frequency measurement for transition from CELL_FACH to CELL_DCH state

8.4.1.7.1	Definition

8.4.1.7.2	Conformance requirement

Upon transition from CELL_FACH to CELL_DCH state, the UE shall:

1>	retrieve each set of measurement control information of measurement type "intra-frequency" stored in the variable MEASUREMENT_IDENTITY;

1>	if the IE "measurement validity" for a measurement has been assigned the value "CELL_DCH:

2>	resume the measurement reporting.

1>	if no intra-frequency measurements applicable to CELL_DCH state are stored in the variable MEASUREMENT_IDENTITY:

2>	continue monitoring the list of neighbouring cells assigned in the IE "intra-frequency cell info list" in System Information Block type 12 (or System Information Block type 11);

2>	if the IE "intra-frequency measurement reporting criteria" was included in System Information Block type 12 (or System Information Block type 11):

3>	send the MEASUREMENT REPORT message when reporting criteria in IE "Reporting information for state CELL_DCH" are fulfilled.

…

Upon cell reselection while in CELL_FACH/CELL_PCH/URA/PCH state and the cell reselection has occurred after the measurement control information was stored, the UE shall:

1>	delete all measurements of type intra-frequency, inter-frequency, and inter-RAT associated with the variable MEASUREMENT_IDENTITY;

1>	delete the traffic volume measurements that have not been set up or modified through a MEASUREMENT CONTROL message.

…

Upon reception of a MEASUREMENT CONTROL message the UE shall perform actions specified in subclause 8.6 unless otherwise specified below.

The UE shall:

1>	read the IE "Measurement command";

1>	if the IE "Measurement command" has the value "setup":

2>	store this measurement in the variable MEASUREMENT_IDENTITY according to the IE "measurement identity", first releasing any previously stored measurement with that identity if that exists;

2>	for measurement types "inter-RAT measurement" or "inter-frequency measurement":

…

2>	for measurement type "UE positioning measurement":

…

2>	for any other measurement type:

3>	if the measurement is valid in the current RRC state of the UE:

4>	begin measurements according to the stored control information for this measurement identity.

1>	if the IE "Measurement command" has the value "modify":

2>	for all IEs present in the MEASUREMENT CONTROL message:

3>	if a measurement was stored in the variable MEASUREMENT_IDENTITY associated to the identity by the IE "measurement identity":

4>	for measurement types "inter-frequency measurement" that require measurements on a frequency other than the actually used frequency, or that require measurements on another RAT:

…

4>	for any other measurement type:

5>	replace the corresponding information stored in variable MEASUREMENT_IDENTITY associated to the identity indicated by the IE "measurement identity" with the one received in the MEASUREMENT CONTROL message;

5>	resume the measurements according to the new stored measurement control information.

3>	otherwise:

…

2>	for all optional IEs that are not present in the MEASUREMENT CONTROL message:

3>	leave the currently stored information elements unchanged in the variable MEASUREMENT_IDENTITY if not stated otherwise for that IE.

1>	if the IE "measurement command" has the value "release":

2>	terminate the measurement associated with the identity given in the IE "measurement identity";

2>	clear all stored measurement control information related associated to this measurement identity in variable MEASUREMENT_IDENTITY.

Reference

3GPP TS 25.331, clause 8.4.1.3, 8.4.1.6a and 8.4.1.7.1

8.4.1.7.3	Test Purpose

-	To confirm that UE retrieves stored measurement control information for intra-frequency measurement measurement type with "measurement validity" assigned to "CELL_DCH", after it enters CELL_DCH state from CELL_FACH state.

-	To confirm that the UE continues to monitor the neighbouring cells listed "intra-frequency cell info" IE in the System Information Block type 11 or 12 messages, if no intra-frequency measurements applicable to CELL_DCH are stored.

-	To confirm that the UE transmits MEASUREMENT REPORT messages if reporting criteria stated in IE "intra-frequency measurement reporting criteria" in System Information Block type 11 or 12 messages are fulfilled.

-	To confirm that a MEASUREMENT CONTROL message received in CELL_DCH state overrides the measurement and associated reporting contexts maintained in the UE by virtue of System Information Block type 11 or 12 messages.

8.4.1.7.4	Method of test

Initial Condition

System Simulator: 3 cells – Cell 1, cell 2 and cell 3 are active.

UE: PS-DCCH+DTCH_FACH (state 6-11).

Test Procedure

Table 8.4.1.7-1 illustrates the downlink power to be applied for the 3 cells at various time instants of the test execution. Column marked "T0" denotes the initial conditions, while columns marked "T1" are to be applied subsequently. The exact instants on which these values shall be applied are described in the text in this clause.

- Table 8.4.1.7-1

| Para-meter | Unit | Cell 1 |  | Cell 2 |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T0 | T1 | T0 | T1 | T0 | T1 |
| UTRA RF Channel Number |  | Ch. 1 |  | Ch. 1 |  | Ch. 1 |  |
| CPICH Ec | dBm/3.84 MHz | -60 | -122 | -70 | -60 | -75 | -75 |

The UE is brought to CELL_FACH state in cell 1. System Information Block type 12 message is changed with respect to the default message contents, with cell 2 included in the IE "intra-frequency cell info". Event 1e is selected in IE "Reporting information for state CELL_DCH", and "Intra-frequency measurement quantity" is set to CPICH RSCP. SS sends SYSTEM INFORMATION CHANGE INDICATION message to UE to inform UE of the modification in the system information.

SS send a RADIO BEARER RECONFIGURATION message to UE, and configures dedicated physical channels on both uplink and downlink directions. The UE shall move to CELL_DCH state and then return RADIO BEARER RECONFIGURATION COMPLETE message. The UE shall send MEASUREMENT REPORT messages containing IE "Measured results" to report cell 2's CPICH RSCP value and IE "event results" to report triggering of event type "1e". After receiving the MEASUREMENT REPORT messages, SS transmits a MEASUREMENT CONTROL message with only cell 3 included in the IE "intra-frequency cell info" and IE "CHOICE reporting criteria" set to "periodic reporting". After receiving such a message, the UE shall transmit another set of MEASUREMENT REPORT messages. SS verifies that only measurement readings for cell 3 's CPICH RSCP are report in IE "cell measured results" in these messages.

Next, SS sends PHYSICAL CHANNEL RECONFIGURATION message. SS configures common physical channels for both the uplink and the downlink directions. The UE shall transit to CELL_FACH state and then reply with a PHYSICAL CHANNEL RECONFIGURATION COMPLETE. SS waits for 16 seconds and checks the uplink RACH to confirm that no MEASUREMENT REPORT messages are received. SS transmits MEASUREMENT CONTROL message on the downlink DCCH. The key parameters specified in this message are: measurement command = 'setup', measurement type = 'intra-frequency measurement', measurement object = 'cell 2', reporting criteria = 'periodic reporting', measurement validity IE is present and "UE state" = "CELL_DCH". SS waits for 16 seconds, verifies that no MEASUREMENT REPORT messages are detected on the uplink DCCH. SS sends RADIO BEARER RECONFIGURATION message and configures dedicated physical channels. The UE shall return to CELL_DCH state, transmit a RADIO BEARER RECONFIGURATION COMPLETE message and start to monitor cell 2. The UE shall also resume periodic reporting of cell 2's CPICH RSCP measured results by sending MEASUREMENT REPORT messages. Following the reception of the MEASUREMENT REPORT message, SS commands the UE using MEASUREMENT CONTROL message to release measurement control information stored in "measurement identity" = 12. Thereafter, SS verifies that no MEASUREMENT REPORT messages are detected on the uplink DCCH. After this requirement is satisfied, SS sends MEASUREMENT CONTROL on the downlink DCCH once more. This message is identical to the one sent in step 5 (see specific message content).

SS transmits a PHYSICAL CHANNEL RECONFIGURATION message on the downlink DCCH and configures common physical channel. The UE shall transit to CELL_FACH state and then respond with a PHYSICAL CHANNEL RECONFIGURATION COMPLETE message. SS monitor the uplink DCCH once more to verify that no MEASUREMENT REPORT messages are detected. SS modifies the contents of Master Information Block and System Information Block type 12 messages, and then send SYSTEM INFORMATION CHANGE INDICATION message to UE. This is followed by a reconfiguration of the downlink transmission power of the respect cells according to the settings in columns "T1" in table 8.4.1.7-1. SS starts timer T305 and then waits for it to expire. The UE shall discover an "out-of-service" condition and initiate a cell re-selection procedure. This is verified in the SS when a CELL UPDATE message is received on the uplink CCCH with the "cell update cause" IE set to "cell reselection". SS transmits a CELL UPDATE CONFIRM message, which includes "New C-RNTI", on the DCCH. Then the UE shall reply with UTRAN MOBILITY INFORMATION CONFIRM message. Next, SS sends a RADIO BEARER RECONFIGURATION message on the downlink DCCH, assigning dedicated physical channels in both uplink and downlink directions. The UE shall respond with a RADIO BEARER RECONFIGURATION COMPLETE message and then return to CELL_DCH state. UE shall then send MEASUREMENT REPORT messages reporting cell 1 and 3’s CPICH RSCP according ot the content in System Information Block type 12 messages broadcasted in cell 2.

Expected Sequence

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 |  |  | System Information Block type 12 | UE is initially in PS-DCCH+DTCH_FACH (state 6-11) in cell 1. System Information Block type 12 messages are changed with respect to the default contents according to the descriptions in "Specific Message Contents" clause. |
| 1a |  |  | SYSTEM INFORMATION CHANGE INDICATION |  |
| 2 |  |  | RADIO BEARER RECONFIGURATION | SS configures dedicated physical channels. |
| 3 |  |  | RADIO BEARER RECONFIGURATION COMPLETE | UE shall move to CELL_DCH state. |
| 4 |  |  | MEASUREMENT REPORT | Reports cell 2's CPICH RSCP measurement value. |
| 5 |  |  | MEASUREMENT CONTROL | Only cell 3 is included in the IE "intra-frequency cell info". |
| 6 |  |  | MEASUREMENT REPORT | UE shall report cell 3's CPICH RSCP reading in IE "cell measured results". |
| 7 |  |  | PHYSICAL CHANNEL RECONFIGURATION | SS configures PRACH and S-CCPCH physical channels. |
| 8 |  |  | PHYSICAL CHANNEL RECONFIGURATION COMPLETE | UE shall move to CELL_FACH state. |
| 9 |  |  |  | SS waits for 16 seconds and checks that no MEASUREMENT REPORT messages are sent by UE. |
| 10 |  |  | MEASUREMENT CONTROL | SS instructs the UE to setup intra-frequency measurement and reporting for cell 2. Measurement validity" IE is set to CELL_DCH state. |
| 11 |  |  |  | SS waits for 16 seconds and verifies that no MEASUREMENT REPORT messages are sent by UE. |
| 12 |  |  | RADIO BEARER RECONFIGURATION | SS configures dedicated physical channels. |
| 13 |  |  | RADIO BEARER RECONFIGURATION COMPLETE | UE shall return to CELL_DCH state. |
| 14 |  |  | MEASUREMENT REPORT | UE begins to report cell 2's measured results for CPICH RSCP. |
| 15 |  |  | MEASUREMENT CONTROL | Terminate all the intra-frequency measurement and reporting activitiest related to "measurement identity" = 12. |
| 16 |  |  |  | SS waits for 16 seconds and verifies that UE stop transmitting MEASUREMENT REPORT messages. |
| 17 |  |  | MEASUREMENT CONTROL | This message is the same as in step 5 |
| 18 |  |  | PHYSICAL CHANNEL RECONFIGURATION | Allocates common physical channels. |
| 19 |  |  | PHYSICAL CHANNEL RECONFIGURATION COMPLETE | UE shall move to CELL_FACH state. |
| 20 |  |  |  | SS checks that no MEASUREMENT REPORT messages are received. |
| 21 |  |  | Master Information Block System Information Block type 12 | System Information Block type 12 messages are modified to include cell 2 and cell 3 into neighbouring cells list for intra-frequency type measurements. |
| 21a |  |  | SYSTEM INFORMATION CHANGE INDICATION | SS reconfigures the downlink transmission power settings for cell 1 to cell 3 according to columns "T1" in table 8.4.1.7-1, runs timer T305, and then waits until T305 expires. |
| 22 |  |  | CELL UPDATE | UE shall re-selects to cell 2 and then perform a cell update procedure. |
| 23 |  |  | CELL UPDATE CONFIRM | UE shall stay in CELL_FACH state. |
| 23a |  |  | UTRAN MOBILITY INFORMATION CONFIRM |  |
| 24 |  |  | RADIO BEARER RECONFIGURATION | Dedicated physical channels are assigned to the UE in this message. |
| 25 |  |  | RADIO BEARER RECONFIGURATION COMPLETE | UE shall return to CELL_DCH state. |
| 26 |  |  | MEASUREMENT REPORT | UE begins to report cell 1 and 3's measured results for CPICH RSCP. |

Specific Message Content

Master Information Block (Step 1)

| Information Element | Value/Remarks |
| --- | --- |
| MIB Value Tag | 3 |

System Information Block type 12 (Step 1)

| Information Element | Value/Remark |
| --- | --- |
| FACH measurement occasion info Measurement control system information - Intra-frequency measurement system information - Intra-frequency measurement identity - Intra-frequency cell info list - CHOICE intra-frequency cell removal - New intra-frequency info list - Intra-frequency cell id - Cell info - Cell individual offset - Reference time difference to cell - Read SFN Indicator - CHOICE mode - Primary CPICH Info - Primary Scrambling Code - Primary CPICH TX power - TX Diversity Indicator - Cell selection and Re-selection info - Intra-frequency measurement quantity - Filter Coefficient - Measurement quantity - Intra-frequency measurement for RACH reporting - Maximum number of reported cells on RACH - Reporting information for state CELL_DCH - Intra-frequency reporting quantity - Reporting quantities for active set cells - SFN-SFN observed time difference reporting indicator - Cell synchronisation information reporting indicator - Cell identity reporting indicator - CPICH Ec/No reporting indicator - CPICH RSCP reporting indicator - Pathloss reporting indicator - Reporting quantities for monitored set cells - SFN-SFN observed time difference reporting indicator - Cell synchronisation information reporting indicator - Cell identity reporting indicator - CPICH Ec/No reporting indicator - CPICH RSCP reporting indicator - Pathloss reporting indicator - Reporting quantities for detected cells - CHOICE report criteria - Parameter required for each event - Intra-frequency event identity - Triggering condition 1 - Triggering condition 2 - Reporting range - Cells forbidden to affect reporting - CHOICE Mode - Primary CPICH Info - Primary scrambling code - W - Hysteresis - Threshold used frequency - Reporting deactivation threshold - Replacement activation threshold - Time to trigger - Amount of reporting - Reporting Interval - Reporting cell status - CHOICE reported cells - Maximum number of reported cells - Inter-frequency measurement system information - Inter-RAT measurement system information - Traffic volume measurement system information - UE internal measurement system information | Not Present 10 Remove no intra-frequency cells 2 0 dB 0 chips FALSE FDD Set to same code as used for cell 2 Not Present FALSE Not Present 0 CPICH RSCP Not Present No report No report FALSE FALSE FALSE FALSE FALSE No report FALSE TRUE FALSE TRUE FALSE Not present Intra-frequency measurement reporting criteria 1e Not Present Monitored set cells Not present Not present FDD Set to the scrambling code of cell 2 Not present 0 dB -80 dBm Not present Not present 0 Infinity 16 seconds Report cells within monitored set cells on used frequency 1 Not Present Not Present Not Present Not Present |

SYSTEM INFORMATION CHANGE INDICATION (Step 1a)

| Information Element | Value/Remarks |
| --- | --- |
| BCCH modification info |  |
| - MIB Value Tag | 3 |
| - BCCH modification time | Not Present |

SYSTEM INFORMATION CHANGE INDICATION (Step 21a)

| Information Element | Value/Remarks |
| --- | --- |
| BCCH modification info |  |
| - MIB Value Tag | 2 |
| - BCCH modification time | Not Present |

RADIO BEARER RECONFIGURATION (Step 2, Step 12 and Step 24)

Use the same message type found in Annex A, with condition set to A4.

MEASUREMENT REPORT (Step 4)

| Information Element | Value/Remarks |
| --- | --- |
| Measurement identity Measured Results - CHOICE measurement - Intra-frequency measurement results - Cell measured results - Cell Identity - SFN-SFN observed time difference - Cell synchronisation information - Primary CPICH Info - Primary Scrambling Code - CPICH Ec/No - CPICH RSCP - Pathloss Measured Results on RACH Additional measured results Event Results - CHOICE event result - Intra-frequency event identity - Cell measurement event results - Primary CPICH info - Primary scrambling code | Check to see if set to 10 Check to see if set to "Intra-frequency measured results list" Check to see if this IE is absent Check to see if this IE is absent Check to see if it's the same code for cell 2 Check to see if this IE is absent Check to see if this IE is present Check to see if this IE is absent Check to see if this IE is absent Check to see if this IE is absent Check to see if it's set to 'Intra-frequency measurement event results' Check to see if this IE is set to '1e' Check to see if it's the same code for cell 2 |

MEASUREMENT CONTROL (Step 5 and Step 17)

| Information Element | Value/Remark |
| --- | --- |
| Measurement Identity Measurement Command Measurement Reporting Mode - Measurement Reporting Transfer Mode - Periodic Reporting / Event Trigger Reporting Mode Additional measurements list CHOICE measurement type - Intra-frequency cell info list - CHOICE intra-frequency cell removal - New intra-frequency info list - Intra-frequency cell id - Cell info - Cell individual offset - Reference time difference to cell - Read SFN Indicator - CHOICE mode - Primary CPICH Info - Primary Scrambling Code - Primary CPICH TX power - TX Diversity Indicator - Cells selection and Re-selection info - Cells for measurement - Intra-frequency cell id - Intra-frequency measurement quantity - Filter Coefficient - Measurement quantity - Intra-frequency reporting quantity - Reporting quantities for active set cells - SFN-SFN observed time difference reporting indicator - Cell synchronisation information reporting indicator - Cell identity reporting indicator - CPICH Ec/No reporting indicator - CPICH RSCP reporting indicator - Pathloss reporting indicator - Reporting quantities for monitored set cells - SFN-SFN observed time difference reporting indicator - Cell synchronisation information reporting indicator - Cell identity reporting indicator - CPICH Ec/No reporting indicator - CPICH RSCP reporting indicator - Pathloss reporting indicator - Reporting quantities for detected cells - Reporting cell status - Measurement validity - CHOICE report criteria - Parameters required for each event - Intra-frequency event identity - Triggering condition 1 - Triggering condition 2 - Reporting Range - Cells forbidden to affect Reporting range - CHOICE Mode - Primary CPICH Info - Primary Scrambling Code - W - Hysteresis - Reporting deactivation threshold - Replacement activation threshold - Threshold used frequency - Time to Trigger - Amount of reporting - Reporting interval - Reporting cell status - CHOICE reported cells - Maximum number of reported cells DPCH compressed mode status info | 10 Setup Acknowledged Mode RLC Event Trigger Not Present Intra-frequency measurement Remove no intra-frequency cells 3 0 dB 0 chips FALSE FDD Set to same code as used for cell 3 Not Present FALSE Not Present 3 0 CPICH RSCP No report FALSE FALSE FALSE FALSE FALSE No report FALSE TRUE FALSE TRUE FALSE Not present Not present Not present Intra-frequency measurement criteria 1e Not Present Monitored set cells Not Present Not Present FDD Set to the same scrambling code for cell 3 Not Present 0 dB Not Present Not Present -90 dBm 0 Infinity 16 seconds Report cells within monitored set cells on used frequency 1 Not Present |

MEASUREMENT REPORT (Step 6)

| Information Element | Value/Remarks |
| --- | --- |
| Measurement identity Measured Results - CHOICE measurement - Intra-frequency measurement results - Cell measured results - Cell Identity - SFN-SFN observed time difference - Cell synchronisation information - Primary CPICH Info - Primary Scrambling Code - CPICH Ec/No - CPICH RSCP - Pathloss Measured Results on RACH Additional measured results Event Results - CHOICE event result - Intra-frequency event identity - Cell measurement event results - Primary CPICH info - Primary scrambling code | Check to see if set to 10 Check to see if set to "Intra-frequency measured results list" Check to see if this IE is absent Check to see if this IE is absent Check to see if this IE is absent Check to see if it's the same code for cell 3 Check to see if this IE is absent Check to see if this IE is present Check to see if this IE is absent Check to see if this IE is absent Check to see if this IE is absent Check to see if it's set to 'Intra-frequency measurement event results' Check to see if this IE is set to '1e' Check to see if it's the same code for cell 3 |

PHYSICAL CHANNEL RECONFIGURATION (Step 7 and 18)

Use the same message sub-type found in Annex A, which is entitled "Packet to CELL_FACH from CELL_DCH in PS".

MEASUREMENT CONTROL (Step 10)

| Information Element | Value/Remark |
| --- | --- |
| Measurement Identity Measurement Command Measurement Reporting Mode - Measurement Reporting Transfer Mode - Periodic Reporting / Event Trigger Reporting Mode Additional measurements list CHOICE measurement type - Intra-frequency cell info list - CHOICE intra- frequency cell removal - New intra-frequency info list - Intra-frequency cell id - Cell info - Cell individual offset - Reference time difference to cell - Read SFN Indicator - CHOICE Mode - Primary CPICH Info - Primary Scrambling Code - Primary CPICH TX power - TX Diversity Indicator - Cell selection and Re-selection info - Cells for measurement - Intra-frequency cell id - Intra-frequency measurement quantity - Filter Coefficient - Measurement quantity - Intra-frequency reporting quantity - Reporting quantities for active set cells - SFN-SFN observed time difference reporting indicator - Cell synchronisation information reporting indicator - Cell identity reporting indicator - CPICH Ec/No reporting indicator - CPICH RSCP reporting indicator - Pathloss reporting indicator - Reporting quantities for monitored set cells - SFN-SFN observed time difference reporting indicator - Cell synchronisation information reporting indicator - Cell identity reporting indicator - CPICH Ec/No reporting indicator - CPICH RSCP reporting indicator - Pathloss reporting indicator - Reporting quantities for detected cells - Reporting cell status - Measurement validity - UE state - CHOICE report criteria - Parameters required for each event - Intra-frequency event identity - Triggering condition 1 - Triggering condition 2 - Reporting Range - Cells forbidden to affect Reporting range - Primary CPICH Info - Primary Scrambling Code - W - Hysteresis - Reporting deactivation threshold - Replacement activation threshold - Threshold Used Frequency - Time to Trigger - Amount of reporting - Reporting interval - Reporting cell status - CHOICE reported cell - Maximum number of reported cells DPCH compressed mode status info | 12 Setup Acknowledged Mode RLC Event Trigger Not Present Intra-frequency measurement Remove no intra-frequency cells 2 0 dB 0 chips FALSE FDD Set to same code as used for cell 2 Not Present FALSE Not Present cell 2 0 CPICH RSCP No report FALSE FALSE FALSE FALSE FALSE No report FALSE TRUE FALSE TRUE FALSE Not present Not present CELL_DCH Intra-frequency measurement criteria 1e Not Present Monitored set cells Not Present Not Present Set to the same scrambling code for cell 2 Not Present 0 dB Not Present Not Present -80 dBm 0 Infinity 16 seconds Report cells within monitored set cells on used frequency 1 Not Present |

MEASUREMENT REPORT (Step 14)

| Information Element | Value/Remarks |
| --- | --- |
| Measurement identity Measured Results - CHOICE measurement - Intra-frequency measurement results - Cell measured results - Cell Identity - SFN-SFN observed time difference - Cell synchronisation information - Primary CPICH Info - Primary Scrambling Code - CPICH Ec/No - CPICH RSCP - Pathloss Measured Results on RACH Additional measured results Event Results - CHOICE event result - Intra-frequency event identity - Cell measurement event results - Primary CPICH info - Primary scrambling code | Check to see if set to 12 Check to see if set to "Intra-frequency measured results list" Check to see if this IE is absent Check to see if this IE is absent Check to see if it's the same code for cell 2 Check to see if this IE is absent Check to see if this IE is present Check to see if this IE is absent Check to see if this IE is absent Check to see if this IE is absent Check to see if it's set to 'Intra-frequency measurement event results' Check to see if this IE is set to '1e' Check to see if it's the same code for cell 2 |

MEASUREMENT CONTROL (Step 15)

| Information Element | Value/Remarks |
| --- | --- |
| Measurement Identity Measurement Command Measurement Reporting Mode Additional measurements list CHOICE Measurement type DPCH compressed mode status info | 12 Release Not Present Not Present Not Present Not Present |

Master Information Block (Step 21)

| Information Element | Value/Remarks |
| --- | --- |
| MIB Value Tag | 2 |

System Information Block type 12 (Step 21)

| Information Element | Value/Remark |
| --- | --- |
| FACH measurement occasion info Measurement control system information - Use of HCS - Cell_selection_and_reselection_-quality_measure - Intra-frequency measurement system information - Intra-frequency measurement identity - Intra-frequency cell info list - CHOICE intra-frequency cell removal - New intra-frequency info list - Intra-frequency cell id - Cell info - Cell individual offset - Reference time difference to cell - Read SFN Indicator - CHOICE Mode - Primary CPICH Info - Primary Scrambling Code - Primary CPICH TX power - TX Diversity Indicator - Cell selection and Re-selection info - Intra-frequency cell id - Cell info - Cell individual offset - Reference time difference to cell - Read SFN Indicator - CHOICE Mode - Primary CPICH Info - Primary Scrambling Code - Primary CPICH TX power - TX Diversity Indicator - Cell selection and Re-selection info - Intra-frequency measurement quantity - Filter Coefficient - Measurement quantity - Intra-frequency measurement for RACH reporting - Maximum number of reported cells on RACH - Reporting information for state CELL_DCH - Intra-frequency reporting quantity - Measurement reporting mode - Measurement Reporting Transfer Mode - Periodic Reporting / Event Triggering Report Mode - CHOICE report criteria - Amount of reporting - Reporting interval - Inter-frequency measurement system information - Inter-RAT measurement system information - Traffic volume measurement system information - UE internal measurement system information | Not Present Not used CPICH_Ec/No 13 Cell 2 and Cell 3 are added Remove no intra-frequency cells Set to id of cell 2 0 dB 0 chips FALSE FDD Set to same code as used for cell 2 Not Present FALSE Not Present – use default values Set to id of cell 3 0 dB 0 chips FALSE FDD Set to same code as used for cell 3 Not Present FALSE Not Present – use default values 0 CPICH RSCP Not Present No report CPICH RSCP Acknowledged mode RLC Periodic Reporting Perioidical reporting criteria Infinity 250 msec Not Present Not Present Not Present Not Present |

CELL UPDATE (Step 22)

| Information Element | Value/Remarks |
| --- | --- |
| U-RNTI - SRNC Identity - S-RNTI Cell Update Cause Protocol error indicator Measured results on RACH Protocol error information | Check to see if set to '0000 0000 0001' Check to see if set to '0000 0000 0000 0000 0001' Check to see if set to 'Cell Re-selection' Check to see if it is absent or set to 'FALSE' Check to see if it is absent Check to see if it is absent |

CELL UPDATE CONFIRM (Step 23)

Use the default message content of the same message type in Annex A, with the following exceptions.

| Information Element | Value/Remarks |
| --- | --- |
| New C-RNTI | '1010 1010 1010 1010' |

UTRAN MOBILITY INFORMATION CONFIRM (Step 23a)

Only the message type is checked.

MEASUREMENT REPORT (Step 26)

| Information Element | Value/Remarks |
| --- | --- |
| Measurement identity Measured Results - CHOICE measurement - Intra-frequency measurement results - Cell measured results - Cell Identity - SFN-SFN observed time difference - Cell synchronisation information - Primary CPICH Info - Primary Scrambling Code - CPICH Ec/No - CPICH RSCP - Pathloss Measured Results on RACH Additional measured results Event Results - CHOICE event result - Intra-frequency event identity - Cell measurement event results - CHOICE mode - Primary CPICH info - Primary scrambling code - Primary CPICH info - Primary scrambling code | Check to see if set to 1 Check to see if set to "Intra-frequency measured results list" Check to see if this IE is present Check to see if this IE is absent Check to see if this IE is present Check to see if it's the same code for cell 1 Check to see if this IE is absent Check to see if this IE is present Check to see if this IE is absent Check to see if this IE is absent Check to see if this IE is absent Check to see if it's set to 'Intra-frequency measurement event results' Check to see if this IE is set to '1b' FDD Check to see if it's the same code for cell 1 Check to see if it's the same code for cell 3 |

8.4.1.7.5	Test Requirement

After step 3 the UE shall report cell 2's CPICH RSCP value by transmitting MEASUREMENT REPORT messages.

After step 5 the UE shall delete all measurement and reporting contexts obtained from System Information Block type 12 messages. It shall transmit MEASUREMENT REPORT messages which contain measured results of cell 3's CPICH RSCP value only

After step 9 and step 11 the UE shall not transmit MEASUREMENT REPORT messages, which pertain to intra-frequency type measurement reporting.

After step 13 the UE shall resume the measurement and reporting activities as specified in MEASUREMENT CONTROL message received in step 10. The UE shall transmit MEASUREMENT REPORT messages, containing measured results of cell 2's CPICH RSCP value.

After step 15 the UE shall stop measurement activities pertaining to periodic reporting of cell 2's CPICH RSCP, no MEASUREMENT REPORT messages shall be detectable by the SS on the uplink DCCH.

After step 21 the UE shall re-select to cell 2 and initiate a cell update procedure. SS shall receive a CELL UPDATE message on the uplink CCCH of cell 2, with the "cell update cause" IE stated as "cell re-selection".

After step 23, the UE shall transmit UTRAN MOBILITY INFORMATION CONFIRM message on the uplink DCCH AM RLC.

After step 25 the UE shall report cell 1 and 3's CPICH RSCP value by transmitting MEASUREMENT REPORT messages.

#### 8.4.1.8	Measurement Control and Report: Inter-frequency measurement for transition from CELL_FACH to CELL_DCH state

8.4.1.8.1	Definition

8.4.1.8.2	Conformance requirement

Upon transition from CELL_FACH to CELL_DCH state, the UE shall:

1>	stop monitoring the list of cells assigned in the IE "inter-frequency cell info list" in System Information Block type 12 (or System Information Block type 11);

1>	retrieve each set of measurement control information of measurement type "inter-frequency" stored in the variable MEASUREMENT_IDENTITY; and

1>	if the IE "measurement validity" for a measurement has been assigned the value "CELL_DCH":

2>	resume the measurement reporting.

If the IE "DPCH compressed mode info" is included, and if the IE group "transmission gap pattern sequence configuration parameters" is included, the UE shall for each transmission gap pattern sequence perform the following consistency checks:

1>	if UE, according to its measurement capabilities, and for the measurement purpose indicated by IE "TGMP", requires UL compressed mode for measurements on any of the cells to be measured according to UE variable CELL_INFO_LIST, and CHOICE 'UL/DL mode' indicates 'DL only':

2>	set the variable INVALID_CONFIGURATION to TRUE.

1>	if UE, according to its measurement capabilities, and for the measurement purpose indicated by IE "TGMP", requires DL compressed mode for measurements on any of the cells to be measured according to UE variable CELL_INFO_LIST, and CHOICE 'UL/DL mode' indicates 'UL only':

2>	set the variable INVALID_CONFIGURATION to TRUE.

1>	if UE already has an active transmission gap pattern sequence that, according to IE "TGMP", has the same measurement purpose, and both patterns will be active after the new configuration has been taken into use:

2>	set the variable INVALID_CONFIGURATION to TRUE.

If variable INVALID_CONFIGURATION has value FALSE after UE has performed the checks above, the UE shall:

1>	if pattern sequence corresponding to IE "TGPSI" is already active (according to "TGPS Status Flag"):

2>	deactivate this pattern sequence at the beginning of the frame, indicated by IE "Activation time" received in this message, when the new configuration received in this message is taken into use.

1>	update each pattern sequence to the variable TGPS_IDENTITY according to the IE "TGPSI";

1>	update into the variable TGPS_IDENTITY the configuration information defined by IE group" transmission gap pattern sequence configuration parameters ";

1>	after the new configuration has been taken into use:

2>	activate the stored pattern sequence corresponding to each IE "TGPSI" for which the "TGPS status flag" is set to "activate" at the time indicated by IE "TGCFN"; and

2>	begin the inter-frequency corresponding to the pattern sequence measurement purpose of each activated pattern sequence;

2>	if the new configuration is taken into use at the same CFN as indicated by IE "TGCFN":

3>	start the concerned pattern sequence immediately at that CFN.

1>	monitor if the parallel transmission gap pattern sequences create an illegal overlap, and in case of overlap, take actions as specified in TS 25.331 subclause 8.2.11.2.

Reference

3GPP TS 25.331 clause 8.4.1.7.2, 8.4.1.3

8.4.1.8.3	Test Purpose

- To confirm that the UE stops monitoring the list of cells assigned in the IE "inter-frequency cell info" in System Information Block type 11 or 12 when it transits from CELL_FACH state to CELL_DCH state.

- To confirm that the UE resumes inter-frequency measurements and reporting stored for which the measurement control information has IE "measurement validity" assigned to the value "CELL_DCH", after it re-enters CELL_DCH state from CELL_FACH state.

- To confirm that the UE resumes inter-frequency measurement and reporting activities after it has received a MEASUREMENT CONTROL message specifying that a stored compressed mode pattern sequence be re-activated.

8.4.1.8.4	Method of test

Initial Condition

System Simulator: 3 cells – Cells 1, cell 4 and cell 5 are active.

UE: PS-DCCH+DTCH_DCH (State 6-10) in cell 1 as specified in clause 7.4 of TS 34.108.

Related ICS/IXIT statements

-	Compressed mode required 	yes/no

Test Procedure

Table 8.4.1.8-1 illustrates the downlink power to be applied for the 3 cells in this test.

- Table 8.4.1.8-1

| Para-meter | Unit | Cell 1 | Cell 4 | Cell 5 |
| --- | --- | --- | --- | --- |
| UTRA RF Channel Number |  | Ch. 1 | Ch. 2 | Ch. 2 |
| CPICH Ec | dBm/3.84 MHz | -60 | -75 | -75 |

The UE is in CELL_DCH state in cell 1. SS transmits MEASUREMENT CONTROL message to add cell 5 into the IE "inter-frequency cell info". In the MEASUREMENT CONTROL message, the parameters of the IE "inter-frequency measurement reporting criteria" are as follow: event-triggered with event identity ='2c', Measurement quantity = "CPICH RSCP", threshold for non-used frequency = '-85 dBm', hysteresis = '1.0dB', and time to trigger = '10 seconds',. In the same message, IE "Measurement validity" is present and "UE state" is assigned the value 'CELL_DCH'. If UE requires compressed mode, SS checks that no MEASUREMENT REPORT messages are detected on the uplink DCCH after it has transmitted the MEASUREMENT CONTROL message. If UE do not require compressed mode, SS checks that UE sends MEASUREMENT REPORT message on the uplink DCCH.

SS sends a PHYSICAL CHANNEL RECONFIGURATION message on the downlink DCCH and configures PRACH and S-CCPCH physical channels. The UE shall reconfigure itself to receive and transmit using the new common physical channels assigned, and send PHYSICAL CHANNEL RECONFIGURATION COMPLETE on the uplink DCCH. SS modifies the content of Master Information Block and System Information Block type 12 messages, such that cell 4 is added in the list of cells assigned in the IE "inter-frequency cell info". SS transmits SYSTEM INFORMATION CHANGE INDICATION message to UE. Once again, SS verifies that the UE does not transmit MEASUREMENT REPORT messages in the uplink direction.

SS sends PHYSICAL CHANNEL RECONFIGURATION message, and configures dedicated physical. If UE requires compressed mode, in this message, SS commands the UE to start applying compressed mode mechanism for DPCH. The UE shall move to CELL_DCH state and then reply with PHYSICAL CHANNEL RECONFIGURATION COMPLETE message. SS waits for 10 seconds. The UE shall transmit 1 MEASUREMENT REPORT message, containing the selected frequency quality estimate (in this case CPICH RSCP) of cell 5. The UE shall also report the triggering of event '2c' in the IE "Event results" of MEASUREMENT REPORT message. SS verifies that this message does not contain measured results for cell 4.

SS transmits a MEASUREMENT CONTROL message on the downlink DCCH using AM-RLC. In this message, SS modifies the measurement control information for measurement identity = "14" and set IE "CHOICE reporting criteria" to "periodic reporting criteria".The UE shall transmit MEASUREMENT REPORT messages at 2 seconds interval.

If UE requires compressed mode, SS transmits a PHYSICAL CHANNEL RECONFIGURATION message and deactivates the compressed mode pattern sequence with "TGPSI" IE set to 1. The UE shall respond by sending PHYSICAL CHANNEL RECONFIGURATION COMPLETE message and also stop the periodic reporting activities.

Following this if UE requires compressed mode, SS sends a MEASUREMENT CONTROL message and re-activates the compressed mode pattern sequence by using the "DPCH compressed mode status" IE. SS confirms that the UE has reconfigured itself to start measurement reporting again. The SS shall be able to receive MEASUREMENT REPORT messages continuously at 2 seconds interval.

Expected Sequence

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 |  |  |  | The initial state of UE is in CELL_DCH state of cell 1. |
| 2 |  |  | MEASUREMENT CONTROL | SS specifies inter-frequency measurement and reporting parameters for cell 5, with "measurement validity" IE present and "UE state" set to "CELL_DCH". |
| 3 |  |  | MEASUREMENT REPORT | If compresed mode is not required ( refer ICS/IXIT ), , SS checks that UE transmit this message, or else SS checks that no MEASUREMENT REPORT messages are detected on the uplink DCCH. |
| 4 |  |  | PHYSICAL CHANNEL RECONFIGURATION | SS configures PRACH and S-CCPCH physical resources. |
| 5 |  |  | PHYSICAL CHANNEL RECONFIGURATION COMPLETE | UE shall move to CELL_FACH state. |
| 6 |  |  | Master Information Block System Information Block type 12 | SS modifies MIB and SIB 12 in order to include cell 4 into the list of cells in IE "inter-frequency cell info". |
| 7 |  |  | SYSTEM INFORMATION CHANGE INDICATION | After SS transmits this message, SS confirms that there are no transmissions of MEASUREMENT REPORT message in the uplink direction. |
| 8 |  |  | PHYSICAL CHANNEL RECONFIGURATION | See specific message content below. |
| 9 |  |  | PHYSICAL CHANNEL RECONFIGURATION COMPLETE | UE shall move to CELL_DCH state. |
| 10 |  |  | MEASUREMENT REPORT | UE shall resume inter-frequency measurement task for cell 5 and report the measured CPICH RSCP value for cell 5. |
| 11 |  |  | MEASUREMENT CONTROL | SS changes the reporting criteria for cell 5 to 'periodic reporting' |
| 12 |  |  | MEASUREMENT REPORT | UE shall begin to transmit this message at 2 seconds interval. If compresed mode is not required ( refer ICS/IXIT ), , the test ends here. |
| 13 |  |  | PHYSICAL CHANNEL RECONFIGURATION | SS deactivates the currently used pattern sequence for compressed mode operation. |
| 14 |  |  | PHYSICAL CHANNEL RECONFIGURATION COMPLETE | UE stays in CELL_DCH state. SS verifies that no MEASUREMENT REPORT messages are received. |
| 15 |  |  | MEASUREMENT CONTROL | SS activates the pattern sequence stored by the UE. |
| 16 |  |  | MEASUREMENT REPORT | SS checks that MEASURE-MENT REPORT messages are received at 2 seconds interval. |

# Specific Message Content

MEASUREMENT CONTROL (Step 2)

| Information Element | Value/Remark |
| --- | --- |
| Measurement Identity Measurement Command Measurement Reporting Mode - Measurement Reporting Transfer Mode - Periodic Reporting / Event Trigger Reporting Mode Additional measurements list CHOICE measurement type - Inter-frequency cell info list - CHOICE inter-frequency cell removal - New inter-frequency info list - Inter-frequency cell id - Frequency info - UARFCN uplink (Nu) - UARFCN downlink (Nd) - Cell info - Cell individual offset - Reference time difference to cell - Read SFN Indicator - CHOICE Mode - Primary CPICH Info - Primary Scrambling Code - Primary CPICH TX power - TX Diversity Indicator - Cells for measurement - Inter-frequency cell id - Inter-frequency measurement quantity - CHOICE reporting criteria - Filter Coefficient - Measurement quantity for frequency quality estimate - Inter-frequency reporting quantity - UTRA Carrier RSSI - Frequency quality estimate - Non frequency related cell reporting quantities - SFN-SFN observed time difference reporting indicator - Cell synchronisation information reporting indicator - Cell Identity reporting indicator - CPICH Ec/No reporting indicator - CPICH RSCP reporting indicator - Pathloss reporting indicator - Reporting cell status - Measurement validity - UE State - Inter-frequency set update - UE autonomous update mode - CHOICE report criteria - Parameters required for each event - Inter-frequency event identity - Threshold used frequency - W used frequency - Hysteresis - Time to trigger - Reporting cell status - CHOICE reported cell - Maximum number of reported cells - Parameters required for each non-used frequency - Threshold non used frequency - W non-used frequency DPCH compressed mode status info | 14 Setup Acknowledged Mode RLC Event Trigger Not Present Inter-frequency measurement No inter-frequency cells removed 5 UARFCN of the uplink frequency for cell 5 UARFCN of the downlink frequency for cell 5 0 dB 0 chips FALSE FDD Set to same code as used for cell 5 Not Present FALSE 5 Inter-frequency reporting criteria 0 CPICH RSCP FALSE FALSE No report FALSE TRUE FALSE TRUE FALSE Not present CELL_DCH On with no reporting Inter-frequency measurement reporting criteria 2c Not Present Not Present 1.0 dB 10 seconds Report cells within active and/or monitored set on used frequency or within active and/or monitored set on non-used frequency 2 -85 dBm 0.0 Not Present |

PHYSICAL CHANNEL RECONFIGURATION (Step 4)

Use the same message sub-type found in Annex A titled "(Packet to CELL_FACH from CELL_DCH in PS)".

Master Information Block (Step 6)

| Information Element | Value/Remark |
| --- | --- |
| Value Tag | 2 |

System Information Block type 12 (Step 6)

| Information Element | Value/Remark |
| --- | --- |
| FACH measurement occasion info - FACH Measurement occasion cycle length coefficient - Inter-frequency FDD measurement indicator - Inter-frequency TDD measurement indicator - Inter-RAT measurement indicators Measurement control system information - Intra-frequency measurement system information - Inter-frequency measurement system information - Inter-frequency cell info list - CHOICE inter-frequency cells removal - New inter-frequency info list - Inter-frequency cell id - Cell info - Cell individual offset - Reference time difference to cell - Read SFN Indicator - CHOICE Mode - Primary CPICH Info - Primary Scrambling Code - Primary CPICH TX power - TX Diversity Indicator - Cell selection and Re-selection info - Inter-RAT measurement system information - Traffic volume measurement system information - UE internal measurement system information | 2 TRUE FALSE Not Present Not Present No inter-frequency cells removed Set to id of cell 4 0 dB 0 chips FALSE FDD Set to same code as used for cell 4 Not Present FALSE Not Present – use default values Not Present Not Present Not Present |

PHYSICAL CHANNEL RECONFIGURATION (Step 8)

If UE do not require compressed mode, use the same message sub-type found in [9] TS 34.108 clause 9, which is entitled "(Packet to CELL_DCH from CELL_FACH in PS)".

If UE requires compressed mode, use the same message sub-type found in Annex A, which is entitled "(Packet to CELL_DCH from CELL_FACH in PS)", with the following exceptions in the IE(s) concerned:

| Information Element | Value/Remarks |
| --- | --- |
| Downlink information common for all radio links - Downlink DPCH info common for all RL - DPCH compressed mode info - TGPSI - TGPS Status Flag - TGCFN - Transmission gap pattern sequence configuration parameters - TGMP - TGPRC - TGSN - TGL1 - TGL2 - TGD - TGPL1 - TGPL2 - RPP - ITP - CHOICE UL/DL Mode - Downlink compressed mode method - Uplink compressed mode method - Downlink frame type - DeltaSIR1 - DeltaSIRAfter1 - DeltaSIR2 - DeltaSIRAfter2 - N identify abort - T Reconfirm abort - TX Diversity Mode - SSDT information - Default DPCH Offset Value | Not Present 1 Activate (Current CFN+(256 – TTI/10msec)) mod 256 FDD Measurement Infinity 4 7 Not Present 0 3 Not Present Mode 0 Mode 0 UL and DL or DL only depending on UE capability SF/2 SF/2 or Not present depending on UE capability B 2.0 1.0 Not Present Not Present Not Present Not Present None Not Present 0 |

MEASUREMENT REPORT (Step 3 and 10)

| Information Element | Value/Remarks |
| --- | --- |
| Measurement identity Measured Results - CHOICE measurement - Inter-frequency measurement results - Frequency info - UARFCN (uplink) - UARFCN (downlink) - UTRA carrier RSSI - Inter-frequency cell measurement results - Cell measured results - Cell Identity - SFN-SFN observed time difference - Cell synchronisation information - Primary CPICH Info - Primary Scrambling Code - CPICH Ec/No - CPICH RSCP - Pathloss Measured Results on RACH Event Results - CHOICE event result - Inter-frequency event identity - Inter-frequency cells - Frequency Info - UARFCN (uplink) - UARFCN (downlink) - Non frequency related measurement event results - Primary CPICH Info - Primary Scrambling Code | Check to see if set to 14 Check to see if set to "Inter-frequency measured results list" Check to see if set to the UARFCN of the uplink frequency for cell 5 Check to see if set to the UARFCN of the downlink frequency for cell 5 Check to see if it is absent Check to see if it is absent Check to see if it is absent Check to see if it is absent Check to see if set to the same code for cell 5 Check to see if it is absent Check to see if it is present Check to see if it is absent Check to see if it is absent Inter-frequency event results Check to see if it's set to '2c' Check to see if set to the UARFCN of the uplink frequency for cell 5 Check to see if set to the UARFCN of the downlink frequency for cell 5 Check to see if set to the same code for cell 5 |

MEASUREMENT CONTROL (Step 11)

| Information Element | Value/Remark |
| --- | --- |
| Measurement Identity Measurement Command Measurement Reporting Mode - Measurement Reporting Transfer Mode - Periodic Reporting / Event Trigger Reporting Mode Additional measurements list CHOICE measurement type - Inter-frequency cell info list - CHOICE inter-frequency cell removal - New inter-frequency info list - Inter-frequency cell id - Frequency info - UARFCN uplink (Nu) - UARFCN downlink (Nd) - Cell info - Cell individual offset - Reference time difference to cell - Read SFN Indicator - CHOICE Mode - Primary CPICH Info - Primary Scrambling Code - Primary CPICH TX power - TX Diversity Indicator - Cells for measurement - Inter-frequency cell id - Inter-frequency measurement quantity - CHOICE reporting criteria - Filter Coefficient - Measurement quantity for frequency quality estimate - Inter-frequency reporting quantity - UTRA Carrier RSSI - Frequency quality estimate - Non frequency related cell reporting quantities - SFN-SFN observed time difference reporting indicator - Cell synchronisation information reporting indicator - Cell Identity reporting indicator - CPICH Ec/No reporting indicator - CPICH RSCP reporting indicator - Pathloss reporting indicator - Reporting cell status - CHOICE reported cell - Maximum number of reported cells - Measurement validity - Inter-frequency set update - CHOICE report criteria - Amount of reporting - Reporting interval DPCH compressed mode status info | 14 Modify Acknowledged Mode RLC Event Trigger Not Present Inter-frequency measurement No inter-frequency cells removed 5 UARFCN of the uplink frequency for cell 5 UARFCN of the downlink frequency for cell 5 0 dB 0 chips FALSE FDD Set to same code as used for cell 5 Not Present FALSE 5 Inter-frequency reporting criteria 0 CPICH RSCP FALSE FALSE No report FALSE TRUE FALSE TRUE FALSE Report cells within active and/or monitored set on used frequency or within active and/or monitored set on non-used frequency 2 Not Present Not Present Periodic reporting criteria Infinity 2000 milliseconds Not Present |

MEASUREMENT REPORT (Step 12, 16)

| Information Element | Value/Remarks |
| --- | --- |
| Measurement identity Measured Results - CHOICE measurement - Inter-frequency measurement results - Frequency info - UARFCN (uplink) - UARFCN (downlink) - UTRA carrier RSSI - Inter-frequency cell measurement results - Cell measured results - Cell Identity - SFN-SFN observed time difference - Cell synchronisation information - Primary CPICH Info - Primary Scrambling Code - CPICH Ec/No - CPICH RSCP - Pathloss - CFN-SFN observed time difference Measured Results on RACH Event Results | Check to see if set to 14 Check to see if set to "Inter-frequency measured results list" Check to see if set to the UARFCN of the uplink frequency for cell 5 Check to see if set to the UARFCN of the downlink frequency for cell 5 Check to see if it is absent Check to see if is absent Check to see if it is absent Check to see if it is absent Check to see if set to the same code for cell 5 Check to see if it is absent Check to see if it is present Check to see if it is absent Check to see if it is absent Check to see if it is absent Check to see if it is absent |

PHYSICAL CHANNEL RECONFIGURATION (Step 13)

Use the same message transmitted in step 8 with the following modifications:

| Information Element | Value/Remarks |
| --- | --- |
| Downlink information common for all radio links - DPCH compressed mode info - TGPSI - TGPS Status Flag - TGCFN - Transmission gap pattern sequence configuration parameters | 1 deactivate Not Present Not Present |

MEASUREMENT CONTROL (Step 15)

| Information Element | Value/Remark |
| --- | --- |
| Measurement Identity Measurement Command Measurement Reporting Mode Additional measurements list CHOICE measurement type DPCH compressed mode status info - TGPS reconfiguration CFN - Transmission gap pattern sequence - TGPSI - TGPS Status Flag - TGCFN | 14 Modify Not Present Not Present Not Present (Current CFN+(256 – TTI/10msec)) mod 256 1 Activate (Current CFN+(256 – TTI/10msec)) mod 256 |

8.4.1.8.5	Test Requirement

After step 2, if UE requires compressed mode, the UE shall not send any MEASUREMENT REPORT messages on the uplink DCCH of cell 1. If UE do not require compressed mode, the UE shall send a MEASUREMENT REPORT message on the uplink DCCH of cell 1.

After step 4 and 8, UE shall transmit PHYSICAL CHANNEL RECONFIGURATION COMPLETE message on the uplink DCCH using AM RLC.

After step 9 the UE shall transmit a MEASUREMENT REPORT message, containing the IE "measured results" reporting cell 5's CPICH RSCP value. The UE shall also report the triggering of event '2c' by including IE "Event results" in the MEASUREMENT REPORT message. The UE shall not transmit any MEASUREMENT REPORT messages pertaining to cell 4's measurements.

After step 11 the UE shall send MEASUREMENT REPORT messages, containing cell 5's CPICH RSCP measured value in IE "Measured results" at 2 seconds interval. The "Event results" IE shall be omitted in these messages.

If UE requires compressed mode, after step 14, the UE shall not transmit any MEASUREMENT REPORT messages.

If UE requires compressed mode, after step 15, the UE shall resume the transmission of MEASUREMENT REPORT messages with identical contents as in those received after step 11.

#### 8.4.1.14	Measurement Control and Report: Cell forbidden to affect reporting range

8.4.1.14.1	Definition

8.4.1.14.2	Conformance requirement

The reporting range affects the reporting events 1A and 1B. The reporting range is defined as a function of all the Primary CPICHs in the active set. If the parameter W is set to 0, the reporting range is defined relative to the best Primary CPICH. However, there could be cases where it is good to forbid a specific Primary CPICH to affect the reporting range. This mechanism could be effective if the operator knows by experience that the quality of a Primary CPICH is very unstable in a specific area and therefore should not affect the reporting of the other Primary CPICHs.

The UE shall ignore that a Primary CPICH is forbidden to affect the reporting range if all of the following conditions are fulfilled:

-	the Primary CPICH is included in active set; and

-	all cells in active set are defined as Primary CPICHs forbidden to affect the reporting range.

Reference

3GPP TS 25.331 clause 14.1.2.1, 14.1.2.2, clause 14.1.5.4

8.4.1.14.3	Test Purpose

- To confirm that the UE reports the triggering of event 1A to the SS, if a primary CPICH currently measured by the UE enters the reporting range.

- To confirm that the UE reports the triggering of event 1B to the SS, if a primary CPICH currently measured by the UE leaves the reporting range.

- To confirm that the UE use the forbidden cell indicated in the MEASUREMENT CONTROL message to affect the reporting range.

- To confirm that the UE ignores that a primary CPICH is forbidden to affect the reporting range when (a) the primary CPICH concerned is included in active set and (b) all cells in the active set are defined as primary CPICHs forbidden to affect the reporting range.

8.4.1.14.4	Method of test

Initial Condition

System Simulator: 3 cells – Cell 1, cell 2 and cell 3 are active.

UE: CS-DCCH+DTCH _DCH (State 6-9) or PS-DCCH+DTCH_DCH (State 6-10) in cell 1 as specified in clause 7.4 of TS 34.108, depending on the CN domain supported by the UE.

Test Procedure

Table 8.4.1.14-1 illustrates the downlink power to be applied for the 3 cells.

- Table 8.4.1.14-1

| Para-meter |  | Unit |  |  | Cell 1 |  | Cell 2 |  |  | Cell 3 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UTRA RF Channel Number |  |  |  |  | Ch. 1 |  | Ch. 1 |  |  | Ch. 1 |  |  |  |  |  |  |
| CPICH Ec |  | dBm/3.84 MHz |  |  | -60 |  | -75 |  |  | -70 |  |  |  |  |  |  |

The UE is initially in CELL_DCH state of cell 1.

SS sends a MEASUREMENT CONTROL message with cell 1, cell 2 and cell 3 listed in IE "intra-frequency cell info list". In this message the IE "CHOICE reporting criteria" is set to "intra-frequency measurement report criteria", with the IE "intra-frequency event identity" set to "1A" and “1B”. The IE "reporting range" is set to 12 dB for both events in the MEASUREMENT CONTROL message. The UE shall send a MEASUREMENT REPORT on the uplink DCCH, which contains the IE "Event results" to report that intra-frequency event 1A is triggered by cell 3.

SS executes the active set update procedure, requesting that cell 3 be added to the active set. The UE shall respond with ACTIVE SET UPDATE COMPLETE message on the uplink DCCH and then include cell 3 into its current active set. The UE shall transmit a MEASUREMENT REPORT message on the uplink DCCH to report the triggering of intra-frequency event 1A. In these messages, the IE "Events results" shall indicate that intra-frequency event 1A and 1B is triggered by cell 2. Upon reception of MEASUREMENT REPORT message,

SS sends a MEASUREMENT CONTROL message to command that cell 3 in the active set is forbidden to update the reporting range for event 1A and 1B. The UE shall transmit a MEASUREMENT REPORT message on the uplink DCCH to report the triggering of intra-frequency event 1B. In these message, the IE "Events results" shall indicate that intra-frequency event 1B is triggered by cell 2. SS sends a MEASUREMENT CONTROL message to command that cell 1 in the active set is forbidden to update the reporting range for event 1A and 1B. The UE shall transmit a MEASUREMENT REPORT message on the uplink DCCH to report the triggering of intra-frequency event 1A. In these message, the IE "Events results" shall indicate that intra-frequency event 1A is triggered by cell 2. SS sends a MEASUREMENT CONTROL message to command that all cells in the active set are forbidden to update the reporting range for event 1A and 1B. The UE shall transmit a MEASUREMENT REPORT message on the uplink DCCH to report the triggering of intra-frequency event 1A. In these messages, the IE "Events results" shall indicate that intra-frequency event 1A and 1B is triggered by cell 2.

Expected sequence

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 |  |  | MEASUREMENT CONTROL | UE is initially in CELL_DCH state in cell 1. Cell 1, cell 2 and cell 3 are listed in IE "Intra-frequency cell info list". The IE "CHOICE reporting criteria" is set to "Intra-frequency measurement reporting criteria" and IE "Intra-frequency event identity" is set to "1A" and “1B”, with IE "reporting range" set to 12 dB for both events. |
| 2 |  |  | MEASUREMENT REPORT |  |
| 3 |  |  | ACTIVE SET UPDATE | SS requests UE to add cell 3 into active set. |
| 4 |  |  | ACTIVE SET UPDATE COMPLETE |  |
| 5 |  |  | Void |  |
| 6 |  |  | MEASUREMENT REPORT | Measurement made on cell 2 should trigger event 1A and 1B. |
| 7 |  |  | MEASUREMENT CONTROL | SS request UE to monitor cell 2 for event ‘1A’ and ‘1B’. SS set cell 3 to be forbidden to affect reporting range. |
| 8 |  |  | MEASUREMENT REPORT | Measurement made on cell 2 should trigger event 1B. |
| 9 |  |  | MEASUREMENT CONTROL | SS request UE to monitor cell 2 for event ‘1A’ and ‘1B’. SS set cell 1 to be forbidden to affect reporting range. |
| 10 |  |  | MEASUREMENT REPORT | Measurement made on cell 2 should trigger event 1A. |
| 11 |  |  | MEASUREMENT CONTROL | SS request UE to monitor cell 2 for event ‘1A’ and ‘1B’. SS forbids all cells in active list to affect the reporting range |
| 12 |  |  | MEASUREMENT REPORT | Same as step 3. |

# Specific Message Contents

MEASUREMENT CONTROL (Step 1)

The contents of MEASUREMENT CONTROL message for this test step is identical to the same message found in [9] TS 34.108 clause 9 with the following exceptions:

| Information Element | Value/Remark |
| --- | --- |
| RRC transaction identifier Measurement Identity Measurement Command Measurement Reporting Mode - Measurement Reporting Transfer Mode - Periodic Reporting / Event Trigger Reporting Mode Additional measurements list CHOICE measurement type - Intra-frequency cell info list - CHOICE intra-frequency cell removal - New intra-frequency info list - Intra-frequency cell id - Cell info - Cell individual offset - Reference time difference to cell - Read SFN Indicator - CHOICE Mode - Primary CPICH Info - Primary Scrambling Code - Primary CPICH TX power - TX Diversity Indicator - Intra-frequency cell id - Cell info - Cell individual offset - Reference time difference to cell - Read SFN Indicator - CHOICE Mode - Primary CPICH Info - Primary Scrambling Code - Primary CPICH TX power - TX Diversity Indicator - Intra-frequency cell id - Cell info - Cell individual offset - Reference time difference to cell - Read SFN Indicator - CHOICE Mode - Primary CPICH Info - Primary Scrambling Code - Primary CPICH TX power - TX Diversity Indicator - Cell for measurement - Intra-frequency cell id - Intra-frequency measurement quantity - Filter Coefficient - Measurement quantity - Intra-frequency reporting quantity - Reporting quantities for active set cells - SFN-SFN observed time difference reporting indicator - Cell synchronisation information reporting indicator - Cell identity reporting indicator - CPICH Ec/No reporting indicator - CPICH RSCP reporting indicator - Pathloss reporting indicator - Reporting quantities for monitored set cells - SFN-SFN observed time difference reporting indicator - Cell synchronisation information reporting indicator - Cell identity reporting indicator - CPICH Ec/No reporting indicator - CPICH RSCP reporting indicator - Pathloss reporting indicator - Reporting quantities for detected cells - Reporting cell status - Measurement validity - CHOICE report criteria - Parameters required for each events - Intra-frequency event identity - Triggering conditions 1 - Triggering conditions 2 - Reporting range - Cells forbidden to affect reporting range - W - Hysteresis - Threshold used frequency - Reporting deactivation threshold - Replacement activation threshold - Time to trigger - Amount of reporting - Reporting interval - Reporting cell status - CHOICE reported cells - Maximum number of reported cells - Intra-frequency event identity - Triggering conditions 1 - Triggering conditions 2 - Reporting range - Cells forbidden to affect reporting range - W - Hysteresis - Threshold used frequency - Reporting deactivation threshold - Replacement activation threshold - Time to trigger - Amount of reporting - Reporting interval - Reporting cell status - CHOICE reported cells - Maximum number of reported cells DPCH compressed mode status info | 1 1 Setup Acknowledged Mode RLC Event Trigger Not Present Intra-frequency measurement Remove no intra-frequency 1 0 dB 0 chips FALSE FDD Set to same code as used for cell 1 Not Present FALSE 2 0 dB 0 chips FALSE FDD Set to same code as used for cell 2 Not Present FALSE 3 0 dB 0 chips FALSE FDD Set to same code as used for cell 3 Not Present FALSE 1, 2 and 3 0 CPICH RSCP No report FALSE FALSE FALSE FALSE FALSE No report FALSE FALSE FALSE TRUE FALSE Not present Not present Not present Intra-frequency measurement reporting criteria 1a Not Present monitored set cells 12.0 dB Not Present 0 0 dB Not Present 3 Not Present 0 msec Infinity 4000 Report cells within monitored set on used frequency 3 1b monitored set cells Not Present 12.0 dB Not Present 0 0 dB Not Present 3 Not Present 0 msec Infinity 4000 Report cells within monitored set on used frequency 3 Not Present |

MEASUREMENT REPORT (Step 2)

| Information Element | Value/Remarks |
| --- | --- |
| RRC transaction identifier Measurement identity Measured Results - CHOICE measurement - Intra-frequency measurement results - Cell measured results - Cell Identity - SFN-SFN observed time difference - Cell synchronisation information - Primary CPICH Info - Primary Scrambling Code - CPICH Ec/No - CPICH RSCP - Pathloss - Cell measured results - Cell Identity - SFN-SFN observed time difference - Cell synchronisation information - Primary CPICH Info - Primary Scrambling Code - CPICH Ec/No - CPICH RSCP - Pathloss Measured Results on RACH Additional Measured Results Event Results - Intra-frequency event identity - Cell measurement event results - CHOICE Mode - Primary CPICH info - Primary Scrambling Code - Intra-frequency event identity - Cell measurement event results - CHOICE Mode - Primary CPICH info - Primary Scrambling Code | Check to see if set to 1 Check to see if set to 1 Check to see if set to "Intra-frequency measured results list" Check to see if it is absent Check to see if this IE is absent Check to see if this IE is absent Check to see if it's the same code for cell 3 Check to see if this IE is absent Check to see if this IE is present Check to see if this IE is absent Check to see if it is absent Check to see if this IE is absent Check to see if this IE is absent Check to see if it's the same code for cell 2 Check to see if this IE is absent Check to see if this IE is present Check to see if this IE is absent Check to see if this IE is absent Check to see if this IE is absent Check to see if set to 'Intra-frequency measurement event results' Check to see if set to '1a' Check to see if set to 'FDD' Check to see if set to the same code for cell 3 Check to see if set to '1b' Check to see if set to 'FDD' Check to see if set to the same code for cell 2 |

ACTIVE SET UPDATE (Step 3)

The contents of ACTIVE SET UPDATE message for this test step is identical to the same message found in Annex A with the following exceptions:

| Information Element | Value/remark |
| --- | --- |
| RRC transaction identifier Radio link addition information - Primary CPICH Info - Primary Scrambling Code - Downlink DPCH info for each RL - CHOICE mode - Primary CPICH usage for channel estimation - DPCH frame offset - Secondary CPICH info - DL channelisation code - Secondary scrambling code - Spreading factor - Code Number - Scrambling code change - TPC Combination Index - SSDT Cell Identity - Close loop timing adjustment mode - TFCI Combining Indicator - SCCPCH information for FACH Radio link removal information | 0 Set to same code as assigned for cell 3 FDD P-CPICH can be used. 0 chips Not Present This IE is repeated for all existing downlink DPCHs allocated to the UE Not Present 512 For each DPCH, assign the same code number in the current code given in cell 1. Not Present 0 Not Present Not Present Not Present Not Present Not Present |

ACTIVE SET UPDATE COMPLETE (Step 4)

| Information Element | Value/remark |
| --- | --- |
| RRC transaction identifier | Check to see if it is set to 0 |

MEASUREMENT REPORT (Step 6 and 12)

| Information Element | Value/Remarks |
| --- | --- |
| RRC transaction identifier Measurement identity Measured Results - CHOICE measurement - Intra-frequency measurement results - Cell measured results - Cell Identity - SFN-SFN observed time difference - Cell synchronisation information - Primary CPICH Info - Primary Scrambling Code - CPICH Ec/No - CPICH RSCP - Pathloss Measured Results on RACH Additional Measured Results Event Results - Intra-frequency event identity - Cell measurement event results - CHOICE Mode - Primary CPICH info - Primary Scrambling Code - Intra-frequency event identity - Cell measurement event results - CHOICE Mode - Primary CPICH info - Primary Scrambling Code | Check to see if set to 1 Check to see if set to 1 Check to see if set to "Intra-frequency measured results list" Check to see if it is absent Check to see if this IE is absent Check to see if this IE is absent Check to see if it's the same code for cell 2 Check to see if this IE is absent Check to see if this IE is present Check to see if this IE is absent Check to see if this IE is absent Check to see if this IE is absent Check to see if set to 'Intra-frequency measurement event results' Check to see if set to '1a' Check to see if set to 'FDD' Check to see if set to the same code for cell 2 Check to see if set to '1b' Check to see if set to 'FDD' Check to see if set to the same code for cell 2 |

MEASUREMENT CONTROL (Step 7)

The contents of MEASUREMENT CONTROL message for this test step is identical to the same message found in [9] TS 34.108 clause 9 with the following exceptions:

| Information Element | Value/Remark |
| --- | --- |
| RRC transaction identifier Measurement Identity Measurement Command Measurement Reporting Mode - Measurement Reporting Transfer Mode - Periodic Reporting / Event Trigger Reporting Mode Additional measurements list CHOICE measurement type - Intra-frequency cell info list - Intra-frequency measurement quantity - Intra-frequency reporting quantity - CHOICE report criteria - Parameters required for each events - Intra-frequency event identity - Triggering conditions 1 - Triggering conditions 2 - Reporting range - Cells forbidden to affect reporting range - CHOICE Mode - Primary CPICH info - Primary scrambling code - W - Hysteresis - Threshold used frequency - Reporting deactivation threshold - Replacement activation threshold - Time to trigger - Amount of reporting - Reporting interval - Reporting cell status - CHOICE reported cells - Maximum number of reported cells - Intra-frequency event identity - Triggering conditions 1 - Triggering conditions 2 - Reporting range - Cells forbidden to affect reporting range - CHOICE Mode - Primary CPICH info - Primary scrambling code - W - Hysteresis - Threshold used frequency - Reporting deactivation threshold - Replacement activation threshold - Time to trigger - Amount of reporting - Reporting interval - Reporting cell status - CHOICE reported cells - Maximum number of reported cells DPCH compressed mode status info | 1 1 Modify Acknowledged Mode RLC Event Trigger Not Present Intra-frequency measurement Not Present Not Present Not Present Intra-frequency measurement reporting criteria 1a Not Present monitored set cells 12.0 dB FDD Set to the same code as in cell 3 0 0 dB Not Present 3 Not Present 0 msec Infinity 4000 Report cells within monitored set on used frequency 3 1b monitored set cells Not Present 12.0 dB FDD Set to the same code as in cell 3 0 0 dB Not Present 3 Not Present 0 msec Infinity 4000 Report cells within monitored set on used frequency 3 Not Present |

MEASUREMENT REPORT (Step 8)

| Information Element | Value/Remarks |
| --- | --- |
| RRC transaction identifier Measurement identity Measured Results - CHOICE measurement - Intra-frequency measurement results - Cell measured results - Cell Identity - SFN-SFN observed time difference - Cell synchronisation information - Primary CPICH Info - Primary Scrambling Code - CPICH Ec/No - CPICH RSCP - Pathloss Measured Results on RACH Additional Measured Results Event Results - Intra-frequency event identity - Cell measurement event results - CHOICE Mode - Primary CPICH info - Primary Scrambling Code | Check to see if set to 1 Check to see if set to 1 Check to see if set to "Intra-frequency measured results list" Check to see if it is absent Check to see if this IE is absent Check to see if this IE is absent Check to see if it's the same code for cell 2 Check to see if this IE is absent Check to see if this IE is present Check to see if this IE is absent Check to see if this IE is absent Check to see if this IE is absent Check to see if set to 'Intra-frequency measurement event results' Check to see if set to '1b' Check to see if set to 'FDD' Check to see if set to the same code for cell 2 |

MEASUREMENT CONTROL (Step 9)

The contents of MEASUREMENT CONTROL message for this test step is identical to the same message found in [9] TS 34.108 clause 9 with the following exceptions:

| Information Element | Value/Remark |
| --- | --- |
| RRC transaction identifier Measurement Identity Measurement Command Measurement Reporting Mode - Measurement Reporting Transfer Mode - Periodic Reporting / Event Trigger Reporting Mode Additional measurements list CHOICE measurement type - Intra-frequency cell info list - Intra-frequency measurement quantity - Intra-frequency reporting quantity - CHOICE report criteria - Parameters required for each events - Intra-frequency event identity - Triggering conditions 1 - Triggering conditions 2 - Reporting range - Cells forbidden to affect reporting range - CHOICE Mode - Primary CPICH info - Primary scrambling code - W - Hysteresis - Threshold used frequency - Reporting deactivation threshold - Replacement activation threshold - Time to trigger - Amount of reporting - Reporting interval - Reporting cell status - CHOICE reported cells - Maximum number of reported cells - Intra-frequency event identity - Triggering conditions 1 - Triggering conditions 2 - Reporting range - Cells forbidden to affect reporting range - CHOICE Mode - Primary CPICH info - Primary scrambling code - W - Hysteresis - Threshold used frequency - Reporting deactivation threshold - Replacement activation threshold - Time to trigger - Amount of reporting - Reporting interval - Reporting cell status - CHOICE reported cells - Maximum number of reported cells DPCH compressed mode status info | 1 1 Modify Acknowledged Mode RLC Event Trigger Not Present Intra-frequency measurement Not Present Not Present Not Present Intra-frequency measurement reporting criteria 1a Not Present monitored set cells 12.0 dB FDD Set to the same code as in cell 1 0 0 dB Not Present 3 Not Present 0 msec Infinity 4000 Report cells within monitored set on used frequency 3 1b monitored set cells Not Present 12.0 dB FDD Set to the same code as in cell 1 0 0 dB Not Present 3 Not Present 0 msec Infinity 4000 Report cells within monitored set on used frequency 3 Not Present |

MEASUREMENT REPORT (Step 10)

| Information Element | Value/Remarks |
| --- | --- |
| RRC transaction identifier Measurement identity Measured Results - CHOICE measurement - Intra-frequency measurement results - Cell measured results - Cell Identity - SFN-SFN observed time difference - Cell synchronisation information - Primary CPICH Info - Primary Scrambling Code - CPICH Ec/No - CPICH RSCP - Pathloss Measured Results on RACH Additional Measured Results Event Results - Intra-frequency event identity - Cell measurement event results - CHOICE Mode - Primary CPICH info - Primary Scrambling Code | Check to see if set to 1 Check to see if set to 1 Check to see if set to "Intra-frequency measured results list" Check to see if it is absent Check to see if this IE is absent Check to see if this IE is absent Check to see if it's the same code for cell 2 Check to see if this IE is absent Check to see if this IE is present Check to see if this IE is absent Check to see if this IE is absent Check to see if this IE is absent Check to see if set to 'Intra-frequency measurement event results' Check to see if set to '1a' Check to see if set to 'FDD' Check to see if set to the same code for cell 2 |

with the following exceptions:

MEASUREMENT CONTROL (Step 11)

The contents of MEASUREMENT CONTROL message for this test step is identical to the same message found in Annex A with the following exceptions:

| Information Element | Value/Remark |
| --- | --- |
| RRC transaction identifier Measurement Identity Measurement Command Measurement Reporting Mode - Measurement Reporting Transfer Mode - Periodic Reporting / Event Trigger Reporting Mode Additional measurements list CHOICE measurement type - Intra-frequency cell info list - Intra-frequency measurement quantity - Intra-frequency reporting quantity - CHOICE report criteria - Parameters required for each events - Intra-frequency event identity - Triggering conditions 1 - Triggering conditions 2 - Reporting range - Cells forbidden to affect reporting range - CHOICE Mode - Primary CPICH info - Primary scrambling code - CHOICE Mode - Primary CPICH info - Primary scrambling code - CHOICE Mode - Primary CPICH info - Primary scrambling code - W - Hysteresis - Threshold used frequency - Reporting deactivation threshold - Replacement activation threshold - Time to trigger - Amount of reporting - Reporting interval - Reporting cell status - CHOICE reported cells - Maximum number of reported cells - Intra-frequency event identity - Triggering conditions 1 - Triggering conditions 2 - Reporting range - Cells forbidden to affect reporting range - CHOICE Mode - Primary CPICH info - Primary scrambling code - CHOICE Mode - Primary CPICH info - Primary scrambling code - CHOICE Mode - Primary CPICH info - Primary scrambling code - W - Hysteresis - Threshold used frequency - Reporting deactivation threshold - Replacement activation threshold - Time to trigger - Amount of reporting - Reporting interval - Reporting cell status - CHOICE reported cells - Maximum number of reported cells DPCH compressed mode status info | 1 1 Modify Acknowledged Mode RLC Event Trigger Not Present Intra-frequency measurement Not Present Not Present Not Present Intra-frequency measurement reporting criteria Only 2 events are specified 1a Not Present Monitored set cells 12 dB FDD Set to the same code as in cell 1 FDD Set to the same code as in cell 2 FDD Set to the same code as in cell 3 0 0 dB Not Present 3 Not Present 0 msec 1 0 Report cells within monitored set 3 1b Monitored set cells Not Present 12 dB FDD Set to the same code as in cell 1 FDD Set to the same code as in cell 2 FDD Set to the same code as in cell 3 0 0 dB Not Present 3 Not Present 0 msec 1 0 Report cells within monitored set 3 Not Present |

MEASUREMENT REPORT (Step 15)

8.4.1.14.5	Test requirement

After step 1, the UE shall send a MEASUREMENT REPORT message on the uplink DCCH. The message shall contain the IE "Event results" to report that cell 3 has triggered intra-frequency event 1A and cell 2 has triggered intra-frequency event 1B.

After step 3, the UE shall send a ACTIVE SET UPDATE COMPLETE message on the uplink DCCH..

After step 4, the UE shall transmit MEASUREMENT REPORT message on the uplink DCCH.  The message shall contain IE "Event results" to report that cell 2 has triggered intra-frequency event 1A and 1B.

After step 7, the UE shall transmit MEASUREMENT REPORT message on the uplink DCCH.  The message shall contain IE "Event results" to report that cell 2 has triggered intra-frequency event 1B.

After step 9, the UE shall transmit MEASUREMENT REPORT message on the uplink DCCH.  The message shall contain IE "Event results" to report that cell 2 has triggered intra-frequency event 1A.

After step 11, the UE shall send a MEASUREMENT REPORT message on the uplink DCCH. The message shall contain IE "Event results" to report that cell 2 has triggered intra-frequency event 1A and 1B.

#### 8.4.1.16	Measurement Control and Report: Traffic volume measurement for transition from idle mode to CELL_FACH state

8.4.1.16.1	Definition

8.4.1.16.2 	Conformance requirement

Upon transition from idle mode to CELL_FACH state, the UE shall:

1>	store the measurement control information from the IE "Traffic volume measurement system information" received in System Information Block type 12 (or System Information Block type 11, according to subclause 8.1.1.6.11) in the variable MEASUREMENT_IDENTITY;

1>	begin traffic volume measurement reporting according to the assigned information.

Upon reception of a MEASUREMENT CONTROL message the UE shall perform actions specified in TS 25.331 subclause 8.6 unless otherwise specified below.

The UE shall:

1>	read the IE "Measurement command";

1>	if the IE "Measurement command" has the value "setup":

2>	store this measurement in the variable MEASUREMENT_IDENTITY according to the IE "measurement identity", first releasing any previously stored measurement with that identity if that exists;

2>	for measurement types "inter-RAT measurement" or "inter-frequency measurement":

…

2>	for measurement type "UE positioning measurement":

…

2>	for any other measurement type:

3>	if the measurement is valid in the current RRC state of the UE:

4>	begin measurements according to the stored control information for this measurement identity.

Reference

3GPP TS 25.331 clause 8.4.1.9.4, 3GPP TS 25.331 clause 8.4.1.3

8.4.1.16.3	Test Purpose

- To confirm that after a state transition from idle mode to CELL_FACH state, the UE shall begin a traffic volume type measurement, as specified in System Information Block type 11 or 12 messages on BCCH.

- To confirm that in CELL_FACH state, the UE shall send a MEASUREMENT REPORT message when reporting criteria is satisfied.  During CELL_FACH state, if the UE receives a MEASUREMENT CONTROL message, it shall perform the measurement and reporting tasks based on the MEASUREMENT CONTROL message received.

8.4.1.16.4	Method of test

Initial Condition

System Simulator: 1cell

UE: "Registered idle mode on PS" (state 3) in cell 1 as specified in clause 7.4 of TS 34.108.  If the UE supports both CS and PS domains, the initial UE state shall be "Registered idle mode on CS/PS" (state 7).

Test Procedure

The UE is initially in idle mode. The System Information Block type 11 message is modified with respect to the default settings to request UE to perform traffic volume measurements. Key measurement parameters are as follows: measurement quantity = "RLC Buffer Payload", report criteria = "periodic reporting criteria", reporting interval = "6 seconds", reporting amount = 'infinity'.  The System Information type 12 message is not broadcasted.

SS prompts the operator to make an outgoing call for one of the traffic classes supported by the UE. SS and UE shall execute procedure P6. Next SS and UE shall execute procedure P10. Then SS and UE shall execute procedure P14. SS

UE shall begin traffic volume measurements, and shall send MEASUREMENT REPORT message after completing first measurement. UE shall send second MEASUREMENT REPORT message 6 seconds after first MEASUREMENT REPORT message.

SS sends MEASUREMENT CONTROL message to the UE. This message overwrites measurement information saved from System information type 11. Key measurement parameters are as follow: measurement type = "traffic volume measurement", measurement quantity = "RLC Buffer Payload", report criteria = "Event triggered, event 4B: Transport Channel Traffic Volume becomes smaller than an absolute threshold ", Time to trigger = "5 seconds", pending time after trigger = "16 seconds", "reporting threshold = '4K'. Since there is no uplink traffic, UE shall send MEASUREMENT REPORT message after 5 seconds (time to trigger interval). SS calls for generic procedure C.2 to check that UE is in CELL_FACH state.

Expected Sequence

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 |  |  | System Information Block type 11 | The UE is idle mode and camped onto cell 1. System Information Block type 11 to be transmitted is different from the default settings (see specific message contents) |
| 2 |  |  | SS executes procedure P6 (clause 7.4.2.2.2) specified in TS 34.108. | SS prompts the test operator to make an outgoing call. |
| 3 |  |  | SS executes procedure P10 (clause 7.4.2.4.2) specified in TS 34.108. |  |
| 4 |  |  | SS executes procedure P14 (clause 7.4.2.6.2) specified in TS 34.108. |  |
| 5 |  |  | Void |  |
| 6 |  |  | MEASUREMENT REPORT |  |
| 7 |  |  | MEASUREMENT REPORT | Time difference between earlier and this MEASUREMENT REPORT message should be 6 Seconds. |
| 8 |  |  | MEASUREMENT CONTROL | Traffic volume measurement reporting is requested if measurement is below threshold. |
| 9 |  |  |  | SS monitors the uplink DCCH to confirm that no MEASUREMENT REPORT messages are received in 5 seconds. |
| 10 |  |  | MEASUREMENT REPORT | Measurement report because event 4b is triggered |
| 11 |  |  | CALL C.2 | If the test result of C.2 indicates that UE is in CELL_FACH state, the test passes, otherwise it fails. |

Specific Message Content

System Information Block type 11 (Step 1)

| Information Element | Value/Remarks |
| --- | --- |
| SIB12 indicator FACH measurement occasion info Measurement control system information - Use of HCS - Cell selection and reselection quality measure - Intra-frequency measurement system information - Inter-frequency measurement system information - Inter-RAT measurement system information - Traffic volume measurement system information - Traffic volume measurement ID - Traffic volume measurement object list - Traffic volume measurement quantity - Traffic volume reporting quantity - RB buffer payload - RB buffer payload average - RB buffer payload variance - Traffic volume measurement reporting criteria - Measurement validity - Measurement reporting mode - Measurement report transfer mode - Periodical or event trigger - Report criteria system Information - Reporting amount - Reporting interval - UE internal measurement system information | FALSE Not Present Not used CPICH RSCP Not Present Not Present Not Present 4 Rach RLC Buffer Payload True False False Not Present All States except CELL_DCH Acknowledged Mode Periodical Periodical reporting criteria Infinity 6 seconds Not Present |

RRC CONNECTION REQUEST (Step 3)

MEASUREMENT REPORT (Step 6,7)

| Information Element | Value/Remarks |
| --- | --- |
| Measurement identity Measured Results - CHOICE measurement - Traffic volume measurement results - RB identity - RLC buffer payload - RLC buffer payload average - RLC buffer payload variance - RB identity - RLC buffer payload - RLC buffer payload average - RLC buffer payload variance - RB identity - RLC buffer payload - RLC buffer payload average - RLC buffer payload variance - RB identity - RLC buffer payload - RLC buffer payload average - RLC buffer payload variance Measured results on RACH Additional measured results Event results | Check to see if set to 4 Check to see if set to "traffic volume measured results list" 1 Check to see if this IE is present Check to see if this IE is absent Check to see if this IE is absent 2 Check to see if this IE is present Check to see if this IE is absent Check to see if this IE is absent 3 Check to see if this IE is present Check to see if this IE is absent Check to see if this IE is absent 4 Check to see if this IE is present Check to see if this IE is absent Check to see if this IE is absent Check to see if this IE is absent Check to see if this IE is absent Check to see if this IE is absent |

MEASUREMENT CONTROL (Step 8)

| Information Element | Value/Remark |
| --- | --- |
| Measurement Identity Measurement Command - CHOICE measurement type - Traffic volume measurement object list - Traffic volume measurement quantity - Traffic volume reporting quantity - RB buffer payload - RB buffer payload average - RB buffer payload variance - Measurement validity - Report criteria - UL transport channel id - Event specific parameters - Event id - Reporting threshold - Time to trigger - Pending time after trigger - Tx interruption after trigger Measurement reporting mode - Transfer Mode - Periodical or event trigger Additional measurement list DPCH compressed mode status | 4 Setup Traffic Volume Measurement Not Present RLC Buffer Payload True False False Not Present Traffic Volume Reporting Criteria Rach Null 4B 4K 5000 ms 16000 ms Not Present Acknowledged mode Event trigger Not Present Not Present |

MEASUREMENT REPORT (Step 10)

| Information Element | Value/Remarks |
| --- | --- |
| Measurement identity Measured Results - CHOICE measurement - Traffic volume measurement results - RB identity - RLC buffer payload - RLC buffer payload average - RLC buffer payload variance - RB identity - RLC buffer payload - RLC buffer payload average - RLC buffer payload variance - RB identity - RLC buffer payload - RLC buffer payload average - RLC buffer payload variance - RB identity - RLC buffer payload - RLC buffer payload average - RLC buffer payload variance Measured results on RACH Additional measured results Event results - UL transport channel causing event - Traffic volume event identity | 4 Traffic volume measured results list 1 Check to see if this IE is present Check to see if this IE is absent Check to see if this IE is absent 2 Check to see if this IE is present Check to see if this IE is absent Check to see if this IE is absent 3 Check to see if this IE is present Check to see if this IE is absent Check to see if this IE is absent 4 Check to see if this IE is present Check to see if this IE is absent Check to see if this IE is absent Check to see if this IE is absent Check to see if this IE is absent Traffic Volume Event Results Rach Null 4B |

8.4.1.16.5	Test Requirement

After step 5 the UE shall send MEASUREMENT REPORT messages on the uplink DCCH containg RLC buffer payload information for all SRBs. After 6 seconds UE shall send second MEASUREMENT REPORT messages containg RLC buffer payload information for all SRBs.

After step 8 the UE shall overwrite measurement information received from system information type 11 with measurement information in MEASUREMENT CONTROL message. The UE shall not send MEASUREMENT REPORT message within time to trigger interval. After step 9 the UE shall transmit MEASUREMENT REPORT messages with event identity 4B.

#### 8.4.1.17	Measurement Control and Report: Traffic volume measurement for transition from idle mode to CELL_DCH state

8.4.1.17.1	Definition

8.4.1.17.2 	Conformance requirement

Upon transition from idle mode to CELL_DCH state, the UE shall:

1>	begin a traffic volume type measurement, assigned in System Information Block type 11 (or System Information Block type 12.

Upon reception of a MEASUREMENT CONTROL message the UE shall perform actions specified in TS 25.331 subclause 8.6 unless otherwise specified below.

The UE shall:

1>	read the IE "Measurement command";

1>	if the IE "Measurement command" has the value "setup":

2>	store this measurement in the variable MEASUREMENT_IDENTITY according to the IE "measurement identity", first releasing any previously stored measurement with that identity if that exists;

2>	for measurement types "inter-RAT measurement" or "inter-frequency measurement":

…

2>	for measurement type "UE positioning measurement":

…

2>	for any other measurement type:

3>	if the measurement is valid in the current RRC state of the UE:

4>	begin measurements according to the stored control information for this measurement identity.

Reference

3GPP TS 25.331 clause 8.4.1.8.4, 3GPP TS 25.331 clause 8.4.1.3

8.4.1.17.3	Test Purpose

- To confirm that after a state transition from idle mode to CELL_DCH state, the UE begin a traffic volume type measurement, as specified in System Information Block type 11 or 12 messages on BCCH.  When entering CELL_DCH state, the UE shall send a MEASUREMENT REPORT message when reporting criteria is satisfied.

- During CELL_DCH state, if the UE receives a MEASUREMENT CONTROL message, it shall perform the measurement and reporting tasks based on the MEASUREMENT CONTROL message received.

8.4.1.17.4	Method of test

Initial Condition

System Simulator: 1cell

UE: "Registered idle mode on CS" (state 2) or "Registered idle mode on PS" (state 3) in cell 1 as specified in clause 7.4 of TS 34.108, depending on the CN domain supported by the UE.  If the UE supports both CS and PS domains, the initial UE state shall be "Registered idle mode on CS/PS" (state 7).

Test Procedure

The UE is initially in idle mode. The System Information Block type 11 message is modified with respect to the default settings to request UE to perform traffic volume measurements. Key measurement parameters are as follows: measurement quantity = "Average RLC Buffer Payload", report criteria = "Event triggered, event 4B", reporting threshold = "8K", report transfer mode = "Unacknowledged mode". The System Information type 12 message is not broadcasted.

SS prompts the operator to make an outgoing call of a supported traffic class. SS and UE shall execute procedure P3 (for CS service) or P5 (for PS service). Next SS and UE shall execute procedure P7 (for CS service) or P9 (for PS service). Then SS and UE shall execute procedure P11 (for CS service) or P13 (for PS service).

UE shall begin traffic volume measurements after entering in CELL_DCH state. The UE shall send MEASUREMENT REPORT message because uplink traffic is below threshold.

SS sends MEASUREMENT CONTROL message to the UE. This message reconfigures measurement information saved from System information type 11. Key measurement parameters are as follow: measurement type = "traffic volume measurement", measurement quantity = "RLC Buffer Payload", report criteria = "Periodic reporting criteria", reporting interval = "8 seconds", reporting amount = "8". The UE shall periodically send MEASUREMENT REPORT message to report RLC Buffer Payload for each RB.

SS sends MEASUREMENT CONTROL message to release traffic volume measurement. UE shall not send measurement report after receiving this message. SS calls for generic procedure C.3 to check that UE is in CELL_DCH state.

Expected Sequence

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 |  |  | System Information Block type 11 | The UE is idle mode and camped onto cell 1. System Information Block type 11 to be transmitted is different from the default settings (see specific message contents) |
| 2 |  |  | SS executes procedure P3 (clause 7.4.2.1.2) or P5 (clause 7.4.2.2.2) specified in TS 34.108. |  |
| 3 |  |  | SS executes procedure P7 (clause 7.4.2.3.2) or P9 (clause 7.4.2.4.2) specified in TS 34.108. |  |
| 4 |  |  | SS executes procedure P11 (clause 7.4.2.5.2) or P13 (clause 7.4.2.6.2) specified in TS 34.108. |  |
| 5 |  |  | Void |  |
| 6 |  |  | MEASUREMENT REPORT | Event 4B is triggered. This message should come on RB1. |
| 7 |  |  | MEASUREMENT CONTROL | Periodic Traffic volume measurement reporting is requested. |
| 8 |  |  | MEASUREMENT REPORT | This message should come on RB2. |
| 9 |  |  | MEASUREMENT REPORT | Time difference between earlier and this MEASUREMENT REPORT message should be 8 Seconds. |
| 10 |  |  | MEASUREMENT CONTROL | Release traffic volume measurement. |
| 11 |  |  |  | Wait for 8 Seconds to confirm that UE does not send measurement report message. |
| 12 |  |  | CALL C.3 | If the test result of C.3 indicates that UE is in CELL_DCH state, the test passes, otherwise it fails. |

Specific Message Content

System Information Block type 11 (Step 1)

| Information Element | Value/Remarks |
| --- | --- |
| SIB12 indicator FACH measurement occasion info Measurement control system information - Use of HCS - Cell selection and reselection quality measure - Intra-frequency measurement system information - Inter-frequency measurement system information - Inter-RAT measurement system information - Traffic volume measurement system information - Traffic volume measurement ID - Traffic volume measurement object list - Traffic volume measurement quantity - Time Interval to take an average - Traffic volume reporting quantity - RB buffer payload - RB buffer payload average - RB buffer payload variance - Traffic volume measurement reporting criteria - Measurement validity - Measurement reporting mode - Measurement report transfer mode - Periodical or event trigger - Report criteria system Information - UL transport channel id - Event specific parameters - Event id - Reporting threshold - Time to trigger - Pending time after trigger - Tx interruption after trigger - UE internal measurement system information | FALSE Not Present Not used CPICH RSCP Not Present Not Present Not Present 2 Not Present Average RLC Buffer Payload 200 msec False True False Not Present CELL_DCH Unacknowledged Mode Event Trigger Traffic Volume Reporting Criteria Not Present 4B 8K Not Present Not Present Not Present Not Present |

RRC CONNECTION REQUEST (Step 3)

MEASUREMENT REPORT (Step 6)

| Information Element | Value/Remarks |
| --- | --- |
| Measurement identity Measured Results - CHOICE measurement - Traffic volume measurement results - RB identity - RLC buffer payload - RLC buffer payload average - RLC buffer payload variance - RB identity - RLC buffer payload - RLC buffer payload average - RLC buffer payload variance - RB identity - RLC buffer payload - RLC buffer payload average - RLC buffer payload variance - RB identity - RLC buffer payload - RLC buffer payload average - RLC buffer payload variance Measured results on RACH Additional measured results Event results - UL transport channel causing event - Traffic volume event identity | Check to see if set to 2 Check to see if set to "traffic volume measured results list" 1 Check to see if this IE is absent Check to see if this IE is present Check to see if this IE is absent 2 Check to see if this IE is absent Check to see if this IE is present Check to see if this IE is absent 3 Check to see if this IE is absent Check to see if this IE is present Check to see if this IE is absent 4 Check to see if this IE is absent Check to see if this IE is present Check to see if this IE is absent Check to see if this IE is absent Check to see if this IE is absent DCH 5 4B |

MEASUREMENT CONTROL (Step 7)

| Information Element | Value/Remark |
| --- | --- |
| Measurement Identity Measurement Command Measurement reporting mode - Transfer Mode - Periodical or event trigger Additional measurement list CHOICE measurement type - Traffic volume measurement object list - Uplink transport channel type - UL Target Transport Channel ID - Traffic volume measurement quantity - Measurement quantity - Time Interval to take an average or a variance - Traffic volume reporting quantity - RLC Buffer Payload for each RB - Average of RLC Buffer Payload for each RBe - Variance of RLC Buffer Payload for each RB - Measurement validity - CHOICE Reporting criteria - Amount of reporting - Reporting interval DPCH compressed mode status | 2 Modify Acknowledged mode Periodic Not Present Traffic Volume Measurement DCH 5 RLC Buffer Payload Not Present True False False Not Present Periodical Reporting Criteria 8 8 Sec Not Present |

MEASUREMENT REPORT (Step 8,9)

| Information Element | Value/Remarks |
| --- | --- |
| Measurement identity Measured Results - CHOICE measurement - Traffic volume measurement results - RB identity - RLC buffer payload - RLC buffer payload average - RLC buffer payload variance - RB identity - RLC buffer payload - RLC buffer payload average - RLC buffer payload variance - RB identity - RLC buffer payload - RLC buffer payload average - RLC buffer payload variance - RB identity - RLC buffer payload - RLC buffer payload average - RLC buffer payload variance Measured results on RACH Additional measured results Event results | 2 Traffic volume measured results list 1 Check to see if this IE is present Check to see if this IE is absent Check to see if this IE is absent 2 Check to see if this IE is present Check to see if this IE is absent Check to see if this IE is absent 3 Check to see if this IE is present Check to see if this IE is absent Check to see if this IE is absent 4 Check to see if this IE is present Check to see if this IE is absent Check to see if this IE is absent Check to see if this IE is absent Check to see if this IE is absent Check to see if this IE is absent |

MEASUREMENT CONTROL (Step 10)

| Information Element | Value/Remark |
| --- | --- |
| Measurement Identity Measurement Command Measurement reporting mode Additional measurement list DPCH compressed mode status | 2 Release Not Present Not Present Not Present |

8.4.1.17.5	Test Requirement

After step 5, due to triggering of event 4B, the UE shall send MEASUREMENT REPORT message using unacknowledged mode of RLC.  After step 7, UE shall send MEASUREMENT REPORT message using Acknowledged mode of RLC. After 8 seconds UE shall send second MEASUREMENT REPORT message. After step 10, the UE shall not send MEASUREMENT REPORT message.

#### 8.4.1.18	Measurement Control and Report: Traffic volume measurement for transition from CELL_FACH state to CELL_DCH state

8.4.1.18.1	Definition

8.4.1.18.2	Conformance requirement

Upon transition from CELL_FACH to CELL_DCH state, the UE shall:

-	retrieve each set of measurement control information of measurement type "traffic volume" stored;

-	if the optional IE "measurement validity" for this measurement has not been included:

-	delete the measurement;

-	if the IE "measurement validity" for the measurement has been included, and the IE "UE state" has been assigned to value "all states except CELL_DCH":

-	stop measurement reporting; and

-	save the measurement to be used after the next transition to CELL_FACH state;

-	if the IE "measurement validity" for the measurement has been included, and the IE "UE state" has been assigned to value "all states":

-	continue measurement reporting;

-	if the IE "measurement validity" has been included and the IE "UE state" has been assigned to value "CELL_DCH":

-	resume this measurement and associated reporting;

-	if no traffic volume type measurement has been assigned to the UE with a MEASUREMENT CONTROL message when transiting to CELL_DCH state:

-	continue an ongoing traffic volume type measurement, assigned in System Information Block type 11  or System Information Block type 12.

Reference

3GPP TS 25.331 clause 8.4.1.7.4

8.4.1.18.3	Test Purpose

1.     To confirm that the UE performs traffic volume measurements and the associated reporting when it enters CELL_DCH state from CELL_FACH state, and that such measurement contexts (and optionally, the reporting context) valid for CELL_DCH state have been previously stored.

2.     To confirm that the UE shall continue to perform traffic volume measurement listed in the System Information Block type 11 or 12 messages, if no previously assigned measurements are present. The UE shall transmit MEASUREMENT REPORT messages if reporting conditions stated in System Information Block type 11 or 12 messages have been satisfied.

8.4.1.18.4	Method of test

Initial Condition

System Simulator: 1 cell

UE: PS-DCCH+DTCH_FACH (state 6-11) as specified in clause 7.4 of TS 34.108

Test Procedure

Initially the UE is in CELL_FACH state. MEASUREMENT CONTROL message is sent to the UE to establish traffic volume measurement context with optional IE "measurement validity" is not present. The UE shall perform measurement and reporting as assigned in MEASUREMENT CONTROL message. RADIO BEARER RECONFIGURATION procedure is used to take the UE from CELL_FACH state to CELL_DCH state. While entering CELL_DCH state from CELL_FACH state, the UE shall delete traffic volume measurement contexts if optional IE "measurement validity" is not present. So, in CELL_DCH state UE shall not perform traffic volume measurement and reporting. UE is taken to the CELL_FACH state from CELL_DCH state using RADIO BEARER RECONFIGURATION procedure.  The UE shall not send MEASUREMENT REPORT message as measurement context is already deleted.

Similarly behavior of the UE when moved from CELL_FACH state to CELL_DCH state and assigned traffic volume measurement context is present with IE "measurement validity" is set to "All But CELL_DCH state" or "CELL_DCH state" or "All states" is tested.

When the UE is in CELL_FACH state, System Information is modified to assign traffic volume measurement and reporting to the UE. No previously assigned traffic volume measurement contexts are present in the UE. The UE is taken to CELL_DCH state from CELL_FACH state using RADIO BEARER RECONFIGURATION procedure. In CELL_DCH state the UE shall continue traffic volume measurement and reporting as assigned in System Information. Traffic volume measurement and reporting is released by sending MEASUREMENT CONTROL message.

Expected Sequence

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 |  |  | MEASUREMENT CONTROL | Optional IE "measurement validity" is not included. |
| 2 |  |  | MEASUREMENT REPORT |  |
| 3 |  |  | RADIO BEARER RECONFIGURATION |  |
| 4 |  |  | RADIO BEARER RECONFIGURATION COMPLETE | While entering in CELL_DCH state from CELL_FACH state UE shall delete measurement context setup by MEASUREMENT CONTROL message (Step 1). |
| 5 |  |  |  | SS waits for 8 seconds to confirm that there is no MEASUREMENT REPORT message from UE. |
| 6 |  |  | RADIO BEARER RECONFIGURATION |  |
| 7 |  |  | RADIO BEARER RECONFIGURATION COMPLETE | UE is in CELL_FACH state. |
| 8 |  |  |  | SS waits for 8 seconds to confirm that there is no MEASUREMENT REPORT message from UE. |
| 9 |  |  | MEASUREMENT CONTROL | IE "measurement validity" is set to "All But CELL_DCH". |
| 10 |  |  | MEASUREMENT REPORT | . |
| 11 |  |  | RADIO BEARER RECONFIGURATION |  |
| 12 |  |  | RADIO BEARER RECONFIGURATION COMPLETE | While entering in CELL_DCH state from CELL_FACH state UE shall stop traffic volume measurement setup by MEASUREMENT CONTROL message (Step 9). |
| 13 |  |  |  | SS waits for 8 seconds to confirm that there is no MEASUREMENT REPORT message from UE. |
| 14 |  |  | MEASUREMENT CONTROL | UE shall release measurement context setup by MEASUREMENT CONTROL message (Step 9). |
| 15 |  |  | RADIO BEARER RECONFIGURATION |  |
| 16 |  |  | RADIO BEARER RECONFIGURATION COMPLETE | UE is in CELL_FACH state. |
| 17 |  |  | MEASUREMENT CONTROL | IE "measurement validity" is set to "CELL_DCH". |
| 18 |  |  |  | SS waits for 8 seconds to confirm that there is no MEASUREMENT REPORT message from UE. |
| 19 |  |  | RADIO BEARER RECONFIGURATION |  |
| 20 |  |  | RADIO BEARER RECONFIGURATION COMPLETE | While entering in CELL_DCH state from CELL_FACH state UE shall start traffic volume measurement setup by MEASUREMENT CONTROL message (Step 17). |
| 21 |  |  | MEASUREMENT REPORT |  |
| 22 |  |  | MEASUREMENT CONTROL | UE shall release measurement context setup by MEASUREMENT CONTROL message (Step 17) |
| 23 |  |  | RADIO BEARER RECONFIGURATION |  |
| 24 |  |  | RADIO BEARER RECONFIGURATION COMPLETE | UE is in CELL_FACH state. |
| 25 |  |  | MEASUREMENT CONTROL | IE "measurement validity" is set to "All states". |
| 26 |  |  | MEASUREMENT REPORT |  |
| 27 |  |  | RADIO BEARER RECONFIGURATION |  |
| 28 |  |  | RADIO BEARER RECONFIGURATION COMPLETE | While entering in CELL_DCH state from CELL_FACH state UE shall continue traffic volume measurement setup by MEASUREMENT CONTROL message (Step 25). |
| 29 |  |  | MEASUREMENT REPORT |  |
| 30 |  |  | MEASUREMENT CONTROL | UE shall release measurement context setup by MEASUREMENT CONTROL message (Step 25) |
| 31 |  |  | RADIO BEARER RECONFIGURATION |  |
| 32 |  |  | RADIO BEARER RECONFIGURATION COMPLETE | UE is in CELL_FACH state. |
| 33 |  |  | MIB and SIB11 modified | Traffic volume measurements and reporting is assigned to Ues |
| 33a |  |  | SYSTEM INFORMATION CHANGE INDICATION |  |
| 34 |  |  | MEASUREMENT REPORT |  |
| 35 |  |  | RADIO BEARER RECONFIGURATION |  |
| 36 |  |  | RADIO BEARER RECONFIGURATION COMPLETE | While entering in CELL_DCH state from CELL_FACH state UE shall continue traffic volume measurement assigned in System Information (Step 33). |
| 37 |  |  | MEASUREMENT REPORT |  |
| 38 |  |  | MEASUREMENT CONTROL | UE shall release measurement context assigned in System Information (Step 33). |

Specific Message Content

MEASUREMENT CONTROL (Step 1)

| Information Element | Value/Remark |
| --- | --- |
| Measurement Identity Measurement Command - CHOICE measurement type - Traffic volume measurement object list - Traffic volume measurement quantity - Traffic volume reporting quantity - RB buffer payload - RB buffer payload average - RB buffer payload variance - Measurement validity - Report criteria - Reporting amount - Reporting interval Measurement reporting mode - Transfer Mode - Periodical or event trigger Additional measurement list DPCH compressed mode status | 1 Setup Traffic Volume Measurement Not Present RLC Buffer Payload True False False Not Present Periodical Reporting Criteria 8 8 Sec Acknowledged mode Periodic Not Present Not Present |

MEASUREMENT REPORT (Step 2)

| Information Element | Value/Remarks |
| --- | --- |
| Measurement identity Measured Results - CHOICE measurement - Traffic volume measurement results - RB identity - RLC buffer payload - RLC buffer payload average - RLC buffer payload variance - RB identity - RLC buffer payload - RLC buffer payload average - RLC buffer payload variance - RB identity - RLC buffer payload - RLC buffer payload average - RLC buffer payload variance - RB identity - RLC buffer payload - RLC buffer payload average - RLC buffer payload variance - RB identity - RLC buffer payload - RLC buffer payload average - RLC buffer payload variance Measured results on RACH Additional measured results Event results | 1 Traffic volume measured results list 1 Check to see if this IE is present Check to see if this IE is absent Check to see if this IE is absent 2 Check to see if this IE is present Check to see if this IE is absent Check to see if this IE is absent 3 Check to see if this IE is present Check to see if this IE is absent Check to see if this IE is absent 4 Check to see if this IE is present Check to see if this IE is absent Check to see if this IE is absent 20 Check to see if this IE is present Check to see if this IE is absent Check to see if this IE is absent Check to see if this IE is absent Check to see if this IE is absent Check to see if this IE is absent |

RADIO BEARER RECONFIGURATION (Step 3, 11, 19, 27, and 35)

Use the same message type found in Annex A with condition set to A4.

RADIO BEARER RECONFIGURATION (Step 6, 15, 23, and 31)

Use the same message type found in Annex A with condition set to A5.

MEASUREMENT CONTROL (Step 9)

The contents of this message are identical to MEASUREMENT CONTROL (Step 1) message with the following exceptions:

| Information Element | Value/Remark |
| --- | --- |
| Measurement Identity Measurement Command - CHOICE measurement type - Measurement validity | 2 Setup Traffic Volume Measurement All But CELL_DCH |

MEASUREMENT REPORT (Step 10)

The contents of this message are identical to MEASUREMENT REPORT (Step 2) message with the following exceptions:

| Information Element | Value/Remarks |
| --- | --- |
| Measurement identity | 2 |

MEASUREMENT CONTROL (Step 14)

| Information Element | Value/Remark |
| --- | --- |
| Measurement Identity Measurement Command Measurement reporting mode Additional measurement list DPCH compressed mode status | 2 Release Not Present Not Present Not Present |

MEASUREMENT CONTROL (Step 17)

The contents of this message are identical to MEASUREMENT CONTROL (Step 1) message with the following exceptions:

| Information Element | Value/Remark |
| --- | --- |
| Measurement Identity Measurement Command - CHOICE measurement type - Measurement validity | 3 Setup Traffic Volume Measurement CELL_DCH |

MEASUREMENT REPORT (Step 21)

The contents of this message are identical to MEASUREMENT REPORT (Step 2) message with the following exceptions:

| Information Element | Value/Remarks |
| --- | --- |
| Measurement identity | 3 |

MEASUREMENT CONTROL (Step 22)

The contents of this message are identical to MEASUREMENT CONTROL (Step 14) message with the following exceptions:

| Information Element | Value/Remark |
| --- | --- |
| Measurement Identity | 3 |

MEASUREMENT CONTROL (Step 25)

The contents of this message are identical to MEASUREMENT CONTROL (Step 1) message with the following exceptions:

| Information Element | Value/Remark |
| --- | --- |
| Measurement Identity Measurement Command - CHOICE measurement type - Traffic volume measurement object list - UL transport channel identity - UL transport channel identity - UL transport channel identity - Measurement validity | 4 Setup Traffic Volume Measurement RACH DCH :1 DCH : 5 All States |

MEASUREMENT REPORT (Step 26, and 29)

The contents of this message are identical to MEASUREMENT REPORT (Step 2) message with the following exceptions:

| Information Element | Value/Remarks |
| --- | --- |
| Measurement identity | 4 |

MEASUREMENT CONTROL (Step 30)

The contents of this message are identical to MEASUREMENT CONTROL (Step 14) message with the following exceptions:

| Information Element | Value/Remark |
| --- | --- |
| Measurement Identity | 4 |

Master Information Block (Step 33)

| Information Element | Value/Remarks |
| --- | --- |
| MIB Value Tag | 2 |

System Information Block type 11 (Step 33)

| Information Element | Value/Remarks |
| --- | --- |
| SIB12 indicator FACH measurement occasion info Measurement control system information - Use of HCS - Cell selection and reselection quality measure - Intra-frequency measurement system information - Inter-frequency measurement system information - Inter-RAT measurement system information - Traffic volume measurement system information - Traffic volume measurement ID - Traffic volume measurement object list - Traffic volume measurement quantity - Traffic volume reporting quantity - RB buffer payload - RB buffer payload average - RB buffer payload variance - Traffic volume measurement reporting criteria - Measurement validity - Measurement reporting mode - Measurement report transfer mode - Periodical or event trigger - Report criteria system Information - Reporting amount - Reporting interval - UE internal measurement system information | FALSE Not Present Not used CPICH RSCP Not Present Not Present Not Present 5 Not Present RLC Buffer Payload True False False Not Present all states Acknowledged Mode Periodical Periodical reporting criteria Infinity 8 seconds Not Present |

SYSTEM INFORMATION CHANGE INDICATION (Step 33a)

| Information Element | Value/Remarks |
| --- | --- |
| Paging record list | Not Present |
| BCCH modification info |  |
| - MIB Value Tag | 3 |
| - BCCH modification time | Not Present |

MEASUREMENT REPORT (Step 34, and 37)

The contents of this message are identical to MEASUREMENT REPORT (Step 2) message with the following exceptions:

| Information Element | Value/Remarks |
| --- | --- |
| Measurement identity | 5 |

MEASUREMENT CONTROL (Step 38)

The contents of this message are identical to MEASUREMENT CONTROL (Step 14) message with the following exceptions:

| Information Element | Value/Remark |
| --- | --- |
| Measurement Identity | 5 |

8.4.1.18.5	Test Requirement

The UE shall send MEASUREMENT REPORT message in steps 21, 29 and 37. The UE shall not send MEASUREMENT REPORT message in steps 5, 8, and 13.

#### 8.4.1.19	Measurement Control and Report: Traffic volume measurement for transition from CELL_DCH to CELL_FACH state

8.4.1.19.1	Definition

8.4.1.19.2	Conformance requirement

Upon transition from CELL_DCH to CELL_FACH state, the UE shall:

-	retrieve each set of measurement control information of measurement type "traffic volume" stored in the UE; and

-	if the optional IE "measurement validity" for this measurement has not been included:

-	delete the associated measurement;

-	if the IE "measurement validity" for the measurement has been included, and the IE "UE state" has been assigned to value "CELL_DCH":

-	stop measurement reporting;

-	save the associated measurement to be used after the next transition to CELL_DCH state;

-	if the IE "measurement validity" for the measurement has been included, and the IE "UE state" has been assigned to value "all states":

-	continue measurement reporting;

-	if the IE "measurement validity" has been included and the IE "UE state" has been assigned to value "all states except CELL_DCH":

-	resume this measurement and associated reporting;

-	if no traffic volume type measurements applicable to CELL_FACH states are stored in the UE:

-	store the measurement control information from the IE "Traffic volume measurement system information" received in System Information Block type 12 or System Information Block type 11;

-    begin traffic volume measurement reporting according to the assigned information.

Reference

3GPP TS 25.331 clauses 8.4.1.6.6.

8.4.1.19.3	Test Purpose

1.    The UE shall performs traffic volume measurements and the associated reporting when it enters CELL_FACH state from CELL_DCH state, and that such measurement contexts (and optionally, the reporting context) valid for CELL_FACH state have been previously stored.

2.    The UE shall perform traffic volume measurement listed in the System Information Block type 11 or 12 messages, if no previously assigned measurements are present. The UE shall transmit MEASUREMENT REPORT messages if reporting conditions has been satisfied.

Reference

3GPP TS 25.331 clause 8.4.1.6.6

8.4.1.19.4	Method of test

Initial Condition

System Simulator: 1 cell

UE: PS-DCCH+DTCH_DCH (state 6-10) as specified in clause 7.4 of TS 34.108

Test Procedure

Initially the UE is in CELL_DCH state. MEASUREMENT CONTROL message is sent to the UE to establish traffic volume measurement context with optional IE "measurement validity" is not present. The UE shall perform measurement and reporting as assigned in MEASUREMENT CONTROL message. RADIO BEARER RECONFIGURATION procedure is used to take the UE from CELL_DCH state to CELL_FACH state. While entering CELL_FACH state from CELL_DCH state, the UE shall delete traffic volume measurement contexts if optional IE "measurement validity" is not present. So, in CELL_FACH state UE shall not perform traffic volume measurement and reporting. UE is taken to the CELL_DCH state from CELL_FACH state using RADIO BEARER RECONFIGURATION procedure.  The UE shall not send MEASUREMENT REPORT message as measurement context is already deleted.

Similarly behavior of the UE when moved from CELL_DCH state to CELL_FACH state and assigned traffic volume measurement context is present with IE "measurement validity" is set to "All But CELL_DCH state" or "CELL_DCH state" or "All states" is tested.

When the UE is in CELL_DCH state, System Information is modified to assign traffic volume measurement and reporting to the UE. No previously assigned traffic volume measurement contexts are present in the UE. The UE is taken to CELL_FACH state from CELL_DCH state using RADIO BEARER RECONFIGURATION procedure. In CELL_FACH state the UE shall perform traffic volume measurement and reporting as assigned in System Information. Traffic volume measurement and reporting is released by sending MEASUREMENT CONTROL message.

Expected Sequence

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 |  |  | MEASUREMENT CONTROL | Optional IE "measurement validity" is not included. |
| 2 |  |  | MEASUREMENT REPORT |  |
| 3 |  |  | RADIO BEARER RECONFIGURATION |  |
| 4 |  |  | RADIO BEARER RECONFIGURATION COMPLETE | While entering in CELL_FACH state from CELL_DCH state UE shall delete measurement context setup by MEASUREMENT CONTROL message (Step 1). |
| 5 |  |  |  | SS waits for 8 seconds to confirm that there is no MEASUREMENT REPORT message from UE. |
| 6 |  |  | RADIO BEARER RECONFIGURATION |  |
| 7 |  |  | RADIO BEARER RECONFIGURATION COMPLETE | UE is in CELL_DCH state. |
| 8 |  |  |  | SS waits for 8 seconds to confirm that there is no MEASUREMENT REPORT message from UE. |
| 9 |  |  | MEASUREMENT CONTROL | IE "measurement validity" is set to "All But CELL_DCH". |
| 10 |  |  |  | SS waits for 8 seconds to confirm that there is no MEASUREMENT REPORT message from UE. |
| 11 |  |  | RADIO BEARER RECONFIGURATION |  |
| 12 |  |  | RADIO BEARER RECONFIGURATION COMPLETE | While entering in CELL_FACH state from CELL_DCH state UE shall start traffic volume measurement setup by MEASUREMENT CONTROL message (Step 9). |
| 13 |  |  | MEASUREMENT REPORT |  |
| 14 |  |  | MEASUREMENT CONTROL | UE shall release measurement context setup by MEASUREMENT CONTROL message (Step 9). |
| 15 |  |  | RADIO BEARER RECONFIGURATION |  |
| 16 |  |  | RADIO BEARER RECONFIGURATION COMPLETE | UE is in CELL_DCH state. |
| 17 |  |  | MEASUREMENT CONTROL | IE "measurement validity" is set to "CELL_DCH". |
| 18 |  |  | MEASUREMENT REPORT |  |
| 19 |  |  | RADIO BEARER RECONFIGURATION |  |
| 20 |  |  | RADIO BEARER RECONFIGURATION COMPLETE | While entering in CELL_FACH state from CELL_DCH state UE shall stop traffic volume measurement setup by MEASUREMENT CONTROL message (Step 17). |
| 21 |  |  |  | SS waits for 8 seconds to confirm that there is no MEASUREMENT REPORT message from UE. |
| 22 |  |  | MEASUREMENT CONTROL | UE shall release measurement context setup by MEASUREMENT CONTROL message (Step 17) |
| 23 |  |  | RADIO BEARER RECONFIGURATION |  |
| 24 |  |  | RADIO BEARER RECONFIGURATION COMPLETE | UE is in CELL_DCH state. |
| 25 |  |  | MEASUREMENT CONTROL | IE "measurement validity" is set to "All states". |
| 26 |  |  | MEASUREMENT REPORT |  |
| 27 |  |  | RADIO BEARER RECONFIGURATION |  |
| 28 |  |  | RADIO BEARER RECONFIGURATION COMPLETE | While entering in CELL_FACH state from CELL_DCH state UE shall continue traffic volume measurement setup by MEASUREMENT CONTROL message (Step 25). |
| 29 |  |  | MEASUREMENT REPORT |  |
| 30 |  |  | MEASUREMENT CONTROL | UE shall release measurement context setup by MEASUREMENT CONTROL message (Step 25) |
| 31 |  |  | RADIO BEARER RECONFIGURATION |  |
| 32 |  |  | RADIO BEARER RECONFIGURATION COMPLETE | UE is in CELL_DCH state. |
| 33 |  |  | SIB12 modified | Traffic volume measurements and reporting is assigned to UEs |
| 34 |  |  | RADIO BEARER RECONFIGURATION |  |
| 35 |  |  | RADIO BEARER RECONFIGURATION COMPLETE | While entering in CELL_FACH state from CELL_DCH state UE shall start traffic volume measurement as assigned in System Information (Step 33). |
| 36 |  |  | MEASUREMENT REPORT |  |
| 37 |  |  | MEASUREMENT CONTROL | UE shall release measurement context assigned in System Information (Step 33). |

Specific Message Content

MEASUREMENT CONTROL (Step 1)

| Information Element | Value/Remark |
| --- | --- |
| Measurement Identity Measurement Command - CHOICE measurement type - Traffic volume measurement object list - Traffic volume measurement quantity - Traffic volume reporting quantity - RB buffer payload - RB buffer payload average - RB buffer payload variance - Measurement validity - Report criteria - Reporting amount - Reporting interval Measurement reporting mode - Transfer Mode - Periodical or event trigger Additional measurement list DPCH compressed mode status | 1 Setup Traffic Volume Measurement Not Present RLC Buffer Payload True False False Not Present Periodical Reporting Criteria 8 8 Sec Acknowledged mode Periodic Not Present Not Present |

MEASUREMENT REPORT (Step 2)

| Information Element | Value/Remarks |
| --- | --- |
| Measurement identity Measured Results - CHOICE measurement - Traffic volume measurement results - RB identity - RLC buffer payload - RLC buffer payload average - RLC buffer payload variance - RB identity - RLC buffer payload - RLC buffer payload average - RLC buffer payload variance - RB identity - RLC buffer payload - RLC buffer payload average - RLC buffer payload variance - RB identity - RLC buffer payload - RLC buffer payload average - RLC buffer payload variance - RB identity - RLC buffer payload - RLC buffer payload average - RLC buffer payload variance Measured results on RACH Additional measured results Event results | 1 Traffic volume measured results list 1 Check to see if this IE is present Check to see if this IE is absent Check to see if this IE is absent 2 Check to see if this IE is present Check to see if this IE is absent Check to see if this IE is absent 3 Check to see if this IE is present Check to see if this IE is absent Check to see if this IE is absent 4 Check to see if this IE is present Check to see if this IE is absent Check to see if this IE is absent 20 Check to see if this IE is present Check to see if this IE is absent Check to see if this IE is absent Check to see if this IE is absent Check to see if this IE is absent Check to see if this IE is absent |

RADIO BEARER RECONFIGURATION (Step 3, 11, 19, 27, and 34)

Use the same message type found in Annex A with condition set to A5.

RADIO BEARER RECONFIGURATION (Step 6, 15, 23, and 31)

Use the same message type found in Annex A with condition set to A4.

MEASUREMENT CONTROL (Step 9)

The contents of this message are identical to MEASUREMENT CONTROL (Step 1) message with the following exceptions:

| Information Element | Value/Remark |
| --- | --- |
| Measurement Identity Measurement Command - CHOICE measurement type - Measurement validity | 2 Setup Traffic Volume Measurement All But CELL_DCH |

MEASUREMENT REPORT (Step 13)

The contents of this message are identical to MEASUREMENT REPORT (Step 2) message with the following exceptions:

| Information Element | Value/Remarks |
| --- | --- |
| Measurement identity | 2 |

MEASUREMENT CONTROL (Step 14)

| Information Element | Value/Remark |
| --- | --- |
| Measurement Identity Measurement Command Measurement reporting mode Additional measurement list DPCH compressed mode status | 2 Release Not Present Not Present Not Present |

MEASUREMENT CONTROL (Step 17)

The contents of this message are identical to MEASUREMENT CONTROL (Step 1) message with the following exceptions:

| Information Element | Value/Remark |
| --- | --- |
| Measurement Identity Measurement Command - CHOICE measurement type - Measurement validity | 3 Setup Traffic Volume Measurement CELL_DCH |

MEASUREMENT REPORT (Step 18)

The contents of this message are identical to MEASUREMENT REPORT (Step 2) message with the following exceptions:

| Information Element | Value/Remarks |
| --- | --- |
| Measurement identity | 3 |

MEASUREMENT CONTROL (Step 22)

The contents of this message are identical to MEASUREMENT CONTROL (Step 14) message with the following exceptions:

| Information Element | Value/Remark |
| --- | --- |
| Measurement Identity | 3 |

MEASUREMENT CONTROL (Step 25)

The contents of this message are identical to MEASUREMENT CONTROL (Step 1) message with the following exceptions:

| Information Element | Value/Remark |
| --- | --- |
| Measurement Identity Measurement Command - CHOICE measurement type - Traffic volume measurement object list - UL transport channel identity - UL transport channel identity - UL transport channel identity - Measurement validity | 4 Setup Traffic Volume Measurement RACH DCH :1 DCH : 5 All States |

MEASUREMENT REPORT (Step 26, and 29)

The contents of this message are identical to MEASUREMENT REPORT (Step 2) message with the following exceptions:

| Information Element | Value/Remarks |
| --- | --- |
| Measurement identity | 4 |

MEASUREMENT CONTROL (Step 30)

The contents of this message are identical to MEASUREMENT CONTROL (Step 14) message with the following exceptions:

| Information Element | Value/Remark |
| --- | --- |
| Measurement Identity | 4 |

System Information Block type 12 (Step 33)

| Information Element | Value/Remarks |
| --- | --- |
| FACH measurement occasion info Measurement control system information - Use of HCS - Cell_selection_and_reselection_quality - Intra-frequency measurement system information - Inter-frequency measurement system information - Inter-RAT measurement system information - Traffic volume measurement system information - Traffic volume measurement ID - Traffic volume measurement object list - Traffic volume measurement quantity - Traffic volume reporting quantity - RB buffer payload - RB buffer payload average - RB buffer payload variance - Traffic volume measurement reporting criteria - Measurement validity - Measurement reporting mode - Measurement report transfer mode - Periodical or event trigger - Report criteria system Information - Reporting amount - Reporting interval - UE internal measurement system information | Not Present Not used CPICH RSCP Not Present Not Present Not Present 5 Not Present RLC Buffer Payload True False False Not Present Not Present Acknowledged Mode Periodical Periodical reporting criteria Infinity 8 seconds Not Present |

MEASUREMENT REPORT (Step 36)

The contents of this message are identical to MEASUREMENT REPORT (Step 2) message with the following exceptions:

| Information Element | Value/Remarks |
| --- | --- |
| Measurement identity | 5 |

MEASUREMENT CONTROL (Step 37)

The contents of this message are identical to MEASUREMENT CONTROL (Step 14) message with the following exceptions:

| Information Element | Value/Remark |
| --- | --- |
| Measurement Identity | 5 |

8.4.1.19.5	Test Requirement

The UE shall send MEASUREMENT REPORT message in steps 13, 29 and 36. The UE shall not send MEASUREMENT REPORT message in steps 5, 8, and 21.

#### 8.4.1.23	Measurement Control and Report: Intra-frequency measurement for events 1C and 1D

8.4.1.23.1	Definition

8.4.1.23.2	Conformance requirement

1.	When event 1C is configured in the UE, the UE shall:

-	if "Measurement quantity" is "pathloss" and Equation 1 below is fulfilled for a primary CPICH, or if "Measurement quantity" is "CPICH Ec/N0" or "CPICH RSCP", and Equation 2 below is fulfilled for a primary CPICH:

-	if the equations have been fulfilled during the time "Time to trigger", and if the primary CPICH that is better is not included in the active set but the other primary CPICH is any of the primary CPICHs included in the active set, and if that primary CPICH is not included in the "cells triggered" in the variable TRIGGERED_1C_EVENTS:

-	include that primary CPICH in the "cells triggered" in the variable TRIGGERED_1C_EVENTS;

-	if the value of "Replacement activation threshold" for this event is lower than the current number of cells in the active set or equal to 0:

-	if "Reporting interval" for this event is not equal to 0:

-	start a timer for that primary CPICH with the value of "Reporting interval" for this event;

-	set "sent reports" for that primary CPICH in the variable TRIGGERED_1C_EVENTS;

-	send a measurement report with IEs set as below:

-	in "intra-frequency event results": "Intrafrequency event identity" to "1c" and the first entry in "cell measurement event results" to the CPICH info of the primary CPICH not in the active set that triggered the report; and

-	the second entry in "cell measurement event results" to the CPICH info of the primary CPICH in the active set that now is worse than the new primary CPICH and has the best measured value (lowest measured result for pathloss and highest measured result for other measurements); and

-	the rest of the entries to other primary CPICHs that are now worse than this new primary CPICH in the order of their measured value;

-	"measured results" and possible "additional measured results" according to TS 25.331 clause 8.4.2;

2.	When event 1D is configured in the UE, the UE shall:

-	if "Measurement quantity" is "pathloss" and Equation 1 below is fulfilled for a primary CPICH that is not stored in "Best cell" in variable BEST_CELL_1D_EVENT, or if "Measurement quantity" is "CPICH Ec/N0" or "CPICH RSCP", and Equation 2 below is fulfilled for a primary CPICH that is not stored in "Best cell" in variable BEST_CELL_1D_EVENT:

-	if the equations have been fulfilled during the time "Time to trigger":

-	set "best cell" in the variable BEST_CELL_1D_EVENT to that primary CPICH that triggered the event;

-	send a measurement report with IEs set as below:

-	in "intra-frequency event results"; "Intrafrequency event identity" to "1d" and "cell measurement event results" to the CPICH info of the primary CPICH that triggered the report.

-	"measured results" and possible "additional measured results" according to TS 25.331 clause 8.4.2;

Reference

3GPP TS 25.331 clause 14.1.2.3, 14.1.2.4.

8.4.1.23.3	Test Purpose

1.A	To confirm that the UE sends MEASUREMENT REPORT message if event 1C is configured, and number of cells in active set is greater than or equal to 'Replacement activation threshold' parameter, and if monitored or detected primary CPICH on same frequency becomes better than a primary CPICH in active set.

1.B	To confirm that the UE does not send MEASUREMENT REPORT message indicating event 1C if number of cells in active set is less than 'Replacement activation threshold' parameter, and if monitored or detected primary CPICH on same frequency becomes better than a primary CPICH in active set.

1.C	To confirm that the UE stops periodic reporting of event 1C if the cell that triggered event 1C is added into active set.

2.	To confirm that the UE sends MEASUREMENT REPORT message if event 1D is configured and intra-frequency measurement indicates change in best cell.

8.4.1.23.4	Method of test

Initial Condition

System Simulator: 3 cells – The initial configurations of the 3 cells in the SS shall follow the values indicated in the column marked "T0" in table 8.4.1.23-1. The table is found in "Test Procedure" clause.

UE: CS-DCCH+DTCH_DCH (State 6-9) or PS-DCCH+DTCH_DCH (State 6-10) in cell 1 as specified in clause 7.4 of TS 34.108, depending on the CN domain supported by the UE.

Test Procedure

Table 8.4.1.23-1 illustrates the downlink power to be applied for the 3 cells at various time instants of the test execution. Column marked "T0" denotes the initial conditions, while columns marked "T1" and "T2" are to be applied subsequently. The exact instants on which these values shall be applied are described in the text in this clause.

- Table 8.4.1.23-1

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  | Cell 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T0 | T1 | T2 | T0 | T1 | T2 | T0 | T1 | T2 |
| UTRA RF Channel Number |  | Ch. 1 |  |  | Ch. 1 |  |  | Ch. 1 |  |  |
| CPICH Ec | dBm | -60 | -60 | -66 | -70 | -70 | Switched off | Switched off | -70 | -60 |

The UE is initially in CELL_DCH state of cell 1. SS then performs a soft handover procedure by sending ACTIVE SET UPDATE message on the downlink DCCH. The UE shall reply with an ACTIVE SET UPDATE COMPLETE message on the uplink DCCH, and include cell 2 to the active set when the activation time specified has elapsed.

SS then ask the UE to perform Intra-frequency measurement and report event 1C and event 1D. In MEASUREMENT CONTROL message, IE 'Replacement activation threshold' is set to 3 and IE 'Cell individual offset' is set to +6 dBm for Cell 3. SS configures itself according to the values in columns "T1" shown above. Cell 3 becomes better than Cell 2 that is in active set of the UE, due to parameter 'Cell Individual offset' for Cell 3. However the UE shall not send MEASUREMENT REPORT message indicating event 1C because number of cells in active set is less than parameter 'Replacement Activation Threshold'.

SS then sends MEASUREMENT CONTROL message to the UE to modify earlier configured intra-frequency measurement. Now, IE 'Replacement activation threshold' is set to 1. MEASUREMENT CONTROL message contains only those IEs that are modified and the UE shall continue to use current values of parameters that are not modified. The UE sends MEASUREMENT REPORT message reporting event 1C, monitored Cell 3 is better than Cell 2 that is in active set. The UE sends second MEASUREMENT REPORT message reporting event 1C after 4 seconds, equals to parameter 'Reporting interval'.

SS then performs soft handover procedure by sending ACTIVE SET UPDATE message on the downlink DCCH. In this message SS commands UE to add Cell 3 and remove Cell 2 from active set. The UE shall reply with an ACTIVE SET UPDATE COMPLETE message. The UE shall also stop periodic reporting of event 1C because the Cell that triggered it is added into active set.  SS then configures itself according to the values in columns "T2" shown above. This triggers event 1D and the UE sends MEASUREMENT REPORT message indicating Cell 3 as a best cell. SS calls for generic procedure C.3 to check that UE is in CELL_DCH state.

Expected Sequence

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 |  |  | ACTIVE SET UPDATE | SS command the UE to add Cell 2 in active set. |
| 2 |  |  | ACTIVE SET UPDATE COMPLETE |  |
| 3 |  |  | MEASUREMENT CONTROL | Event 1C and 1D are configured. IE "Replacement activation threshold" is set to 3. |
| 4 |  |  |  | SS re-adjusts the downlink transmission power settings according to columns "T1" in table 8.4.1.23-1. |
| 5 |  |  |  | Check for 10 seconds, the UE shall not send MEASUREMENT REPORT message. |
| 6 |  |  | MEASUREMENT CONTROL | Measurement configured in step 3 is modified to set parameter 'replacement activation threshold' to 1. |
| 7 |  |  | MEASUREMENT REPORT | Event 1C is triggered. The UE shall report that Cell 3 is better than Cell 2. |
| 8 |  |  | MEASUREMENT REPORT | The UE shall send second report after 4 seconds (Reporting interval) |
| 9 |  |  | ACTIVE SET UPDATE | SS command the UE to replace Cell 2 in active set by Cell 3. |
| 10 |  |  | ACTIVE SET UPDATE COMPLETE |  |
| 11 |  |  |  | Check for 10 seconds, the UE shall not send MEASUREMENT REPORT message. |
| 12 |  |  |  | SS re-adjusts the downlink transmission power settings according to columns "T2" in table 8.4.1.23-1. |
| 13 |  |  | MEASUREMENT REPORT | The UE shall report event 1D change of best cell |
| 14 |  |  | CALL C.3 | If the test result of C.3 indicates that UE is in CELL_DCH state, the test passes, otherwise it fails. |

# Specific Message Contents

All messages indicated below shall use the same content as described in default message content, with the following exceptions:

ACTIVE SET UPDATE (Step 1)

| Information Element | Value/Remark |  |
| --- | --- | --- |
| Radio link addition information - Primary CPICH Info - Primary scrambling code - Downlink DPCH info for each RL - CHOICE mode - Primary CPICH usage for channel estimation - DPCH frame offset - Secondary CPICH info - DL channelisation code - Secondary scrambling code - Spreading factor - Code number - Scrambling code change - TPC combination index - SSDT cell identity - Close loop timing adjustment mode - TFCI combining indicator - SCCPCH information for FACH Radio link removal information |  | Primary scrambling code of Cell 2 FDD P-CPICH may be used. 0 chips Not present This IE is repeated for all existing downlink DPCHs allocated to the UE Not present Refer to the parameter set in TS 34.108 For each DPCH, assign the same code number in the current code given in cell 1. Not present 0 Not present Not present Not present Not present Not present |

MEASUREMENT CONTROL (Step 3)

| Information Element | Value/Remark |
| --- | --- |
| Measurement identity Measurement command - CHOICE measurement type - Intra-frequency cell info list - Intra-frequency cell removal - New intra-frequency info list - Intra-frequency cell id - Cell info - Cell individual offset - Reference time difference to cell - CHOICE mode - Read SFN Indicator - Primary CPICH Info - Primary scrambling code - Primary CPICH TX power - TX Diversity Indicator - Cell for measurement - Intra-frequency cell id list - Intra-frequency measurement quantity - Filter Coefficient - Measurement quantity - Intra-frequency reporting quantity - Reporting quantities for active set cells - SFN-SFN observed time difference reporting indicator - Cell synchronisation information reporting indicator - Cell identity reporting indicator - CPICH Ec/No reporting indicator - CPICH RSCP reporting indicator - Pathloss reporting indicator - Reporting quantities for monitored set cells - SFN-SFN observed time difference reporting indicator - Cell synchronisation information reporting indicator - Cell identity reporting indicator - CPICH Ec/No reporting indicator - CPICH RSCP reporting indicator - Pathloss reporting indicator - Reporting quantities for detected cells - Measurement validity - CHOICE report criteria - Parameters required for each events - Intra-frequency event identity - Replacement activation threshold - Reporting amount - Reporting interval - Hysteresis - Time to trigger - Reporting cell status - Intra-frequency event identity - Hysteresis - Time to trigger - Reporting cell status Measurement reporting mode - Measurement reporting transfer mode - Periodic reporting / Event trigger reporting mode Additional measurement list DPCH compressed mode status info | 1 Setup Intra-frequency measurement Not present Id of Cell 3 6 dBm Not present FDD FALSE Primary scrambling code of Cell 3 Not present FALSE Set to id of cell 1, 2 and 3. 0 CPICH RSCP Same as in default message content No report FALSE FALSE FALSE FALSE FALSE No report FALSE FALSE FALSE FALSE FALSE Not present Not present Intra-frequency measurement reporting criteria 1C 3 16 4 seconds 4 10 mSec Not present 1D 4 10 mSec Not present Acknowledged mode RLC Event trigger Not present Not present |

MEASUREMENT CONTROL (Step 6)

| Information Element | Value/Remark |
| --- | --- |
| Measurement identity Measurement command - CHOICE measurement type - Intra-frequency cell info list - Intra-frequency measurement quantity - Intra-frequency reporting quantity - Measurement validity - CHOICE report criteria - Parameters required for each events - Intra-frequency event identity - Replacement activation threshold - Reporting amount - Reporting interval - Hysteresis - Time to trigger - Reporting cell status - Intra-frequency event identity - Hysteresis - Time to trigger - Reporting cell status Measurement reporting mode Additional measurement list DPCH compressed mode status info | 1 Modify Intra-frequency measurement Not present Not present Not present Not present Intra-frequency measurement reporting criteria 1C 1 16 4 seconds 4 10 mSec Not present 1D 4 10 mSec Not present Not present Not present Not present |

MEASUREMENT REPORT (Step 7 and 8)

| Information Element | Value/Remarks |
| --- | --- |
| Measurement identity Measured results Measured results on RACH Additional Measured results Event results - Event ID - Cell measurement event results - Primary scrambling code - Primary scrambling code | 1 Check to see if this IE is absent Check to see if this IE is absent Check to see if this IE is absent Intra Frequency Event results 1C Primary scrambling code of Cell 3 Primary scrambling code of Cell 2 |

ACTIVE SET UPDATE (Step 9)

| Information Element | Value/Remark |  |
| --- | --- | --- |
| Radio link addition information - Primary CPICH Info - Primary scrambling code - Downlink DPCH info for each RL - CHOICE mode - Primary CPICH usage for channel estimation - DPCH frame offset - Secondary CPICH info - DL channelisation code - Secondary scrambling code - Spreading factor - Code Number - Scrambling code change - TPC Combination Index - SSDT Cell Identity - Close loop timing adjustment mode - TFCI Combining Indicator - SCCPCH information for FACH Radio link removal information - Primary CPICH Info - Primary scrambling code |  | Primary scrambling code of Cell 3 FDD P-CPICH may be used. 0 chips Not present This IE is repeated for all existing downlink DPCHs allocated to the UE Not present Refer to the parameter set in TS 34.108 For each DPCH, assign the same code number in the current code given in cell 1. Not present 0 Not present Not present Not present Not present Primary scrambling code of Cell 2 |

MEASUREMENT REPORT (Step 13)

| Information Element | Value/Remarks |
| --- | --- |
| Measurement identity Measured results Measured results on RACH Additional Measured results Event results - Event ID - Cell measurement event results - Primary scrambling code | 1 Check to see if this IE is absent Check to see if this IE is absent Check to see if this IE is absent Intra-frequency event results 1D Primary scrambling code of Cell 3 |

8.4.1.23.5	Test Requirement

1.A	In steps 7 and 8 the UE shall send MEASUREMENT REPORT message indicating event 1C.  IE 'Cell measurement event results' in MEASUREMENT REPORT message shall contain primary scrambling code of Cell 3 and Cell 2 in that order.

1.B	In step 5 the UE shall not send MEASUREMENT REPORT message.

1.C	In step 11 the UE shall not send MEASUREMENT REPORT message.

2.	In step 13 the UE shall send MEASUREMENT REPORT message indicating event 1D. IE 'Cell measurement event results' in MEASUREMENT REPORT message shall contain primary scrambling code of Cell 3.