3GPP TSG-T1 SIG Meeting #24 	T1S-020457

YOKOHAMA, Japan, 29th – 31st July 2002

| CR-Form-v5.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 34-123-1 |  |  |  | CR | CRNum |  |  |  rev |  |  |  | - |  Current version: |  |  |  |  | 5.0.1 |  |  |  |
| For HELP on using this form, see bottom of this page or look at the pop-up text over the  symbols. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed change affects:  |  |  |  |  | (U)SIM |  |  | ME/UE |  | X |  | Radio Access Network |  |  |  |  |  |  |  | Core Network |  |  |
| Title:  |  | Integrity correction to TC 12.9.11 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source:  |  | Nokia |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Work item code:  |  | TEI |  |  |  |  |  |  |  |  |  |  |  |  | Date:  |  | 01-July-2002 |  |  |  |  |  |
| Category:  |  | F |  |  |  |  |  |  |  |  |  |  |  |  | Release:  |  | REL-5 |  |  |  |  |  |
|  |  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  |  |  |  |  |  |  | Use one of the following releases: 2 (GSM Phase 2) R96 (Release 1996) R97 (Release 1997) R98 (Release 1998) R99 (Release 1999) REL-4 (Release 4) REL-5 (Release 5) |  |  |  |  |  |  |
| Reason for change:  |  |  | The TC 12.9.11 is against 3GPP 24.008. UE GMM does ignore the DETACH REQUEST if integrity protection is not started. See 3GPP 24.008 Chapter 4.1.1.1.1. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Summary of change:  |  |  | DETACH REQUEST is now sent before SERVICE REQUEST for data. The procedure is established via integrity protected signaling connection. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  |  | TC specification is against 3GPP specification 24.008 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected:  |  |  | TC 12.9.11 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other specs  |  |  |  | Other core specifications  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| affected: |  |  |  | Test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | O&M Specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other comments:  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

How to create CRs using this form:

Comprehensive information and tips about how to create CRs can be found at http://www.3gpp.org/specs/CR.htm.  Below is a brief summary:

1)	Fill out the above form. The symbols above marked  contain pop-up help information about the field that they are closest to.

2)	Obtain the latest version for the release of the specification to which the change is proposed. Use the MS Word "revision marks"  feature (also known as "track changes") when making the changes. All 3GPP specifications can be downloaded from the 3GPP server under ftp://ftp.3gpp.org/specs/ For the latest version, look for the directory name with the latest date e.g. 2001-03 contains the specifications resulting from the March 2001 TSG meetings.

3)	With "track changes" disabled, paste the entire CR form (use CTRL-A to select it) into the specification just in front of the clause containing the first piece of changed text.  Delete those parts of the specification which are not relevant to the change request.

### 12.9.11	Service Request / Abnormal cases / Service request procedure collision

12.9.11.1	Definition

12.9.11.2	Conformance requirement

When the UE in GMM-SERVICE-REQUEST-INITIATED state receives a DETACH REQUEST message from the network, UE shall:

-	perform the PS detach procedure.

-	abort Service request procedure.

Reference

TS 24.008 clauses 4.7.13.5

12.9.11.3	Test purpose

To test the behaviour of the UE in case of collision between Service request procedure and PS detach procedure.

12.9.11.4	Method of test

Initial condition

System Simulator:

One cell operating in network operation mode II.

User Equipment:

The UE has a valid P-TMSI-1, P-TMSI-1 signature and RAI-1.

Related ICS/IXIT statements

Support of PS service Yes/No

UE operation mode A Yes/No
UE operation mode C Yes/No

Switch off on button Yes/No

Test procedure

a)	The UE sends a SERVICE REQUEST message to the SS in order to establish the PS signalling connection for the upper layer signalling.

b)	The SS does not respond to SERVICE REQUEST for data. Instead it sends a DETACH REQUEST message to the UE..

c)	After the UE receives the DETACH REQUEST message, the UE aborts the Service request  procedure.

Expected Sequence

| Step | Direction |  | Message | Comments |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
|  |  |  |  | The following message are sent and shall be received on cell A. |
| 1 | UE |  |  | The UE is set in UE operation mode C (see ICS). |
| 2 | SS |  |  | The SS is set in network operation mode II and activates cell A. |
| 3 | UE |  |  | The UE is powered up or switched on and initiates an attach (see ICS). Cell A is preferred by the UE. |
| 4 | -> |  | ATTACH REQUEST | Attach type = 'PS attach' Mobile identity = P-TMSI-1 P-TMSI-1 signature Routing area identity = RAI-1 |
| 5 | <- |  | ATTACH ACCEPT | No new mobile identity assigned. P-TMSI and P-TMSI signature not included. Routing area identity = RAI-1 Attach result = 'PS only attached' |
| 6 |  |  | Void |  |
| 7a | UE |  |  | The UE initiates an upper-layer signalling, e.g., Active PDP Context request, by MMI or by AT command. |
| 7b | -> |  | SERVICE REQUEST | Service type =”signalling” |
| 7c | SS |  |  | The SS starts ciphering and integrity protection. |
| 8 | -> |  | SERVICE REQUEST | Service type = "data" |
| 9 | SS |  |  | The SS does not respond to SERVICE REQUEST message. |
| 10 | <- |  | DETACH REQUEST | GMM cause = "reattach request" |
| 11 | -> |  | ATTACH REQUEST | Attach type = 'PS attach' Mobile identity = P-TMSI-1 P-TMSI-1 signature Routing area identity = RAI-1 |
| 12 | <- |  | ATTACH ACCEPT | No new mobile identity assigned. P-TMSI and P-TMSI signature not included. Routing area identity = RAI-1 Attach result = 'PS only attached' |
| 13 | -> |  | ATTACH COMPLETE |  |
| 14 | UE |  |  | The UE is switched off or power is removed (see ICS). |
| 15 | -> |  | DETACH REQUEST | Message not sent if power is removed. Detach type = 'power switched off, PS detach' |

Specific message contents

None.

12.9.11.5	Test requirements

At step4, when the UE is powered on or switched on, UE shall:

-	initiate the PS attach procedure with information elements specified in the above Expected Sequence.

At step11, when the UE receives a DETACH REQUEST message from the network before the Service request procedure completes, UE shall;

-	abort the Service request procedure.

-	perform the PS detach procedure.