3GPP TSG-T1/SIG Meeting #24	Tdoc  T1S-020492

- Yokohama, Japan, 29-31 July 2002

| CR-Form-v7 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | TS 34.123-1 | CR | CRNum |  rev | - |  Current version: | 5.0.1 |  |
| For HELP on using this form, see bottom of this page or look at the pop-up text over the  symbols. |  |  |  |  |  |  |  |  |

| Proposed change affects:  | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title:  | CR to 34.123-1 REL-5; Corrections to Clause 8.1.10 for Package 2 (System Information) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source:  | Ericsson, Panasonic. |  |  |  |  |  |  |  |  |  |
| Work item code:  | TEI |  |  |  |  |  |  | Date:  |  | 29/07/2002 |
| Category:  | F |  |  |  |  |  |  | Release:  |  | Rel 5 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: 2 (GSM Phase 2) R96 (Release 1996) R97 (Release 1997) R98 (Release 1998) R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) |  |
| Reason for change:  |  | Reason for changes from Ericsson CR T1S-020409: Section 8.1.10 contains a list of System Information for the “Minimum” configuration and the “Maximum” configuration. This System Information is based on the System Information that exists in section 6 of 34.108. This System Information in 34.108 has changed but the changes are not reflected in section 8.1.10 of 34.123. Instead of explicitly outlining all of the System for the “Minimum” configuration and the “Maximum” configuration, section 8.1.10 shall refer to the System Information in section 6 of 34.108 and outline the changes required to that System Information for the tests required there. The existing schedules for System Information listed in section 8.1.10 and in section 6 of 34.108 do not allow for all segment combinations. New schedules are required to ensure that all these segment combinations are tested. Reason for Changes from Panasonic CR T1S-020356: - As SIB message contents in TS34.108 were revised at the last T1SIG meeting, these are reflected into specific message contents in TS34.123-1. - The range of Intra-frequency measurement identity is 1 to 16. |  |  |  |  |  |  |  |  |
| Summary of change:  |  | Summary of changes from Ericsson CR T1S-020409: The following changes to section 8.1.10 are proposed: - Refer to the relevant System Information configurations in section 6 of 34.108 instead of explicitly listing the System Information required for the tests in section 8.1.10. Indicate the changes required to this System Information in section 8.1.10. Configuration 1 in 34.108 can be used for the “Minimum” configuration (includes MIB, SIB1, SIB3, SIB5, SIB7, SIB11) and the “Maximum” configuration (includes the entire Configuration 1 plus one “dummy” SIB). - New schedules are required to ensure that all segment combinations are tested. Summary of changes from Panasonic CR T1S-020356: - All SIB definition except for SIB 5 that includes SCCPCHs information for separated PCH and FACHs are revised according to TS34.108. - SIB 5 for separated PCH and FACHs is defined as 2 SCCPCHs can be configured according to TS34.108 clause 6.10.2.4.3.1 and 6.10.2.4.3.2. - Intra-frequency measurement identity in SIB 11 and 12 should be changed from 0 to 1. All changes from Panasonic CR except the renaming “SIB and SB type” were removed as the Ericsson CR now uses default message contents instead. |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  | Faulty system Information in section 8.1.10 is not corrected. A schedule that incorporates all segment combinations is not defined therefore all segment combinations are not tested. |  |  |  |  |  |  |  |  |
| Clauses affected:  |  | Section 8.1.10. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs  |  |  | x | Other core specifications  |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
|  |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments:  |  | Affects R99, R4 and R5 UE test cases. |  |  |  |  |  |  |  |  |

- How to create CRs using this form:

Comprehensive information and tips about how to create CRs can be found at http://www.3gpp.org/specs/CR.htm.  Below is a brief summary:

1)	Fill out the above form. The symbols above marked  contain pop-up help information about the field that they are closest to.

2)	Obtain the latest version for the release of the specification to which the change is proposed. Use the MS Word "revision marks"  feature (also known as "track changes") when making the changes. All 3GPP specifications can be downloaded from the 3GPP server under ftp://ftp.3gpp.org/specs/ For the latest version, look for the directory name with the latest date e.g. 2001-03 contains the specifications resulting from the March 2001 TSG meetings.

3)	With "track changes" disabled, paste the entire CR form (use CTRL-A to select it) into the specification just in front of the clause containing the first piece of changed text.  Delete those parts of the specification which are not relevant to the change request.

### 8.1.10	Broadcast of system information

#### 8.1.10.1	Dynamic change of segmentation, concatenation & scheduling and handling of unsupported information blocks

8.1.10.1.1	Definition

8.1.10.1.2	Conformance requirement

1.	The RRC layer in the UE shall perform re-assembly of segments. All segments belonging to the same master information block, scheduling block or system information block shall be assembled in ascending order with respect to the segment index. When all segments of the master information block, scheduling block or a system information block have been received, the UE shall perform decoding of the complete master information block, scheduling block or system information block.

2.	For all system information blocks or scheduling blocks that are supported by the UE referenced in the master information block or the scheduling blocks, the UE shall perform the following actions:

……..

-	read and store the IEs of that system information block;

NOTE: 	There are options with and without scheduling blocks.

3.	For system information blocks, not supported by the UE …..

-	skip reading this system information block;

-	skip monitoring changes to this system information block.

4.	However, to enable future introduction of new system information blocks, the UE shall also be able to receive system information blocks other than the ones indicated within the scheduling information. The UE may ignore contents of such system information block.

Reference

3GPP TS 25.331 clause 8.1.1.1.3, 8.1.1.1.4, 8.1.1.1.5, 8.1.1.5 and 8.1.1.6.

8.1.10.1.4	Test Purpose

1.	To verify that dynamic change of System Information is identified, new information read and used.

2.	To verify that the UE can support all segment types and "all" segment combinations.

3.	To verify that the UE can dynamically use different configurations

- To verify that the UE properly uses combinations of Default and assigned values.

NOTE: 	There are 4 segment types and 11 different SYSTEM INFORMATION messages to interpret when re-assembling segments. There are many alternative SIB position offsets and repetition rates.

The allowed segment types are:

- First segment

- Subsequent segment

- Last segment

- Complete

The allowed segment combinations are:

- No segment

- First segment

- Subsequent segment

- Last segment

- Last segment + First segment

- Last segment + one or several Complete

- Last segment + one or several Complete + First segment

- One or several Complete

- One or several Complete + First segment

- One Complete of size 215 to 226 (not tested)

- Last segment of size 215 to 222 (not tested)

8.1.10.1.5	Method of test

Alternate two sets of System Information and generate a call after one or the other set has been broadcasted.

These two sets of System Information are based on the System Information specified in 34.108, section 6.

A "Minimum" configuration and a "Maximum" configuration of System Information are defined. The "Minimum" configuration does not contain all of the Information Blocks defined for Configuration 1 in section 6 of 34.108, while the "Maximum" configuration does. The contents of the SIBs remains the same (for the "Minimum" configuration, the contents of SIB11 changes for the "Maximum" configuration) while the contents of the MIB and SB is altered depending on the nature of the test, i.e. the schedule changes between the "Minimum" and "Maximum" configurations.

The four segment types and nine of the eleven segment combinations are tested using the two configurations (segment combinations 10 and 11 are not tested).

NOTE: 	The decoding of system information in the UE is only measurable by functional tests. A large number of functions utilize system information. An extensive test of the system information decoding thus creates a large number of functional tests, which is impractical. This test specification uses a "sample test", where only a few functions are invoked.

Initial Condition

System Simulator:  3 cells (C1, C2, C3)

UE: Idle state (state 2 or state 3 or state 7) as specified in clause 7.4 of TS 34.108 with a CN UE identity (set to IMSI), depending on the CN domain(s) supported by the UE.

Test procedure

a)	SS broadcasts the "Minimum" system information. The UE selects C1, set the parameters (cell selection/reselection parameters in SIB3) associated with the S criteria to ensure that C1 is selected.

b)	Call setup according to clause 7.2.3.2 in TS 34.108

c)	Disconnect call according to clause 10.1.2.6.4. UE shall enter IDLE state and performs cell reselection. C1 is selected again.

d)	SS broadcasts the "Maximum" system information and notifies the UE as described in clause 8.1.1.5. The UE reads all relevant new/changed Information Blocks. Set the cell selection/reselection parameters in SIB3 and those associated with C2 in SIB11 to ensure that C2 is a better option for the UE. The UE performs a cell reselection (selects C2).

e)Call Setup according to clause 10.1.3, procedure 1.

Specific message content for "Minimum" configuration

The Minimum configuration is the same as the Configuration 1 System Information on 34.108, section 6 with some differences:

-	Only SIB1, SIB3, SIB5, SIB7, SIB11 are used, i.e. the Minimum number of SIBs is used.

-	No SB is used, all scheduling information is contained in the MIB. The contents of this changed MIB are shown below.

-	A different schedule is used. Details below.

-	SIB11 lists eight cells (one serving cell and seven neighbouring cells). Only the first three of these are considered relevant.

Other characteristics of the "minimum" configuration are:

-	no "unknown future" blocks

- First Segment, First Segment (short), Last Segment and Last Segment (short) are used

The following tables show (based on SIB_REP and SIB_POS in the MIB and SB) the schedule used for these tests.

Table 1: The schedule in this table incorporates segment combinations 1, 2, 3, 4, 7, 8.

| Block Type | MIB | SB1 | SIB1 | SIB2 | SIB3 | SIB4 | SIB5 | SIB6 | SIB7 | SIB11 | SIB12 | SIB18 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SIB_REP | 8 | 16 | 64 | 64 | 64 | 64 | 64 | 64 | 16 | 64 | 64 | 64 |
| SEG_ COUNT | 1 | 1 | 1 | 1 | 1 | 1 | 3 | 3 | 1 | 3 | 3 | 1 |

| Frame No / SIB_POS | 0 | 2 | 4 | 6 | 8 | 10 | 12 | 14 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Block Type | MIB |  | SIB7 |  | MIB |  |  |  |
| Frame No / SIB_POS | 16 | 18 | 20 | 22 | 24 | 26 | 28 | 30 |
| Block Type | MIB |  | SIB7/SIB3 | SIB1 | MIB |  |  | SIB11 |
| Frame No / SIB_POS | 32 | 34 | 36 | 38 | 40 | 42 | 44 | 46 |
| Block Type | MIB | SIB11 | SIB11/SIB7/SIB5 | SIB5 | MIB | SIB5 |  |  |
| Frame No / SIB_POS | 48 | 50 | 52 | 54 | 56 | 58 | 60 | 62 |
| Block Type | MIB |  | SIB7 |  | MIB |  |  |  |

Contents of Master Information Block PLMN type is the case of GSM-MAP

| - MIB value tag | 1 |
| --- | --- |
| - Supported PLMN types |  |
| - PLMN type | GSM-MAP |
| - PLMN identity |  |
| - MCC digit | Set to the same Mobile Country Codes stored in the test USIM card (TS 34.108 clause 8.3.2.2 EF IMSI(IMSI)). |
| - MNC digit | Set to the same Mobile Network Codesstored in the test USIM card (TS 34.108 clause 8.3.2.2 EF IMSI(IMSI)). |
| - ANSI-41 Core Network information | Not Present |
| - References to other system information blocks and scheduling blocks |  |
| - References to other system information blocks |  |
| - Scheduling information |  |
| - CHOICE Value tag | PLMN Value tag |
| - PLMN Value tag | 1 |
| - SEG_COUNT | 1 |
| - SIB_REP | 64 |
| - SIB_POS | 22 |
| - SIB_POS offset info | Not Present – use default |
| - SIB type | System Information Type 1 |
| - Scheduling information |  |
| - CHOICE Value tag | Cell Value tag |
| - Cell Value tag | 1 |
| - SEG_COUNT | 1 |
| - SIB_REP | 64 |
| - SIB_POS | 20 |
| - SIB_POS offset info | Not Present – use default |
| - SIB type | System Information Type 3 |
| - Scheduling information |  |
| - CHOICE Value tag | Cell Value tag |
| - Cell Value tag | 1 |
| - SEG_COUNT | 3 |
| - SIB_REP | 64 |
| - SIB_POS | 36 |
| - SIB_POS offset info |  |
| - SIB_OFF | 2 |
| - SIB_OFF | 4 |
| - SIB type | System Information Type 5 |
| - Scheduling information |  |
| - CHOICE Value tag | Not Present |
| - SEG_COUNT | 1 |
| - SIB_REP | 16 |
| - SIB_POS | 4 |
| - SIB_POS offset info | Not Present |
| - SIB and SB type | System Information Type 7 |
| - Scheduling information |  |
| - CHOICE Value tag | Cell Value tag |
| - Cell Value tag | 1 |
| - SEG_COUNT | 3 |
| - SIB_REP | 64 |
| - SIB_POS | 30 |
| - SIB_POS offset info |  |
| - SIB_OFF | 4 |
| - SIB_OFF | 2 |
| - SIB and SB type | System Information Type 11 |

Specific message content for "Maximum" configuration

The "Maximum" configuration is the same as the Configuration 1 System Information on 34.108, section 6 with some differences:

-	A different schedule is used. Details below.

-	SIB11 lists eight cells (one serving cell and seven neighbouring cells). Only the first three of these are considered relevant.

Other characteristics of the maximum configuration are:

-	one "unknown future" block. A "dummy" SIB (SIB50) is added. This is used to ensure that the UE can receive an Information Block that it does not support and still process the Information Blocks that it does support in the correct way.

- First Segment, First Segment (short), Last Segment and Last Segment (short) are used

The following tables show (based on SIB_REP and SIB_POS in the MIB and SB) the schedule used for these tests.

Table 2: The schedule in this table incorporates segment combinations 1, 2, 3, 4, 5, 6, 8, 9.

| Block Type | MIB | SB1 | SIB1 | SIB2 | SIB3 | SIB4 | SIB5 | SIB6 | SIB7 | SIB11 | SIB12 | SIB18 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SIB_REP | 8 | 16 | 64 | 64 | 64 | 64 | 64 | 64 | 16 | 64 | 64 | 64 |
| SEG_ COUNT | 1 | 1 | 1 | 1 | 1 | 1 | 3 | 3 | 1 | 3 | 3 | 1 |

| Frame No / SIB_POS | 0 | 2 | 4 | 6 | 8 | 10 | 12 | 14 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Block Type | MIB | SB1 | SIB7/SIB6 | SIB6 | MIB | SIB6 | SIB3 |  |
| Frame No / SIB_POS | 16 | 18 | 20 | 22 | 24 | 26 | 28 | 30 |
| Block Type | MIB | SB1 | SIB11 | SIB11 | MIB | SIB11/SIB12 | SIB12 | SIB12 |
| Frame No / SIB_POS | 32 | 34 | 36 | 38 | 40 | 42 | 44 | 46 |
| Block Type | MIB | SB1 | SIB7/SIB18 | SIB5 | MIB | SIB5 | SIB5/SIB2 |  |
| Frame No / SIB_POS | 48 | 50 | 52 | 54 | 56 | 58 | 60 | 62 |
| Block Type | MIB | SB1 | SIB4 |  | MIB | SIB1 | SIB50 |  |

Contents of Master Information Block PLMN type is the case of GSM-MAP

| - MIB value tag | 1 |
| --- | --- |
| - Supported PLMN types |  |
| - PLMN type | GSM-MAP |
| - PLMN identity |  |
| - MCC digit | Set to the same Mobile Country Codes stored in the test USIM card (TS 34.108 clause 8.3.2.2 EF IMSI(IMSI)). |
| - MNC digit | Set to the same Mobile Network Codesstored in the test USIM card (TS 34.108 clause 8.3.2.2 EF IMSI(IMSI)). |
| - ANSI-41 Core Network information | Not Present |
| - References to other system information blocks and scheduling blocks |  |
| - References to other system information blocks |  |
| - Scheduling information |  |
| - CHOICE Value tag | Cell Value Tag |
| - Cell Value tag | 1 |
| - Scheduling |  |
| - SEG_COUNT | 1 |
| - SIB_REP | 16 |
| - SIB_POS | 2 |
| - SIB_POS offset info | Not Present – use default |
| - SIB type | Scheduling Block 1 |
| - Scheduling information |  |
| - CHOICE Value tag | PLMN Value tag |
| - PLMN Value tag | 1 |
| - SEG_COUNT | 1 |
| - SIB_REP | 64 |
| - SIB_POS | 58 |
| - SIB_POS offset info | Not Present – use default |
| - SIB type | System Information Type 1 |
| - Scheduling information |  |
| - CHOICE Value tag | Cell Value tag |
| - Cell Value tag | 1 |
| - SEG_COUNT | 1 |
| - SIB_REP | 64 |
| - SIB_POS | 44 |
| - SIB_POS offset info | Not Present – use default |
| - SIB type | System Information Type 2 |
| - Scheduling information |  |
| - CHOICE Value tag | Cell Value tag |
| - Cell Value tag | 1 |
| - SEG_COUNT | 1 |
| - SIB_REP | 64 |
| - SIB_POS | 12 |
| - SIB_POS offset info | Not Present – use default |
| - SIB type | System Information Type 3 |
| - Scheduling information |  |
| - CHOICE Value tag | Cell Value tag |
| - Cell Value tag | 1 |
| - SEG_COUNT | 1 |
| - SIB_REP | 64 |
| - SIB_POS | 52 |
| - SIB_POS offset info | Not Present – use default |
| - SIB type | System Information Type 4 |
| - Scheduling information |  |
| - CHOICE Value tag | Cell Value tag |
| - Cell Value tag | 1 |
| - SEG_COUNT | 3 |
| - SIB_REP | 64 |
| - SIB_POS | 38 |
| - SIB_POS offset info |  |
| - SIB_OFF | 4 |
| - SIB_OFF | 2 |
| - SIB type | System Information Type 5 |

Contents of Scheduling Block 1 (FDD)

| - References to other system information blocks |  |
| --- | --- |
| - Scheduling information |  |
| - CHOICE Value tag | Cell Value tag |
| - Cell Value tag | 1 |
| - SEG_COUNT | 3 |
| - SIB_REP | 64 |
| - SIB_POS | 4 |
| - SIB_POS offset info |  |
| - SIB_OFF | 2 |
| - SIB_OFF | 4 |
| - SIB and SB type | System Information Type 6 |
| - Scheduling information |  |
| - CHOICE Value tag | Not Present |
| - SEG_COUNT | 1 |
| - SIB_REP | 32 |
| - SIB_POS | 4 |
| - SIB_POS offset info | Not Present |
| - SIB and SB type | System Information Type 7 |
| - Scheduling information |  |
| - CHOICE Value tag | Cell Value tag |
| - Cell Value tag | 1 |
| - SEG_COUNT | 3 |
| - SIB_REP | 64 |
| - SIB_POS | 20 |
| - SIB_POS offset info |  |
| - SIB_OFF | 2 |
| - SIB_OFF | 4 |
| - SIB and SB type | System Information Type 11 |
| - Scheduling information |  |
| - CHOICE Value tag | Cell Value tag |
| - Cell Value tag | 1 |
| - SEG_COUNT | 3 |
| - SIB_REP | 64 |
| - SIB_POS | 26 |
| - SIB_POS offset info |  |
| - SIB_OFF | 2 |
| - SIB_OFF | 2 |
| - SIB and SB type | System Information Type 12 |
| - Scheduling information |  |
| - CHOICE Value tag | Cell Value tag |
| - Cell Value tag | 1 |
| - SEG_COUNT | 1 |
| - SIB_REP | 64 |
| - SIB_POS | 36 |
| - SIB_POS offset info | Not Present |
| - SIB and SB type | System Information Type 18 |
| - Scheduling information |  |
| - CHOICE Value tag | Cell Value tag |
| - Cell Value tag | 1 |
| - SEG_COUNT | 1 |
| - SIB_REP | 64 |
| - SIB_POS | 60 |
| - SIB_POS offset info | Not Present |
| - SIB and SB type | System Information Type 50 |

Contents of System Information Block type 11 (FDD)

Same contents as that defined in Configuration 1, Section 6, 34.108 except for the following:

| Information Element | Value/remark |
| --- | --- |
| Qqualmin | -18 (for first adjacent cell) |

Contents of System Information Block type 50

| Information Element | Value/remark |
| --- | --- |
| Data 1 |  |
| Data 3 |  |

Contents of Master Information Block PLMN type is the case of GSM-MAP

Contents of System Information Block type 3 (FDD)

Contents of System Information Block type 5 (FDD)

Contents of System Information Block type 7 (FDD)

Contents of System Information Block type 11 (FDD)

Default settings for cell No.1 (FDD):

The contents of SYSTEM INFORMATION BLOCK TYPE 1 to 16 messages for cell No.2 are identical to those of cell No.1 with the following exceptions:

Default settings for cell No.2 (FDD):

Cell No.3

The contents of SYSTEM INFORMATION BLOCK TYPE 1 to 16 messages for cell No.3 are identical to those of cell No.1 with the following exceptions:

Default settings for cell No.3 (FDD):

Cell No.4

The contents of SYSTEM INFORMATION BLOCK TYPE 1 to 16 messages for cell No.4 are identical to those of cell No.1 with the following exceptions:

Default settings for cell No.4 (FDD):

The contents of SYSTEM INFORMATION BLOCK TYPE 1 to 16 messages for cell No.5 are identical to those of cell No.1 with the following exceptions:

Default settings for cell No.5 (FDD):

The contents of SYSTEM INFORMATION BLOCK TYPE 1 to 16 messages for cell No.6 are identical to those of cell No.1 with the following exceptions:

Default settings for cell No.6 (FDD):

The contents of SYSTEM INFORMATION BLOCK TYPE 1 to 16 messages for cell No.7 are identical to those of cell No.1 with the following exceptions:

Default settings for cell No.7 (FDD):

The contents of SYSTEM INFORMATION BLOCK TYPE 1 to 16 messages for cell No.8 are identical to those of cell No.1 with the following exceptions:

Default settings for cell No.8 (FDD):

Contents of Master Information Block PLMN type is the case of GSM-MAP

Contents of Scheduling Block 1 (FDD)

Contents of System Information Block type 2

Contents of System Information Block type 3 (FDD)

Contents of System Information Block type 4 in connected mode (FDD)

Contents of System Information Block type 5 (FDD)

Contents of System Information Block type 6 in connected mode (FDD)

Contents of System Information Block type 7 (FDD)

Contents of System Information Block type 11 (FDD)

Contents of System Information Block type 12 in connected mode (FDD)

Contents of System Information Block type 15

Contents of System Information Block type 16

Contents of System Information Block type 18

Default settings for cell No.1 (FDD):

The contents of SYSTEM INFORMATION BLOCK TYPE 1 to 16 messages for cell No.2 are identical to those of cell No.1 with the following exceptions:

Default settings for cell No.2 (FDD):

The contents of SYSTEM INFORMATION BLOCK TYPE 1 to 16 messages for cell No.3 are identical to those of cell No.1 with the following exceptions:

Default settings for cell No.3 (FDD):

Cell No.4

The contents of SYSTEM INFORMATION BLOCK TYPE 1 to 16 messages for cell No.4 are identical to those of cell No.1 with the following exceptions:

Default settings for cell No.4 (FDD):

The contents of SYSTEM INFORMATION BLOCK TYPE 1 to 16 messages for cell No.5 are identical to those of cell No.1 with the following exceptions:

Default settings for cell No.5 (FDD):

The contents of SYSTEM INFORMATION BLOCK TYPE 1 to 16 messages for cell No.6 are identical to those of cell No.1 with the following exceptions:

Default settings for cell No.6 (FDD):

The contents of SYSTEM INFORMATION BLOCK TYPE 1 to 16 messages for cell No.7 are identical to those of cell No.1 with the following exceptions:

Default settings for cell No.7 (FDD):

The contents of SYSTEM INFORMATION BLOCK TYPE 1 to 16 messages for cell No.8 are identical to those of cell No.1 with the following exceptions:

Default settings for cell No.8 (FDD):

8.1.10.1.6		Test requirement

After step b the UE shall be in Connected state U10 in Cell 1.

After step e the UE shall be in Connected state U10 in Cell 2.