3GPP TSG-T1/SIG Meeting #24	Tdoc  T1S-020504

- Yokohama, Japan, 29-31 July 2002

| CR-Form-v6.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | TS 34.123-1 |  |  |  | CR | CR Num |  |  |  rev |  |  |  | - |  Current version: |  |  |  |  | 5.0.0 |  |  |  |
|  | Spec Title: |  |  |  | User Equipment (UE) conformance specification; Part 1: Protocol conformance specification |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| For HELP on using this form, see bottom of this page or look at the pop-up text over the  symbols. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed change affects:  |  |  |  |  | (U)SIM |  |  | ME/UE |  | X |  | Radio Access Network |  |  |  |  |  |  |  | Core Network |  |  |
| Title:  |  | CR to 34.123-1 REL-5; Additional test case for timing re-initialise inter-frequency handover |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source:  |  | Ericsson |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Work item code:  |  | TEI |  |  |  |  |  |  |  |  |  |  |  |  | Date:  |  | 30/07/2002 |  |  |  |  |  |
| Category:  |  | F |  |  |  |  |  |  |  |  |  |  |  |  | Release:  |  | REL-5 |  |  |  |  |  |
|  |  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  |  |  |  |  |  |  | Use one of the following releases: 2 (GSM Phase 2) R96 (Release 1996) R97 (Release 1997) R98 (Release 1998) R99 (Release 1999) REL-4 (Release 4) REL-5 (Release 5) |  |  |  |  |  |  |
| Reason for change:  |  |  | No test case covers timing re-initialised inter-frequency handover Changes in this document compared to T1S-020438 marked in yellow. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Summary of change:  |  |  | - One test case is proposed to be added to chapter 8.2.6, aiming at testing timing re-initialised inter-frequency handover. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  |  | No test case covering timing re-initialised inter-frequency handover, while this is an important function to test. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected:  |  |  | 8.2.6.37 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other specs  |  |  |  | Other core specifications  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| affected: |  |  |  | Test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | O&M Specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other comments:  |  |  | Affects R99, REL-4 and REL-5 test cases. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

- How to create CRs using this form:

Comprehensive information and tips about how to create CRs can be found at: http://www.3gpp.org/3G_Specs/CRs.htm.  Below is a brief summary:

1)	Fill out the above form. The symbols above marked  contain pop-up help information about the field that they are closest to.

2)	Obtain the latest version for the release of the specification to which the change is proposed. Use the MS Word "revision marks"  feature (also known as "track changes") when making the changes. All 3GPP specifications can be downloaded from the 3GPP server under ftp://ftp.3gpp.org/specs/ For the latest version, look for the directory name with the latest date e.g. 2001-03 contains the specifications resulting from the March 2001 TSG meetings.

3)	With "track changes" disabled, paste the entire CR form (use CTRL-A to select it) into the specification just in front of the clause containing the first piece of changed text.  Delete those parts of the specification which are not relevant to the change request.

#### 8.2.6.37	Physical channel reconfiguration for transition from CELL_DCH to CELL_DCH (Hard handover to another frequency with timing re-initialised)

8.2.6.37.1	Definition

8.2.6.37.2	Conformance requirement

- In case the reconfiguration procedure is used to remove all existing RL(s) in the active set while new RL(s) are established the UE shall:

1>	apply the hard handover procedure as specified (below);

- be able to perform this procedure even if no prior UE measurements have been performed on the target cell and/or frequency.

…

In case the procedure was triggered by reception of a PHYSICAL CHANNEL RECONFIGURATION message, the UE shall:

1>	transmit a PHYSICAL CHANNEL RECONFIGURATION COMPLETE as response message on the uplink DCCH using AM RLC.

…

When performing hard handover with change of frequency, the UE shall:

1>	stop all intra-frequency and inter-frequency measurements on the cells listed in the variable CELL_INFO_LIST until a MEASUREMENT CONTROL message is received from UTRAN.

…

When the UE is in CELL_DCH state and receives any of the messages causing the UE to perform a hard handover, the UE shall check the IE "Timing indication" in that message and:

1>	if IE "Timing indication" has the value "initialise" (i.e. timing re-initialised hard handover):

2>	read SFN on target cell identified by the first radio link listed in the IE "Downlink information per radio link list" included in that message;

2>	set the CFN according to the following formula:

3>	for FDD:

CFN = (SFN - (DOFF div 38400)) mod 256

where the formula gives the CFN of the downlink DPCH frame which starts at the same time as or which starts during the PCCPCH frame with the given SFN.

…

If the IE "DPCH compressed mode info" is included, and if the IE group "transmission gap pattern sequence configuration parameters" is not included, the UE shall:

1>	if pattern sequence corresponding to IE "TGPSI" is already active (according to "Current TGPS Status Flag" in the variable TGPS_IDENTITY):

2>	if the "TGPS Status Flag" in this message is set to "deactivate" for the corresponding pattern sequence:

3>	deactivate this pattern sequence at the beginning of the frame, indicated by IE "Activation time" received in this message, when the new configuration received in this message is taken into use;

…

If the IE "Downlink DPCH info common for all RL" is included in a message used to perform a Timing re-initialised hard handover or the IE "Downlink DPCH info common for all RL" is included in a message other than RB SETUP used to transfer the UE from a state different from Cell_DCH to Cell_DCH, and ciphering is active for any radio bearer using RLC-TM, the UE shall, after having activated the dedicated physical channels indicated by that IE:

1>	set the 20 MSB of the HFN component of COUNT-C for TM-RLC to the value of the latest transmitted IE "START" or "START List" for this CN domain, while not incrementing the value of the HFN component of COUNT-C at each CFN cycle; and

1>	set the remaining LSBs of the HFN component of COUNT-C to zero;

1>	start to perform ciphering on the radio bearer in lower layers while not incrementing the HFN;

1>	include the IE "COUNT-C activation time" in the response message and specify a CFN value other than the default, "Now" for this IE;

1>	calculate the START value according to subclause 8.5.9;

1>	include the calculated START values for each CN domain in the IE "START list" in the IE "Uplink counter synchronisation info" in the response message;

1>	at the CFN value as indicated in the response message in the IE "COUNT-C activation time":

2>	set the 20 MSB of the HFN component of the COUNT-C variable to the START value as indicated in the IE "START list" of the response message for the relevant CN domain; and

2>	set the remaining LSBs of the HFN component of COUNT-C to zero;

2>	increment the HFN component of the COUNT-C variable by one;

2>	set the CFN component of the COUNT-C to the value of the IE "COUNT-C activation time" of the response message. The HFN component and the CFN component completely initialise the COUNT-C variable;

2>	step the COUNT-C variable, as normal, at each CFN value, i.e. the HFN component is no longer fixed in value but incremented at each CFN cycle.

Reference

3GPP TS 25.331 clauses 8.2.2.3, 8.3.5, 8.5.15.2, 8.6.6.15,  8.6.6.28

8.2.6.37.3	Test Purpose

To confirm that the UE is able to perform a hard-handover with change of frequency, with and without prior measurements on the target frequency.

To confirm that the UE answers with a PHYSICAL CHANNEL RECONFIGURATION COMPLETE message when the procedure has been initiated with the PHYSICAL CHANNEL RECONFIGURATION message.

To confirm that the UE stops intra-frequency measurements after the inter-frequency handover has been performed, until a MEASUREMENT CONTROL message is received from the SS.

To confirm that the UE computes as it shall the CFN to be used after the handover.

To confirm that the UE deactivates compressed mode when it has been ordered to do so in the PHYSICAL CHANNEL RECONFIGURATION message.

To confirm that the UE includes the IE "COUNT-C activation time" and the IE "START list" (in the IE "Uplink counter synchronisation info") in the response message if ciphering is active for any radio bearer using RLC-TM.

8.2.6.37.4	Method of test

Initial Condition

System Simulator: 5 cells – Cell 1 and cell 2 on frequency f1, cell 3 and cell 4 on frequency f2, and cell 5 on frequency f3. Cells 2 and 4 shall have the same primary scrambling code.

UE: "CS-DCCH+DTCH_DCH" (state 6-9) or "PS-DCCH+DTCH_DCH" (state 6-10) as specified in clause 7.4 of TS 34.108, depending on the CN domain supported by the UE. If the UE supports both CS and PS domains, the test case shall be run twice, once starting from state 6-9, once starting from state 6-10. Ciphering shall be activated.

Test Procedure

Table 8.2.6.37-1 illustrates the downlink power to be applied for the 4 cells, as well as the frequency and scrambling code for each cell.

- Table 8.2.6.37-1

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  | Cell 3 |  |  | Cell 4 |  |  | Cell 5 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Frequency |  | f1 |  |  | f1 |  |  | f2 |  |  | f2 |  |  | f3 |  |  |
| Scrambling code |  | Scrambling code 1 |  |  | Scrambling code 2 |  |  | Scrambling code 3 |  |  | Scrambling code 2 |  |  | Scrambling code 4 |  |  |
|  |  | T0 | T1 | T2 | T0 | T1 | T2 | T0 | T1 | T2 | T0 | T1 | T2 | T0 | T1 | T2 |
| CPICH Ec | dBm/3.84 MHz | -60 | -60 | -75 | -95 | -60 | -75 | -60 | -60 | -60 | -60 | -60 | -62 | -50 | -50 | -50 |

The UE is initially in CELL_DCH, and has only cell 1 in its active set.

At instant T1, the downlink power is changed according to what is shown in table 8.2.6.37-1. Cell 2 should then trigger event 1a as has been configured through the default System Information Block Type 11. The UE shall thus send a MEASUREMENT REPORT to the SS, triggered by cell 2.

The SS adds then cell 2 to the active set of the Ue, by sending an ACTIVE SET UPDATE message to the UE. The UE shall answer with an ACTIVE SET UPDATE COMPLETE message.

The SS configures then compressed mode, to prepare the UE for inter-frequency measurements, by sending a PHYSICAL CHANNEL RECONFIGURATION message on DCCH using AM-RLC. The UE shall answer with a PHYSICAL CHANNEL RECONFIGURATION COMPLETE message.

The SS then sets up inter-frequency measurements (event 2b), by sending a MEASUREMENT CONTROL message to the UE. Compressed mode is started at the same time in that message.

At instant T2, the downlink power is changed according to what is shown in table 8.2.6.37-1. Frequency f2 shall then trigger event 2b, and the UE shall transmit a MEASUREMENT REPORT message to the SS.

SS then transmits a PHYSICAL CHANNEL RECONFIGURATION message to the UE on DCCH using AM-RLC, to order it to perform timing reinitialised inter-frequency handover to cell 3 on frequency f2. The UE is also ordered to stop compressed mode after the handover.

The UE shall then  transmit a PHYSICAL CHANNEL RECONFIGURATION COMPLETE message to the SS when the inter-frequency handover has succeeded. In case the initial condition was CS-DCCH+DTCH_DCH, that message shall contain the IEs "COUNT-C activation time" and the IE "START list" (in the IE "Uplink counter synchronisation info"), indicating to the SS when and from which value to start incrementing the HFN part of the COUNT-C variable used for ciphering. The SS shall restart incrementing the HFN part of the COUNT-C variable from the value specified in the IE START from the CFN indicated in the IE COUNT-C activation time.

The SS then waits for 20 seconds, and checks that no MEASUREMENT REPORT is received from the UE.

The SS sends then a MEASUREMENT CONTROL message to the UE, to modify the intra-frequency cell info list of the UE.  About 640 ms after, a MEASUREMENT REPORT message shall be received from the UE, triggered by cell 4. Subsequent MEASUREMENT REPORT messages shall be received at 4 seconds interval.

The SS sends then a MEASUREMENT CONTROL message to the UE that sets up inter-frequency measurements, but does not activate compressed mode in that message. It waits then for 20 seconds, and checks that no MEASUREMENT REPORT message triggered by cell 5 is received.

The SS then sends a  PHYSICAL CHANNEL RECONFIGURATION message to the UE on DCCH using AM-RLC, to order it to perform timing reinitialised inter-frequency handover to cell 5 on frequency f3.

The UE shall then  transmit a PHYSICAL CHANNEL RECONFIGURATION COMPLETE message to the SS when the inter-frequency handover has succeeded. In case the initial condition was CS-DCCH+DTCH_DCH, that message shall contain the IEs "COUNT-C activation time" and the IE "START list" (in the IE "Uplink counter synchronisation info"), indicating to the SS when and from which value to start incrementing the HFN part of the COUNT-C variable used for ciphering.

# Expected Sequence

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 |  |  |  | The SS changes the power of the cells according to column T1 in table 8.2.6.37-1 |
| 2 |  |  | MEASUREMENT REPORT | Event 1a is triggered by cell 2 in the UE, which sends a MEASUREMENT REPORT message to the SS |
| 3 |  |  | ACTIVE SET UPDATE | The SS adds cell 2 to the active set of the UE. |
| 4 |  |  | ACTIVE SET UPDATE COMPLETE | The UE answers with an ACTIVE SET UPDATE COMPLETE message to the SS |
| 5 |  |  | PHYSICAL CHANNEL RECONFIGURATION | The SS downloads the compressed mode parameters in the UE. |
| 6 |  |  | PHYSICAL CHANNEL RECONFIGURATION COMPLETE | The UE acknowleges the downloading of the compressed mode parameters. |
| 7 |  |  | MEASUREMENT CONTROL | The SS configures inter-frequency measurements in the UE, and activates compressed mode. |
| 8 |  |  |  | The SS changes the power of the cells according to column T2 in table 8.2.6.37-1. |
| 9 |  |  | MEASUREMENT REPORT | Frequency f2 triggers event 2b in the UE, which sends a MEASUREMENT REPORT message to the SS. |
| 10 |  |  | PHYSICAL CHANNEL RECONFIGURATION | The SS orders the UE to perform timing re-initialised inter-frequency handover to cell 3 on frequency f2. |
| 11 |  |  | PHYSICAL CHANNEL RECONFIGURATION COMPLETE | After the UE has succeeded in performing the inter-frequency handover, it shall send a PHYSICAL CHANNEL RECONFIGURATION COMPLETE message to the SS using the new configuration. |
| 12 |  |  |  | The SS waits for 20 seconds and monitors that no MEASUREMENT REPORT message is received from the UE. |
| 13 |  |  | MEASUREMENT CONTROL | The SS updates the list of intra-frequency cells in the UE. |
| 14 |  |  | MEASUREMENT REPORT | Cell 4 triggers event 1a in the UE, which sends a MEASUREMENT REPORT message to the SS. Subsequent MEASUREMENT REPORT messages shall be received from the UE at 4 seconds interval. |
| 15 |  |  | MEASUREMENT CONTROL | The SS sets up an inter-frequency measurement in the UE, but does not activate compressed mode. |
| 16 |  |  |  | The SS waits for 20 seconds and monitors that no MEASUREMENT REPORT message is received from the UE. |
| 17 |  |  | PHYSICAL CHANNEL RECONFIGURATION | The SS orders the UE to perform timing re-initialised inter-frequency handover to cell 5 on frequency f3. |
| 18 |  |  | PHYSICAL CHANNEL RECONFIGURATION COMPLETE | After the UE has succeeded in performing the inter-frequency handover, it shall send a PHYSICAL CHANNEL RECONFIGURATION COMPLETE message to the SS using the new configuration. |

# Specific Message Content

All messages indicated below shall use the same content as described in default message content, with the following exceptions:

###### MEASUREMENT REPORT (Step 2)

| Information Element | Value/remark |
| --- | --- |
| Message Type |  |
| Integrity check info | The presence of this IE is dependent on IXIT statements in TS 34.123-2. If integrity protection is indicated to be active, this IE shall be present with the values of the sub IEs as stated below. Else, this IE and the sub-IEs shall be absent. |
| - Message authentication code | This IE is checked to see if it is present. The value is compared against the XMAC-I value computed by SS. |
| - RRC Message sequence number | This IE is checked to see if it is present. The value is used by SS to compute the XMAC-I value. |
| Measurement identity | 1 |
| Measured Results |  |
| - Intra-frequency measured results |  |
| - Cell measured results |  |
| - Cell Identity | Check that this IE is absent |
| - SFN-SFN observed time difference | Check that this IE is absent |
| - Cell synchronisation information | Check that this IE is absent |
| - Primary CPICH info |  |
| - Primary scrambling code | Scrambling code 1 (or scrambling code 2) |
| - CPICH Ec/N0 | Check that this IE is absent |
| - CPICH RSCP | Check that this IE is present |
| - Pathloss | Check that this IE is absent |
| - Cell measured results |  |
| - Cell Identity | Check that this IE is absent |
| - SFN-SFN observed time difference | Check that this IE is absent |
| - Cell synchronisation information | Check that this IE is present and includes IE COUNT-C-SFN frame difference |
| - Primary CPICH info |  |
| - Primary scrambling code | Scrambling code 2 (or scrambling code 1 if the previous scrambling code included by the UE was scrambling code 2) |
| - CPICH Ec/N0 | Check that this IE is absent |
| - CPICH RSCP | Check that this IE is present |
| - Pathloss | Check that this IE is absent |
| Measured results on RACH | Check that this IE is absent |
| Additional measured results | Check that this IE is absent |
| Event results |  |
| - Intra-frequency measurement event results |  |
| - Intra-frequency event identity | 1a |
| - Cell measurement event results |  |
| - Primary CPICH info |  |
| - Primary scrambling code | Scrambling code 2 |

ACTIVE SET UPDATE (Step 3)

The message to be used in this test is defined in TS 34.108, clause 9, with the following exceptions:

| Information Element | Value/remark |
| --- | --- |
| Radio link addition information |  |
| - Primary CPICH Info |  |
| - Primary Scrambling Code | Scrambling code 2 |
| - Downlink DPCH info for each RL | Reference to TS34.108 clause 6.10 Parameter Set |
| - CHOICE mode | FDD |
| - Primary CPICH usage for channel estimation | P-CPICH can be used. |
| - DPCH frame offset | Calculated value from Cell synchronisation information |
| - Secondary CPICH info | Not Present |
| - DL channelisation code |  |
| - Secondary scrambling code | Not Present |
| - Spreading factor | Refer to TS 34.108 clause 6.10.2.4 "Typical radio parameter sets" |
| - Code Number | Any value between 0 and Spreading factor-1 (use different values for each DPCH in case several DPCHs are allocated to the UE). |
| - Scrambling code change | Not Present |
| - TPC Combination Index | 0 |
| - SSDT Cell Identity | Not Present |
| - Close loop timing adjustment mode | Not Present |
| - TFCI Combining Indicator | Not Present |
| - SCCPCH information for FACH | Not Present |

PHYSICAL CHANNEL RECONFIGURATION (Step 5 for the CS case)

| Information Element | Value/Remark |
| --- | --- |
| Activation time | Not Present |
| New U-RNTI | Not Present |
| New C-RNTI | Not Present |
| New DSCH-RNTI | Not Present |
| RRC State indicator | CELL_DCH |
| UTRAN DRX cycle length coefficient | Not Present |
| CN information info | Not Present |
| URA identity | Not Present |
| Downlink counter synchronisation info | Not Present |
| Frequency info | Not Present |
| Maximum allowed UL TX power | Not Present |
| CHOICE channel requirement | Not Present |
| CHOICE mode | FDD |
| - Downlink PDSCH information | Not Present |
| Downlink information common for all radio links |  |
| - Downlink DPCH info common for all RL | Not Present |
| - DPCH compressed mode info - TGPSI - TGPS Status Flag - TGCFN - Transmission gap pattern sequence configuration parameters - TGMP - TGPRC - TGSN - TGL1 - TGL2 - TGD - TGPL1 - TGPL2 - RPP - ITP - CHOICE UL/DL Mode - Downlink compressed mode method - Uplink compressed mode method - Downlink frame type - DeltaSIR1 - DeltaSIRAfter1 - DeltaSIR2 - DeltaSIRAfter2 - N identify abort - T Reconfirm abort | 1 Inactive Not present FDD Measurement Infinity 4 7 Not Present 0 3 Not Present Mode 0 Mode 0 UL and DL, UL only or DL only (depending on the UE capability) SF/2 (or not sent, depending on the UE capability) SF/2 (or not sent, depending on the UE capability) B 2.0 1.0 Not Present Not Present Not Present Not Present |
| - TX Diversity mode | Not Present |
| - SSDT information | Not Present |
| - Default DPCH Offset Value | Not Present |
| Downlink information per radio link list | 2 radio links |
| Downlink information for each radio link |  |
| - CHOICE mode | FDD |
| - Primary CPICH info | Scrambling code 1 |
| - Cell ID | Not present |
| - PDSCH with SHO DCH info | Not present |
| - PDSCH code mapping | Not present |
| - Downlink DPCH info for each RL |  |
| - CHOICE mode | FDD |
| - Primary CPICH usage for channel estimation | Primary CPICH may be used |
| - DPCH frame offset | 0 |
| - Secondary CPICH info | Not present |
| - DL channelisation code |  |
| - Secondary scrambling code | Not present |
| - Spreading factor | Reference to TS34.108 clause 6.10 Parameter Set |
| - Code number | Same as the code currently allocated to the UE in cell 1 |
| - Scrambling code change | Code change |
| - TPC combination index | 0 |
| - SSDT cell identity | Not present |
| - Closed loop timing adjustment mode | Not present |
| Downlink information for each radio link |  |
| - CHOICE mode | FDD |
| - Primary CPICH info | Scrambling code 2 |
| - Cell ID | Not present |
| - PDSCH with SHO DCH info | Not present |
| - PDSCH code mapping | Not present |
| - Downlink DPCH info for each RL |  |
| - CHOICE mode | FDD |
| - Primary CPICH usage for channel estimation | Primary CPICH may be used |
| - DPCH frame offset | 0 |
| - Secondary CPICH info | Not present |
| - DL channelisation code |  |
| - Secondary scrambling code | Not present |
| - Spreading factor | Reference to TS34.108 clause 6.10 Parameter Set |
| - Code number | Same as the code currently allocated to the UE in cell 2 |
| - Scrambling code change | No code change |
| - TPC combination index | 0 |

###### PHYSICAL CHANNEL RECONFIGURATION MESSAGE (Step 5 for the PS case)

| Information Element | Value/Remark |
| --- | --- |
| Activation time | Not Present |
| New U-RNTI | Not Present |
| New C-RNTI | Not Present |
| New DSCH-RNTI | Not Present |
| RRC State indicator | CELL_DCH |
| UTRAN DRX cycle length coefficient | Not Present |
| CN information info | Not Present |
| URA identity | Not Present |
| Downlink counter synchronisation info | Not Present |
| Frequency info | Not Present |
| Maximum allowed UL TX power | Not Present |
| CHOICE channel requirement | Not Present |
| CHOICE mode | FDD |
| - Downlink PDSCH information | Not Present |
| Downlink information common for all radio links |  |
| - Downlink DPCH info common for all RL | Not Present |
| - DPCH compressed mode info - TGPSI - TGPS Status Flag - TGCFN - Transmission gap pattern sequence configuration parameters - TGMP - TGPRC - TGSN - TGL1 - TGL2 - TGD - TGPL1 - TGPL2 - RPP - ITP - CHOICE UL/DL Mode - Downlink compressed mode method - Uplink compressed mode method - Downlink frame type - DeltaSIR1 - DeltaSIRAfter1 - DeltaSIR2 - DeltaSIRAfter2 - N identify abort - T Reconfirm abort | 1 Active (Current CFN+(256 – TTI/10msec)) mod 256 FDD Measurement Infinity 4 7 Not Present 0 3 Not Present Mode 0 Mode 0 UL and DL, UL only or DL only (depending on the UE capability) HLS(or not sent, depending on the UE capability) HLS(or not sent, depending on the UE capability) B 2.0 1.0 Not Present Not Present Not Present Not Present |
| - TX Diversity mode | Not Present |
| - SSDT information | Not Present |
| - Default DPCH Offset Value | Not Present |
| Downlink information for each radio link | N ot Present |

###### MEASUREMENT CONTROL (Step 7)

| Information elements Measurement Identity Measurement Command Measurement Reporting Mode - Measurement Reporting Transfer Mode - Periodical Reporting / Event Trigger Reporting Mode Additional measurements list CHOICE measurement type - Inter-frequency cell info list - CHOICE inter-frequency cell removal - New inter-frequency info list - Inter-frequency cell id - Frequency info - UARFCN uplink (Nu) - UARFCN downlink (Nd) - Cell info - Cell individual offset - Reference time difference to cell - Read SFN Indicator - CHOICE Mode - Primary CPICH Info - Primary Scrambling Code - Primary CPICH TX power - TX Diversity Indicator - Inter-frequency cell id - Frequency info - UARFCN uplink (Nu) - UARFCN downlink (Nd) - Cell info - Cell individual offset - Reference time difference to cell - Read SFN Indicator - CHOICE Mode - Primary CPICH Info - Primary Scrambling Code - Primary CPICH TX power - TX Diversity Indicator - Cells for measurement - Inter-frequency measurement quantity - CHOICE reporting criteria - Filter Coefficient - Measurement quantity for frequency quality estimate - Inter-frequency reporting quantity - UTRA Carrier RSSI - Frequency quality estimate - Non frequency related cell reporting quantities - SFN-SFN observed time difference reporting indicator - Cell synchronisation information reporting indicator - Cell Identity reporting indicator - CPICH Ec/No reporting indicator - CPICH RSCP reporting indicator - Pathloss reporting indicator - Reporting cell status - Measurement validity - UE State - Inter-frequency set update - UE autonomous update - Non autonomous update mode - CHOICE report criteria - Parameters required for each event - Inter-frequency event identity - Threshold used frequency - W used frequency - Hysteresis - Time to trigger | Value/Remark 2 Setup Acknowledged Mode RLC Event Trigger Not Present Inter-frequency measurement No inter-frequency cells removed 2 inter-frequency cells 3 UARFCN for the uplink corresponding to f2 UARFCN for the downlink corresponding to f2 0 dB Not present FALSE FDD Scrambling code 3 Not Present FALSE 4 UARFCN for the uplink corresponding to f2 UARFCN for the downlink corresponding to f2 0 dB Not present FALSE FDD Not present Scrambling code 2 Not Present FALSE Not present Inter-frequency reporting criteria 0 CPICH RSCP FALSE FALSE No report TRUE TRUE FALSE TRUE FALSE Not present CELL_DCH On with no reporting Not present Inter-frequency measurement reporting criteria 2b -70 dBm 0.0 1.0 dB 10 seconds |
| --- | --- |
| - Reporting cell status - CHOICE reported cell - Maximum number of reported cells - Parameters required for each non-used frequency - Threshold non used frequency - W non-used frequency DPCH compressed mode status info - TGPS reconfiguration CFN - Transmission gap pattern sequence - TGPSI - TGPS Status Flag - TGCFN | Report cells within active and/or monitored set on used frequency or within active and/or monitored set on non-used frequency 2 1 frequency -70 dBm 0.0 (Current CFN+(256 – TTI/10msec)) mod 256 1 sequence 1 Active (Current CFN+(256 – TTI/10msec)) mod 256 |

###### MEASUREMENT REPORT (Step 9)

| Information Element | Value/remark |
| --- | --- |
| Message Type |  |
| Integrity check info | The presence of this IE is dependent on IXIT statements in TS 34.123-2. If integrity protection is indicated to be active, this IE shall be present with the values of the sub IEs as stated below. Else, this IE and the sub-IEs shall be absent. |
| - Message authentication code | This IE is checked to see if it is present. The value is compared against the XMAC-I value computed by SS. |
| - RRC Message sequence number | This IE is checked to see if it is present. The value is used by SS to compute the XMAC-I value. |
| Measurement identity | 2 |
| Measured Results |  |
| - Inter-frequency measured results list |  |
| - Frequency info |  |
| -CHOICE mode | FDD |
| - UARFCN uplink | Check that the value of this IE is set to UARFCN for the uplink corresponding to f2 (Could be absent in case the duplex distance is 190 MHz) |
| - UARFCN downlink | Check that the value of this IE is set to UARFCN for the downlink corresponding to f2 |
| - UTRA carrier RSSI | Check that this IE is absent |
| - Inter-frequency cell measurement results | Check that the value of this IE is set to 2 cells reported |
| - Cell measured results |  |
| - Cell Identity | Check that this IE is absent |
| - SFN-SFN observed time difference | Check that this IE is absent |
| - Cell synchronisation information | Check that this IE is absent |
| - Primary CPICH info |  |
| - Primary scrambling code | Check that the value of this IE is set to Scrambling code 3 (or scrambling code 2) |
| - CPICH Ec/N0 | Check that this IE is absent |
| - CPICH RSCP | Check that this IE is present |
| - Pathloss | Check that this IE is absent |
| - Cell measured results |  |
| - Cell Identity | Check that this IE is absent |
| - SFN-SFN observed time difference | Check that this IE is absent |
| - Cell synchronisation information | Check that this IE is present and includes IE COUNT-C-SFN frame difference |
| - Primary CPICH info |  |
| - Primary scrambling code | Check that the value of this IE is set to Scrambling code 2 (or scrambling code 3 if the previous scrambling code included by the UE was scrambling code 2) |
| - CPICH Ec/N0 | Check that this IE is absent |
| - CPICH RSCP | Check that this IE is present |
| - Pathloss | Check that this IE is absent |
| Measured results on RACH | Check that this IE is absent |
| Additional measured results | Check that this IE is absent |
| Event results |  |
| - Inter-frequency measurement event results |  |
| - Intra-frequency event identity | 2b |
| - Cell measurement event results |  |
| - Primary CPICH info |  |
| - Primary scrambling code | Check that the value of this IE is set to Scrambling code 3 (or scrambling code 2) |

###### PHYSICAL CHANNEL RECONFIGURATION (Step 10)

| Information Element | Value/Remark |
| --- | --- |
| Activation time | Not Present |
| New U-RNTI | Not Present |
| New C-RNTI | Not Present |
| New DSCH-RNTI | Not Present |
| RRC State indicator | CELL_DCH |
| UTRAN DRX cycle length coefficient | Not Present |
| CN information info | Not Present |
| URA identity | Not Present |
| Downlink counter synchronisation info | Not Present |
| Frequency info |  |
| - CHOICE mode | FDD |
| - UARFCN uplink (Nu) | UARFCN for the uplink corresponding to f2 (Could be absent in case the duplex distance is 190 MHz) |
| - UARFCN downlink (Nd) | UARFCN for the downlink corresponding to f2 |
| Maximum allowed UL TX power | Not Present |
| CHOICE channel requirement | Not Present |
| CHOICE mode | FDD |
| - Downlink PDSCH information | Not Present |
| Downlink information common for all radio links |  |
| - Downlink DPCH info common for all RL |  |
| - Timing indication | Initialise |
| - CFN-targetSFN frame offset | 0 |
| - Downlink DPCH power control information | Not Present |
| - Downlink rate matching restriction information | Not Present |
| - Spreading factor | Reference to TS34.108 clause 6.10 Parameter Set |
| - Fixed or flexible position | Reference to TS34.108 clause 6.10 Parameter Set |
| - TFCI existence | Reference to TS34.108 clause 6.10 Parameter Set |
| - CHOICE SF | Reference to TS34.108 clause 6.10 Parameter Set |
| - DPCH compressed mode info - TGPSI - TGPS Status Flag - TGCFN - Transmission gap pattern sequence configuration parameters | 1 Inactive Not present Not present |
| - TX Diversity mode | Not Present |
| - SSDT information | Not Present |
| - Default DPCH Offset Value | Arbitrary value between 0..306688 by step of 512 |
| Downlink information per radio link list | 1 radio link |
| Downlink information for each radio link |  |
| - CHOICE mode | FDD |
| - Primary CPICH info | Scrambling code 3 |
| - Cell ID | Not present |
| - PDSCH with SHO DCH info | Not present |
| - PDSCH code mapping | Not present |
| - Downlink DPCH info for each RL |  |
| - CHOICE mode | FDD |
| - Primary CPICH usage for channel estimation | Primary CPICH may be used |
| - DPCH frame offset | Set to value of DPCH Frame Offset modulo 38400 |
| - Secondary CPICH info | Not present |
| - DL channelisation code | Reference to TS34.108 clause 6.10 Parameter Set |
| - Secondary scrambling code | Not present |
| - Spreading factor | Reference to TS34.108 clause 6.10 Parameter Set |
| - Code number | Any value between 0 and Spreading factor-1 |
| - Scrambling code change | Not Present |
| - TPC combination index | 0 |
| - SSDT cell identity | Not present |
| - Closed loop timing adjustment mode | Not present |

###### PHYSICAL CHANNEL RECONFIGURATION COMPLETE (Steps 11 and 18 for the CS case)

| Information Element | Value/remark |  |
| --- | --- | --- |
| Message Type |  |  |
| RRC transaction identifier | Checked to see if it is set to identical value of the same IE in the downlink PHYSICAL CHANNEL RECONFIGURATION message |  |
| Integrity check info | The presence of this IE is dependent on IXIT statements in TS 34.123-2. If integrity protection is indicated to be active, this IE shall be present with the values of the sub IEs as stated below. Else, this IE and the sub-IEs shall be absent. |  |
| - Message authentication code | This IE is checked to see if it is present. The value is compared against the XMAC-I value computed by SS. |  |
| - RRC Message sequence number | This IE is checked to see if it is present. The value is used by SS to compute the XMAC-I value. |  |
| Uplink integrity protection activation info | Check that not present |  |
| CHOICE mode | FDD |  |
| COUNT-C activation time | Check that this IE shall be present |  |
| Radio bearer uplink ciphering activation time info | Check that not present |  |
| Uplink counter synchronisation info | Check that present |  |
| >RB with PDCP information list | Check that absent |  |
| >START list | Check that this IE is set to 1 |  |
| >>CN Domain identity | Check that this IE is set to CS Domain |  |
| >>START | Not checked |  |

###### PHYSICAL CHANNEL RECONFIGURATION COMPLETE (Steps 11 and 18 for the PS case)

| Information Element | Value/remark |  |
| --- | --- | --- |
| Message Type |  |  |
| RRC transaction identifier | Checked to see if it is set to identical value of the same IE in the downlink PHYSICAL CHANNEL RECONFIGURATION message |  |
| Integrity check info | The presence of this IE is dependent on IXIT statements in TS 34.123-2. If integrity protection is indicated to be active, this IE shall be present with the values of the sub IEs as stated below. Else, this IE and the sub-IEs shall be absent. |  |
| - Message authentication code | This IE is checked to see if it is present. The value is compared against the XMAC-I value computed by SS. |  |
| - RRC Message sequence number | This IE is checked to see if it is present. The value is used by SS to compute the XMAC-I value. |  |
| Uplink integrity protection activation info | Check that not present |  |
| CHOICE mode | FDD |  |
| COUNT-C activation time | Check that not present |  |
| Radio bearer uplink ciphering activation time info | Check that not present |  |
| Uplink counter synchronisation info | Check that not present |  |

###### MEASUREMENT CONTROL (Step 13)

| Information Element | Value/Remark |
| --- | --- |
| Measurement Identity Measurement Command Measurement Reporting Mode Additional measurements list CHOICE measurement type - Intra-frequency cell info list - CHOICE intra-frequency cell removal - New intra-frequency info list - Intra-frequency cell id - Cell info - Cell individual offset - Reference time difference to cell - Read SFN Indicator - CHOICE mode - Primary CPICH Info - Primary Scrambling Code - Primary CPICH TX power - TX Diversity Indicator - Cells selection and Re-selection info - Intra-frequency cell id - Cell info - Cell individual offset - Reference time difference to cell - Read SFN Indicator - CHOICE mode - Primary CPICH Info - Primary Scrambling Code - Primary CPICH TX power - TX Diversity Indicator - Cells selection and Re-selection info - Cells for measurement - Intra-frequency measurement quantity - Intra-frequency reporting quantity - Reporting cell status - Measurement validity - CHOICE report criteria | 1 Modify Not present Not Present Intra-frequency measurement Remove all intra-frequency cells 2 new intra-frequency cells 3 0 dB Not present FALSE FDD Scrambling code 3 (for cell 3) Not Present FALSE Not Present 4 0 dB Not present FALSE FDD Scrambling code 2 (for cell 4) Not Present FALSE Not Present Not Present Not Present Not Present Not Present Not Present Not Present |

###### MEASUREMENT REPORT (Step 14)

| Information Element | Value/remark |
| --- | --- |
| Message Type |  |
| Integrity check info | The presence of this IE is dependent on IXIT statements in TS 34.123-2. If integrity protection is indicated to be active, this IE shall be present with the values of the sub IEs as stated below. Else, this IE and the sub-IEs shall be absent. |
| - Message authentication code | This IE is checked to see if it is present. The value is compared against the XMAC-I value computed by SS. |
| - RRC Message sequence number | This IE is checked to see if it is present. The value is used by SS to compute the XMAC-I value. |
| Measurement identity | 1 |
| Measured Results |  |
| - Intra-frequency measured results |  |
| - Cell measured results |  |
| - Cell Identity | Check that this IE is absent |
| - SFN-SFN observed time difference | Check that this IE is absent |
| - Cell synchronisation information | Check that this IE is absent |
| - Primary CPICH info |  |
| - Primary scrambling code | Check that this IE is set to Scrambling code 2 (or scrambling code 3) |
| - CPICH Ec/N0 | Check that this IE is absent |
| - CPICH RSCP | Check that this IE is present |
| - Pathloss | Check that this IE is absent |
| - Cell measured results |  |
| - Cell Identity | Check that this IE is absent |
| - SFN-SFN observed time difference | Check that this IE is absent |
| - Cell synchronisation information | Check that this IE is present and includes IE COUNT-C-SFN frame difference |
| - Primary CPICH info |  |
| - Primary scrambling code | Check that this IE is set to Scrambling code 3 (or scrambling code 2 if scrambling code 3 was indicated first) |
| - CPICH Ec/N0 | Checked that this IE is absent |
| - CPICH RSCP | Checked that this IE is present |
| - Pathloss | Checked that this IE is absent |
| Measured results on RACH | Checked that this IE is absent |
| Additional measured results | Checked that this IE is absent |
| Event results |  |
| - Intra-frequency measurement event results |  |
| - Intra-frequency event identity | 1a |
| - Cell measurement event results |  |
| - Primary CPICH info |  |
| - Primary scrambling code | Check that this IE is set to Scrambling code 2 |

###### MEASUREMENT CONTROL (Step 15)

| Measurement Identity Measurement Command Measurement Reporting Mode - Measurement Reporting Transfer Mode - Periodical Reporting / Event Trigger Reporting Mode Additional measurements list CHOICE measurement type - Inter-frequency cell info list - CHOICE inter-frequency cell removal - New inter-frequency info list - Inter-frequency cell id - Frequency info - UARFCN uplink (Nu) - UARFCN downlink (Nd) - Cell info - Cell individual offset - Reference time difference to cell - Read SFN Indicator - CHOICE Mode - Primary CPICH Info - Primary Scrambling Code - Primary CPICH TX power - TX Diversity Indicator - Inter-frequency cell id - Frequency info - UARFCN uplink (Nu) - UARFCN downlink (Nd) - Cell info - Cell individual offset - Reference time difference to cell - Read SFN Indicator - CHOICE Mode - Primary CPICH Info - Primary Scrambling Code - Primary CPICH TX power - TX Diversity Indicator - Cells for measurement - Inter-frequency measurement quantity - CHOICE reporting criteria - Filter Coefficient - Measurement quantity for frequency quality estimate - Inter-frequency reporting quantity - UTRA Carrier RSSI - Frequency quality estimate - Non frequency related cell reporting quantities - SFN-SFN observed time difference reporting indicator - Cell synchronisation information reporting indicator - Cell Identity reporting indicator - CPICH Ec/No reporting indicator - CPICH RSCP reporting indicator - Pathloss reporting indicator - Reporting cell status - Measurement validity - UE State - Inter-frequency set update - UE autonomous update - Non autonomous update mode - CHOICE report criteria - Parameters required for each event - Inter-frequency event identity - Threshold used frequency - W used frequency - Hysteresis - Time to trigger | 2 Setup Acknowledged Mode RLC Event Trigger Not Present Inter-frequency measurement No inter-frequency cells removed 2 inter-frequency cells 1 UARFCN for the uplink corresponding to f1 UARFCN for the downlink corresponding to f1 0 dB Not present FALSE FDD Scrambling code 1 Not Present FALSE 2 UARFCN for the uplink corresponding to f1 UARFCN for the downlink corresponding to f1 0 dB Not present FALSE FDD Not present Scrambling code 2 Not Present FALSE Not present Inter-frequency reporting criteria 0 CPICH RSCP FALSE FALSE No report FALSE TRUE FALSE TRUE FALSE Not present CELL_DCH On with no reporting Not present Inter-frequency measurement reporting criteria 2c Not present Not present 1.0 dB 10 seconds |
| --- | --- |
| - Reporting cell status - CHOICE reported cell - Maximum number of reported cells - Parameters required for each non-used frequency - Threshold non used frequency - W non-used frequency DPCH compressed mode status info | Report cells within active and/or monitored set on used frequency or within active and/or monitored set on non-used frequency 2 1 frequency -90 dBm 0.0 Not present |

###### PHYSICAL CHANNEL RECONFIGURATION (Step 17)

| Information Element | Value/Remark |
| --- | --- |
| Activation time | Not Present |
| New U-RNTI | Not Present |
| New C-RNTI | Not Present |
| New DSCH-RNTI | Not Present |
| RRC State indicator | CELL_DCH |
| UTRAN DRX cycle length coefficient | Not Present |
| CN information info | Not Present |
| URA identity | Not Present |
| Downlink counter synchronisation info | Not Present |
| Frequency info |  |
| - CHOICE mode | FDD |
| - UARFCN uplink (Nu) | UARFCN for the uplink corresponding to f3 (Could be absent in case the duplex distance is 190 MHz) |
| - UARFCN downlink (Nd) | UARFCN for the downlink corresponding to f3 |
| Maximum allowed UL TX power | Not Present |
| CHOICE channel requirement | Not Present |
| CHOICE mode | FDD |
| - Downlink PDSCH information | Not Present |
| Downlink information common for all radio links |  |
| - Downlink DPCH info common for all RL | Not Present |
| - Timing indication | Initialise |
| - CFN-targetSFN frame offset | 0 |
| - Downlink DPCH power control information | Not Present |
| - Downlink rate matching restriction information | Not Present |
| - Spreading factor | Reference to TS34.108 clause 6.10 Parameter Set |
| - Fixed or flexible position | Reference to TS34.108 clause 6.10 Parameter Set |
| - TFCI existence | Reference to TS34.108 clause 6.10 Parameter Set |
| - CHOICE SF | Reference to TS34.108 clause 6.10 Parameter Set |
| - DPCH compressed mode info | Not present |
| - TX Diversity mode | Not Present |
| - SSDT information | Not Present |
| - Default DPCH Offset Value | Arbitrary set to value 0..306688 by step of 512 |
| Downlink information per radio link list | 1 radio link |
| Downlink information for each radio link |  |
| - CHOICE mode | FDD |
| - Primary CPICH info | Scrambling code 4 |
| - Cell ID | Not present |
| - PDSCH with SHO DCH info | Not present |
| - PDSCH code mapping | Not present |
| - Downlink DPCH info for each RL |  |
| - CHOICE mode | FDD |
| - Primary CPICH usage for channel estimation | Primary CPICH may be used |
| - DPCH frame offset | Set to value of DPCH Frame Offset modulo 38400 |
| - Secondary CPICH info | Not present |
| - DL channelisation code | Reference to TS34.108 clause 6.10 Parameter Set |
| - Secondary scrambling code | Not present |
| - Spreading factor | Reference to TS34.108 clause 6.10 Parameter Set |
| - Code number | Any value between 0 and Spreading factor-1 |
| - Scrambling code change | Not Present |
| - TPC combination index | 0 |
| - SSDT cell identity | Not present |
| - Closed loop timing adjustment mode | Not present |

8.2.6.37.5	Test Requirement

After step 1, the UE shall send a MEASUREMENT REPORT message triggered by event 1a for cell 2.

After step 3, the UE shall send an ACTIVE SET UPDATE COMPLETE message to acknowledge that it has added cell 2 to its active set.

After step 5, the UE shall transmit a PHYSICAL CHANNEL RECONFIGURATION COMPLETE message to the SS to acknowledge the downloading of the compressed mode parameters that were sent in the PHYSICAL CHANNEL RECONFIGURATION message of step 4.

After step 8, the UE shall transmit a MEASUREMENT REPORT message triggered by frequency f2. In that message, cell 3 shall be the only cell included in the IE event results.

After step 10, the UE shall send a PHYSICAL CHANNEL RECONFIGURATION COMPLETE message to the SS in cell 3 to acknowledge the successful timing re-initialised inter-frequency handover, and in case the UE was in state 6-9 as defined in clause 7.4 of 34.108 as initial condition to the test, the IEs “COUNT-C activation time” and “START list” (in the IE “Uplink counter synchronisation info”) shall be included in that message. The UE shall also start incrementing the HFN part of the COUNT-C variable from the value specified in the IE START from the CFN indicated in the IE COUNT-C activation time.

After step 11, the UE shall not send any MEASUREMENT REPORT message triggered by event 1a for cell 4.

After step 13, the UE send a MEASUREMENT REPORT message triggered by event 1a for cell 4.

After step 15, the UE shall not send any MEASUREMENT REPORT message triggered by event 2c for frequency 1.

After step 17, the UE shall send a PHYSICAL CHANNEL RECONFIGURATION COMPLETE message to the SS in cell 3 to acknowledge the successful timing re-initialised inter-frequency handover, and in case the UE was in state 6-9 as defined in clause 7.4 of 34.108 as initial condition to the test, the IEs “COUNT-C activation time” and “START list” (in the IE “Uplink counter synchronisation info”) shall be included in that message.