3GPP TSG-T WG1 Meeting #20 	Tdoc T1-031176

- Munich, Germany, 28th July – 1st August 2003

| CR-Form-v7 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.108 | CR | -- |  rev | 1 |  Current version: | 3.12.0 |  |
| For HELP on using this form, see bottom of this page or look at the pop-up text over the  symbols. |  |  |  |  |  |  |  |  |

| Proposed change affects:  | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title:  | CR to 34.108 REL-99; Correction to section 7.3 Test procedures for RF test |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source:  | 7 layers AG |  |  |  |  |  |  |  |  |  |
| Work item code:  | TEI |  |  |  |  |  |  | Date:  |  | 22/07/2003 |
| Category:  | F |  |  |  |  |  |  | Release:  |  | Rel-99 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: 2 (GSM Phase 2) R96 (Release 1996) R97 (Release 1997) R98 (Release 1998) R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) |  |
| Reason for change:  |  | According to 3GPP TS 33.102, Sections 6.4.5 - 6.5.1 (V3.13, 2002-12 or earlier), the allowed procedures to be performed before the execution of the security procedure to start integrity are explicied defined. This has been taken into account in the sections 7.2 and 7.4 of the current 34.108. In contrast to this section 7.3 describes test procedures for RF test which are not conform to 33.102 and will cause problems with a complient UE. So avoid this the - Authentication procedure and - Security procedure should be added as used in 7.2 and 7.4 to be in line with 33.102. The Authentication procedure is required to assure the successfully execution of the security procedure Otherwise a non-conformant UE would be required to perform the signalling procedures for the RF tests. |  |  |  |  |  |  |  |  |
| Summary of change:  |  | - 7.3.2.1 / 7.3.3.1 - refer to the valid procedures to assure the UE being in idle mode and updated/attached using the valid procedures - 7.3.2.3 / 7.3.3.3 - Add the required steps for Authentication and security |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  | Test procedure inconsistent with core specifications |  |  |  |  |  |  |  |  |
| Clauses affected:  |  | 7.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs  |  |  | X | Other core specifications  |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
|  |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments:  |  | REL-5 must also be checked. |  |  |  |  |  |  |  |  |

- How to create CRs using this form:

Comprehensive information and tips about how to create CRs can be found at http://www.3gpp.org/specs/CR.htm.  Below is a brief summary:

1)	Fill out the above form. The symbols above marked  contain pop-up help information about the field that they are closest to.

2)	Obtain the latest version for the release of the specification to which the change is proposed. Use the MS Word "revision marks"  feature (also known as "track changes") when making the changes. All 3GPP specifications can be downloaded from the 3GPP server under ftp://ftp.3gpp.org/specs/ For the latest version, look for the directory name with the latest date e.g. 2001-03 contains the specifications resulting from the March 2001 TSG meetings.

3)	With "track changes" disabled, paste the entire CR form (use CTRL-A to select it) into the specification just in front of the clause containing the first piece of changed text.  Delete those parts of the specification which are not relevant to the change request.

## 7.3	Test procedures for RF test

### 7.3.1	UE Test States for RF testing

In this clause, the states of the UE for the test are defined.

|  |  | RRC | CC | MM | SM | GMM |
| --- | --- | --- | --- | --- | --- | --- |
| State1 | Power OFF | ----- | null | detached | inactive | detached |
| State2 | CS Registered Idle Mode | idle | null | idle | inactive | detached |
| State3 | PS Registered Idle Mode | idle | null | detached | inactive | idle |
| State4 | Test Mode | connected | null | detached | inactive | detached |

### 7.3.2	Test procedure for TX, RX and Performance Requirement (without handover)

#### 7.3.2.1 	Initial conditions

System Simulator

-	1cell, default parameters.

User Equipment

The UE shall be operated under RF test conditions.

The Test-USIM shall be inserted.

The UE has a valid TMSI (CS) after the execution of the procedure described in 7.2.2.1

The UE has a valid P-TMSI (PS) after the execution of the procedure described in 7.2.2.2

#### 7.3.2.2	Definition of system information messages

The default system information messages specified in clause 6.1 are used with the following exceptions.

# Contents of System information block type 1: RRC

| Information Element | Value/remark |
| --- | --- |
| - CN domain system information |  |
| - CN domain identity | PS |
| - CHOICE CN Type | GSM-MAP |
| - CN domain specific NAS system information |  |
| - GSM-MAP NAS system information | 05 00 |
| - CN domain specific DRX cycle length coefficient | 7 |
| - CN domain identity | CS |
| - CHOICE CN Type | GSM-MAP |
| - CN domain specific NAS system information |  |
| - GSM-MAP NAS system information | 00(T3212 is set to infinity) 01 |
| - CN domain specific DRX cycle length coefficient | 7 |
| - UE Timers and constants in connected mode |  |
| - T305 | Infinity |

#### 7.3.2.3	Procedure

For UE supporting CS

| Step | Direction |  | Message | Comments |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 | <-- |  | SYSTEM INFORMATION (BCCH) | Broadcast |
| 2 | <-- |  | PAGING TYPE1 (PCCH) | Paging (CS domain, TMSI) |
| 3 | --> |  | RRC CONNECTION REQUEST (CCCH) | RRC |
| 4 | <-- |  | RRC CONNECTION SETUP (CCCH) | RRC |
| 5 | --> |  | RRC CONNECTION SETUP COMPLETE (DCCH) | RRC |
| 6 | --> |  | PAGING RESPONSE | RR |
| 7 | <-- |  | AUTHENTICATION REQUEST | MM |
| 8 | --> |  | AUTHENTICATION RESPONSE | MM |
| 9 | <-- |  | SECURITY MODE COMMAND | RRC |
| 10 | --> |  | SECURITY MODE COMPLETE | RRC |
| 11 | <-- |  | ACTIVATE RB TEST MODE | TC |
| 12 | --> |  | ACTIVATE RB TEST MODE COMPLETE | TC |
| 13 | <-- |  | RADIO BEARER SETUP | RRC (RAB SETUP) |
| 14 | --> |  | RADIO BEARER SETUP COMPLETE | RRC |
| 15 | <-- |  | CLOSE UE TEST LOOP (DCCH) | TC (UE test loop mode set up) |
| 16 | --> |  | CLOSE UE TEST LOOP COMPLETE | TC (confirms that loopback entities for the radio bearer(s) have been created and loop back is activated) |
| 17 | <-- |  | OPEN UE TEST LOOP | TC |
| 18 | --> |  | OPEN UE TEST LOOP COMPLETE | TC |
| 19 | <-- |  | RRC CONNECTION RELEASE | RRC |
| 20 | --> |  | RRC CONNECTION RELEASE COMPLETE | RRC |

For UE supporting PS only

| Step | Direction |  | Message | Comments |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 | <-- |  | SYSTEM INFORMATION (BCCH) | Broadcast |
| 2 | <-- |  | PAGING TYPE1 (PCCH) | Paging (PS domain, P-TMSI) |
| 3 | --> |  | RRC CONNECTION REQUEST (CCCH) | RRC |
| 4 | <-- |  | RRC CONNECTION SETUP (CCCH) | RRC |
| 5 | --> |  | RRC CONNECTION SETUP COMPLETE (DCCH) | RRC |
| 6 | --> |  | SERVICE REQUEST | GMM |
| 7 | <-- |  | AUTHENTICATION AND CIPHERING REQUEST | GMM |
| 8 | --> |  | AUTHENTICATION AND CIPHERING RESPONSE | GMM |
| 9 | <-- |  | SECURITY MODE COMMAND | RRC |
| 10 | --> |  | SECURITY MODE COMPLETE | RRC |
| 11 | <-- |  | ACTIVATE RB TEST MODE | TC |
| 12 | --> |  | ACTIVATE RB TEST MODE COMPLETE | TC |
| 13 | <-- |  | RADIO BEARER SETUP | RRC (RAB SETUP) |
| 14 | --> |  | RADIO BEARER SETUP COMPLETE | RRC |
| 15 | <-- |  | CLOSE UE TEST LOOP (DCCH) | TC (UE test loop mode set up) |
| 16 | --> |  | CLOSE UE TEST LOOP COMPLETE | TC (confirms that loopback entities for the radio bearer(s) have been created and loop back is activated) |
| 17 | <-- |  | OPEN UE TEST LOOP | TC |
| 18 | --> |  | OPEN UE TEST LOOP COMPLETE | TC |
| 19 | <-- |  | RRC CONNECTION RELEASE | RRC |
| 20 | --> |  | RRC CONNECTION RELEASE COMPLETE | RRC |

#### 7.3.2.4	Specific message contents

The default message contents specified in clause 9.2 are used with the following exceptions.

##### 7.3.2.4.1	ATTCH ACCEPT

This message is sent from the SS  to the UE, used for the UE supporting PS only.

Contents of Attach Accept message: GMM

| Information Element | Value/remark |
| --- | --- |
| Periodic RA update timer | E0 (timer is deactivated) |

##### 7.3.2.4.2	Reference measurement channels

The configurations of the reference measurement channels for RF tests are described in TS 34.121 [2] Annex C for FDD and TS 34.122 [5] Annex C for TDD.

##### 7.3.2.4.3	UE test loop mode

The messages in this sub-clause are sent from the SS to the UE, determining the UE test loop mode for the RF tests.

UE test loop mode 1 without DCCH dummy transmission

Default. See clause 9.2.

UE test loop mode 1 with DCCH dummy transmission

Contents of CLOSE UE TEST LOOP: TC

| Information Element | Value/remark |
| --- | --- |
| UE test loop mode | UE test loop mode 1 DCCH dummy transmission set to "enabled". 00000100B |

UE test loop mode 2 without DCCH dummy transmission

Contents of CLOSE UE TEST LOOP: TC

| Information Element | Value/remark |
| --- | --- |
| UE test loop mode | UE test loop mode 2 DCCH dummy transmission set to "disabled". 00000001B |

##### 7.3.2.4.4	Compressed mode

[T.B.D.]

##### 7.3.2.4.5	Transmit diversity mode

[T.B.D.]

### 7.3.3	Test procedure for Rx Spurious Emission

#### 7.3.3.1 	Initial conditions

System Simulator

-	1cell, default parameters.

User Equipment

The UE shall be operated under RF test conditions.

The Test-USIM shall be inserted.

The UE has a valid TMSI (CS) after the execution of the procedure described in 7.2.2.1

The UE has a valid P-TMSI (PS) after the execution of the procedure described in 7.2.2.2

#### 7.3.3.2	Definition of system information messages

The default system information messages specified in clause 6.1 are used with the following exceptions.

# Contents of System information block type 1: RRC

| Information Element | Value/remark |
| --- | --- |
| - CN domain system information |  |
| - CN domain identity | PS |
| - CHOICE CN Type | GSM-MAP |
| - CN domain specific NAS system information |  |
| - GSM-MAP NAS system information | 05 00 |
| - CN domain specific DRX cycle length coefficient | 7 |
| - CN domain identity | CS |
| - CHOICE CN Type | GSM-MAP |
| - CN domain specific NAS system information |  |
| - GSM-MAP NAS system information | 00(T3212 is set to infinity) 01 |
| - CN domain specific DRX cycle length coefficient | 7 |
| - UE Timers and constants in connected mode |  |
| - T305 | Infinity |

#### 7.3.3.3	Procedure

For UE supporting CS

| Step | Direction |  | Message | Comments |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 | <-- |  | SYSTEM INFORMATION (BCCH) | Broadcast |
| 2 | <-- |  | PAGING TYPE1 (PCCH) | Paging (CS domain, TMSI) |
| 3 | --> |  | RRC CONNECTION REQUEST (CCCH) | RRC |
| 4 | <-- |  | RRC CONNECTION SETUP (CCCH) | RRC |
| 5 | --> |  | RRC CONNECTION SETUP COMPLETE (DCCH) | RRC |
| 6 | --> |  | PAGING RESPONSE | RR |
| 7 | <-- |  | AUTHENTICATION REQUEST | MM |
| 8 | --> |  | AUTHENTICATION RESPONSE | MM |
| 9 | <-- |  | SECURITY MODE COMMAND | RRC |
| 10 | --> |  | SECURITY MODE COMPLETE | RRC |
| 11 | <-- |  | ACTIVATE RB TEST MODE | TC |
| 12 | --> |  | ACTIVATE RB TEST MODE COMPLETE | TC |
| 13 | <-- |  | RADIO BEARER SETUP | RRC - RAB SETUP using Reference Radio Bearer Configuration - RRC state indicator is set to "CELL_FACH" |
| 14 | --> |  | RADIO BEARER SETUP COMPLETE | RRC |
| 15 | <-- |  | RRC CONNECTION RELEASE | RRC |
| 16 | --> |  | RRC CONNECTION RELEASE COMPLETE | RRC |

For UE supporting PS only

| Step | Direction |  | Message | Comments |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 | <-- |  | SYSTEM INFORMATION (BCCH) | Broadcast |
| 2 | <-- |  | PAGING TYPE1 (PCCH) | Paging (PS domain, P-TMSI) |
| 3 | --> |  | RRC CONNECTION REQUEST (CCCH) | RRC |
| 4 | <-- |  | RRC CONNECTION SETUP (CCCH) | RRC |
| 5 | --> |  | RRC CONNECTION SETUP COMPLETE (DCCH) | RRC |
| 6 | --> |  | SERVICE REQUEST | GMM |
| 7 | <-- |  | AUTHENTICATION AND CIPHERING REQUEST | GMM |
| 8 | --> |  | AUTHENTICATION AND CIPHERING RESPONSE | GMM |
| 9 | <-- |  | SECURITY MODE COMMAND | RRC |
| 10 | --> |  | SECURITY MODE COMPLETE | RRC |
| 11 | <-- |  | ACTIVATE RB TEST MODE | TC |
| 12 | --> |  | ACTIVATE RB TEST MODE COMPLETE | TC |
| 13 | <-- |  | RADIO BEARER SETUP | RRC - RAB SETUP using Reference Radio Bearer Configuration - RRC state indicator is set to "CELL_FACH" |
| 14 | --> |  | RADIO BEARER SETUP COMPLETE | RRC |
| 15 | <-- |  | RRC CONNECTION RELEASE | RRC |
| 16 | --> |  | RRC CONNECTION RELEASE COMPLETE | RRC |

#### 7.3.3.4	Specific message contents

The default message contents specified in clause 9.2 are used with the following exceptions.

Contents of RADIO BEARER SETUP message: RRC

| Information Element | Value/remark |
| --- | --- |
| New C-RNTI | '1010 1010 1010 1010' |
| RRC State indicator | CELL_FACH |

Contents of Attach Accept message: GMM

| Information Element | Value/remark |
| --- | --- |
| Periodic RA update timer | E0 (timer is deactivated) |

### 7.3.4	Test procedure for Handover

FFS

### 7.3.5	Test procedure for Measurement Performance Requirement

FFS