3GPP TSG-T1 Meeting #20 	T1 030604

Munich, Germany 28 July-1August 2003

| CR-Form-v7 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | TS 34.123-3 | CR |  |  rev |  |  Current version: | 3.1.0 |  |
| For HELP on using this form, see bottom of this page or look at the pop-up text over the  symbols. |  |  |  |  |  |  |  |  |

| Proposed change affects:  | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title:  | Test Case 8.3.4.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source:  | Anritsu Ltd |  |  |  |  |  |  |  |  |  |
| Work item code:  |  |  |  |  |  |  |  | Date:  |  | 21/05/2003 |
| Category:  | F |  |  |  |  |  |  | Release:  |  | R99 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: 2 (GSM Phase 2) R96 (Release 1996) R97 (Release 1997) R98 (Release 1998) R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) |  |
| Reason for change:  |  | To introduce test case 8.3.4.2 to RRCv310 |  |  |  |  |  |  |  |  |
| Summary of change:  |  | - 0 table(s) deleted from RRCv310 - 5 table(s) modified in RRCv310 - 4 new table(s) added - 4 tables added from RRCv143 of which - 2 tables have been modified For more details see below. |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  | Test case 8.3.4.2 will not be added |  |  |  |  |  |  |  |  |
| Clauses affected:  |  | N/A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs  |  |  | X | Other core specifications  |  |  |  |  |  |  |
| Affected: |  |  | X | Test specifications |  |  |  |  |  |  |
|  |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments:  |  |  |  |  |  |  |  |  |  |  |

How to create CRs using this form:

Comprehensive information and tips about how to create CRs can be found at http://www.3gpp.org/specs/CR.htm.  Below is a brief summary:

1)	Fill out the above form. The symbols above marked  contain pop-up help information about the field that they are closest to.

2)	Obtain the latest version for the release of the specification to which the change is proposed. Use the MS Word "revision marks"  feature (also known as "track changes") when making the changes. All 3GPP specifications can be downloaded from the 3GPP server under ftp://ftp.3gpp.org/specs/ For the latest version, look for the directory name with the latest date e.g. 2001-03 contains the specifications resulting from the March 2001 TSG meetings.

3)	With "track changes" disabled, paste the entire CR form (use CTRL-A to select it) into the specification just in front of the clause containing the first piece of changed text.  Delete those parts of the specification which are not relevant to the change request.

| TSG-T Working Group1 SWG SIG E-mail Approval |  |  | T1-030604 |
| --- | --- | --- | --- |
| Seoul, Korea |  | 12-15 May 2003 |  |
| Title | Introducing test case 8.3.4.2 required to RRCv310 |  |  |
| Source | Anritsu |  |  |
| Agenda Item | N/A |  |  |
| Document for | Approval |  |  |
| Contact | Dan Fox (Anritsu) dan.fox@eu.anritsu.com Tel: +44 1582 433357 |  |  |

- Table Of Contents

1	Overview	4

2	Changes required for test-case 8.3.4.2	4

2.1	Tables deleted from RRCv310	4

2.2	Tables modified in RRCv310	5

2.2.1	cb_SIB11_Def	5

2.2.2	ts_SHO_ConfigureAdditionalDL_DPCH	6

2.2.3	ts_SHO_ReleaseDL_DPCH	7

2.2.4	cas_UE_CapabilityInfoCnfAM	8

2.2.5	ts_SS_Rel	9

2.3	Tables added to RRCv310	10

2.3.1	Tables added from RRCv143– No changes necessary	10

2.3.2	Other tables  added	11

2.3.2.1	cr_108_MeasReportIntraFreqPeriodic2cell_e1a	11

2.3.2.2	cr_108_MeasReportIntraFreqPeriodic2cell_e1b	12

2.3.2.3	cs_CellUpdateCnfGenericDCCH_noURA	13

2.3.2.4	ts_CMAC_Reconf_SCCPCH_to_2FACH	14

2.4	Modifications to tables added from RRCv143	15

2.4.1	cs_ActSetUpdateAdd	15

2.4.2	tc_8_3_4_2	17

# Overview

This document details the changes needed introduce test case 8.3.4.2 to RRCv310 by using RRCv143 as the primary source of the new tables and applying only essential fixes to the TTCN.

# Changes required for test-case 8.3.4.2

## Tables deleted from RRCv310

None

## Tables modified in RRCv310

### cb_SIB11_Def

Reason for change

The existing constraint contains incorrect event 1a reportCriteria  intraFreqReportingCriteria which causes step 9a failure

Summary of Change

Update the constraint as follows.

Change the Constraint Declaration from:

| Constraint Name | cb_SIB11_Def ( p_ActiveCellInfo, p_IntraCellInfo2, p_IntraCellInfo3, p_IntraCellInfo4, p_IntraCellInfo5, p_InterCellInfo6, p_InterCellInfo7, p_InterCellInfo8 : CellInfoCfg ) |
| --- | --- |
| ASN1 Type | SysInfoType11 |
| Derivation Path |  |
| Encoding Variation |  |
| Comments | Default system information block type 11. To be used by cell A,B,C,G and H: - 5 intra cells frequency of the same frequency - 3 inter cell frequency of the same frequency. |
| … reportCriteria intraFreqReportingCriteria : { eventCriteriaList {{ event e1a : { triggeringCondition monitoredSetCellsOnly, reportingRange 5, w 1, reportDeactivationThreshold t2, reportingAmount ra4, reportingInterval ri4 }, … |  |

To:

| Constraint Name | cb_SIB11_Def ( p_ActiveCellInfo, p_IntraCellInfo2, p_IntraCellInfo3, p_IntraCellInfo4, p_IntraCellInfo5, p_InterCellInfo6, p_InterCellInfo7, p_InterCellInfo8 : CellInfoCfg ) |
| --- | --- |
| ASN1 Type | SysInfoType11 |
| Derivation Path |  |
| Encoding Variation |  |
| Comments | Default system information block type 11. To be used by cell A,B,C,G and H: - 5 intra cells frequency of the same frequency - 3 inter cell frequency of the same frequency. |
| … reportCriteria intraFreqReportingCriteria : { eventCriteriaList {{ event e1a : { triggeringCondition activeSetAndMonitoredSetCells, -- refer to TS25.331 for active and monitiored set definition reportingRange 5, w 1, reportDeactivationThreshold t2, reportingAmount ra4, reportingInterval ri4 }, … |  |

Change the TTCN PDU Constraint Declaration from:

To:

Change the TCN PDU Constraint Declaration from:

To:

Change the Structured Type Constraint Declaration from:

To:

Change the Structured Type Constraint Declaration from:

To:

### ts_SHO_ConfigureAdditionalDL_DPCH

Reason for change

Local tree lt_ConfigSpeech contains incorrect configuration detail as follow:

- incorrect Radio Link configuration for 12.2 K AMR

- configuration of the transport channel for SHO is not required per TS25.303 section 6.4.4

Summary of Change

Update the test step from :

| Test Step Name |  |  | ts_SHO_ConfigureAdditionalDL_DPCH ( p_CellId : INTEGER) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Nr | Label | Behaviour Description |  | Constraints Ref | Verdict | Comments |
|  |  | ….. |  |  |  |  |
|  |  | lt_ConfigSpeech |  |  |  |  |
| 19 |  | CPHY!CPHY_RL_Setup_REQ |  | ca_DL_DPCH_Info ( p_CellId, tsc_DL_DPCH1, cb_DL_DPCH_64K_CS ( c_DL_CommonInformationRB_SetUp ( tsc_DL_DPCH1_SFP_Speech ), tcv_TmpCellInfo.dl_DPCH_2ndScrCode) ) |  |  |
| 20 |  | CPHY?CPHY_RL_Setup_CNF |  | ca_RL_SetupCnf ( p_CellId, tsc_DL_DPCH1 ) |  |  |
| 21 |  | CPHY!CPHY_TrCH_Config_REQ |  | ca_4DCH_DL_InfoActNow ( p_CellId, tsc_DL_DPCH1 ) |  |  |
| 22 |  | CPHY?CPHY_TrCH_Config_CNF |  | ca_TrChCfgCnf ( p_CellId, tsc_DL_DPCH1 ) |  |  |
|  |  | lt_Config64kCS |  |  |  |  |
|  |  | …. |  |  |  |  |

To:

| Test Step Name |  |  | ts_SHO_ConfigureAdditionalDL_DPCH ( p_CellId : INTEGER) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Nr | Label | Behaviour Description |  | Constraints Ref | Verdict | Comments |
|  |  | ….. |  |  |  |  |
|  |  | lt_ConfigSpeech |  |  |  |  |
| 19 |  | CPHY!CPHY_RL_Setup_REQ |  | ca_DL_DPCH_Info ( p_CellId, tsc_DL_DPCH1, cb_DL_DPCH_122_AMR ( c_DL_CommonInformationRB_SetUpSpeech ( tsc_DL_DPCH1_SFP_Speech ), tcv_TmpCellInfo.dl_DPCH_2ndScrCode) ) |  |  |
| 20 |  | CPHY?CPHY_RL_Setup_CNF |  | ca_RL_SetupCnf ( p_CellId, tsc_DL_DPCH1 ) |  |  |
|  |  | lt_Config64kCS |  |  |  |  |
|  |  | …. |  |  |  |  |

### ts_SHO_ReleaseDL_DPCH

Reason for change:

Release of Transport Channel for SHO is not required. Refer to section 2.2.2 for explanation.

Summary of Change

Modify the test step as follows:

Change test step from:

| Test Step Name |  |  | ts_SHO_ReleaseDL_DPCH (p_CellId : INTEGER) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Nr | Label | Behaviour Description |  | Constraints Ref | Verdict | Comments |
| 1 |  | + ts_SetTmpCellInfo ( p_CellId ) |  |  |  |  |
| 2 |  | + ts_CPHY_TrChRel ( p_CellId , tsc_DL_DPCH1 ) |  |  |  | Release of this step is not necessary as this channel is not configured in the first place |
| 3 |  | + ts_SS_StopRL ( p_CellId , tsc_DL_DPCH1 ) |  |  |  |  |
| 4 |  | ( tcv_TmpCellInfo.DL_DPCH_SHO := FALSE ) |  |  |  |  |
| 5 |  | + ts_SaveCellInfo ( p_CellId ) |  |  |  |  |

To:

| Test Step Name |  |  | ts_SHO_ReleaseDL_DPCH (p_CellId : INTEGER) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Nr | Label | Behaviour Description |  | Constraints Ref | Verdict | Comments |
| 1 |  | + ts_SetTmpCellInfo ( p_CellId ) |  |  |  |  |
| 2 |  | + ts_SS_StopRL ( p_CellId , tsc_DL_DPCH1 ) |  |  |  |  |
| 3 |  | ( tcv_TmpCellInfo.DL_DPCH_SHO := FALSE ) |  |  |  |  |
| 4 |  | + ts_SaveCellInfo ( p_CellId ) |  |  |  |  |

### cas_UE_CapabilityInfoCnfAM

Reason for change

The existing constraint does not set the “confirm request” to “confirmationRequested” as the name may suggest. Therefore the AM Data confirm was not received which cause the test case to fail.

Summary of Change

Update the constraint as follows.

Change the Constraint Declaration from:

| Constraint Name | cas_UE_CapabilityInfoCnfAM( p_CellId: INTEGER; p_RB_Id: INTEGER; p_PDU: DL_DCCH_Message ) |
| --- | --- |
| ASP Type | RLC_AM_DATA_REQ |
| Derivation Path |  |
| Encoding Variation |  |
| Comments |  |
| { cellId p_CellId, routingInfo rB_Identity: p_RB_Id, confirmationRequest noConfirmationRequest: NULL, aM_message dL_DCCH_Message : p_PDU } |  |

To:

| Constraint Name | cas_UE_CapabilityInfoCnfAM( p_CellId: INTEGER; p_RB_Id: INTEGER; p_Mui : Mui; p_PDU: DL_DCCH_Message ) |
| --- | --- |
| ASP Type | RLC_AM_DATA_REQ |
| Derivation Path |  |
| Encoding Variation |  |
| Comments |  |
| { cellId p_CellId, routingInfo rB_Identity: p_RB_Id, confirmationRequest confirmationRequested: p_Mui, aM_message dL_DCCH_Message : p_PDU } |  |

### ts_SS_Rel

Reason for change

The test step contain in correct qualifier logic to release non-existent radio bearers RB20. (i.e. RB20 was never established in this test case prior to the CS Cell DCH to Cell FACH transition)

Summary of Change

Change the test step behaviour line as follows:

Change test step from:

| Test Step Name |  |  | Ts_SS_Rel ( p_CellId : INTEGER ) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Nr | Label | Behaviour Description |  | Constraint Ref | Verdict | Comments |
| 1 | … .. | ….. |  | ….. | … .. | ….. |
| 11 |  | [ ( tcv_TmpCellInfo.cellConfig = cell_FACH_PS ) OR ( tcv_TmpCellInfo.cellConfig = cell_FACH ) OR ( tcv_TmpCellInfo.cellConfig = cell_FACH_NoConn ) ] |  |  |  |  |
| 12 |  | +ts_CRLC_Rel ( tsc_CellDedicated, tsc_RB20 ) |  |  |  |  |
| 13 |  | + ts_CRLC_Rel ( p_CellId , tsc_RB_BCCH_FACH ) |  |  |  |  |
|  | … .. | … .. |  | … .. | … .. | … .. |

To:

| Test Step Name |  |  | Ts_SS_Rel ( p_CellId : INTEGER ) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Nr | Label | Behaviour Description |  | Constraint Ref | Verdict | Comments |
| 1 | … .. | ….. |  | ….. | … .. | ….. |
| 11 |  | [ ( tcv_TmpCellInfo.cellConfig = cell_FACH_PS ) OR ( tcv_TmpCellInfo.cellConfig = cell_FACH ) ] |  |  |  |  |
| 12 |  | +ts_CRLC_Rel ( tsc_CellDedicated, tsc_RB20 ) |  |  |  |  |
| 13 |  | + ts_CRLC_Rel ( p_CellId , tsc_RB_BCCH_FACH ) |  |  |  |  |
| 14 |  | + lt_RelSRB1_4 |  |  |  |  |
| 15 |  | +lt_ReleaseCommonCh |  |  |  |  |
| 16 |  | + lt_Release_BCCH |  |  |  |  |
| 17 |  | + ts_SetCellCfg ( p_CellId, cell_NotConfigured ) |  |  |  |  |
| 18 |  | [ ( tcv_TmpCellInfo.cellConfig = cell_FACH_NoConn ) ] |  |  |  |  |
| 19 |  | + lt_RelSRB1_4 |  |  |  |  |
| 20 |  | +lt_ReleaseCommonCh |  |  |  |  |
| 21 |  | + lt_Release_BCCH |  |  |  |  |
| 22 |  | + ts_SetCellCfg ( p_CellId, cell_NotConfigured ) |  |  |  |  |
|  | … .. | … .. |  | … .. | … .. | … .. |

Tables added to RRCv310

### Tables added from RRCv143– No changes necessary

| cr_UTRAN_MobilityInfoCnf |  |
| --- | --- |
| cs_ActSetUpdateRemv |  |

Other tables  added

#### cr_108_MeasReportIntraFreqPeriodic2cell_e1a

#### adde

This table is not based on one in any existing ATS.

Reason for change

Test case  expects the periodic measurement report to delivered the measurement results in a specific cell order. However, this may not be applicable for the cell when equal CPICH RSCP is received. Therefore it is necessary to create a constraint to accept flexible cell ordering per event 1a

Summary of Change

Table added to suite.

Add Constraint Declaration:

| Constraint Name | cr_108_MeasReportIntraFreqPeriodic2cell_e1a ( p_MeasId : MeasurementIdentity ; p_PriScmbCode : INTEGER; p_CellSync : CellSynchronisationInfo; p_PrimaryScr2:INTEGER; p_CellSync2 : CellSynchronisationInfo; p_PriScmbCode_event: INTEGER) |
| --- | --- |
| PDU Type | UL_DCCH_Message |
| Derivation Path |  |
| Encoding Variation |  |
| Comments |  |
| { integrityCheckInfo *, message measurementReport : { measurementIdentity p_MeasId, measuredResults intraFreqMeasuredResultsList : {{ cellIdentity OMIT, sfn_SFN_ObsTimeDifference *, cellSynchronisationInfo p_CellSync, modeSpecificInfo fdd : { primaryCPICH_Info { primaryScramblingCode p_PriScmbCode }, cpich_Ec_N0 OMIT, cpich_RSCP ?, pathloss OMIT } }, { cellIdentity OMIT, sfn_SFN_ObsTimeDifference OMIT, cellSynchronisationInfo p_CellSync2, modeSpecificInfo fdd : { primaryCPICH_Info { primaryScramblingCode p_PrimaryScr2 }, cpich_Ec_N0 OMIT, cpich_RSCP ?, pathloss OMIT } } }, measuredResultsOnRACH OMIT, additionalMeasuredResults OMIT, eventResults intraFreqEventResults : { eventID e1a, cellMeasurementEventResults fdd : { { primaryScramblingCode p_PriScmbCode_event}}}, v390nonCriticalExtensions * } } |  |

#### cr_108_MeasReportIntraFreqPeriodic2cell_e1b

This table is not based on one in any existing ATS

Reason for change

Test case  expects the periodic measurement report to deliver the measurement results in a specific cell order. However, this may not be applicable for the when equal CPICH RSCP is received. Therefore it is necessary to create a contarin to accept flexible cell ordering per event 1b

Summary of Change

Table added to suite.

Add Constraint Declaration:

| Constraint Name | cr_108_MeasReportIntraFreqPeriodic2cell_e1b ( p_MeasId : MeasurementIdentity ; p_PriScmbCode : INTEGER; p_CellSync : CellSynchronisationInfo; p_PrimaryScr2:INTEGER; p_CellSync2 : CellSynchronisationInfo; p_PriScmbCode_event: INTEGER) |
| --- | --- |
| PDU Type | UL_DCCH_Message |
| Derivation Path |  |
| Encoding Variation |  |
| Comments |  |
| { integrityCheckInfo *, message measurementReport : { measurementIdentity p_MeasId, measuredResults intraFreqMeasuredResultsList : {{ cellIdentity OMIT, sfn_SFN_ObsTimeDifference *, cellSynchronisationInfo p_CellSync, modeSpecificInfo fdd : { primaryCPICH_Info { primaryScramblingCode p_PriScmbCode }, cpich_Ec_N0 OMIT, cpich_RSCP ?, pathloss OMIT } }, { cellIdentity OMIT, sfn_SFN_ObsTimeDifference OMIT, cellSynchronisationInfo p_CellSync2, modeSpecificInfo fdd : { primaryCPICH_Info { primaryScramblingCode p_PrimaryScr2 }, cpich_Ec_N0 OMIT, cpich_RSCP ?, pathloss OMIT } } }, measuredResultsOnRACH OMIT, additionalMeasuredResults OMIT, eventResults intraFreqEventResults : { eventID e1b, cellMeasurementEventResults fdd : { { primaryScramblingCode p_PriScmbCode_event}}}, v390nonCriticalExtensions * } } |  |

#### cs_CellUpdateCnfGenericDCCH_noURA

This table is not based on one in any existing ATS

Reason for change

URA ID is only required in the PCH state. Therefore a new constraint is introduced

Summary of Change

Table added to suite.

Add Constraint Declaration:

| Constraint Name | cs_CellUpdateCnfGenericDCCH_noURA ( p_IntegrityCheckInfo : IntegrityCheckInfo; p_RRC_TI: RRC_TransactionIdentifier; p_U_RNTI_New : U_RNTI; p_C_rnti: C_RNTI; p_State_Ind: RRC_StateIndicator; p_RB_Rls: RB_InformationReleaseList ; p_RB_Affected: RB_InformationAffectedList ; p_Ul_reconf_trch : UL_AddReconfTransChInfoList; p_Reconfpower:MaxAllowedUL_TX_Power; p_Dl_reconf_trch:DL_AddReconfTransChInfoList; p_Rb_infoReconf:RB_InformationReconfigList) |
| --- | --- |
| PDU Type | DL_DCCH_Message |
| Derivation Path |  |
| Encoding Variation |  |
| Comments | Based on cs_CellUpdateCnfGenericDCCH |
| { |  |
| integrityCheckInfo p_IntegrityCheckInfo, |  |
| message cellUpdateConfirm: r3:{ |  |
| cellUpdateConfirm_r3 { --CellUpdateConfirm_r3_IEs |  |
| rrc_TransactionIdentifier p_RRC_TI, |  |
| integrityProtectionModeInfo OMIT, |  |
| cipheringModeInfo OMIT, |  |
| activationTime OMIT , |  |
| new_U_RNTI p_U_RNTI_New , |  |
| new_C_RNTI p_C_rnti, |  |
| rrc_StateIndicator p_State_Ind, |  |
| utran_DRX_CycleLengthCoeff OMIT, |  |
| rlc_Re_establishIndicatorRb2_3or4 FALSE, |  |
| rlc_Re_establishIndicatorRb5orAbove FALSE, |  |
| cn_InformationInfo OMIT, |  |
| ura_Identity OMIT, -- '0000000000000001'B, |  |
| rb_InformationReleaseList p_RB_Rls, |  |
| rb_InformationReconfigList p_Rb_infoReconf, |  |
| rb_InformationAffectedList p_RB_Affected, |  |
| ul_CommonTransChInfo OMIT, |  |
| ul_deletedTransChInfoList OMIT, |  |
| ul_AddReconfTransChInfoList p_Ul_reconf_trch, |  |
| modeSpecificTransChInfo fdd: { |  |
| cpch_SetID OMIT, |  |
| addReconfTransChDRAC_Info OMIT |  |
| }, |  |
| dl_CommonTransChInfo OMIT, |  |
| dl_DeletedTransChInfoList OMIT, |  |
| dl_AddReconfTransChInfoList p_Dl_reconf_trch, |  |
| frequencyInfo OMIT, |  |
| maxAllowedUL_TX_Power p_Reconfpower, |  |
| ul_ChannelRequirement OMIT, |  |
| modeSpecificPhysChInfo fdd:{ |  |
| dl_PDSCH_Information OMIT |  |
| } , |  |
| dl_CommonInformation OMIT, |  |
| dl_InformationPerRL_List OMIT |  |
| }, |  |
| v3a0NonCriticalExtensions OMIT |  |
| } |  |
| } |  |

#### ts_CMAC_Reconf_SCCPCH_to_2FACH

Reason for change

Prior to the the transmission of Cell Confrim Update PDU the S-CCPCH is required to be  re-configured to re-route the downlink PDU through DDCH over FACH

Summary of Change

Table Added to suite

| Test Step Name |  |  | ts_CMAC_Reconf_SCCPCH_to_2FACH ( p_CellId : INTEGER; p_DL_TrLogMapping: TrCH_LogCHMappingList1) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Nr | Label | Behaviour Description |  | Constraint Ref | Verdict | Comments |
| 1 |  | + ts_SetTmpCellInfo ( p_CellId ) |  |  |  |  |
| 2 |  | CMAC ! CMAC_Config_REQ |  | ca_CMAC_ReconfigInfoActNow ( p_CellId, tsc_S_CCPCH1, c_UE_Info(tcv_TmpCellInfo.uRNTI, OMIT), c_TrChInfoPCH_FACH_PS, p_DL_TrLogMapping ) |  |  |
| 3 |  | CMAC ? CMAC_Config_CNF |  | ca_CMAC_CfgCnf(p_CellId, tsc_S_CCPCH1 ) |  |  |

#### Modifications to tables added from RRCv143

### cs_ActSetUpdateAdd

Reason for change

The existing constraint contains an incorrect IE as follows:

- Secondary Scrambling Code shall not be omitted

- Incorrect Spreading factor and channelisation code

- Inocrrect tfci_CombiningIndicator

Summary of Change

Update the constraint as follows.

Change the Constraint Declaration from:

| Constraint Name | cs_ActSetUpdateAdd ( p_IntegrityCheckInfo : IntegrityCheckInfo; p_RRC_TI: RRC_TransactionIdentifier; p_Act_time: ActivationTime; p_PrimScramblingCode : PrimaryScramblingCode; p_Dpch_FrameOffset:DPCH_FrameOffset ) |
| --- | --- |
| PDU Type | DL_DCCH_Message |
| Derivation Path |  |
| Encoding Variation |  |
| Comments |  |
| { |  |
| integrityCheckInfo p_IntegrityCheckInfo, |  |
| message activeSetUpdate: r3:{ |  |
| activeSetUpdate_r3 { --ActiveSetUpdate_r3_IEs, |  |
| rrc_TransactionIdentifier p_RRC_TI, |  |
| activationTime p_Act_time, |  |
| newU_RNTI OMIT, |  |
| cn_InformationInfo OMIT, |  |
| maxAllowedUL_TX_Power OMIT, |  |
| rl_AdditionInformationList {{ primaryCPICH_Info { primaryScramblingCode p_PrimScramblingCode }, |  |
| dl_DPCH_InfoPerRL fdd: { |  |
| pCPICH_UsageForChannelEst mayBeUsed, |  |
| dpch_FrameOffset p_Dpch_FrameOffset, |  |
| secondaryCPICH_Info OMIT, |  |
| dl_ChannelisationCodeList {{ --DL_ChannelisationCode |  |
| secondaryScramblingCode OMIT, |  |
| sf_AndCodeNumber sf256: 255, |  |
| scramblingCodeChange noCodeChange |  |
| }}, |  |
| tpc_CombinationIndex 0, |  |
| ssdt_CellIdentity OMIT, |  |
| closedLoopTimingAdjMode OMIT |  |
| }, |  |
| tfci_CombiningIndicator TRUE, |  |
| sccpch_InfoforFACH OMIT |  |
| }} , |  |
| rl_RemovalInformationList OMIT, |  |
| tx_DiversityMode noDiversity, |  |
| ssdt_Information OMIT |  |
| }, |  |
| nonCriticalExtensions OMIT |  |
| } |  |
| } |  |

To:

| Constraint Name | cs_ActSetUpdateAdd ( p_IntegrityCheckInfo : IntegrityCheckInfo; p_RRC_TI: RRC_TransactionIdentifier; p_Act_time: ActivationTime; p_PrimScramblingCode : PrimaryScramblingCode; p_Dpch_FrameOffset:DPCH_FrameOffset; p_SecondaryScramblingCode : SecondaryScramblingCode ) |
| --- | --- |
| PDU Type | DL_DCCH_Message |
| Derivation Path |  |
| Encoding Variation |  |
| Comments |  |
| { |  |
| integrityCheckInfo p_IntegrityCheckInfo, |  |
| message activeSetUpdate: r3:{ |  |
| activeSetUpdate_r3 { --ActiveSetUpdate_r3_IEs, |  |
| rrc_TransactionIdentifier p_RRC_TI, |  |
| activationTime p_Act_time, |  |
| newU_RNTI OMIT, |  |
| cn_InformationInfo OMIT, |  |
| maxAllowedUL_TX_Power OMIT, |  |
| rl_AdditionInformationList {{ primaryCPICH_Info { primaryScramblingCode p_PrimScramblingCode }, |  |
| dl_DPCH_InfoPerRL fdd: { |  |
| pCPICH_UsageForChannelEst mayBeUsed, |  |
| dpch_FrameOffset p_Dpch_FrameOffset, |  |
| secondaryCPICH_Info OMIT, |  |
| dl_ChannelisationCodeList {{ --DL_ChannelisationCode |  |
| secondaryScramblingCode p_SecondaryScramblingCode, --OMIT, |  |
| sf_AndCodeNumber tsc_DL_DPCH1_ChC_Speech, -- sf256: 255, |  |
| scramblingCodeChange noCodeChange |  |
| }}, |  |
| tpc_CombinationIndex 0, |  |
| ssdt_CellIdentity OMIT, |  |
| closedLoopTimingAdjMode OMIT |  |
| }, |  |
| tfci_CombiningIndicator FALSE, --TRUE, |  |
| sccpch_InfoforFACH OMIT |  |
| }} , |  |
| rl_RemovalInformationList OMIT, |  |
| tx_DiversityMode noDiversity, |  |
| ssdt_Information OMIT |  |
| }, |  |
| nonCriticalExtensions OMIT |  |
| } |  |
| } |  |

### tc_8_3_4_2

Reason for change

The existing test step  has bee modified as follows:

- Line 9  - Postamble po_SHO_ConnectionAndSS_Rel is not suitable for Cell FACH stte release replaced with po_ConnectionAndSS_Rel (see modified table line 9 and 10)

- Line 13 contains a rigid scheme w.r.t. to cell ordering – replaced this with a local tree (lt_mea_report_cellb_e1a) to allow flexible permutation of cell ordering (see modified table line 14)

- Line 15 - ts_Compute_DPCH_FrameOffset fails to acquire the real time SFN as expected moved the computation during Activate Set Update PDU transmission using px_SFN_OffSetA and  px_SFN_OffSetB (see modified table line 17)

- Line 24 contains a rigid scheme w.r.t. to cell ordering – replaced this with a local tree (lt_mea_report_cellb_e1b) to allow flexible permutation of cell ordering (see modified table line 24)

- Line 34 disallow single mode UE to be tested (i.e InterRAT_UE_RadioAccessCapabilityList must be present) Replace ‘?’ with a ‘*’ to allow single mode UE to pass the test case.

- Line 36 – missing awaiting UE Capability Confirm primitive which causes the RLC engine to reset and subsequently leads to test case failure. (see modified table line 36) Note:- This change must be rippled through all the test cases which use the cas_UE_CapabilityInfoCnfAM constraint

- Line 38 – Cell B power level is not consistent with corresponding 34.123-1 clause modified the (T3) power level accordingly (see modified table line 39)

- Line 41 – missing cell FACH state assignment and S-CCPCH re-configuration for the Cell Update to be transmitted through DCCH over FACH ( see modified table line 42 and 43)

- Line 42 – transmission of Cell Update Confrim PDU in Cell FACH state does not require U-RNTI and URA ID. (see modified table line 42 & 43)

- Line 44 – contains incorrect Annex C test procedure for Cell FACH, replaces procedure with UE Cell FACH state checker accordingly (see modified table line 47)

Summary of Change

Update the test as follows.

From:

| Nr | Label | Behaviour Description | Constraints Ref | Verdict | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 |  | START t_Guard |  |  |  |
| 2 |  | +lt_RRC_InitVariables |  |  | Initial Test Case Variables |
| 3 |  | +pr_GotoState6_9_Or6_10_MO ( tsc_CellA ) |  |  | Initial conditions: DCH state either PS or CS according to TS 34.108 clause 7.4 |
| 4 |  | +ts_SS_CreateCellDCH(tsc_CellB) |  |  | Create also cell B |
| 5 |  | +ts_SendDefSysInfo(tsc_CellB) |  |  |  |
| 6 |  | (tcv_TestBody:=TRUE) |  |  |  |
| 7 |  | +lt_TestBody |  |  |  |
| 8 |  | (tcv_TestBody:=FALSE) |  |  |  |
| 9 |  | +po_SHO_ConnectionAndSS_Rel |  |  | Postamble |
|  |  | lt_TestBody |  |  |  |
| 10 |  | +ts_SS_IncrementCellPowerLevel (tsc_CellB ,15) |  |  |  |
| 11 |  | START t_WaitMS |  |  |  |
| 12 | TBF1 | ? TIMEOUT t_WaitMS |  | (F) | Wait for 13 secs |
| 13 | TBP1 | AM ? RLC_AM_DATA_IND CANCEL t_WaitS | car_MeasurementReport ( tsc_CellDedicated, tsc_RB2, cr_108_MeasReportIntraFreqPeriodic ( 1, INT_TO_BIT ( tsc_CellB,28 ), tcv_CellInfoA.priScrmCode,tcv_CellInfoB.priScrmCode ) ) | (P) | Step 2. MEASUREMENT REPORT This message is sent by the UE when it detects cell B (at -60 dBm) wich has been created above. |
| 14 |  | +ts_SHO_ConfigureAdditionalDL_DPCH ( tsc_CellB ) |  |  | To configure an additional radio link (downlink) for cell B |
| 15 |  | +ts_Compute_DPCH_FrameOffset ( tsc_CellA, tsc_CellB) |  |  |  |
| 16 |  | + ts_CalculateActTime (tsc_CellA ) |  |  |  |
| 17 |  | AM ! RLC_AM_DATA_REQ | cas_ActSetUpdate ( tsc_CellDedicated, tsc_RB2, cs_ActSetUpdateAdd ( tcv_CellIndInfo.dl_IntegrityCheckInfo, tcv_RRC_Ti, tcv_ActTime, tcv_CellInfoB.priScrmCode, tcv_DPCH_FrameOffset ) ) |  | Step 3. ACTIVE SET UPDATE message including "Radio Link Addition Information" |
| 18 |  | START t_WaitMS |  |  |  |
| 19 | TBF2 | ? TIMEOUT t_WaitMS |  | (F) | Wait for 13 secs |
| 20 | TBP2 | AM ? RLC_AM_DATA_IND CANCEL t_WaitMS | car_ActSetUpdateCmpl( tsc_CellDedicated, tsc_RB2, cr_108_ActSetUpdateCmpl ( tcv_RRC_Ti ) ) | (P) | Step 4 . UE configures a new radio link to cell 2 |
| 21 |  | +ts_SS_DecrementCellPowerLevel (tsc_CellA, 15) |  |  | Step 5. Set power levels according to column T2 in Table 8.3.4.2 (Cell A -75 dBm, Cell B -60 dBm) |
| 22 |  | START t_WaitMS |  |  |  |
| 23 | TBF3 | ? TIMEOUT t_WaitMS |  | (F) | Wait for 13 secs |
| 24 | TBP3 | AM ? RLC_AM_DATA_IND CANCEL t_WaitMS | car_MeasurementReport ( tsc_CellDedicated, tsc_RB2, cr_MeasReportIntraFreqPeriodic2(1, INT_TO_BIT(tsc_CellB,28), tcv_CellInfoB.priScrmCode, tcv_CellInfoA.priScrmCode)) | (P) | Step 6. MEASUREMENT REPORT |
| 25 |  | + ts_CalculateActTime (tsc_CellA ) |  |  |  |
| 26 |  | AM ! RLC_AM_DATA_REQ | cas_ActSetUpdate ( tsc_CellDedicated, tsc_RB2, cs_ActSetUpdateRemv ( tcv_CellIndInfo.dl_IntegrityCheckInfo, tcv_RRC_Ti, tcv_ActTime, tcv_CellInfoA.priScrmCode ) ) |  | Step 7 . ACTIVE SET UPDATE message including "Radio Link Removal Information" for Cell A |
| 27 |  | START t_WaitMS |  |  |  |
| 28 | TBF4 | ? TIMEOUT t_WaitMS |  | (F) | Wait for 13 secs |
| 29 | TBP4 | AM ? RLC_AM_DATA_IND CANCEL t_WaitMS | car_ActSetUpdateCmpl ( tsc_CellDedicated, tsc_RB2, cr_108_ActSetUpdateCmpl ( tcv_RRC_Ti ) ) | (P) | Step 8. ACTIVE SET UPDATE COMPLETE (removal of link to cell A completed) |
| 30 |  | +ts_SHO_ReleaseDL_DPCH (tsc_CellA) |  |  | Remove DL DPCH on cell A, it is not used any more. |
| 31 |  | AM ! RLC_AM_DATA_REQ | cas_UE_CapabilityEnqy( tsc_CellDedicated, tsc_RB2, cs_108_UE_CapabilityEnq ( tcv_CellIndInfo.dl_IntegrityCheckInfo, tcv_RRC_Ti)) |  | Step 9. UE CAPABILITY ENQUIRY |
| 32 |  | START t_WaitMS |  |  |  |
| 33 | TBF5 | ? TIMEOUT t_WaitMS |  | (F) | Wait for 13 secs |
| 34 | TBP5 | AM?RLC_AM_DATA_IND CANCEL t_WaitMS | car_UE_CapabilityInfoAM ( tsc_CellDedicated, tsc_RB2, cr_108_UE_CapabilityInfoAM ( ?, ?, ? ) ) | (P) | Step 10. UE CAPABILITY INFORMATION |
| 35 |  | AM ! RLC_AM_DATA_REQ | cas_UE_CapabilityInfoCnfAM ( tsc_CellDedicated, tsc_RB2, cs_108_UE_CapabilityInfoCnfAM ( tcv_CellIndInfo.dl_IntegrityCheckInfo, tcv_RRC_Ti ) ) |  | Step 11. UE CAPABILITY INFORMATION CONFIRM. |
| 36 |  | +ts_SHO_ReleaseDL_DPCH (tsc_CellB) |  |  | Step 12. Set power levels according to column T2 in Table 8.3.4.2. Cell B is "Off". (and RL to cell A has been released in step3) |
| 37 |  | +ts_SS_IncrementCellPowerLevel (tsc_CellA ,15) |  |  | Step 12; |
| 38 |  | +ts_SS_DecrementCellPowerLevel (tsc_CellB ,15) |  |  | Deactivate Cell B; |
| 39 |  | +localTree |  |  |  |
|  |  | localTree |  |  |  |
| 40 | TBP6 | +ts_RRC_ReceiveCellUpdate(tsc_CellA, cbr_108_CellUpdate ( tcv_CellInfoA.uRNTI, cellReselection),(tsc_MaxCampingTime * 1000) ) |  |  | Step 13. UE sends CELL UPDATE with "Cell update cause" cell reselection " |
| 41 |  | UM ! RLC_UM_DATA_REQ | cas_RRC_CellUpdateCnf( tsc_CellDedicated, tsc_RB1, cs_CellUpdateCnfGenericDCCH( tcv_CellIndInfo.dl_IntegrityCheckInfo, tcv_RRC_Ti, c_U_RNTI_2, tsc_CRNTI_Id2, cell_FACH, OMIT, OMIT, OMIT, OMIT,OMIT,OMIT)) |  | Step 14 . SS sends CELL UPDATE CONFIRM with IEs "new C-RNTI" |
| 42 |  | +ts_CMAC_NewU_RNTI_Reconf (tsc_CellA, c_U_RNTI_2, tsc_CRNTI_Id2) |  |  | SS reconfiguration |
| 43 | TBP7 | AM ? RLC_AM_DATA_IND | car_RRC_UtranMobilityInfoCnf( tsc_CellDedicated, tsc_RB2, cr_UTRAN_MobilityInfoCnf( tcv_RRC_Ti) ) | (P) | Step 15 |
| 44 |  | + ts_C3_CheckCellDCH ( tsc_CellA ) |  |  |  |
|  |  | lt_RRC_InitVariables |  |  |  |
| 45 |  | +ts_RRC_InitVariables ( cell_DCH ) |  |  |  |
| 46 |  | (tcv_CellInfoA.attenuationLevel := tsc_AttLevToPower60_dBm, tcv_CellInfoB.attenuationLevel := tsc_AttLevToPower75_dBm) |  |  |  |

To:

| Nr | Label | Behaviour Description | Constraints Ref | Verdict | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 |  | START t_Guard |  |  |  |
| 2 |  | +lt_RRC_InitVariables |  |  | Initial Test Case Variables |
| 3 |  | +pr_GotoState6_9_Or6_10_MO ( tsc_CellA ) |  |  | Initial conditions: DCH state either PS or CS according to TS 34.108 clause 7.4 |
| 4 |  | +ts_SS_CreateCellDCH(tsc_CellB) |  |  | Create also cell B |
| 5 |  | +ts_SendDefSysInfo(tsc_CellB) |  |  |  |
| 6 |  | (tcv_TestBody:=TRUE) |  |  |  |
| 7 |  | +lt_TestBody |  |  |  |
| 8 |  | (tcv_TestBody:=FALSE) |  |  |  |
| 9 |  | +po_ConnectionAndSS_Rel ( tsc_CellA ) |  |  | Postamble - cna not use DCH postamble +po_SHO_ConnectionAndSS_Rel |
| 10 |  | +po_ConnectionAndSS_Rel ( tsc_CellB ) |  |  | release Cell B Common channels |
|  |  | lt_TestBody |  |  |  |
| 11 |  | +ts_SS_IncrementCellPowerLevel (tsc_CellB ,15) |  |  |  |
| 12 |  | START t_WaitMS |  |  |  |
| 13 | TBF1 | ? TIMEOUT t_WaitMS |  | (F) | Wait for 13 secs |
| 14 |  | +lt_mea_report_cellb_e1a |  |  | Step 2. MEASUREMENT REPORT This message is sent by the UE when it detects cell B (at -60 dBm) wich has been created above. |
| 15 |  | +ts_SHO_ConfigureAdditionalDL_DPCH ( tsc_CellB ) |  |  | To configure an additional radio link (downlink) for cell B |
| 16 |  | + ts_CalculateActTime (tsc_CellA ) |  |  |  |
| 17 |  | AM ! RLC_AM_DATA_REQ | cas_ActSetUpdate ( tsc_CellDedicated, tsc_RB2, cs_ActSetUpdateAdd ( tcv_CellIndInfo.dl_IntegrityCheckInfo, tcv_RRC_Ti, tcv_ActTime, tcv_CellInfoB.priScrmCode, px_SFN_OffsetB - px_SFN_OffsetA, tcv_TmpCellInfo.dl_DPCH_2ndScrCode ) ) |  | Step 3. ACTIVE SET UPDATE message including "Radio Link Addition Information" cas_ActSetUpdate ( tsc_CellDedicated, tsc_RB2, cs_ActSetUpdateAdd ( tcv_CellIndInfo.dl_IntegrityCheckInfo, tcv_RRC_Ti, tcv_ActTime, tcv_CellInfoB.priScrmCode, tcv_DPCH_FrameOffset ) ) |
| 18 |  | START t_WaitMS |  |  |  |
| 19 | TBF2 | ? TIMEOUT t_WaitMS |  | (F) | Wait for 13 secs |
| 20 | TBP2 | AM ? RLC_AM_DATA_IND CANCEL t_WaitMS | car_ActSetUpdateCmpl( tsc_CellDedicated, tsc_RB2, cr_108_ActSetUpdateCmpl ( tcv_RRC_Ti ) ) | (P) | Step 4 . UE configures a new radio link to cell 2 |
| 21 |  | +ts_SS_DecrementCellPowerLevel (tsc_CellA, 15) |  |  | Step 5. Set power levels according to column T2 in Table 8.3.4.2 (Cell A -75 dBm, Cell B -60 dBm) |
| 22 |  | START t_WaitMS |  |  |  |
| 23 | TBF3 | ? TIMEOUT t_WaitMS |  | (F) | Wait for 13 secs |
| 24 |  | +lt_mea_report_cella_e1b |  |  | Step 6. MEASUREMENT REPORT |
| 25 |  | + ts_CalculateActTime (tsc_CellA ) |  |  |  |
| 26 |  | AM ! RLC_AM_DATA_REQ | cas_ActSetUpdate ( tsc_CellDedicated, tsc_RB2, cs_ActSetUpdateRemv ( tcv_CellIndInfo.dl_IntegrityCheckInfo, tcv_RRC_Ti, tcv_ActTime, tcv_CellInfoA.priScrmCode ) ) |  | Step 7 . ACTIVE SET UPDATE message including "Radio Link Removal Information" for Cell A |
| 27 |  | START t_WaitMS |  |  |  |
| 28 | TBF4 | ? TIMEOUT t_WaitMS |  | (F) | Wait for 13 secs |
| 29 | TBP4 | AM ? RLC_AM_DATA_IND CANCEL t_WaitMS | car_ActSetUpdateCmpl ( tsc_CellDedicated, tsc_RB2, cr_108_ActSetUpdateCmpl ( tcv_RRC_Ti ) ) | (P) | Step 8. ACTIVE SET UPDATE COMPLETE (removal of link to cell A completed) |
| 30 |  | +ts_SHO_ReleaseDL_DPCH (tsc_CellA) |  |  | Remove DL DPCH on cell A, it is not used any more. |
| 31 |  | AM ! RLC_AM_DATA_REQ | cas_UE_CapabilityEnqy( tsc_CellDedicated, tsc_RB2, cs_108_UE_CapabilityEnq ( tcv_CellIndInfo.dl_IntegrityCheckInfo, tcv_RRC_Ti)) |  | Step 9. UE CAPABILITY ENQUIRY |
| 32 |  | START t_WaitMS |  |  |  |
| 33 | TBF5 | ? TIMEOUT t_WaitMS |  | (F) | Wait for 13 secs |
| 34 | TBP5 | AM?RLC_AM_DATA_IND CANCEL t_WaitMS | car_UE_CapabilityInfoAM ( tsc_CellDedicated, tsc_RB2, cr_108_UE_CapabilityInfoAM ( ?, ?, * ) ) | (P) | Step 10. UE CAPABILITY INFORMATION car_UE_CapabilityInfoAM ( tsc_CellDedicated, tsc_RB2, cr_108_UE_CapabilityInfoAM ( ?, ?, ? ) ) |
| 35 |  | AM ! RLC_AM_DATA_REQ | cas_UE_CapabilityInfoCnfAM ( tsc_CellDedicated, tsc_RB2, tsc_Mui, cs_108_UE_CapabilityInfoCnfAM ( tcv_CellIndInfo.dl_IntegrityCheckInfo, tcv_RRC_Ti ) ) |  | Step 11. UE CAPABILITY INFORMATION CONFIRM. cas_UE_CapabilityInfoCnfAM ( tsc_CellDedicated, tsc_RB2, cs_108_UE_CapabilityInfoCnfAM ( tcv_CellIndInfo.dl_IntegrityCheckInfo, tcv_RRC_Ti ) ) |
| 36 |  | AM ? RLC_AM_DATA_CNF | car_AM_DataMuiCnf (tsc_CellDedicated, tsc_RB2, tsc_Mui) |  |  |
| 37 |  | +ts_SHO_ReleaseDL_DPCH (tsc_CellB) |  |  | Step 12. Set power levels according to column T2 in Table 8.3.4.2. Cell B is "Off". (and RL to cell A has been released in step3) |
| 38 |  | +ts_SS_IncrementCellPowerLevel (tsc_CellA ,15) |  |  | Step 12; |
| 39 |  | +ts_SS_DecrementCellPowerLevel (tsc_CellB ,30) |  |  | Deactivate Cell B; +ts_SS_DecrementCellPowerLevel (tsc_CellB ,15) |
| 40 |  | +localTree |  |  |  |
|  |  | localTree |  |  |  |
| 41 | TBP6 | +ts_RRC_ReceiveCellUpdate(tsc_CellA, cbr_108_CellUpdate ( tcv_CellInfoA.uRNTI, radiolinkFailure),(tsc_MaxCampingTime * 1000) ) |  |  | Step 13. UE sends CELL UPDATE with "Cell update cause" cell reselection " +ts_RRC_ReceiveCellUpdate(tsc_CellA, cbr_108_CellUpdate ( tcv_CellInfoA.uRNTI, cellReselection),(tsc_MaxCampingTime * 1000) ) |
| 42 |  | + ts_SetCellCfg ( tsc_CellA, cell_FACH ) |  |  | UE is in Cell FACH State Now |
| 43 |  | +ts_CMAC_Reconf_SCCPCH_to_2FACH ( tsc_CellA,c_TrLogMappingPCH_FACH) |  |  | SS reconfiguration the SSCPCH to send Cell Update Confirm using DCCH over FACH |
| 44 |  | UM ! RLC_UM_DATA_REQ | cas_RRC_CellUpdateCnf( tsc_CellDedicated, tsc_RB1, cs_CellUpdateCnfGenericDCCH_noURA( tcv_CellIndInfo.dl_IntegrityCheckInfo, tcv_RRC_Ti, OMIT, tsc_CRNTI_Id2, cell_FACH, OMIT, OMIT, OMIT, OMIT,OMIT,OMIT)) |  | Step 14 . SS sends CELL UPDATE CONFIRM with IEs "new C-RNTI" cas_RRC_CellUpdateCnf( tsc_CellDedicated, tsc_RB1, cs_CellUpdateCnfGenericDCCH( tcv_CellIndInfo.dl_IntegrityCheckInfo, tcv_RRC_Ti, c_U_RNTI_2, tsc_CRNTI_Id2, cell_FACH, OMIT, OMIT, OMIT, OMIT,OMIT,OMIT)) |
| 45 |  | +ts_CMAC_NewU_RNTI_Reconf (tsc_CellA, c_U_RNTI_2, tsc_CRNTI_Id2) |  |  | SS reconfiguration |
| 46 | TBP7 | AM ? RLC_AM_DATA_IND | car_RRC_UtranMobilityInfoCnf( tsc_CellDedicated, tsc_RB2, cr_UTRAN_MobilityInfoCnf( tcv_RRC_Ti) ) | (P) | Step 15 |
| 47 |  | + ts_C2_CheckCellFACH ( tsc_CellA ) |  |  |  |
|  |  | lt_RRC_InitVariables |  |  |  |
| 48 |  | +ts_RRC_InitVariables ( cell_DCH ) |  |  |  |
| 49 |  | (tcv_CellInfoA.attenuationLevel := tsc_AttLevToPower60_dBm, tcv_CellInfoB.attenuationLevel := tsc_AttLevToPower75_dBm) |  |  |  |
|  |  | lt_mea_report_cellb_e1a |  |  |  |
| 50 | TBP1 | AM ? RLC_AM_DATA_IND CANCEL t_WaitS | car_MeasurementReport ( tsc_CellDedicated, tsc_RB2, (cr_108_MeasReportIntraFreqPeriodic2cell_e1a( 1, tcv_CellInfoA.priScrmCode,OMIT,tcv_CellInfoB.priScrmCode,?,tcv_CellInfoB.priScrmCode )) ) | (P) | Step 2. MEASUREMENT REPORT This message is sent by the UE when it detects cell B (at -60 dBm) wich has been created above. |
| 51 | TBP1A | AM ? RLC_AM_DATA_IND CANCEL t_WaitS | car_MeasurementReport ( tsc_CellDedicated, tsc_RB2, (cr_108_MeasReportIntraFreqPeriodic2cell_e1a( 1, tcv_CellInfoB.priScrmCode,?,tcv_CellInfoA.priScrmCode,OMIT,tcv_CellInfoB.priScrmCode )) ) | (P) | Step 2. MEASUREMENT REPORT This is necessary when both cell has exactly the same quality |
|  |  | lt_mea_report_cella_e1b |  |  |  |
| 52 | TBP3 | AM ? RLC_AM_DATA_IND CANCEL t_WaitS | car_MeasurementReport ( tsc_CellDedicated, tsc_RB2, (cr_108_MeasReportIntraFreqPeriodic2cell_e1b( 1, tcv_CellInfoA.priScrmCode,OMIT,tcv_CellInfoB.priScrmCode,OMIT,tcv_CellInfoA.priScrmCode )) ) | (P) | Step 5a. MEASUREMENT REPORT |
| 53 | TBP3A | AM ? RLC_AM_DATA_IND CANCEL t_WaitS | car_MeasurementReport ( tsc_CellDedicated, tsc_RB2, (cr_108_MeasReportIntraFreqPeriodic2cell_e1b( 1, tcv_CellInfoB.priScrmCode,OMIT,tcv_CellInfoA.priScrmCode,OMIT,tcv_CellInfoA.priScrmCode )) ) | (P) | Step 5a. MEASUREMENT REPORT |