ETSI SMG4 / 3GPP TSG-T MExE	Tdoc SMG4 4M99-050

Helsinki, Finland

17th-19th February, 1999

| CHANGE REQUEST No : |  |  |  |  |  |  | Please see embedded help file at the bottom of this page for instructions on how to fill in this form correctly. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Technical Specification GSM / UMTS: |  |  |  | 03.57 |  | Version |  | 1.4.0 |  |  |  |
| Submitted to SMG |  |  | For approval |  |  | without presentation ("non-strategic") |  |  |  |  |  |
| list plenary meeting or STC here  |  | for information |  |  |  | with presentation ("strategic") |  |  |  |  |  |
| PT SMG CR cover form. Filename: crf26_3.doc |  |  |  |  |  |  |  |  |  |  |  |

| Proposed change affects: | SIM |  | ME | X | Network |  |
| --- | --- | --- | --- | --- | --- | --- |

(at least one should be marked with an X)

| Work item: | MExE |
| --- | --- |

| Source: | Motorola | Date: | 12/01/99 |
| --- | --- | --- | --- |

| Subject: | User profile updates and security |
| --- | --- |

| Category: | F Correction |  | Release: | Phase 2 |  |
| --- | --- | --- | --- | --- | --- |
|  | A Corresponds to a correction in an earlier release |  |  | Release 96 |  |
| (one category | B Addition of feature |  |  | Release 97 |  |
| and one release | C Functional modification of feature | X |  | Release 98 | X |
| only shall be | D Editorial modification |  |  | Release 99 |  |
| marked with an X) |  |  |  | UMTS |  |

| Reason for change: | Text added to clarify that the security profile shall be managed and stored securely. Additional information (including a reference to 22.01) is added, as well as an updated security table (following comments received from Tim Wright) which has been reformatted. The table includes a new final row identifying new permissions/privileges for activation/modification of a user profile. [Note that this CR contains an updated security table featured in a different CR]. |
| --- | --- |

| Clauses affected: |  |
| --- | --- |

| Other specs | Other releases of same spec |  |  List of CRs: |  |
| --- | --- | --- | --- | --- |
| affected: | Other core specifications |  |  List of CRs: |  |
|  | MS test specifications / TBRs |  |  List of CRs: |  |
|  | BSS test specifications |  |  List of CRs: |  |
|  | O&M specifications |  |  List of CRs: |  |

| Other comments: |  |
| --- | --- |

[10]	Java Mail API version 1.0.2, available from http://www. java.sun.com

[11]	UMTS TR 22.70: “Universal Mobile Telecommunications System (UMTS); Service aspects; Provision of Services in UMTS - The Virtual Home Environment”.

[12]	UMTS TS 22.xx: “Universal Mobile Telecommunications System (UMTS); Provision of Services in UMTS - The Virtual Home Environment: Stage 1”.

[13]	ISO 639 International Standard - codes for the representation of language names

[14]	UMTS TS 22.01: “Universal Mobile Telecommunications System (UMTS); Service Aspects; Service Principles”.

# 3	Definitions and abbreviations

## 3.1	Definitions

For the purposes of this TS the following definitions apply:

phonebook: a phonebook is a dataset of personal or entity attributes. The simplest form is a set of name-number pairs as supported by the current GSM SIM

service: a service is set of functions offered to a user by an organisation

Further definitions specific to MExE are in GSM given in 02.57 (MExE stage 1)

## 3.2	Abbreviations

For the purposes of this TS the following abbreviations apply:

API	Application Programming Interface

CGI	Common Gateway Interface

DHCP	Dynamic Host Configuration Protocol

GSM	Global System for Mobile Communication

HTTP	HyperText Transfer Protocol

IP	Internet Protocol

JNDI	Java Naming Directory Interface

JTAPI	Java Telephony Application Programming Interface

JAR file	Java Archive File

MSE	MExE Service Environment

RFC	Request For Comments

SAP	Service Access Point

SMS	Short Message Service

TLS	Transport Layer Security

UDP	User Datagram Protocol

URL	Uniform Resource Locator

USSD	Unstructured Supplementary Service Data

WAE	Wireless Application Environment

WAP	Wireless Application Protocol

WSP	Wireless Session Protocol [WSP]

WTA	Wireless Telephony Applications

WTAI	Wireless Telephony Applications Interface

WTLS	Wireless Transport Layer Security

WTP	Wireless Transaction Protocol

WWW	World Wide Web

<a href="14sunflowers_lowres.jpg">JPEG-format</a>.

<br>

medium resolution (400x600):

<a href="14sunflowers_midres.gif">GIF-format</a>,

<a href="14sunflowers_midres.jpg">JPEG-format</a>.

<br>

high resolution (800x1200):

<a href="14sunflowers_highres.gif">GIF-format</a>,

<a href="14sunflowers_highres.jpg">JPEG-format</a>.

<br>

super-high resolution (1200x1600):

<a href="14sunflowers_shighres.gif">GIF-format</a>,

<a href="14sunflowers_shighres.jpg">JPEG-format</a>.

</body>

</html>

...

Editor’s note: RFC 2068 states that "the server SHOULD include an entity containing the list of resource characteristics and location(s) from which the user or the user-agent can choose the one most appropriate".  The RFC does not state, how a user(-agent) could possible select any format, if the server does NOT send any such further indication about available formats...).

However, the RFC does not name or describe any specific format, in which such a list should be transferred, but rather notes that "the (list) entity format is specified by the media type given in the Content-type header field".  As there appears to be no special "format-list" media-type, the typical format for such a list is probably a simple .html-document describing the miscellaneous format choices (as shown in the above example).  The document can be displayed to the user, and the user then picks the desired format by clicking on one of the embedded hyper-links.

While it is very easy to implement this "carbon-supported" selection mechanism in a Web-browser, the possibility to select the correct format automatically is drastically limited, because the browser does not have any semantic understanding of the kind and attributes of the document versions listed in the .html document.  The RFC authors were probably quite aware of this limitation as they also noted "However, this specification does not define any standard for such automatic selection.". They probably expect(ed) that such a list format will be specified in another document.

In the absence of any such standardisation activities, it will probably be necessary to standardise such an attribute-list format so that MExE-user-agents can "understand" and automatically select from the list, such that automatic content format selection (not to speak of other capability negotiation issues...) may be enabled.

Editor’s note: Final two paragraphs of the above editorial note requires to be reconsidered and preferably deleted, and the editorial note converted into main text.

## 4.5	User profile

The user profile (which may consist of one or more user profiles for a user) contains the characterisation of the MExE MS as defined by the user and service provider.  The user profile is not unique to the MExE MS, and this clause identifies the usage and content of the user profile from a MExE perspective only, and does not identify the generic support of user profiles in general.  Refer to UMTS 22.01 [14] for further details on the user profile.

### Location of, access to, and security of, the user profile

As multiple user profiles may be defined, the user is able to set up or receive calls/connections associated with different user profiles simultaneously by securely activating a user profile (with each user profile being associated with at least one unique address).  Refer to the Security clause for further details on user profile activation.

The user’s characterisation of the MExE MS in the user profile may be modified at any time by the user and the service provider, and changes affected at the earliest possible opportunity.  Refer to the Security clause for further details of user and service provider modification of a user profile.

The user profile is securely managed by the MExE MS as defined in the Security clause, and stored in a secure area (i.e the SIM or the ME) of the MExE MS.  The service provider may also retain parts of the user profile in the network.

### 4.5.2	Characteristics of the user profile

The user profile acts as a repository (which is always available in the MExE MS) defining the MExE MS behaviour, identifying one or more of the following:-

|  | ME specific | Location(s) | Description |
| --- | --- | --- | --- |
| user interface |  |  | Refer to sub-clause 4.6 User interface customisation for details of the specific user interface customisation characterisation which may be stored in the user profile |
| background - colour - pattern/image |  |  |  |
| - font - type - colour - size |  |  |  |
| - interfaces - softkey definition - additional menu items - enabled/disabled extended menus - icon settings |  |  |  |
| - country - language - date - time - currency |  |  |  |
| - Modify supplementary services settings (i.e. diversion, times) | - ? |  | - ? |
| ###### Modify phone settings - ringer tone/vibrate - enabling SIM Lock - Keypad Lock |  |  |  |
| - Preferences |  |  | - 03.57, section 4.4.1 - ”Support of the user profile preferences is subject to the capabilities of the device. Generally the combination of user profile preferences and ME logic will determine the information sent in the capability negotiation from the MExE device to MSE.” |
| - terminal properties - language in which the user wishes to work - preferred input mode - amount of memory authorised by the user for MExE |  |  |  |
| - data transfer - prioritised list of preferred bearers, e.g. SMS, USSD, GPRS - maximum size of transfer - maximum duration of transfer - transfer of location information - transfer of the user profile or parts of it |  |  | - may, or may not be, sent to the MSE - could be shifted in Security section - could be shifted in Security section |
| - codecs - prioritised list of preferred audio codecs - prioritised list of preferred image codecs - prioritised list of preferred video codecs - prioritised list of preferred multimedia codecs |  |  |  |
| Service Management |  |  | Refer to sub-clauses 4.5, 4.7.1, 4.7.2 controls the ability to determine which services are transferred to, resident, configured or executing on the MS, which may be refined by use of one or more of the following characteristics: identified security level specific permitted services types of permitted services date or times for actions priorities of services and identifies service preferences which may be set globally for all services per security level on a per service basis and may be identified at installation of the service, or be subsequently modified. |
| transfer of services |  |  |  |
| automatic configuration |  |  |  |
| execution of services |  |  |  |
| residence on the MExE MS |  |  |  |
| personalisation of services |  |  | 02.57, section 6.1 for further details see section VHE of this table |
| Security |  |  | control of the following permissions maybe refined by use of one or more of the following characteristics: - universal authorisation for all calls/connections/ messages (i.e. yes/no) - authorisation by calls/connections/messages type (i.e. SMS, USSD etc.) - authorisation by identified services - authorisation by service type (i.e. security level) - authorisation by date/time/day of week |
| call authorisation |  |  | 03.57, section 4.5 |
| Implicit permission for the initiation of events (includes responses made by the MS to external events) of transferred applications, applets |  |  | 02.57, section 11.1 and 11.2 (Timothy Wright) |
| Implicit permission for the transfer of applications, applets and content |  |  | 02.57, section 11.1 and 11.2 (Timothy Wright) |
| Network event journal - status of the optional network event journal which is used to maintain a record of application initiated network events (i.e. activated/deactivated) |  |  |  |
| Miscellaneous |  |  |  |
| phonebook entry |  |  | 03.57, section 4.6 |
| ###### Redirect - voicemail box number - fax-mail box number |  |  |  |
| - reset call meters |  |  | - 03.57, section 4.6 |
| Classmark 1 specific Entries |  |  |  |
| Classmark 2 specific Entries |  |  |  |
| ###### WAP browser - pre-installed browser overwrite on upgrade - location of user preferred WAP browser |  |  | - applicable to MExE classmark 2 |
| VHE |  |  | TS 22.xx :Provision of Services in UMTS – The Virtual Home Environment (Tdoc SMG1 825/98) |
| - for each VHE profile: - Incoming session routing address |  |  |  |
| - for each service within one VHE profile: - service state - service appearance - service appearance customisation parameters - service functionality customisation parameters - service data |  |  | - e.g. activate/deactivate/not available - e.g. level of functionality complexity - e.g. forwarded-to numbers |

The above list is not exhaustive.

Support of the user profile preferences is subject to the capabilities of the device.

4.6	User interface customisation

The MS interface consists of the buttons, menus, screens and MMI as designed and provided by the MS manufacturer; the nature of this MS interface is naturally evolving, MS specific and proprietary to the individual manufacturers of the industry.  This interface is the one normally seen by the user in normal operation of his MS.  This specification does not place any requirements or limitations on the individual manufacturers’ MS interface.

The MExE MMI, in turn, is the interface available to the user to support MExE services and functionality on the MS.  The nature of the MExE MMI interface, like the normal MS interface described above, is not standardised in any way, to allow for manufacturer innovation, cater for evolving market needs, and permit manufacturer differentiation.  The MExE MMI, depending on different manufacturer implementations, may consist of the normal MS interface, the normal MS interface with modifications, a different interface to the normal MS interface, or some combinations thereof etc.  MExE services operate within, and using the capabilities of, the MExE MMI.

User interface customisation consists of two parts.  The first part refers to the user’s ability to request, and verify, the preferred changes to the user interface; thus the user’s preferences, as supported by the specific MS, require to be recorded.  The second part refers to the MExE MS’s support of the user’s preferences for the interface, wihtin the capabilities of an MS.  By defining the user interface customisation to consist of two stages, the preferences which have been recorded by the user may be transferred (as part of the user profile), and thereby provide protability of the user’s preferences.

### 4.6.1	MExE user interface customisation

Customisation of the user interface offers the MExE Service Environment and or the user, the ability to inform the MExE MS of the desired extent of customisation.  All support of the user interface customisation is optional, not mandatory on any class of MS, and subject to the capabilities of the MS.  Depending on the capability of the MS, the customisation may be fully supported, partially supported, interpreted or ignored.

Customisation of the user interface is not restricted to the modifying the appearance of the MMI, but also the modification of MMI parameters (e.g. programming of the voicemail number).  The user’s customisation of the interface is retained as part of the user profile.

# 9	Security

Editor‘s Note: input awaited from SMG10

## 9.1		Generic security

In order to manage the MExE and prevent attack from unfriendly sources or transferred applications unintentionally damaging the MExE device a security system is required. This section defines the MExE security architecture.

The basis of MExE security is a framework of permissions which defines the permissions and privileges transferred content, applets or applications have within the MExE. The MExE framework is defined in TS GSM 02.57 and is as follows (there is no implied hierarchy):

- MExE Security Level 1 (used by the HPLMN operator);

- MExE Security Level 2 (system applications, applets and content);

- MExE Security Level 3 (trusted applications, applets and content);

- MExE Security Level 4 (untested applications, applets and content);

The following table specifies the privileges and permissions of each security level.

|  | MExE Security Levels |  |  |  |
| --- | --- | --- | --- | --- |
| Permission / Privilege |  | 1 | 2 | 3 | 4 |
| - Read file - Write file - Get file information - Delete file | Yes (except in user pre-specified directories) |  |  | Yes (only in application/applet’s own directory) (Restriction may be required on space allocated to write) |
| Load native code libraries | No | Yes | No |  |
| - Initiate a voice/data connection - Accept a voice/data connection - Intercept an incoming voice/data connection. (A network connection may be via any supported bearer service). | Yes | Yes (with user permission) |  | No |
| Call forward | Yes (with user permission) | No | Yes (with user permission, to pre-defined numbers in the user profile) | No |
| - Terminate an existing connection - Hold an existing connection - Resume an existing connection | Yes |  | Yes (with user permission) | No |
| Modification of user invoked connection | Yes |  | Yes (with user permission) | No |
| - Add a phonebook entry - Delete a phonebook entry - Get a phonebook entry - Modify a phonebook entry | Yes (with user permission) |  |  | No |
| Send an SMS | Yes | Yes (with user permission) |  | No |
| Add an application or applet | Yes (with user permission) |  |  | No |
| Delete a saved application or applet | Yes (with user permission) |  | Yes (with user permission, but only in pre-specified directory) | No |
| - Add a certificate for a given level - Delete a certificate for a given level - Replace a certificate for a given level | Only the organisation whose public key is certified (or the organisation that certified the public key) can add, delete or replace a particular certificate (with user permission) |  |  | No |
| Audio access (e.g. wake-up service) | Yes |  |  | No |
| HTTP Cache manipulation | Yes |  |  | No |
| Get IMEI | Yes |  |  | No |
| Location Information | Yes (with user permission) | No | Yes (with user permission) | No |
| - Activate a user profile - Modify a user profile | Yes (with user permission) | No | Yes (with user permission) | No |