| CHANGE REQUEST No : |  |  |  |  |  |  | . |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Technical Specification GSM / UMTS: |  |  |  | 03.57 |  | Version |  | 1.7.1 |  |  |  |
| Submitted to SMG | 4 |  | for approval |  | x | without presentation ("non-strategic") |  |  |  |  |  |
|  |  | for information |  |  |  | with presentation ("strategic") |  |  |  |  |  |
| PT SMG CR cover form. Filename: crf26_3.doc |  |  |  |  |  |  |  |  |  |  |  |

| Proposed change affects: | SIM |  | ME | x | Network |  |
| --- | --- | --- | --- | --- | --- | --- |

(at least one should be marked with an X)

| Work item: | MExE Stage 2 |
| --- | --- |

| Source: | Texas Instruments, Motorola, Nokia, IBM | Date: | 26.5.1999 |
| --- | --- | --- | --- |

| Subject: | CR to 03.57 on Java MExE Devices |
| --- | --- |

| Category: | F Correction | x | Release: | Phase 2 |  |
| --- | --- | --- | --- | --- | --- |
|  | A Corresponds to a correction in an earlier release |  |  | Release 96 |  |
| (one category | B Addition of feature |  |  | Release 97 |  |
| and one release | C Functional modification of feature |  |  | Release 98 | x |
| only shall be | D Editorial modification |  |  | Release 99 |  |
| marked with an X) |  |  |  | UMTS |  |

| Reason for change: | Update of Java MExE devices section to incorporate JavaPhone API liason from JavaPhone and Sun Microsystems. |
| --- | --- |

| Clauses affected: | 2.1, 6., Appendix A |
| --- | --- |

| Other specs | Other releases of same spec |  | ® List of CRs: |  |
| --- | --- | --- | --- | --- |
| affected: | Other core specifications |  | ® List of CRs: |  |
|  | MS test specifications / TBRs |  | ® List of CRs: |  |
|  | BSS test specifications |  | ® List of CRs: |  |
|  | O&M specifications |  | ® List of CRs: |  |

| Other comments: | Changes made to Tdoc99023, with the addition of deleting Appendix A from the Stage 2. |
| --- | --- |

## 2.1	Normative references

[1]	GSM 01.04 (ETR 350): “Digital cellular telecommunications system (Phase 2+); Abbreviations and acronyms”.

[2]	GSM 02.57: "Digital cellular telecommunications system (Phase 2+);MExE Stage 1 Description"

[3]	Personal Java 1.1.1, this specification is available from Sun Microsystems’s website at http://java.sun.com/products/personaljava/spec-1-1-1/index.html .

[4]	JavaPhone API version 0.9, available from http://java.sun.com/products/javaphone/.

[5]	JTAPI 1.2, this specification, currently in the ECTF approval process, is available from Sun Microsystems’s website at http://www.java.sun.com.

[6]	Wireless Application Protocol (WAP) version 1.1, this specification is available from WAP‘s website at http://www.wapforum.org.

[7]	vCard – The Electronic Business Card Exchange Format – Version 2.1, The Internet Mail Consortium (IMC), September 1996, http://www.imc.org/pdi/vcard-21.doc.

[8]	vCalendar – The Electronic Calendaring and Scheduling Exchange Format – Version 1.0, The Internet Mail Consortium (IMC), September 1996, http://www.imc.org/pdi/

[9]	Hypertext Transfer Protocol – HTTP/1.1, IETF document RFC2068, http://www.w3.org/Protocols/rfc2068/rfc2068

[10]	Java Mail API version 1.0.2, available from http://www. java.sun.com

[11]	UMTS TR 22.70: “Universal Mobile Telecommunications System (UMTS); Service aspects; Provision of Services in UMTS - The Virtual Home Environment”.

[12]	UMTS TS 22.xx: “Universal Mobile Telecommunications System (UMTS); Provision of Services in UMTS - The Virtual Home Environment: Stage 1”.

[13]	ISO 639 International Standard - codes for the representation of language names

[14]	UMTS TS 22.01: “Universal Mobile Telecommunications System (UMTS); Service Aspects; Service Principles”.

[15]	CC/PP Exchange Protocol based on HTTP Extension Framework; Available at W3C web pages http://www.w3c.org/

[16]	Composite Capability/Preference Profiles (CC/PP):A user side framework for content negotiation;  Available at W3C web pages.

[17]	Uaprof  Specification (Editor´s note: reference needed after the specification is available)

[18]	JDK 1.1 security http://www.javasoft.com/products/jdk/1.1/docs/guide/security/index.html

[19]	Java 2 security http://www.javasoft.com/products/jdk/1.2/docs/guide/security/index.html

[20]	Java security tutorial http://java.sun.com/docs/books/tutorial/security1.2/overview/index.html

[21]	OCF 1.1.: “Smartcard API specified by OpenCard Consortium (http://www.opencard.org)

# 6	Java MExE devices

Java MExE devices shall be based on the MExE API for Java, which defines the required and optional components of Java APIs that shall be used to realise a MExE compliant device.

The MExE API for Java primarily defines the functions available to a Java-based MExE device such that services (specified in the form of Java classes and interfaces) can control such a device in a standardised way.

Many aspects of the MExE API specification are optional. Services and applications shall be able to determine the presence of optional parts of the functionality. When optional parts of the functionality are implemented, the MExE API shall be supported.

6.1	High level architecture

Figure 5: Basic Functional Architecture of Java MExE Device

The functional architecture of a Java MExE device is shown in figure 5. Java applets, applications, and services access functionality via the MExE API for Java. The MExE API is based on a combination of optional Java APIs approved by Sun Microsystems and the Wireless Profile of the JavaPhoneÔ API [4] as defined by the JavaPhone Expert Group. The JavaPhone API is based on the PersonalJavaÔ API [3] defined by Sun Microsystems.

## 6.2	High level functions

### 6.2.1	Optionality

The use of Java encourages development of modular interfaces and minimal required functionality. Additional functionality is provided by optional  APIs specified in terms of the Java language. In general, optionality is specified in terms of Java packages. Packages are containers for the highest level of functionality in the Java language. In some cases, optionality is specified in terms of Java classes and interfaces. Classes and interfaces are elements contained inside packages.

The following table specifies the optionality of the Wireless Profile of the JavaPhone APIs. Within some of the packages, certain classes and methods may be individually specified as optional by the JavaPhone™ API specification.

| JavaPhone API | Java package | Optionality |
| --- | --- | --- |
| Addressbook | Javax.pim.addressbook | Mandatory |
| User Profile | Javax.pim.userprofile | Mandatory |
| Calendar | Javax.pim.calendar | Mandatory |
| Network | Java.net | Mandatory |
| Datagram | Javax.net.datagram | Mandatory |
| Power Monitor | Javax.power.monitor | Mandatory |
| Power Management | Javax.power.management | Optional |
| Install | Javax.install | Optional |
| Communications | Java.comm | Optional |
| SSL | Javax.net.ssl | Optional |
| JTAPI | Javax.telephony | Mandatory |
| JTAPI Core Package | Javax.telephony | Mandatory |
| JTAPI Core Capabilities Package | Javax.telephony.capabilities | Optional |
| JTAPI Core Events Package | Javax.telephony.events | Mandatory |
| JTAPI Call Control Package | Javax.telephony.callcontrol | Optional |
| JTAPI Call Control Capabilities Package | Javax.telephony.callcontrol.capabilities | Optional |
| JTAPI Call Control Events Package | Javax.telephony.callcontrol.events | Optional |
| JTAPI Phone Package | Javax.telephony.phone | Optional |
| JTAPI Phone Capabilities Package | Javax.telephony.phone.capabilities | Optional |
| JTAPI Phone Events Package | Javax.telephony.phone.events | Optional |
| JTAPI Mobile Package | Javax.telephony.mobile | Mandatory |
|  | Java.math | Optional |
|  | Java.rmi | Optional |
|  | Java.rmi.dgc | Optional |
|  | Java.rmi.registry | Optional |
|  | Java.rmi.server | Optional |
|  | Java.security | Optional |
|  | Java.security.interfaces | Optional |
|  | Java.sql | Optional |
|  | Java.io | Optional |
|  | Java.awt | PersonalJava spec |

### 6.2.2	Required and optional PersonalJava APIs

Java MExE devices shall support the PersonalJava specification [3]. The PersonalJava APIs provide a standardised and readily implementable execution environment as a means for applications, applets, and content:

- to access and personalise the user interface via the java.awt packages

- to utilise both Internet and Intranet connections via the java.net package

### 6.2.3	Required and optional JavaPhone APIs

The JavaPhone APIs extend the PersonalJava APIs to provide functionality unique to telephony devices. Java MExE devices shall support the Wireless Profile of the JavaPhone API specification [4]. Java MExE devices shall support all APIs specified as required by the Wireless Profile in the JavaPhone API specification. All APIs that are optional in the Wireless Profile shall be optional in Java MExE devices.

#### 6.2.3.4	Application Installation

Java MexE devices shall support the following JAR file manifest entries (as described in the JavaPhone specification) as described below:

- Implementation-Title		the Implementation-Title shall be used in any textual description of the application which is displayed in the UI element used to launch the application. E.g. the text displayed with an icon.

- Main-Icon			if icons are used as elements to launch the application, then the icon file within the JAR file named by the Main-Icon attribute shall be displayed, and may be scaled if desired.

- Main-Class and Class-Path	when the application is launched, the VM shall be supplied with the classpath and shall call the main() method in the class named by the Main-Class attribute.

6.2.3.5	Power

Java MExE devices shall support the Power Monitor package (javax.power.monitor) as specified by the JavaPhone API to access the power level of the device and receive notifications concerning changes in power states.

Note that the Power Monitor package does not specify the minimum required events that should be generated under certain circumstances. A MExE Java device shall at least implement the following event generation:

- BatteryCritical:		shall be generated when the battery is at a critically low level.

- BatteryNormal:		shall be generated when the battery is no longer low.

All the other event generation should be supported by the implementation.

6.2.4	Required and optional MExE APIs

A Java MExE device shall not be required to support any other Java APIs.

A Java MExE device may optionally support any other Java APIs approved by Sun Microsystems, such as:

- OCF SmartCard API OpenCard, available from http://www.opencard.org/index-docs.html.  If the ME supports smartcards other than the SIM, and the smartcard is open to 3rd party applications, then the  opencard.core.terminal section of the OpenCard API shall be used to access the card.

6.2.5	Mandated Services and Applications

#### 6.2.5.1 WAP Browser Support

To provide backward compatibility to MExE classmark 1, i.e. allow access to services designed for MExE classmark 1 devices, classmark 2 devices must feature a pre-installed or pre-loaded WAP browser that is capable of rendering at least the following content formats:

- tokenized WML documents (“WML decks”)

- WMLscript bytecode

- A WAP service in a MExE Classmark 2 MS shall execute in the same manner as it executes in a MExE classmark 1 MS.

Other WML formats (such as textual WML documents or textual WMLscripts) are optional.

The pre-installed/pre-loaded WAP browser may be upgraded, replaced or extended by transferring, a replacement, extension or plug-in mechanism to the MS.  Depending on user preferences identified in the user profile and the terminal capabilities, the pre-installed or pre-loaded WAP browser may be overwritten or the new browser stored in a different location.

6.2.5.2 Network Protocol Support

Support for network protocols in Java MexE devices is specified in the following table:

| Protocol | Optionality |
| --- | --- |
| HTTP/1.0 | Mandatory |
| HTTPS | Mandatory |
| Gopher | Optional |
| ftp | Optional |
| mailto | Mandatory |
| File | Optional |
| Phoneto | Mandatory |