| TSG-T2 / SMG4 MExE 26th-28th May 1999 Böblingen, Germany | Tdoc TX299035 |
| --- | --- |

| CHANGE REQUEST No : |  |  |  |  |  |  | Please see embedded help file at the bottom of this page for instructions on how to fill in this form correctly. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Technical Specification GSM / UMTS: |  |  |  | 03.57 |  | Version |  | 1.7.1 |  |  |  |
| Submitted to SMG |  |  | for approval |  |  | without presentation ("non-strategic") |  |  |  |  |  |
| list plenary meeting or STC here  |  | for information |  |  |  | with presentation ("strategic") |  |  |  |  |  |
| PT SMG CR cover form. Filename: crf26_3.doc |  |  |  |  |  |  |  |  |  |  |  |

| Proposed change affects: | SIM |  | ME | x | Network |  |
| --- | --- | --- | --- | --- | --- | --- |

(at least one should be marked with an X)

| Work item: |  |
| --- | --- |

| Source: | Motorola | Date: |  |
| --- | --- | --- | --- |

| Subject: | User Profiles |
| --- | --- |

| Category: | F Correction |  | Release: | Phase 2 |  |
| --- | --- | --- | --- | --- | --- |
|  | A Corresponds to a correction in an earlier release |  |  | Release 96 |  |
| (one category | B Addition of feature |  |  | Release 97 |  |
| and one release | C Functional modification of feature | X |  | Release 98 | x |
| only shall be | D Editorial modification |  |  | Release 99 |  |
| marked with an X) |  |  |  | UMTS |  |

| Reason for change: | VHE has recently updated its specification to indicate that user profiles may be based on CC/PP, which was recently adopted by MExE for capability negotiation. It is proposed that MExE Stage 2 is updated to reflect the recent developments for user profile support. Given the generic support of user preferences of CC/PP, the following changes propose that there is no longer any need for MExE to explicitly define the user profile characteristics. Instead, a combination of the direct and indirect referencing mechanisms of CC/PP should be used to define user profile characteristics. MExE-specific user profile characteristics should in future either be - be negotiated with W3C (for direct references), or - be operator/manufacturer/user defined (by indirect references) as supported by W3C. Such a change makes MExE generically compatible with globally recognised standards of the internet. |
| --- | --- |

| Clauses affected: | 4.5 |
| --- | --- |

| Other specs | Other releases of same spec |  |  List of CRs: |  |
| --- | --- | --- | --- | --- |
| affected: | Other core specifications |  |  List of CRs: |  |
|  | MS test specifications / TBRs |  |  List of CRs: |  |
|  | BSS test specifications |  |  List of CRs: |  |
|  | O&M specifications |  |  List of CRs: |  |

| Other comments: |  |
| --- | --- |

<--------- double-click here for help and instructions on how to create a CR.

# 1	Scope

This ETSI Technical Specification defines the stage two description of the Mobile Station Application Execution Environment (MExE). Stage 2 identifies the functional capabilities and information flows needed to support the service described in stage 1.

This TS includes information applicable to network operators, service providers and terminal, switch and database manufacturers.

This TS contains the core functions for a Mobile Station Application Execution Environment (MExE) which are sufficient to provide a complete service.

MExE uses a number of technologies to realise the requirements of the stage 1 description (GSM 02.57). This TS describes how the service requirements are realised with the selected technologies. The TS is devised into sections each covering the aspects relating to particular MExE technologies, it is intended that this specification will evolve along with the MExE technologies. A generic section of the specification covers areas of MExE common to all technologies.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

For a specific reference, subsequent revisions do not apply.

For a non-specific reference, the latest version applies.

A non-specific reference to an ETS shall also be taken to refer to later versions published as an EN with the same number.

## 2.1	Normative references

[1]	GSM 01.04 (ETR 350): “Digital cellular telecommunications system (Phase 2+); Abbreviations and acronyms”.

[2]	GSM 02.57: "Digital cellular telecommunications system (Phase 2+);MExE Stage 1 Description"

[3]	Personal Java, this specification is available from Sun Microsystems’s website at http://java.sun.com/products/personaljava/index.html.

[4]	MNCRS WG 4 specifications are still under development. When completed, MNCRS WG 4 specifications will be available at http://www.mncrs.org/.

[5]	JTAPI 1.2, this specification, currently in the ECTF approval process, is available from Sun Microsystems’s website at http://www.java.sun.com.

[6]	Wireless Application Protocol (WAP) version 1.1, this specification is available from WAP‘s website at http://www.wapforum.org.

[7]	vCard – The Electronic Business Card Exchange Format – Version 2.1, The Internet Mail Consortium (IMC), September 1996, http://www.imc.org/pdi/vcard-21.doc.

[8]	vCalendar – The Electronic Calendaring and Scheduling Exchange Format – Version 1.0, The Internet Mail Consortium (IMC), September 1996, http://www.imc.org/pdi/

[9]	Hypertext Transfer Protocol – HTTP/1.1, IETF document RFC2068, http://www.w3.org/Protocols/rfc2068/rfc2068

[10]	Java Mail API version 1.0.2, available from http://www. java.sun.com

[11]	UMTS TR 22.170: “Universal Mobile Telecommunications System (UMTS); Service aspects; Provision of Services in UMTS - The Virtual Home Environment”.

[12]	UMTS TS 22.121: “Universal Mobile Telecommunications System (UMTS); Provision of Services in UMTS - The Virtual Home Environment: Stage 1”.

[13]	ISO 639 International Standard - codes for the representation of language names

[14]	UMTS TS 22.101: “Universal Mobile Telecommunications System (UMTS); Service Aspects; Service Principles”.

[15]	CC/PP Exchange Protocol based on HTTP Extension Framework; Available at W3C web pages http://www.w3c.org/

[16]	Composite Capability/Preference Profiles (CC/PP):A user side framework for content negotiation;  Available at W3C web pages.

[17]	Uaprof  Specification (Editor´s note: reference needed after the specification is available)

[18]	JDK 1.1 security http://www.javasoft.com/products/jdk/1.1/docs/guide/security/index.html

[19]	Java 2 security http://www.javasoft.com/products/jdk/1.2/docs/guide/security/index.html

[20]	Java security tutorial http://java.sun.com/docs/books/tutorial/security1.2/overview/index.html

[21]	OCF 1.1.: “Smartcard API specified by OpenCard Consortium (http://www.opencard.org)

## 4.5	User profile

The user profile (which may consist of sub user profiles for a user) contains the characterisation of the MExE MS as defined by the user and service provider. Further, it is also possible for multiple users of a MExE MS to each have their own user profiles.  The user profile is not unique to the MExE MS, and this clause identifies the usage and content of the user profile from a MExE perspective only, and does not identify the generic support of user profiles in general.  Refer to UMTS 22.101 [14] for further details on the user profile.

### 4.5.1	Location of, access to, and security of, the user profile

As multiple user profiles may be defined, the user is able to set up or receive calls/connections associated with different user profiles simultaneously by securely activating a user profile (with each user profile being associated with at least one unique address).  Refer to the Security clause for further details on user profile activation.

The user’s characterisation of the MExE MS in the user profile may be modified at any time by the user and the service provider, and changes affected at the earliest possible opportunity. Refer to the Security clause for further details of user and service provider modification of a user profile.

The security clause shall apply to all user profiles at all times, whether activated or not

The user profile is securely managed by the MExE MS as defined in the Security clause, and stored in a secure area of the MExE MS (either SIM or ME).  The service provider may also retain parts of the user profile in the network.

The support of more than one user profile is not mandatory.

### 4.5.2		User profile and capability negotiation relationship

The user profile contains the user’s preferences. Support of the user’s preferences will depend on the capabilities of the device.  If the capabilities change, then the degree of support of the user’s preferences may change too.

The capability negotiation between the MExE terminal and the MSE contains those user preferences which the device is able to support.

In this way the MSE will serve a  MExE terminal with the lowest common denominator of the user’s preferences, the terminal capabilities and the provided service characteristics and support the user’s preferences to the maximum degree.

Figure 3: Model of user profile and capability relationship

### 4.5.3	Support of the user profile

The user profile acts as a repository (which is always available in the MExE MS) defining the MExE MS behaviour.

- MExE preferences and personalisation are supported in the user profile (e.g. UMTS portability and support of VHE defined in [12] and other 22-series specifications), which in turn is based on the Composite Capability/Preference Profile (CC/PP) specification from W3C [16].

- MExE preferences and personalisation may not only be recorded directly in the user profile as supported by CC/PP (the direct referencing mechanism), but indeed may also may be retrieved from a URL (the indirect referencing mechanism).

- Generally, the user profile’s CC/PP framework provides the mechanism for the standardised format of preferences, and its use of Resource Description Framework (RDF) permits the interoperable encoding of MExE preferences and personalisation.  Future extensions will be supported by the W3C mechanism, allowing for evolution and development of MExE preferences and personalisation.

The set of preferences which are supported in the user profile consists of the following:-

- user interface personalisation

- the user’s personalisation of the user interface.

- service personalisation and management

- the user’s generic service management information.

The coding and presentation of the above characteristics in the user profile is defined by the Composite Capability/Preference Profile (CC/PP) specification from W3C [16], and referenced by the MExE capability negotiation in clause 4.

The above list is not exhaustivethe capabilities of the device.