ETSI SMG4	TSGT2#2(99)xxx

Boeblingen, 1926-28 May 1999

| CHANGE REQUEST No : |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Technical Specification GSM / UMTS: |  |  |  | 03.57 |  | Version |  | 1.7.0 |  |  |  |
| Submitted to SMG |  |  | for approval |  | X | without presentation ("non-strategic") |  |  |  |  |  |
|  |  | for information |  |  |  | with presentation ("strategic") |  |  |  |  |  |

| Proposed change affects: | SIM |  | ME | x | Network |  |
| --- | --- | --- | --- | --- | --- | --- |

(at least one should be marked with an X)

| Work item: | MExE Stage 2 03.57 |
| --- | --- |

| Source: | Nokia | Date: | 3.5.1999 |
| --- | --- | --- | --- |

| Subject: | Additional detail for the Security Table |
| --- | --- |

| Category: | F Correction | X | Release: | Phase 2 |  |
| --- | --- | --- | --- | --- | --- |
|  | A Corresponds to a correction in an earlier release |  |  | Release 96 |  |
|  | B Addition of feature |  |  | Release 97 |  |
|  | C Functional modification of feature |  |  | Release 98 | x |
|  | D Editorial modification |  |  | Release 99 |  |
|  |  |  |  | UMTS |  |

| Reason for change: | Some important functions were not mentioned in the security table, and this is a security loop-hole. If they are not mentioned, then it is quite easy to create a new API which accesses this unmentioned functionality, and it will be free of security restrictions. |
| --- | --- |

| Clauses affected: | 8 |
| --- | --- |

| Other specs | Other releases of same spec |  | ® List of CRs: |  |
| --- | --- | --- | --- | --- |
| Affected: | Other core specifications |  | ® List of CRs: |  |
|  | MS test specifications / TBRs |  | ® List of CRs: |  |
|  | BSS test specifications |  | ® List of CRs: |  |
|  | O&M specifications |  | ® List of CRs: |  |

| Other comments: |  |
| --- | --- |

|  | MExE Security Domains |  |  |
| --- | --- | --- | --- |
| Actions |  | Operator | Manufacturer | Trusted Third Party |
| User Private Data Access 10 - Read - Write - Get properties - Delete - Get Location Information 9 - Read SMS - Delete SMS | Yes 2 |  |  |
| Network Access - Initiate a voice/data connection 5 - Accept a voice/data connection 5 - Call forward 6 - Multiparty call 6 - Call deflection 6 - Explicit call transfer 6 - Terminate an existing connection - Hold an existing connection - Resume an existing connection - Send point-point message (e.g. SMS, USSD) - Generate DTMF - Send USSD - Modify/query network status - Get signal level - Get call list | Yes |  |  |
| Lifecycle management - Install a MExE Executable - Uninstall a MExE executable | Yes |  |  |
| Certificate management - Install a certificate for a given domain - Uninstall a certificate for a given domain - Replace a certificate for a given domain | Yes 8 |  |  |
| Audio access - Sound generation to speaker (e.g. via stream) - Select Melody - Set speaker volume - Get melody list | Yes |  |  |
| Power Management - Monitor the power state - Change the power state | Yes |  |  |
| User Network Information - Get IMSI - Get IMEI - Get Available network - Get home network - Select network - Get manuafacturer software version | Yes |  |  |
| Device core function access - Start/stop radio - Turn on/off device | No |  |  |
| Legend for above footnotes:- 2 – File access is not permitted in user pre-specified directories. This list can only be modified by the user.– 5 – A network connection may be via any supported bearer service 6 – Call forwarding, multiparty, call deflection and explicit call transfer permitted only to numbers explicitly supplied by the user to the MExE Executable. Call forward and deflection refers to modification of the number stored in the network. 8 – Only the organisation whose public key is certified (or the organisation that certified the public key) can add, delete or replace a particular certificate. 9 – Getting location information shall always require explicit one-shot user permission. 10 – User private data includes user files, phonebook, etc located on the MS. |  |  |  |

Table 3: Security domains and actions