ETSI SMG4	TSGT2#2(99)053

Boeblingen, 1926-28 May 1999

| CHANGE REQUEST No : |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Technical Specification GSM / UMTS: |  |  |  | 03.57 |  | Version |  | 1.7.0 |  |  |  |
| Submitted to SMG |  |  | for approval |  | X | without presentation ("non-strategic") |  |  |  |  |  |
|  |  | for information |  |  |  | with presentation ("strategic") |  |  |  |  |  |

| Proposed change affects: | SIM |  | ME | x | Network |  |
| --- | --- | --- | --- | --- | --- | --- |

(at least one should be marked with an X)

| Work item: | MExE Stage 2 03.57 |
| --- | --- |

| Source: | Nokia | Date: | 3.5.1999 |
| --- | --- | --- | --- |

| Subject: | Additional detail for the Security Table |
| --- | --- |

| Category: | F Correction | X | Release: | Phase 2 |  |
| --- | --- | --- | --- | --- | --- |
|  | A Corresponds to a correction in an earlier release |  |  | Release 96 |  |
|  | B Addition of feature |  |  | Release 97 |  |
|  | C Functional modification of feature |  |  | Release 98 | x |
|  | D Editorial modification |  |  | Release 99 |  |
|  |  |  |  | UMTS |  |

| Reason for change: | Some important functions were not mentioned in the security table, and this is a security loop-hole. If they are not mentioned, then it is quite easy to create a new API which accesses this unmentioned functionality, and it will be free of security restrictions. |
| --- | --- |

| Clauses affected: | 8 |
| --- | --- |

| Other specs | Other releases of same spec |  | ® List of CRs: |  |
| --- | --- | --- | --- | --- |
| Affected: | Other core specifications |  | ® List of CRs: |  |
|  | MS test specifications / TBRs |  | ® List of CRs: |  |
|  | BSS test specifications |  | ® List of CRs: |  |
|  | O&M specifications |  | ® List of CRs: |  |

| Other comments: |  |
| --- | --- |

|  | MExE Security Domains |  |  |
| --- | --- | --- | --- |
| Actions |  | Operator | Manufacturer | Trusted Third Party |
| User Private Data Access 10 - Read - Write - Get properties - Delete - Get Location Information - Read stored SMS - Delete stored SMS | Yes 2 |  |  |
| Network Access - Initiate a voice/data connection 5 - Accept a voice/data connection 5 - Call forward 6 - Multiparty call 6 - Call deflection 6 - Explicit call transfer 6 - Terminate an existing connection - Hold an existing connection - Resume an existing connection - Send point-point message (e.g. SMS, USSD) - Generate DTMF - Modify/query network status - Get signal level - Get call list | Yes |  |  |
| Lifecycle management - Install a MExE Executable - Uninstall a MExE executable | Yes |  |  |
| Miscelaneous Functions - Get manuafacturer software version | Yes |  |  |
| Certificate management - Install a certificate for a given domain - Uninstall a certificate for a given domain - Replace a certificate for a given domain | Yes 8 |  |  |
| Audio access - Sound generation to speaker speaker (e.g. via stream) - Select Melody - Set speaker volume - Get melody list | Yes |  |  |
| Power Management - Monitor the power state - Change the power state | Yes |  |  |
| User Network Information - Get IMSI - Get IMEI - Get Available network - Get home network - Select network | No |  |  |
| Device core function access - Start/stop radio - Turn on/off device | No |  |  |
| User Profile management - Activate a user profile - Modify a user profile | No |  |  |
| Legend for above footnotes:- 2 – The user shall be able to specify data access permissions within the capabilities of the device. 5 – A network connection may be via any supported bearer service 6 – Multiparty, deflection, and explicit call transfer shall be permitted only to numbers explicitly supplied by the user to the MExE Executable. Modification of call forward numbers stored in the network shall only be permitted to numbers explicitly supplied by the user to the operator. 8 – Only the organisation whose public key is certified (or the organisation that certified the public key) can add, delete or replace a particular certificate. 10 – User private data includes user files, phonebook, etc located on the MS. |  |  |  |

Table 3: Security domains and actions