| 3GPP TSG-T2 SWG1 MExE Tampere, FINLAND, 15-16 July 1999 | T2X99069 |
| --- | --- |

# Liaison Statement

Date : 		1 July, 1999

#### Meeting : 	WAP WTA Meeting, San Francisco, USA

To :		ETSI/3GPP MExE,

CC:		SMSEG, WAG Push

From :	WAP WTA DC

Subject :	WAP public library functions in MEXE,  response to LS T2-99547

WAP WTA thanks the MExE group for their liaison statement and the fact that support for untrusted applications was added at such a late stage to the MExE specification.

Changes will be made to the WTAI specification in order to accommodate the additional MExE requirements imposed upon untrusted applications performing these functions.  Additionally, the specific requirement to restrict the actions of pushed content will be forwarded to the Push drafting committee for consideration in their work.

The group has noted that sending an SMS is not allowed under this scheme and this LS will be copied to the SMS expert group for their consideration.

The WTA group would also like to point out that an additional public library function has been approved that allows a script to create a new phone book entry.  This formally new public function is attached for your consideration on how MExE requirements might apply to its use.

### 7.3 Add Phonebook Entry

| ###### Description |  |
| --- | --- |
| Request the user to add a new entry to the phonebook. In case of duplication of entry, the user must acknowledge the operation. In case of unsuccessful operation, the output contains a negative number identifying the WTAI error code. |  |
| URI: | -wtai://pb/wr; <id>;<number>;<name> [ ? <result>] |
| WMLScript: | addPBEntry(number, name); |
| Function ID: | 2 |
| Parameters: | <number> = String: Phone number to be stored <name> = String: Name that will be associated with the phone number. |
| Output: | <result> = String: The return value is "0" (zero) if successful or a value below zero indicating the WTAI error code. Predefined relevant error are (see error codes values in Appendix B, table 5) : - "illegal number of parameters" - "service failed" - "service failed, out of memory" |
| Examples: | URI: wtai://pb/wr;5; 5551234; GENE; WMLScript: WTAPublic.addPBEntry("5554367", "EINSTEIN"); |
| Associated Events: | - |
| Notes: - |  |

Appendix A. WTA URI and WMLScript Function Libraries

| Lib/Func ID | URI | Script call | Description |
| --- | --- | --- | --- |
| 512.2 |  | WTAPublic.addPBEntry | Request adding a new phonebook entry |