| 3GPP TSG-T2 SWG1 MExE Tampere, FINLAND, 15-16 July 1999 | T2X99074 |
| --- | --- |

ETSI STC SMG1 meeting # 64 (99/2)		SMG1 (99)328

Quebec City 5th - 6th July 1999	Agenda Item: 9.4.5

Source: 	SMG1

To:	3GPP T2, SMG3, SMG3 WPC (CAMEL), TSG CN, TSG CN WG2, SMG9, SMG1/9 SAT ad hoc

Title:	Answer to LS on SAT/MExE  <-> CAMEL Interworking / Traceability

Document for:	Support & Action _______________________________________________________________________________________________

SMG1  wants to thank SMG1/9 SAT ad hoc for its LS (Tdoc SMG1 Ad-hoc 032/99) on: Interworking between SAT/MExE and CAMEL and traceability of SAT/MExE actions and 3GPP T2 SWG1 (Execution Environment) for the support and additions (Tdoc TSGT2#4(99)541).

In that LS SMG1/9 SAT ad hoc have proposed to introduce an information field in the call set up messages between the ME and the MSC that can be used by SAT or MExE applications for application identification and could be extended by a free formatted field for interworking purposes.

SMG1 considers this proposals important contributions, in fact it meets the requirement for traceability of application-initiated chargeable events.

SMG1 supports to include a field for identification purposes into following messages:

- Call set up

- Mobile initiated Short Messages

- GPRS session set-up

- Control messages for Supplementary Services

- possibly also into Mobile initiated USSD messages

SMG1 also supports the proposal to add an additional free formatted information field for application interworking purposes between CAMEL and SAT/MExE that can be used for information exchange between CAMEL and a SAT/MExE application for following Messages:

- MO/MT Call set up

- Mobile initiated Short Messages

- GPRS session set-up

SMG1 will include this requirement in the Camel Stage 1 for release ’99.