3GPP TSG T WG2, SWG1 (MExE)	Tdoc T2X99111

Newbury, England

11-13 August, 1999

Source: Vodafone

Proposed LS to 3GPP TSG T WG3 (T3), SMG9 on requested changes to 11.11 for operator root public key on the (U)SIM

MExE is pleased to forward the enclosed CR  to 11.11 Release 99 to SMG9 for approval, and for information to T3.

GSM release 99

The Release 99 CR describes changes required for the support of an operator root public key (certificate) on the SIM. This root public key will be used to verify applications and certificates on the terminal that have been digitally signed with the private key corresponding to the operator root public key.  The requirements include a new DF below DFGSM, DFMExE, and an EF below DFMExE, EFORPK, which contains the operator root public key, as a transparent file.  At SIM initialization, there is an additional ME procedure to ensure, if the ME is a MExE ME, that any operator root public key on the terminal is not considered valid unless it was issued by the operator that issued that SIM and there is not an operator root public key on the SIM.

MExE request that the enclosed CR to 11.11 be incorporated into the release 99 version of 11.11.

3GPP

MExE also anticipate 3GPP MExE terminals and therefore also request that T3 consider how the requirements embodied in the enclosed CR to 11.11 release 99 could be best incorporated into 31.102.  If a SIM application is present on a UICC then the MExE parameters shall exist within the SIM so that they can be accessed by GSM ME’s into which the UICC is inserted and dual mode ME’s in GSM mode.  However, the operator may well wish an operator root public key to be available to 3GPP MExE ME’s also.  It would be desirable if the duplication of the MExE parameters could be avoided.  T3 are asked to comment on this issue.  If there is more than one USIM application on a UICC, then each shall have its own MExE parameters.

T3 are asked to inform MExE how the changes to 11.11 can be incorporated into the latest version of 31.102 and, if possible, to perform this incorporation.

3GPP TSG T WG2, SWG1 (MExE)	Tdoc T2X99111

New bury, England

11-13 August, 1999

| CHANGE REQUEST No : |  |  |  |  |  |  |  | Please see embedded help file at the bottom of this page for instructions on how to fill in this form correctly. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Technical Specification GSM |  |  |  | 11.11 |  | Version: |  |  | Release 99 |  |  |  |
| Submitted to SMG | 30 |  | for approval |  |  |  | without presentation ("non-strategic") |  |  |  |  |  |
| list SMG plenary meeting no. here  |  | for information |  |  | X |  | with presentation ("strategic") |  |  |  | X |  |
| PT SMG CR cover form is available from: http://docbox.etsi.org/tech-org/smg/Document/smg/tools/CR_form/crf28_1.zip |  |  |  |  |  |  |  |  |  |  |  |  |

| Proposed change affects: | SIM | X | ME | X | Network |  |
| --- | --- | --- | --- | --- | --- | --- |

(at least one should be marked with an X)

| Work item: |  |
| --- | --- |

| Source: | 3GPP TSG T WG2 SWG1 (MExE) | Date: | 4/08/99 |
| --- | --- | --- | --- |

| Subject: | Support of operator root public key (certificate) in the SIM for use by MExE terminals. |
| --- | --- |

| Category: | F Correction |  | Release: | Phase 2 |  |
| --- | --- | --- | --- | --- | --- |
|  | A Corresponds to a correction in an earlier release |  |  | Release 96 |  |
| (one category | B Addition of feature |  |  | Release 97 |  |
| and one release | C Functional modification of feature | X |  | Release 98 |  |
| only shall be | D Editorial modification |  |  | Release 99 | X |
| marked with an X) |  |  |  |  |  |

| Reason for change: | Support of operator root public key (certificate) in the SIM for use by MExE terminals. |
| --- | --- |

| Clauses affected: | References: Addition of MExE stage 2, 23.057 Section 10.2: addition of new DFMExE, below DFGSM Section 10.3.7: addition of entry for “Operator MExE root public key” in SIM service table. Section 10.4.2: New section giving EF’s below DFMExE. Section 10.7: Addition of MExE DF and EF to the Figure 8. Section 11: Addition of MExE related procedures to the SIM application protocol. Section 11.5.16: New section detailing MExE related procedures. |
| --- | --- |

| Other specs | Other releases of same spec |  |  List of CRs: |  |
| --- | --- | --- | --- | --- |
| affected: | Other core specifications |  |  List of CRs: |  |
|  | MS test specifications / TBRs |  |  List of CRs: |  |
|  | BSS test specifications |  |  List of CRs: |  |
|  | O&M specifications |  |  List of CRs: |  |

| Other comments: |  |
| --- | --- |

## Normative References

[35]	3rd Generation Partnership Project, Technical Specification Group Terminals;Mobile Station Application Execution Environment (MExE);Functional description; Stage 2(3G TS 23.057 version 1.2.0)

## 10.2	DFs at the GSM application level

For compatibility with other systems based on the GSM switching platform and for special GSM services, DFs may be present as child directories of DFGSM. The following have been defined:

|  | DFIRIDIUM | '5F30' |
| --- | --- | --- |
|  | DFGLOBALSTAR | '5F31' |
|  | DFICO | '5F32' |
|  | DFACeS | '5F33' |
|  | DFPCS1900 | '5F40' |
|  | DFCTS | '5F60' |
|  | DFSoLSA | '5F70' |
|  | DFMExE | ‘5F80’ |

Only the contents of DFSoLSA and DFMExE are specified in the present document. For details of the EFs contained in the DFCTS, see GSM 11.19 [34].

### 10.3.7	EFSST (SIM service table)

This EF indicates which services are allocated, and whether, if allocated, the service is activated. If a service is not allocated or not activated in the SIM, the ME shall not select this service.

| Identifier: '6F38' |  | Structure: transparent |  |  | Mandatory |  |
| --- | --- | --- | --- | --- | --- | --- |
| File size: X bytes, X  2 |  |  | Update activity: low |  |  |  |
| Access Conditions: READ CHV1 UPDATE ADM INVALIDATE ADM REHABILITATE ADM |  |  |  |  |  |  |
| Bytes | Description |  |  | M/O |  | Length |
| 1 | Services n1 to n4 |  |  | M |  | 1 byte |
| 2 | Services n5 to n8 |  |  | M |  | 1 byte |
| 3 | Services n9 to n12 |  |  | O |  | 1 byte |
| 4 | Services n13 to n16 |  |  | O |  | 1 byte |
| 5 | Services n17 to n20 |  |  | O |  | 1 byte |
| 6 | Services n21 to n24 |  |  | O |  | 1 byte |
| 7 | Services n25 to n28 |  |  | O |  | 1 byte |
| 8 | Services n29 to n32 |  |  | O |  | 1 byte |
| etc. |  |  |  |  |  |  |
| X | Services (4X-3) to (4X) |  |  | O |  | 1 byte |

| -Services |  |  |
| --- | --- | --- |
| Contents: | Service n°1 : | CHV1 disable function |
|  | Service n°2 : | Abbreviated Dialling Numbers (ADN) |
|  | Service n°3 : | Fixed Dialling Numbers (FDN) |
|  | Service n°4 : | Short Message Storage (SMS) |
|  | Service n°5 : | Advice of Charge (AoC) |
|  | Service n°6 : | Capability Configuration Parameters (CCP) |
|  | Service n°7 : | PLMN selector |
|  | Service n°8 : | RFU |
|  | Service n°9 : | MSISDN |
|  | Service n°10: | Extension1 |
|  | Service n°11: | Extension2 |
|  | Service n°12: | SMS Parameters |
|  | Service n°13: | Last Number Dialled (LND) |
|  | Service n°14: | Cell Broadcast Message Identifier |
|  | Service n°15: | Group Identifier Level 1 |
|  | Service n°16: | Group Identifier Level 2 |
|  | Service n°17: | Service Provider Name |
|  | Service n°18: | Service Dialling Numbers (SDN) |
|  | Service n°19: | Extension3 |
|  | Service n°20: | RFU |
|  | Service n°21: | VGCS Group Identifier List (EFVGCS and EFVGCSS) |
|  | Service n°22: | VBS Group Identifier List (EFVBS and EFVBSS) |
|  | Service n°23: | enhanced Multi-Level Precedence and Pre-emption Service |
|  | Service n°24: | Automatic Answer for eMLPP |
|  | Service n°25: | Data download via SMS-CB |
|  | Service n°26: | Data download via SMS-PP |
|  | Service n°27: | Menu selection |
|  | Service n°28: | Call control |
|  | Service n°29: | Proactive SIM |
|  | Service n°30: | Cell Broadcast Message Identifier Ranges |
|  | Service n°31: | Barred Dialling Numbers (BDN) |
|  | Service n°32: | Extension4 |
|  | Service n°33: | De-personalization Control Keys |
|  | Service n°34: | Co-operative Network List |
|  | Service n°35: | Short Message Status Reports |
|  | Service n°36: | Network's indication of alerting in the MS |
|  | Service n°37: | Mobile Originated Short Message control by SIM |
|  | Service n°38: | GPRS |
|  | Service n°39: | Image (IMG) |
|  | Service n°40: | SoLSA (Support of Local Service Area) |
|  | Service n°41: | USSD string data object supported in Call Control |
|  | Service n°42: | MExE |

For a phase 2 SIM, the EF shall contain at least two bytes which correspond to the Phase 1 services. Further bytes may be included, but if the EF includes an optional byte, then it is mandatory for the EF to also contain all bytes before that byte. Other services are possible in the future and will be coded on further bytes in the EF. The coding falls under the responsibility of ETSI.

### 10.4.2	Contents of files at the GSM MExE level

This subclause specifies the EFs in the dedicated file DFMExE. It only applies if support of MExE by the SIM is supported (see 23.057  [nn]).

The EF(s) contains operator MExE parameter(s).

#### 10.4.2.1	EFORPK (Operator root public key)

This EF contains the operator root public key. This EF shall only be allocated if the operator wishes to verify applications and certificates in the MExE operator domain using a root public key held on the SIM

| Identifier: '????' |  | Structure: transparent |  |  | Optional |  |
| --- | --- | --- | --- | --- | --- | --- |
| File size: X + 2 bytes |  |  | Update activity: low |  |  |  |
| Access Conditions: READ CHV1 UPDATE ADM INVALIDATE ADM REHABILITATE ADM |  |  |  |  |  |  |
| Bytes | Description |  |  | M/O |  | Length |
| 1 to 2 | Length indicator |  |  | M |  | 2 bytes |
| 3 to X+2 | Operator root public key |  |  | M |  | X bytes |

## 10.7	Files of GSM

This subclause contains a figure depicting the file structure of the SIM. DFGSM shall be selected using the identifier '7F20'. If selection by this means fails, then DCS 1800 MEs shall, and optionally GSM MEs may then select DFGSM with '7F21'.

NOTE 1:	The selection of the GSM application using the identifier '7F21', if selection by means of the identifier '7F20' fails, is to ensure backwards compatibility with those Phase 1 SIMs which only support the DCS 1800 application using the Phase 1 directory DFDCS1800 coded '7F21'.

NOTE 2:	To ensure backwards compatibility with those Phase 1 DCS 1800 MEs which have no means to select DFGSM two options have been specified. These options are given in GSM 09.91 [17].

|  |  |  |  |  |  |  |  |  | MF |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  | '3F00' |  |  |  |  |  |  |  |  |  |  |
| DFGSM |  |  | DFTELECOM |  |  | DFIS-41 |  |  | DFFP-CTS |  |  |  |  |  | EFICCID |  |  | EFELP |  |
| '7F20' |  |  | '7F10' |  |  | '7F22' |  |  | '7F23' |  |  |  |  |  | '2FE2' |  |  | '2F05' |  |
|  |  |  |  |  |  |  |  |  | see GSM 11.19 |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | EFADN |  |  | EFFDN |  |  | EFSMS |  |  | EFCCP |  |  | EFMSISDN |  |
|  |  |  |  |  |  | '6F3A' |  |  | '6F3B' |  |  | '6F3C' |  |  | '6F3D' |  |  | '6F40' |  |
|  |  |  |  |  |  | EFSMSP |  |  | EFSMSS |  |  | EFLND |  |  | EFSMSR |  |  | EFSDN |  |
|  |  |  |  |  |  | '6F42' |  |  | '6F43' |  |  | '6F44' |  |  | '6F47' |  |  | '6F49' |  |
|  |  |  |  |  |  | EFEXT1 |  |  | EFEXT2 |  |  | EFEXT3 |  |  | EFBDN |  |  | EFEXT4 |  |
|  |  |  |  |  |  | '6F4A' |  |  | '6F4B' |  |  | '6F4C' |  |  | '6F4D' |  |  | '6F4E' |  |
|  |  |  |  |  |  | DFGRAPHICS |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | '5F50' |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  | EFIMG |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  | '4F20' |  |  |  |  |  |  |  |  |  |  |
|  |  |  | DFIRIDIUM |  |  | DFGLOBST |  |  | DFICO |  |  | DFACeS |  |  |  |  |  |  |  |
|  |  |  | '5F30' |  |  | '5F31' |  |  | '5F32' |  |  | '5F33' |  |  |  |  |  |  |  |
|  |  |  | DFPCS1900 |  |  | DFCTS |  |  | DFSoLSA |  |  | DFMExE |  |  |  |  |  |  |  |
|  |  |  | '5F40' |  |  | '5F60' |  |  | '5F70' |  |  | '5F80' |  |  |  |  |  |  |  |
|  |  |  |  |  |  | see GSM 11.19 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | EFSAI |  |  | EFSLL |  |  | EFORPK |  |
|  |  |  |  |  |  |  |  |  |  |  |  | '4F30' |  |  | '4F31' |  |  | ‘????’ |  |
|  |  |  | EFLP |  |  | EFIMSI |  |  | EFKc |  |  | EFPLMNsel |  |  | EFHPLMN |  |  | EFACMmax |  |
|  |  |  | '6F05' |  |  | '6F07' |  |  | '6F20' |  |  | '6F30' |  |  | '6F31' |  |  | '6F37' |  |
|  |  |  | EFSST |  |  | EFACM |  |  | EFGID1 |  |  | EFGID2 |  |  | EFPUCT |  |  | EFCBMI |  |
|  |  |  | '6F38' |  |  | '6F39' |  |  | '6F3E' |  |  | '6F3F' |  |  | '6F41' |  |  | '6F45' |  |
|  |  |  | EFSPN |  |  | EFCBMID |  |  | EFBCCH |  |  | EFACC |  |  | EFFPLMN |  |  | EFLOCI |  |
|  |  |  | '6F46' |  |  | '6F48' |  |  | '6F74' |  |  | '6F78' |  |  | '6F7B' |  |  | '6F7E' |  |
|  |  |  | EFAD |  |  | EFPHASE |  |  | EFVGCS |  |  | EFVGCSS |  |  | EFVBS |  |  | EFVBSS |  |
|  |  |  | '6FAD' |  |  | '6FAE' |  |  | '6FB1' |  |  | '6FB2' |  |  | '6FB3' |  |  | '6FB4' |  |
|  |  |  | EFeMLPP |  |  | EFAAeM |  |  | EFECC |  |  | EFCBMIR |  |  | EFNIA |  |  | EFKcGPRS |  |
|  |  |  | '6FB5' |  |  | '6FB6' |  |  | '6FB7' |  |  | '6F50' |  |  | '6F51' |  |  | '6F52' |  |
|  |  |  | EFLOCIGPRS |  |  | EFSUME |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | '6F53' |  |  | '6F54' |  |  |  |  |  |  |  |  |  |  |  |  |  |

Figure 8: File identifiers and directory structures of GSM

# 11	Application protocol

When involved in GSM administrative management operations, the SIM interfaces with appropriate terminal equipment. These operations are outside the scope of this standard.

When involved in GSM network operations the SIM interfaces with an ME with which messages are exchanged. A message can be a command or a response.

-	A GSM command/response pair is a sequence consisting of a command and the associated response.

-	A GSM procedure consists of one or more GSM command/response pairs which are used to perform all or part of an application-oriented task. A procedure shall be considered as a whole, that is to say that the corresponding task is achieved if and only if the procedure is completed. The ME shall ensure that, when operated according to the manufacturer's manual, any unspecified interruption of the sequence of command/response pairs which realize the procedure, leads to the abortion of the procedure itself.

-	A GSM session of the SIM in the GSM application is the interval of time starting at the completion of the SIM initialization procedure and ending either with the start of the GSM session termination procedure, or at the first instant the link between the SIM and the ME is interrupted.

During the GSM network operation phase, the ME plays the role of the master and the SIM plays the role of the slave.

Some procedures at the SIM/ME interface require MMI interactions. The descriptions hereafter do not intend to infer any specific implementation of the corresponding MMI. When MMI interaction is required, it is marked "MMI" in the list given below.

Some procedures are not clearly user dependent. They are directly caused by the interaction of the MS and the network. Such procedures are marked "NET" in the list given below.

Some procedures are automatically initiated by the ME. They are marked "ME" in the list given below.

The list of procedures at the SIM/ME interface in GSM network operation is as follows:

General Procedures:

-	Reading an EF	ME

-	Updating an EF	ME

-	Increasing an EF	ME

SIM management procedures:

-	SIM initialization	ME

-	GSM session termination	ME

-	Emergency call codes request	ME

-	Extended language preference request	ME

-	Language preference request	ME

-	Administrative information request	ME

-	SIM service table request	ME

-	SIM phase request	ME

CHV related procedures:

-	CHV verification	MMI

-	CHV value substitution	MMI

-	CHV disabling	MMI

-	CHV enabling	MMI

-	CHV unblocking	MMI

GSM security related procedures:

-	GSM algorithms computation	NET

-	IMSI request	NET

-	Access control information request	NET

-	HPLMN search period request	NET

-	Location Information	NET

-	Cipher key	NET

-	BCCH information	NET

-	Forbidden PLMN information	NET

-	LSA information	NET

Subscription related procedures:

-	Dialling Numbers (ADN, FDN, MSISDN, LND, SDN, BDN)	MMI/ME

-	Short messages (SMS)	MMI

-	Advice of Charge (AoC)	MMI

-	Capability Configuration Parameters (CCP)	MMI

-	PLMN Selector	MMI

-	Cell Broadcast Message Identifier (CBMI)	MMI

-	Group Identifier Level 1 (GID1)	MMI/ME

-	Group Identifier Level 2 (GID2)	MMI/ME

-	Service Provider Name (SPN)	ME

-	Voice Group Call Service (VGCS)	MMI/ME

-	Voice Broadcast Service (VBS)	MMI/ME

-	Enhanced Multi Level Pre-emption and Priority (eMLPP)	MMI/ME

-	Depersonalisation Control Keys	ME

-	Short message status reports (SMSR)	MMI

-	Network's indication of alerting	ME

-	MExE related procedures	ME

SIM Application Toolkit related procedures:

-	Data Download via SMS-CB (CBMID)	NET

-	Data Download via SMS-PP	NET

-	Menu selection	MMI

-	Call Control	MMI/ME/NET

-	Proactive SIM	MMI/ME/NET

-	Mobile Originated Short Message control by SIM	MMI/ME/NET

-	Image Request  	MMI/ME

The procedures listed in subclause 11.2 are basically required for execution of the procedures in subclauses 11.3, 11.4 and 11.5. The procedures listed in subclauses 11.3 and 11.4 are mandatory (see GSM 02.17 [6]). The procedures listed in 11.5 are only executable if the associated services, which are optional, are provided in the SIM. However, if the procedures are implemented, it shall be in accordance with subclause 11.5.

If a procedure is related to a specific service indicated in the SIM Service Table, it shall only be executed if the corresponding bits denote this service as "allocated and activated" (see subclause 10.3.7). In all other cases this procedure shall not start.

### 11.5.16	MExE related procedures

Requirement:	Service n°42 "allocated and activated".

Request:	The ME performs the reading procedure with EFORPK