| 3GPP TSG-T2 SWG1 London, UK March 28-29 2000 | T2x00013 |
| --- | --- |

To:	TSG T2 SWG1 (MExE)

Source:	Bosch Telecom Danmark

Contact:	Lars Brenk (lars.brenk@dk.bosch.com)

Title:	Suggestion for a new mandatory entry on the SIM cards

Document for:	Discussion

### Introduction:

The need for MExE to determine how many digits of the IMSI should be used to derive the HPLMN is necessary to identify the operator of a given SIM card. So here is a suggestion for a new entry for 11.11.

The new entry has the same settings as the IMSI entry, since they are closely related. It is mandatory, under CHV1 protection and has the length of only 1 byte.

### 10.3.X	EFHPLMNlen (HPLMNlen)

This EF contains the length of the HPLMN, which is part of the International Mobile Subscriber Identity (IMSI).

| Identifier: '6FXX' |  | Structure: transparent |  |  | Mandatory |  |
| --- | --- | --- | --- | --- | --- | --- |
| File size: 1 byte |  |  | Update activity: low |  |  |  |
| Access Conditions: READ CHV1 UPDATE ADM INVALIDATE ADM REHABILITATE CHV1 |  |  |  |  |  |  |
| Bytes | Description |  |  | M/O |  | Length |
| 1 | length of HPLMN in the IMSI |  |  | M |  | 1 byte |

-	length of HPLMN in the IMSI

Contents:

The length indicator refers to the number of digits, required for extracting the HPLMN from the IMSI. (0x05 = 900/1800 band, 0x06=1900 band).