| CHANGE REQUEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Please see embedded help file at the bottom of this page for instructions on how to fill in this form correctly. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | 23.057 |  |  |  |  |  | CR |  |  |  |  |  |  |  |  |  |  | Current Version: |  |  |  |  |  |  |  | 3.2.0 |  |  |  |  |
| GSM (AA.BB) or 3G (AA.BBB) specification number  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  CR number as allocated by MCC support team |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| For submission to: |  |  |  |  |  |  |  |  | for approval |  |  |  |  |  |  |  | X |  |  | strategic |  |  |  |  |  |  |  |  |  |  |  |  | (for SMG |  |  |  |
| list expected approval meeting # here  |  |  |  |  |  | for information |  |  |  |  |  |  |  |  |  |  |  |  |  | non-strategic |  |  |  |  |  |  |  |  |  |  |  |  | use only) |  |  |  |
| Form: CR cover sheet, version 2 for 3GPP and SMG The latest version of this form is available from: ftp://ftp.3gpp.org/Information/CR-Form-v2.doc |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed change affects: |  |  |  |  |  |  | (U)SIM |  |  |  |  | ME |  |  |  |  | X | UTRAN / Radio |  |  |  |  |  |  |  |  |  | Core Network |  |  |  |  |  |  |  |  |
| (at least one should be marked with an X) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source: |  |  | Ericsson |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Date: |  |  |  |  |  | 2000-08-25 |  |  |  |  |  |
| Subject: |  |  | Added Annex about MExE Executable Life Cycle |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Work item: |  |  | MExE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Category: | F Correction |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release: |  |  |  | Phase 2 |  |  |  |  |  |  |
|  | A Corresponds to a correction in an earlier release |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release 96 |  |  |  |  |  |  |
| (only one category | B Addition of feature |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release 97 |  |  |  |  |  |  |
| shall be marked | C Functional modification of feature |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release 98 |  |  |  |  |  |  |
| with an X) | D Editorial modification |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |  |  | Release 99 |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release 00 |  |  |  | X |  |  |
| Reason for change: |  |  | Adding an informative annex about MExE Executable Life Cycle |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other specs |  | Other 3G core specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| affected: |  | Other GSM core specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | MS test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | BSS test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | O&M specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

<--------- double-click here for help and instructions on how to create a CR.

Annex Z (informative):
MExE Executable Life Cycle

This is a conceptual description of the life cycle of a MExE executable. There can be small deviations in a specific classmark.

Rounded rectangle is a state. Arrow is a transition. A dashed arrow is a transition with some exception.

Figures in parenthesis are references to sections in the specification.

## Z.1 State of a MExE executable

A MExE executable can have the following states in the MExE device (4.7).

| State | Description |
| --- | --- |
| Discovered | This is the state after service discovery (4.7.1). (Only executables suitable for the device should enter this state.) |
| Transferring | The MExE executable is transferred to the MExE device. |
| Resident | The executable is stored in the MExE device. It has been transferred or pre-loaded. This is a composite state. There is a description of the state in Z.2. The state is left when the service is deleted (4.7.5). |
| Deleted | This is a transient state. After deletion the executable normally returns to the Discovered state. |
| Uninstalled Execution | The execution of the executable is started without installation. It is executed as untrusted. This is a composite state. There is a description of the state in Z.4. After Uninstalled Execution the executable normally returns to the Discovered state. It may also enter the Resident state. The state is also left when the user terminates the executable (4.7.4). |

## Z.2 Resident state

The Resident state is a composite state with concurrent states. It is in the sub-state Integrity and Certificate Chain Verification, and at the same time as in one of the sub-states Configuration,  or Released.

| State | Description |
| --- | --- |
| Integrity and Certificate Chain Verification | This is an initial state of the Resident state. This is a composite state. There is a description of the state in Z.5. |
| Configuration | This is an initial state. The executable can be configured, manually or automatically (4.7.2). |
| Released | The executable is resident and configured and released for execution. It can also be pre-installed. This is a composite state. There is a description of the state in Z.3. An installed executable, which changes security domain, enters the Configuration state. The user shall have the possibility to review the configuration before the executable is released for execution with different privileges. |

## Z.3 Released state

| State | Description |
| --- | --- |
| Passive | This is the initial state. The executable can be invoked |
| Activated | The MExE executable is activated. This state is left when the executable terminates by itself, or when the user terminates the executable (4.7.4). |
| Executing | This is the initial state of Activated |
| Waiting for user permission | This is a sub-state to Executing. The MExE executable is waiting for permission to perform some action (8.2.1). |
| Suspended | The execution is suspended (4.7) |

## Z.4 Uninstalled Execution state

The Uninstalled Execution state is a composite state with concurrent states. The Executing state may be entered before the transfer is finished.

| State | Description |
| --- | --- |
| Transferring | This is an initial state of Uninstalled Execution. The MExE executable is transferred to the MExE device. |
| Integrity and Certificate Chain Verification | This is an initial state of Uninstalled Execution. This is a composite state. There is a description of the state in Z.5. |
| Executing | The state is left when the execution terminates by itself. |
| Waiting for user permission | This is a sub-state to Uninstalled Execution. The MExE executable is waiting for permission to perform some action (8.2.1). |
| Suspended | The execution is suspended (4.7) |

## Z.5 Integrity and Certificate Chain Verification

The result of the Integrity and Certificate Chain Verification determines the change of state (8.4.2)..

| State | Description |
| --- | --- |
| Invalid | This is an initial state of Integrity and Certificate Chain Verification state. The integrity is either not checked or destroyed |
| Untrusted | The executable is untrusted (8.1) |
| Trusted in Operator Domain | The executable is verified to belong to the Operator Domain (8.1). |
| Trusted in Manufacturer Domain | The executable is verified to belong to the Manufacturer Domain (8.1). |
| Trusted in Third Party Domain | The executable is verified to belong to the Third Party Domain (8.1). |