| 3GPP TSG T2 MExE Athens, Greece 24-26 July 2001 |  |  |  |  |  |  |  |  |  | T2-MExE-010081 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | CR-Form-v3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CHANGE REQUEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 23.057 |  |  |  | CR | CR-Num |  |  |  rev |  |  |  | - |  |  Current version: |  |  |  |  | 4.2.0 |  |  |  |
|  | For HELP on using this form, see bottom of this page or look at the pop-up text over the  symbols. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Proposed change affects:  |  |  |  |  | (U)SIM |  |  | ME/UE |  | X |  | Radio Access Network |  |  |  |  |  |  |  |  | Core Network |  |  |
|  | Title:  |  | Status of applications when valid RPK not available |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Source:  |  | MExE group |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Work item code:  |  | MEXE-ENHANC |  |  |  |  |  |  |  |  |  |  |  |  |  | Date:  |  | 24-July-2001 |  |  |  |  |  |
|  | Category:  |  | F |  |  |  |  |  |  |  |  |  |  |  |  |  | Release:  |  | REL-4 |  |  |  |  |  |
|  |  |  | Use one of the following categories: F (essential correction) A (corresponds to a correction in an earlier release) B (Addition of feature), C (Functional modification of feature) D (Editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Use one of the following releases: 2 (GSM Phase 2) R96 (Release 1996) R97 (Release 1997) R98 (Release 1998) R99 (Release 1999) REL-4 (Release 4) REL-5 (Release 5) |  |  |  |  |  |  |
|  | Reason for change:  |  |  | The TS is currently unclear on how to handle MExE executables when their valid root public key is not present. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Summary of change:  |  |  | The text in subclause 8.5.1.2 currently only applies to operator applications, however it is generically applicable to all secure MExE executables. The relevant text is therefore moved to a new subclause, and together with other text, identifies the following procedures when the root public key is not available:- - how to handle launching of new secure MExE executables - how to handle currently executing secure MExE executables |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Consequences if  not approved: |  |  | The current requirements for the support of applications when an RPK is invalidated requires clarification to avoid differing implementations by manufacturers. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Clauses affected:  |  |  | Sub-clause 8.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Other specs  |  |  |  | Other core specifications  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | affected: |  |  |  | Test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | O&M Specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Other comments:  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

How to create CRs using this form:

Comprehensive information and tips about how to create CRs can be found at: http://www.3gpp.org/3G_Specs/CRs.htm.  Below is a brief summary:

1)	Fill out the above form. The symbols above marked  contain pop-up help information about the field that they are closest to.

2)	Obtain the latest version for the release of the specification to which the change is proposed. Use the MS Word "revision marks"  feature (also known as "track changes") when making the changes. All 3GPP specifications can be downloaded from the 3GPP server under ftp://www.3gpp.org/specs/ For the latest version, look for the directory name with the latest date e.g. 2000-09 contains the specifications resulting from the September 2000 TSG meetings.

3)	With "track changes" disabled, paste the entire CR form (use CTRL-A to select it) into the specification just in front of the clause containing the first piece of changed text.  Delete those parts of the specification which are not relevant to the change request.

## 8.5	Root Public keys

If the 3 MExE security domains defined in subclause 8.1 "Generic security" are not supported, then the root public key management described in this subclause is optional.

### 8.5.1	Operator root public key

The ME shall support secure storage for at least one certificate containing an operator root public key. The ME shall support the use and management of a certificate containing an operator root public key stored on the MExE-(U)SIM and in the ME. The ME shall behave according to section 8.5.1.1 “ME actions on SIM insertion and/or power up”. For support of public key management on the SIM and the USIM refer to GSM 11.11 [27] and 3GPP TS 31.102 [39] respectively.  The certificate contains a root public key generated either by the operator, or by a CA trusted by the operator. The ME shall get the operator root public key from the secure area every time it needs to verify a signature, rather than cache the root public key for use in subsequent verifications.

If the MExE device does not contain a valid operator root public key, then the certificate chain to MExE executable previously executing in the Operator Domain will be invalid, and the MExE executables will be excluded from the operator domain.

The user shall not be able to add or delete any type of operator public key (root or contained in a certificate).

Optionally, the operator may install a corresponding disaster-recovery root public key stored in the MExE device, enabling the operator to use a secure mechanism (involving the disaster-recovery key) to replace the certificate containing the standard operator root public key. It shall not be possible to use the disaster recovery operator root public key to replace the standard operator root public key unless both public keys are from the same operator.

There shall be no more than one valid operator root public key on the MExE device (excluding the disaster recovery root public key) at any one time.

An application signed by an operator shall not be able to execute in the Operator Domain unless the root public key of that operator is installed in the MExE device (either ME or MExE-(U)SIM) and is marked as trusted.

#### 8.5.1.1	MExE device actions on detection of valid (U)SIM application and/or power up

This subclause defines the sequence of actions on identification by the MExE ME that a valid SIM card, or USIM application on the UICC, has been detected (e.g. through insertion of (U)SIM card, power up of MExE device etc.).  More specifically, these actions relate to the enabling or disabling of the operator domain and the status of the operator applications on the ME.

The requirements in this subclause ensure that the operator domain on the ME belongs to the same operator as the operator that issued the valid (U)SIM application (if detected)  in the MExE device and, if there is an operator root public key (ORPK) on the MExE-(U)SIM, that trusted operator applications on the MExE device were verified using that ORPK.

The ME shall support the use and management of an Operator root public key (ORPK) on the MExE-(U)SIM.

On power up the MExE device shall behave as dictated by Figure 7 "Terminal behaviour on power up" below.

Figure 7: MExE device behaviour on power up

Note that on DCS1900 the MCC+MNC is 6 digits, but elsewhere it is 5 digits. The MExE device needs to know how many digits to use, however this is outside the scope of this specification. The identity of the root public key has to be defined.

The ME shall only read the ORPK from the MExE-(U)SIM when required and shall not store a ORPK from the MExE-(U)SIM on the ME.

When an operator root public key stored on the ME is marked as invalid, all operator applications verified using that root public key or by certificates verified by a chain that terminates with that root public key, shall cease operation as soon as possible and shall be marked as untrusted.

Removal of the (U)SIM shall not cause the status (i.e. valid or invalid) of any operator root public key on the MExE device to change.

#### 8.5.2	Manufacturer root public key

The ME shall support secure storage for a certificate containing a manufacturer root public key. The certificate contains a root public key generated by the manufacturer of the MExE device, or by a CA trusted by the manufacturer of the MExE device.

If the ME does not contain a valid manufacturer root public key, then the certificate chain to MExE executable previously executing in the Manufacturer Domain will be invalid, and the MExE executables will be excluded from the manufacturer domain and marked as untrusted.

The user shall not be able to add or delete any type of manufacturer public key (root or contained in a certificate).

The Manufacturer shall put a root public key and optionally its corresponding disaster-recovery key in the ME at the time of manufacture, and use a proprietary secure mechanism (e.g. using the disaster-recovery key) to replace the certificate containing the manufacturer root public key. It shall not be possible to use the disaster recovery manufacturer root public key to replace the standard manufacturer root public key unless both public keys are from the same manufacturer.

An application signed by a manufacturer shall not be able to run in the Manufacturer Domain unless the root public key of that manufacturer is installed in the ME and is marked as trusted.

There shall be no more than one valid manufacturer root public key on the ME (excluding the disaster recovery root public key).

### 8.5.3	Third party root public key

The ME shall support secure storage for at least one certificate containing a third party root public key. The ME shall support the use and management of certificates containing Third Party root public keys stored on the MExE-(U)SIM and in ME. For support of public key management on the SIM and the USIM refer to GSM 11.11 [27] and 3GPP TS 31.102 [39] respectively. The MExE device may contain root public key (s) generated by CA(s) implicitly trusted by the user. The user will be able to securely install (using a secure transport) or remove Third Party root public keys at any time using a system administrative tool.

The Manufacturer, Operator and Administrator may at their discretion, securely install certificates containing Third Party root public key(s) on behalf of the user, e.g. at the time of manufacture by the Manufacturer. See subclause 8.6 "Certificate management" for details of Administrator control of Third Party certificate download.

If a Third Party public key is deleted or becomes invalid, then the certificate chain to MExE executables previously executing in the Third Party Domain certified by that public key will become "untrusted".

There may be any number of Third Party root public keys on the MExE device.

The third party domain administrator, i.e. the Administrator (user or other body) shall be able to enable and disable Third Party root public keys by using CCM, see subclause 8.7 “Certificate configuration message (CCM)”. The process of adding/removing public keys and enabling/disabling public key are independent.

All third party certificates shall be subject to restrictions imposed by valid certificate configuration messages.

See subclause 8.6 "Certificate management" for the management of Third Party root public keys.

### 8.5.4	Administrator root public key

To help with the control of Third-Party certificates, the ME shall support secure storage for a certificate containing an administrator root public key. The ME shall support the use and management of a certificate containing an Administrator root public key stored on the MExE-(U)SIM and in the ME. The ME shall behave according to section 8.8.1 “Determining the administrator of the MExE MS”. For support of public key management on the SIM and the USIM refer to GSM 11.11 [27] and 3GPP TS 31.102 [39] respectively. Only one administrator root public key shall be valid on the MExE device at any one time.

The MExE device shall support the administrator designation mechanism explained in subclause 8.8 "Provisioned mechanism for designating administrative responsibilities and adding third parties in a MExE device" and the secure downloading of CCMs explained in subclause 8.7.4 “Authorised CCM download mechanisms”.

The user shall not be able to delete an administrator root public key or certificate.

The system shall support a mechanism (as part of a provisioned functionality and/or inherently part of the MExE implementation) allowing the owner of the MExE device to manage the administrator root public key (including the download of a new administrator root public key) as defined in subclause 8.8.1.1 "Administrator of the MExE device is the user". This mechanism shall be secure so that only the owner can use this functionality.

The administrator root public key can be downloaded to the MExE device as described in subclause 8.10.4 "Administrator root certificate download mechanism".

If the Administrator root public key is stored in the (U)SIM, the ME shall only read the Administrator root public key from the MExE-(U)SIM when required and shall not store the Administrator root public key from the MExE-(U)SIM on the ME.

See subclause 8.6 "Certificate management" for the management of Administrator root public keys.

The same root public key may be used for both the Administrator role and the operator or manufacturer domain. This facility does not imply any increased right of the manufacturer or operator to take the Administrator role.

If the same root public key is used for the operator domain and Administrator role and this root public key is stored on the MExE-(U)SIM (see [27] and [39]), there shall be separate entries relating to each use of the root public key in the operator and administrator trusted certificate directory files. These entries in the operator and Administrator trusted certificate directory files may point to the same root public key in the certificate data file.

If the root public key to be shared is not stored on the (U)SIM, then procedures relating to this are out of the scope of this specification.

### Handling of MExE executables when their valid root public key is not available

This subclause considers the effect on MExE executables when the root public key of a secure domain (e.g. operator, manufacturer, third party) is no longer available (e.g. when the UICC is being physically removed, or the root public key is no longer valid).

#### 8.5.5.1	Launching of MExE executables when their valid RPK is not available

It shall not be possible to launch a MExE executable to run in a security domain unless the root public key of that security domain is available and valid.

#### 8.5.5.2	Currently executing secure MExE executables when their valid RPK is no longer available

On detection that the valid root public key of a secure domain is no longer present, the MExE device shall permit MExE executables currently executing in the secure domain controlled by that root public key to continue executing,until a new RPK for that secure domain is validated.

When the new RPK is validated, the currently running MExE executables (under the old RPK) in that secure domain shall be terminated.