| 3GPP TSG-T2 Portland, U.S.A. 23rd - 25th September 2001 |  |  |  |  |  |  |  |  | T2 –MExE-010102 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CR-Form-v3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 23.057 |  |  |  | CR | CR-Num |  |  |  rev |  |  |  | - |  |  Current version: |  |  |  |  | 4.3.0 |  |  |  |
| For HELP on using this form, see bottom of this page or look at the pop-up text over the  symbols. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed change affects:  |  |  |  |  | (U)SIM |  |  | ME/UE |  | X |  | Radio Access Network |  |  |  |  |  |  |  |  | Core Network |  |  |
| Title:  |  | Classmark 4 security |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source:  |  | Microsoft |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Work item code:  |  | MEXE-EHANC |  |  |  |  |  |  |  |  |  |  |  |  |  | Date:  |  | 14.07.2001 |  |  |  |  |  |
| Category:  |  | B |  |  |  |  |  |  |  |  |  |  |  |  |  | Release:  |  | REL-5 |  |  |  |  |  |
|  |  | Use one of the following categories: F (essential correction) A (corresponds to a correction in an earlier release) B (Addition of feature), C (Functional modification of feature) D (Editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Use one of the following releases: 2 (GSM Phase 2) R96 (Release 1996) R97 (Release 1997) R98 (Release 1998) R99 (Release 1999) REL-4 (Release 4) REL-5 (Release 5) |  |  |  |  |  |  |
| Reason for change:  |  |  | Introduction of a new Classmark based on CLI, the Common Language Infrastructure. This CR introduces the changes to the security parts of the specification. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Summary of change:  |  |  | Modifications to executable permissions for untrusted domains section. This document is based on T2-010671 (CR Classmark 4 security), approved as a rolling CR in Edinburgh, September 2001, updated for version 4.3.0 from 4.2.0. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  |  | Classmark 4 executable permissions not defined. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected:  |  |  | 8.2.2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other specs  |  |  |  | Other core specifications  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| affected: |  |  |  | Test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | O&M Specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other comments:  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7: Executable permissions for untrusted MExE executables

|  | Classmark 1 | Classmark 2 | Classmark 4 | Classmark 3 |
| --- | --- | --- | --- | --- |
| User Interface | An untrusted, uninstalled MExE executable (e.g. an applet) can access the user interface output and input without user permission, but the sending of user data to a server to which the MExE executables has a session connection (e.g. as part of a browser session) requires user permission. An installed untrusted MExE executable shall only be able to access the user interface output and input with user permission (clearly, for the usability of untrusted MExE executables such as games, blanket user permission should be sought and given, and this is permissible). |  |  | Untrusted MExE executables can access the user interface output and input without the user permission. |
| File, Persistent Data | File access is not permitted for untrusted MExE executables. |  |  |  |
|  | But, untrusted MExE executables can access files only in the MExE executable's own directory. |  |  | But, persistent data may be stored via the MIDP record management system (stores are shared between MIDlets in the same MIDlet Suite). |
| Transmission over the Access Network | Untrusted MExE executables shall be able to exchange data, voice, HTTP requests, etc. over the Access Network under the following conditions: The recipient of a transmission (e.g. a phone number, a URL, a server name, etc.) shall be presented to the user for permission by a provisioned functionality of the MExE device itself, even if this recipient was already presented by the executable (this facility would support, for example, "click to dial" buttons/links in untrusted MExE executables). It shall not be possible for an application to use a transmission channel that it did not initiate (except for MIDlets within the same MIDlet suite). |  |  |  |
| Generate DTMF | Untrusted MExE executables shall be able to generate DTMF tones under the following conditions: An untrusted MExE executable is only permitted to send DTMF tones in a currently active call. The request to generate DTMF tones in the currently active call, shall result in the characters which the tones represent being presented to the user for permission by a provisioned functionality of the MExE device. |  |  |  |
| Add Phonebook Entry | Untrusted MExE executables shall be able to add a phonebook entry (i.e. name and number only) under the following conditions: The name and the number to be added shall be displayed to the user for permission by a provisioned functionality of the MExE device and not by the MExE executable itself. The phonebook entry shall not be added without user permission. The function shall not be able to modify or delete any phonebook entry. |  |  |  |
| Executable Interaction | Executable interaction is not permitted for untrusted MExE executables (except for MIDlets within the same MIDlet suite). |  |  |  |