| 3GPP TSG-T2_SWG5_Ad-hoc Sweden, 28-30 July 1999 | T2MT-99002 |
| --- | --- |

Agenda Item:

Source:	NEC

Title:	Terminal situations and switching mode

Document for:	Comment/Discussion

____________________________________

1. Introduction

This document provides helpful for considering the Terminal Situations and switching mode for TR.XX.yy V0.2.0 Multi-mode issues.

2.Comments

2.1 Terminal Situation

In Table 1, this contribution denotes that the Terminal Situation for Terminal position. Especially this is related section 4.1 of Multi-mode issues.

A : CDMA area

C : GSM area

B : Common area

Fig. 1 Service area

Table 1 Terminal situation.

| Terminal Type | Service area | Registered mode | Active communication modes |
| --- | --- | --- | --- |
| Ⅰ | A | CDMA | CDMA |
|  | B | CDMA or GSM | CDMA or GSM |
|  | C | GSM | GSM |
| Ⅱ | A | CDMA | CDMA |
|  | B | CDMA and GSM | CDMA or GSM |
|  | C | GSM | GSM |
| Ⅲ | A | CDMA | CDMA |
|  | B | CDMA and GSM | CDMA and GSM |
|  | C | GSM | GSM |

2.2 Active Mode

This Table2 shows "Terminal behavior" when terminals get the incoming connection in active mode.

Especially this table relates to section 4.2.1.2 of Multi-mode issues. (Only Active mode)

<Proposal> All situations included Table 2 should be described in section 4.2.

Table 2 Behavior of terminal.

|  | Action Mode |  | CS |  |  |  |  |  |  |  |  | PS |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Add connection From Mode |  | CS |  |  |  | PS |  |  |  |  | CS |  |  |  | PS |  |  |  |
|  |  |  | CDMA |  | GSM |  | CDMA |  | GSM |  |  | CDMA |  | GSM |  | CDMA |  | GSM |  |
|  | Terminal Mode |  | C | G | C | G | C | G | C | G |  | C | G | C | G | C | G | C | G |
| Type |  | Service area |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| I |  | A(CDMA) | O | - | - | - | O | - | - | - |  | O | - | - | - | O | - | - | - |
|  |  | B(Both) | O | N | N | N | O | N | N | N |  | O | N | N | N | O | N | N | U |
|  |  | C(GSM) | - | - | - | N | - | - | - | N |  | - | - | - | N | - | - | - | U |
| II |  | A(CDMA) | O | - | - | - | O | - | - | - |  | O | - | - | - | O | - | - | - |
|  |  | B(Both) | O | S | S | N | O | S | S | N |  | O | S | S | N | O | S | S | U |
|  |  | C(GSM) | - | - | - | N | - | - | - | N |  | - | - | - | N | - | - | - | U |
| III |  | A(CDMA) | O | - | - | - | O | - | - | - |  | O | - | - | - | O | - | - | - |
|  |  | B(Both) | O | O | O | N | O | O | O | N |  | O | O | O | N | O | O | O | U |
|  |  | C(GSM) | - | - | - | N | - | - | - | N |  | - | - | - | N | - | - | - | U |

Supplement:

- O: Added connection is OK.

- N: Terminal can't know the information of incoming connection.

- S: Terminal can get the information about this connection. User can choose user's preferable mode to set the active mode.

- U: If GPRS support Multi-call services, added connection is OK.

- The hyphenate[-] show that that situation doesn't exit. e.g. In GSM area [C area], we never get the connection of CDMA at present. In this situation, if we would like to get the CDMA connection, we have to change Network System of GSM. We know the necessity of modify. Because we have already discussed in Miami meeting.

2.3.1 The way of switching between modes.

We have two switching mode between CDMA (GSM) area to GSM (CDMA) area. One is "Manually" mode. Another mode is "Automatically". Now in Section 4.1.X of Multi-mode issues, we can see how to switch between modes. However I think that we don't need to mention the way of switching between modes in that specification. Because I think that Ms can have both switching modes.

2.3.1.1 CDMA area to Common area

2.3.1.1.1 Type I

No need switching.

2.3.1.1.2 Type II

Terminal is changed to preferential mode by automatically or user preference by manually.

2.3.1.1.3 Type III

No need switching

2.3.1.2 GSM area to Common area

2.3.1.2.1 Type I

No need switching.

2.3.1.2.2 Type II

Terminal is changed to preferential mode by automatically or user preference by manually.

2.3.1.2.3 Type III

No need switching.

2.3.1.3 Common area to WCDMA (GSM) area

2.3.1.3.1 Type I

2.3.1.3.1.1 Terminal was WCDMA (GSM) mode.

No need switching

2.3.1.3.1.2 Terminal was GSM (WCDMA) mode.

Terminal is changed to preferential mode by automatically or user preference by manually.

2.3.1.3.2 Type II

2.3.1.3.2.1 Terminal was WCDMA (GSM) mode.

No need switching

2.3.1.3.2.2 Terminal was GSM (WCDMA) mode.

Terminal is changed to preferential mode by automatically or user preference by manually.

2.3.1.3.3 Type III

2.3.1.3.3.1 Terminal was WCDMA (GSM) mode.

No need switching

2.3.1.3.3.2 Terminal was GSM (WCDMA) mode.

Terminal is changed to preferential mode by automatically or user preference by manually.

"A" denotes "CDMA area" which is described in Fig 1.

We can make the automatically switching modes that are able to switch the mode based on precedence prioritized by user or any programmability.