| 3GPP TSG-T2 #13 Pusan, KOREA 14-18 May 2001 | T2-010470 |
| --- | --- |

Agenda Item:	MMS Reference Point MM7

Source:	MATERNA

Title:	MM7 – Use Cases, Goals and Requirements

Document for:	Discussion

___________________________________________________________________________

# Introduction

In 3GPP 23.140 Rel-4, the need for a separate reference point MM7 between the MMS Relay/Server and an external application (server) is identified. However, the technical realisation of MM7 is not specified in Rel-4.

The presentation attached to this TDOC should serve as a starting point for discussion. Some use cases for MMS-based applications, part of them being quite successful in today’s SMS world, are highlighted for a goals and requirements analysis.

Some emphasis has been put on several billing issues related to external MMS-based applications.

A number of recommendations for further work on MM7 are given at the end of the presentation.

# Summary

## MM7 Use Cases

- UMS Notifications (Voice/Email/Fax; „I‘m Away“ multimedia announcement)

- Push-type information services (location-based weather and traffic, sports, jokes, ...)

- Pull-type information services (stock quote request, ...)

- Push-reply-type services (opinion polls, quiz games, ...)

- List servers (public or private message boards, MMS Publisher, chat, ...)

- Instant Messaging gateway

- Make all these available to prepaid subscribers

## MM7 Requirements summary

- Appropriate security measures should be taken to minimize fraud or other interception attempts. As a consequence, a session-oriented protocol should be chosen as basis for MM7.

- MM7 operations should include

- Open session / close session operations for VASP authentication

- Submit message with billing information

- Deliver message

- Delivery notification

- Low-credit / no-credit notifications when submitting messages to prepaid subscribers

- RFC822-style addressing should be used for  routing mobile-originated MMS to applications:
 (<application-name>@<server-name>.<operator-domain>.<toplevel-domain>).

- The Value Added Service Provider should have means for controlling basic service charging.

- Applications connected via MM7 should get individual „low-credit“ or „no-credit“ result codes for push messages to prepaid subscribers.

- Reverse charging and Reply-charging should be supported (being part of commercial agreement with VASP).

## Recommendations

As there are no MM7-related requirements present in TS 22.140, a description of MMS-based value added services and a list of MM7-related requirements should be added in this document (LS to SA1).

Standard IETF protocols should be evaluated for their usefulness as a base protocol for MM7. As stated above, a session-oriented protocol should be chosen for security reasons.

OSA should be evaluated as a possible implementation basis for MM7, although no drafting has been done yet in the OSA group concerning Generic Messaging.

A description of the MM7 architecture and a list of required/optional MM7 operations should be included in TS23.140.

MM7-related billing aspects should be included in other billing drafting activities (LS to SA5).