| 3GPP TSG-T2 #13 Pusan, KOREA 14-18 May 2001 |  |  |  |  |  |  |  |  |  |  | T2-010498 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | CR-Form-v3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CHANGE REQUEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 23.140 |  |  |  | CR | CR-Num |  |  |  rev |  |  |  |  | - |  |  Current version: |  |  |  |  | 4.2.0 |  |  |  |
|  | For HELP on using this form, see bottom of this page or look at the pop-up text over the  symbols. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Proposed change affects:  |  |  |  |  | (U)SIM |  |  | ME/UE |  |  |  |  | Radio Access Network |  |  |  |  |  |  |  |  | Core Network |  |  |
|  | Title:  |  | MMS - Media Object Distribution |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Source:  |  | Ericsson |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Work item code:  |  | MMS |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Date:  |  | May 14, 2001 |  |  |  |  |  |
|  | Category:  |  | B |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release:  |  | Rel-5 |  |  |  |  |  |
|  |  |  | Use one of the following categories: F (essential correction) A (corresponds to a correction in an earlier release) B (Addition of feature), C (Functional modification of feature) D (Editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Use one of the following releases: 2 (GSM Phase 2) R96 (Release 1996) R97 (Release 1997) R98 (Release 1998) R99 (Release 1999) REL-4 (Release 4) REL-5 (Release 5) |  |  |  |  |  |  |
|  | Reason for change:  |  |  | There is currently no mechanism of limiting the distribution and accessibility of a media object within a MMS messages. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Summary of change:  |  |  | This CR proposes to add a MIME header that indicates if a media object is free to distribute or not. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Consequences if  not approved: |  |  | A mechanism to limit distribution of media objects is extremely important for the MMS Service. If no standards solution will be available a number of propriety solution may be used. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Clauses affected:  |  |  | 6.1.3, 8.1.3.3 and 8.8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Other specs  |  |  |  | Other core specifications  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | affected: |  |  |  | Test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | O&M Specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Other comments:  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

How to create CRs using this form:

Comprehensive information and tips about how to create CRs can be found at: http://www.3gpp.org/3G_Specs/CRs.htm.  Below is a brief summary:

1)	Fill out the above form. The symbols above marked  contain pop-up help information about the field that they are closest to.

2)	Obtain the latest version for the release of the specification to which the change is proposed. Use the MS Word "revision marks"  feature (also known as "track changes") when making the changes. All 3GPP specifications can be downloaded from the 3GPP server under ftp://www.3gpp.org/specs/ For the latest version, look for the directory name with the latest date e.g. 2000-09 contains the specifications resulting from the September 2000 TSG meetings.

3)	With "track changes" disabled, paste the entire CR form (use CTRL-A to select it) into the specification just in front of the clause containing the first piece of changed text.  Delete those parts of the specification which are not relevant to the change request.

### 6.1.3	Retrieval of a Multimedia Message in the recipient MMSE

The recipient MMS User Agent shall be able to request delivery of an MM from the recipient MMSE based on the information received in the notification.

Upon delivery request the recipient MMSE

- shall deliver the MM to the recipient MMS User Agent

- may perform data adaptation based on user profile and/or MMS User Agent capabilities

- shall not provide the MM originator address to the MM recipient if the originator MMS User Agent requested its address to be hidden from the MM recipient

- shall provide the MM originator address to the MM recipient if the originator MMS User Agent did not request its address to be hidden from the MM recipient and if the MM originator address is available at the recipient MMSE

- may provide an alias or clarifying text (e.g. “anonymous address” or “unknown address”) in the originator address field instead of providing the originator address to the recipient MMS User Agent, if the originator has requested address hiding or the original message does not contain the originator address

- shall give an indication to the recipient MMS User Agent that a delivery report is requested if such a delivery report has been requested by the originator MMS User Agent

- shall give an indication to the recipient MMS User Agent that a read-reply report is requested if such a read reply report has been requested by the originator MMS User Agent

- shall indicate the MIME content type of the MM to the recipient MMS User Agent

- shall provide other available message qualifications unaltered to the recipient MMS User Agent

- shall provide the time stamp of the MM unaltered to the recipient MMS User Agent

- shall be responsible for the storage of messages in the network until the recipient MMS User Agent becomes reachable (e.g. user moves back into coverage, switches MMS User Agent on) or until the MM expires.

- shall provide the recipient MMS User Agent with information if any media object is not free to distribute.

- may provide the recipient MMS User Agent with a count of the number of times that the particular MM was forwarded, if the MM was forwarded and the counter information is available to the recipient MMSE.

- should provide the recipient MMS User Agent  with a list of addresses of forwarding MMS User Agents for the MM if the MM was forwarded and the address information is available to the recipient MMSE.

In a response to an MM’s delivery the recipient MMS User Agent may be able to

- request a delivery report not to be generated by the MMSE.

### 8.1.3	Retrieval of Multimedia Message

This part of MMS service covers the retrieval of an MM. For retrieval purposes an MM shall always be retrieved by the recipient MMS User Agent from the recipient MMS Relay/Server. Involved abstract messages are outlined in Table 7 from type and direction points of view.

Table 7: Abstract messages for retrieval of MM in MMS

| Abstract messages | Type | Direction |
| --- | --- | --- |
| MM1_retrieve.REQ | Request | MMS UA -> MMS Relay/Server |
| MM1_retrieve.RES | Response | MMS Relay/Server -> MMS UA |
| MM1_acknowledgement.REQ | Request | MMS UA -> MMS Relay/Server |

#### 8.1.3.1	Normal Operation

The recipient MMS User Agent shall issue an MM1_ retrieve.REQ to the recipient MMS Relay/Server to initiate the retrieval process. The MMS Relay/Server shall respond with an MM1_retrieve.RES, which contains MMs control information and the MM content.

After receiving the MM1_retrieve.RES, the recipient MMS User Agent shall send an MM1_acknowledgement.REQ to the corresponding MMS Relay/Server, if requested by the MMS Relay/Server. The MM1_acknowledgement.REQ shall unambiguously refer to the corresponding MM1_retrieve.RES.

#### 8.1.3.2	Abnormal Operation

If the recipient MMS Relay/Server can not process the MM1_retrieve.REQ, for example due to invalid content location or expiration of the message, the recipient MMS Relay/Server shall respond with either an MM1_retrieve.RES or a lower protocol layer error message encapsulating a status which indicates the reason to the MMS User Agent the multimedia message was not delivered.

If the MMS Relay/Server does not provide the MM1_retrieve.RES or the lower protocol layer error message the MMS User Agent should be able to recover.

#### 8.1.3.3	Features

Message Reference: The recipient MMS User Agent shall always provide a reference, e.g., URI, for the MM in the MM1_retrieve.REQ.

Addressing: The MM originator address may be provided to the recipient MMS User Agent in the addressing-relevant information field of MM1_retrieve.RES. The MM originator address shall not be provided to the recipient MMS User Agent if the MM originator has requested her address to be hidden from the MM recipient. One or several address(es) of the MM recipient(s) may be provided to the recipient MMS User Agent in the addressing-relevant information field(s) of the MM1_retrieve.RES.

Time stamping: The MM1_retrieve.RES shall carry the time and date of submission of the MM or the time and date of the forwarding of the MM.

Time constraints: In case of reply-charging the deadline for the latest time of submission of a reply-MM shall be conveyed within the MM1_retrieve.RES.

Message class, priority and subject: Information about class, priority, subject of the MM shall be included in the MM1_retrieve.RES according to their presence and value received at the MMS Relay/Server. Information about additional end-to-end qualifiers of the MM should be included in the MM1_retrieve.RES according to their presence and value received at the MMS Relay/Server.

Reporting: If the originator MMS User Agent has requested to have a read-reply report, the recipient MMS Relay/Server shall convey this information in the MM1_retrieve.RES. If the originator MMS User Agent has requested to have a delivery report, the recipient MMS Relay/Server may convey this information to the recipient MMS User Agent in the MM1_retrieve.RES. If a request for a delivery report is included in the MM1_retrieve.RES the recipient MMS User Agent shall convey the information whether it accepts or denies the sending of a delivery report to the MM originator in MM1_acknowledgement.REQ. If a delivery report is not requested, it is up to the recipient MMS User Agent to include this information in MM1_acknowledgement.REQ or not.

Reply-Charging: In case of reply-charging the MMS Relay/Server should indicate in the MM1_retrieve.RES that a reply to this particular original MM is free of charge

Identification: The MMS Relay/Server shall provide a message identification for a message, which it has accepted for delivery in the MM1_retrieve.RES. In case of reply-charging the MMS Relay/Server shall provide the message-ID of the original MM which is replied to in the MM1_retrieve.RES.

Content Type: The type of the MM’s content shall always be identified in the MM1_retrieve.RES.

Content: The content of the multimedia message if added by the originator MMS User Agent of the MM may be conveyed in the MM1_retrieve.RES.

Status: In case of normal operation the recipient MMS Relay/Server may indicate in the MM1_retrieve.RES that the retrieval of the MM was processed correctly. In case of abnormal operation the recipient MMS Relay/Server shall indicate in the MM1_retrieve.RES the reason why the multimedia message could not be retrieved. The corresponding reason codes should cover application level errors (e.g. ‘the media format could not be converted’, ‘insufficient credit for retrieval’). Lower layer errors may be handled by corresponding protocols.

Forward_Counter: A Counter indicating the number of times the particular MM was forwarded.

Forwarded_by: The address of the forwarding MMS User Agent. Multiple addresses are possible. In the multiple address case this is a sequential list of the address(es) of the forwarding MMS User Agents who forwarded the same MM.

Limited Distribution: This facility allows a level of control to be provided over the distribution of media objects contained within an MM. If a media object is marked as “distribution not allowed”, then it shall not be possible to distribute the object even if the object may have been modified in the MMS User Agent. The MMS user agent shall therefore not make a limited distribution object available to any user agent that has the capability to send/release the object to other devices. For detailed information please refer section 8.8

8.1.3.4	Information Elements

Table 8: Information elements in the MM1_retrieve.REQ .

| Information element | Presence | Description |
| --- | --- | --- |
| Message Reference | Mandatory | Location of the content of the MM to be retrieved. |

Table 9: Information elements in the MM1_retrieve.RES .

| Information element | Presence | Description |
| --- | --- | --- |
| Message ID | Mandatory | The message ID of the MM. |
| Sender address | Conditional | The address of the originator of MM unless the originator MMS User Agent has requested her address to be hidden from the MM recipient. |
| Content type | Mandatory | The content type of the MM’s content. |
| Recipient address | Optional | The address of the MM recipient. Multiple addresses are possible. |
| Message class | Optional | The class of the message (e.g., personal, advertisement, information service) |
| Date and time | Mandatory | The time and date of the submission of the MM or the time and date of the forwarding of the MM (time stamp) |
| Delivery report | Optional | A request for delivery report. |
| Priority | Conditional | The priority (importance) of the message if specified by the originator MMS User Agent.. |
| Read reply | Conditional | A request for read-reply report if the originator MMS User Agent of the MM has requested a read-reply report. |
| Subject | Conditional | The title of the whole multimedia message if specified by the originator MMS User Agent of the MM. |
| Status | Optional | The status of the MM retrieve request. |
| Status Text | Optional | Description which qualifies the status of the MM retrieve request. |
| Reply-Charging | Optional | Information that a reply to this particular original MM is free of charge. |
| Reply-Charging-ID | Optional | In case of reply-charging this is the identification of the original MM replied to. |
| Reply-Deadline | Optional | In case of reply-charging the latest time of submission of a reply granted to the recipient. |
| Forward_counter | Conditional | A Counter indicating the number of times the particular MM was forwarded. |
| Forwarded_by | Conditional | The address of the forwarding MMS User Agent. Multiple addresses are possible. In the multiple address case this is a Sequential list of the address(es) of the forwarding MMS User Agents who forwarded the same MM. |
| Content | Conditional | The content of the multimedia message if specified by the originator MMS User Agent of the MM. This information element may also contain information that further defines the MMS service behaviour. This information is described in section 8.8 |

Table 10: Information elements in the MM1_acknowledgement.REQ .

| Information element | Presence | Description |
| --- | --- | --- |
| Report allowed | Optional | Request to allow or disallow the sending of a delivery report to the MM originator |

## 8.8 Content related information

This subclause defines content related information that is used to control the MMS service behaviour that is not coupled to any specific reference point:

- Limited Distribution, allows a level of control over the distribution of media objects.

### 8.8.1	Limited Distribution

Multiple media elements shall be combined into a composite single MM using MIME multipart format as defined in section 5.1.2. To identify that a specific media element shall not be possible to forward, the X-Mms-Distribution-Allowed header shall be included in the MIME header for that media element. The X-Mms-Distribution header is specified in the following way:

"X-Mms-Distribution-Allowed" ":" ( "Yes" | "No" )