| 3GPP TSG T2 #13 Pusan, Korea 14-18 May 2001 | T2-010538 |
| --- | --- |

Agenda Item:	MMS

Source:	Siemens AG

Title:	Discussion on MM Recall

Document for:	Discussion

## Introduction

From the early beginnings of 3GPP TS 22.140 MMS Stage 1 the MM recall functionality was a high-level requirement for in 3G TS 22.140, MMS stage 1. It was deleted by SP-010002 (CR004) / T2-010167 in February 2001 with the intention to align MMS Stage 1 with MMS Stage 2 where the recall feature had not been realised so far.

This document proposes a straightforward mechanism how to realise such an MM recall feature in 3G TS 23.140, MMS Stage 2. It is intended to initiate the discussion in the group with the intention to refine the attached CRs according to the feedback received.

## Summary of the proposed mechanism:

The MM Recall feature enables an originator to recall a multimedia message after submission. I.e. the MM originator requests the network to delete the original MM. The scope of this feature is assumed to be as follows: an MM can be recalled as long as it has not yet been delivered to the recipient. Note that the recipient may already been notified of the orginal MM.

According to the proposed approach the recall of an MM is initiated by the MM originator (of the original MM which is to be recalled) by submitting a new MM to the same recipient(s). This new MM comes along without any content but with an additional information element that identifies the original MM. Depending on where the original MM hangs around in the system it either triggers the original or the recipient MMS Realy/Server to delete both these MMs.

If a recall command is executed by an MMS Relay/Server after the recipient has been notified about the original MM being available for retrieval the recipient will be informed with a notification that the MM isn’t available for retrieval anymore.

Feedback on the success of the MM recall is proposed to be provided in a delivery report which is anticipated to be requested úpon submission of the 2nd MM which requests the recall of the original MM.

| CR-Form-v3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | TS 22.140 |  |  |  | CR |  |  |  |  rev |  |  |  | - |  |  Current version: |  |  |  |  | 4.1.0 |  |  |  |
| For HELP on using this form, see bottom of this page or look at the pop-up text over the  symbols. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed change affects:  |  |  |  |  | (U)SIM |  |  | ME/UE |  | X |  | Radio Access Network |  |  |  |  |  |  |  |  | Core Network |  | X |
| Title:  |  | MM Recall |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source:  |  | Siemens AG |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Work item code:  |  | MMS R’5 |  |  |  |  |  |  |  |  |  |  |  |  |  | Date:  |  | 15 May 2001 |  |  |  |  |  |
| Category:  |  | B |  |  |  |  |  |  |  |  |  |  |  |  |  | Release:  |  | REL-5 |  |  |  |  |  |
|  |  | Use one of the following categories: F (essential correction) A (corresponds to a correction in an earlier release) B (Addition of feature), C (Functional modification of feature) D (Editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Use one of the following releases: 2 (GSM Phase 2) R96 (Release 1996) R97 (Release 1997) R98 (Release 1998) R99 (Release 1999) REL-4 (Release 4) REL-5 (Release 5) |  |  |  |  |  |  |
| Reason for change:  |  |  | From the early beginnings of MMS MM recall was a required feature in 3GPP TS 22.140, MMS Stage 1. Shortly before freeze of R’4 of MMS it was deleted in SP-010002 (CR004) with the intention to align R’4 of MMS Stage 1 with the achievements made on MMS Stage 2 (where the recall feature had not been realised so far). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Summary of change:  |  |  | Introduction of the feature “Recall of Multimedia Messages” |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  |  | The recall of multimedia messages would not be possible. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected:  |  |  | 5.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other specs  |  |  |  | Other core specifications  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| affected: |  |  |  | Test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | O&M Specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other comments:  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

How to create CRs using this form:

Comprehensive information and tips about how to create CRs can be found at: http://www.3gpp.org/3G_Specs/CRs.htm.  Below is a brief summary:

1)	Fill out the above form. The symbols above marked  contain pop-up help information about the field that they are closest to.

2)	Obtain the latest version for the release of the specification to which the change is proposed. Use the MS Word "revision marks"  feature (also known as "track changes") when making the changes. All 3GPP specifications can be downloaded from the 3GPP server under ftp://www.3gpp.org/specs/ For the latest version, look for the directory name with the latest date e.g. 2000-09 contains the specifications resulting from the September 2000 TSG meetings.

3)	With "track changes" disabled, paste the entire CR form (use CTRL-A to select it) into the specification just in front of the clause containing the first piece of changed text.  Delete those parts of the specification which are not relevant to the change request.

## ...

## 5.1	Multimedia message management

-	Terminal-sensitive MM management

The MMS shall be able to support the capability for the terminal and network to take account of the capability of the user's terminal (e.g. deliver a MM / notification in a manner compatible with the terminals capability).

-	Terminal status-sensitive MM Management

The MMS shall be able to support the capability of the  network to take account of the availability, changes of the state of availability of the terminal (e.g. store messages if the recipient is not available).

-	MMS Control by the operator

The MMS shall be able to support a request from the operator to enable/disable MM delivery and submission.

-	MMS Control by the user

The MMS shall be able to support a request from the user to enable/disable MM delivery and submission.

This requirement shall be supported at the application layer in the terminal, and will not be further elaborated.

-	Personalise multimedia messaging

The MMS shall be able to support a request by the user to manage the Service Preferences of his User Service Profile related to this MMS [2](e.g. customise his MM environment within the capabilities of the terminal, network and MM application. This could be unconditional or conditional e.g. depending on roaming conditions or operator restrictions).

-	MM creation

The MMS shall be able to support the request to create a MM by the user or an application.

This requirement shall be supported at the application layer in the terminal, and will not be further elaborated.

MM recall

The MMS shall be able to support the request to delete a MM once submitted (e.g. recalling a message).

-	MM Time Stamping

The MMS shall be able to support the request to include a reliable time value in an MM, a notification and an acknowledgement as appropriate.

-	Multiple Media

Multimedia messages may be composed of either a single medium (e.g. voice) or multi-media (e.g. Voice and video). The MMS shall be able to support a request for media synchronisation / sequencing.

-	Media Type Conversion

The MMS shall be able to support a request to convert between  media types (e.g. Fax to image).

This requirement shall be supported at the application layer in the network, and will not be further elaborated.

-	Media Format Conversion

The MMS shall be able to support a request by the user or the application to  convert between MM media formats (e.g. JPEG to GIF).

This requirement shall be supported at the application layer in the terminal and/or in the network, and will not be further elaborated.

-	Message forwarding

The MMS shall be able to support a request to forward multimedia messages or multimedia message elements without having to first download the MM to the terminal.

-	Storage of Multi-Media Messages

The MMS shall be able to support a request for multimedia messages or message elements to be stored until delivered to the recipient’s terminal, until they expire, or until they are deleted by the user (unless configured differently). The MMS shall be able to support a request to store and manage all MMs in a network based repository rather than on the mobile terminal.

This requirement shall be supported at the application layer in the network, and will not be further elaborated.

NOTE:	There is no requirement for the MMS  to be responsible for the processing/presentation of the MM message, after it has been delivered to the terminal.

-	Prioritisation of Messages

The MMS shall be able to support a request for MM prioritisation subject to the capabilities of the network (e.g. the sender of the MM may request to prioritise the importance of the multimedia messages).

-	Message qualification

The MMS shall be able to support a request for MM qualification (e.g. subject) for the purpose of advanced user experience and awareness.

-	Screening of Messages

The MMS shall be able to support a request for MM screening subject to the capabilities of the network (e.g. automatically delete “junk mail”, anonymous messages without delivery to the recipient’s terminal).

This requirement shall be supported at the application layer in the terminal an/or in the network, and will not be further elaborated.

-	Validity Period

The MMS shall be able to support a request by the originator of a message to define validity periods (earliest and latest desired time) for message delivery (e.g. if a message can not be delivered within a certain time it will be automatically deleted). The MMS service provider shall be able to set the MAXIMUM allowable validity period for any message.

## 5.2	Multimedia message delivery and submission

...

| CR-Form-v3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | TS 23.140 |  |  |  | CR |  |  |  |  rev |  |  |  | - |  |  Current version: |  |  |  |  | 4.2.0 |  |  |  |
| For HELP on using this form, see bottom of this page or look at the pop-up text over the  symbols. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed change affects:  |  |  |  |  | (U)SIM |  |  | ME/UE |  | X |  | Radio Access Network |  |  |  |  |  |  |  |  | Core Network |  | X |
| Title:  |  | Recall of Multimedia Messages |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source:  |  | Siemens AG |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Work item code:  |  | MMS |  |  |  |  |  |  |  |  |  |  |  |  |  | Date:  |  | 10 May 2001 |  |  |  |  |  |
| Category:  |  | B |  |  |  |  |  |  |  |  |  |  |  |  |  | Release:  |  | REL-5 |  |  |  |  |  |
|  |  | Use one of the following categories: F (essential correction) A (corresponds to a correction in an earlier release) B (Addition of feature), C (Functional modification of feature) D (Editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Use one of the following releases: 2 (GSM Phase 2) R96 (Release 1996) R97 (Release 1997) R98 (Release 1998) R99 (Release 1999) REL-4 (Release 4) REL-5 (Release 5) |  |  |  |  |  |  |
| Reason for change:  |  |  | Introduction of the new feature “Recall of Multimedia Messages” |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Summary of change:  |  |  | The introduction of this new feature requires some changes in the service description and in the technical realisation chapters |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  |  | The recall of multimedia messages would not be possible |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected:  |  |  | 5.2, 6.1, 8.1, 8.4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other specs  |  |  |  | Other core specifications  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| affected: |  |  |  | Test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | O&M Specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other comments:  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

How to create CRs using this form:

Comprehensive information and tips about how to create CRs can be found at: http://www.3gpp.org/3G_Specs/CRs.htm.  Below is a brief summary:

1)	Fill out the above form. The symbols above marked  contain pop-up help information about the field that they are closest to.

2)	Obtain the latest version for the release of the specification to which the change is proposed. Use the MS Word "revision marks"  feature (also known as "track changes") when making the changes. All 3GPP specifications can be downloaded from the 3GPP server under ftp://www.3gpp.org/specs/ For the latest version, look for the directory name with the latest date e.g. 2000-09 contains the specifications resulting from the September 2000 TSG meetings.

3)	With "track changes" disabled, paste the entire CR form (use CTRL-A to select it) into the specification just in front of the clause containing the first piece of changed text.  Delete those parts of the specification which are not relevant to the change request.

# …

## 5.2	MMS Relay/Server

The MMS Relay/Server is responsible for storage and handling of messages. It may provide convergence functionality between External Servers and MMS User Agents and thus enable the integration of different server types across different networks. An Example can be found in Annex A.

It should be possible to separate the MMS Relay/Server element into MMS Relay and MMS Server elements, but an allocation of the MMS Relay/Server functionalities to such elements is not defined in this release.

The MMS Relay/Server is responsible for the following functions:-

- receiving and sending MM;

- enabling/disabling MMS function;

- personalising MMS based on user profile information;

- MM deletion based on user profile or filtering information;

- media type conversion;

- media format conversion;

- conversion of messages arriving at the MMSE from legacy messaging systems to MM format (e.g. facsimile to MM)

- conversion of MMs leaving the MMSE to legacy messaging systems to the appropriate message format (e.g. MM to internet email)

- message content retrieval;

- MM forwarding;

- screening of MM;

- negotiation of terminal capabilities;

- checking terminal availability;

- MM notification to the MMS User Agent;

- generating call data records (CDR);

- address translation.

- address hiding

- managing the message properties on servers (e.g. voicemail or email server)  integrated in the MMSE (consistency)

- temporary and/or persistent storage of messages

- ensuring that messages are not lost until successfully delivered to another MMSE element

- controlling the reply-charging feature of MMS

- controlling the recall feature of MMS

## 5.3	External Servers

…

# 6	MMS Service Behaviour Description

## 6.1	MMS services offered

### 6.1.1	Submission of a Multimedia Message in the originator MMSE

When a user intends to send an MM to one or several destinations the MM shall be submitted to the originator MMS Relay/Server.

The support for submission of MMs is optional for MMS User Agents. The support for submission of MMs is mandatory for MMS Relay/Servers.

If an MMS User Agent supports submission of MMs the MMS User Agent shall be able to:

- Indicate the address of the MM recipient

- Identify the MIME content type of the message.

If a MMS User Agent supports submission of MMs the MMS User Agent may be able to:

- Request a delivery report for the message

- Request a read-reply report for the message

- Provide a time stamp for the time of submission of the message

- Set the earliest desired time of delivery for the message

- Set the desired time of expiry for the message

- Indicate the address of the MM originator

- Set further message qualifications (e.g. priority, message class, subject)

- Request the MM originator’s address being hidden from the recipient MMS User Agent.

- Upon reception of an MM from an originator MMS User Agent the originator MMSE

- shall assign a Message Identification to the MM and immediately provide the originator MMS User Agent with this Message Identification

- is responsible for retaining the MM until the earliest desired time of delivery, if the optional feature of earliest time of delivery is supported by the originator MMSE. If this feature is not supported then the MM is immediately routed forward.

- may provide a time stamp, i.e. it may also override the MMS User Agent’s time stamp,

- shall insert the originator’s address into the MM if not yet provided by the originator MMS User Agent

- shall pass the originator’s address to the peer entity if the peer entity is known to be a MMSE

- shall route forward the request for address hiding unaltered to the recipient MMSE if the peer entity is known to be an MMSE.

- shall pass the originator’s address to the peer entity if the peer entity is not known to be an MMSE and address hiding has not been requested by the originator MMS User Agent

- shall not pass the originator’s address to the peer entity and should override the address provided by the originator MMS User Agent in the MM to an "anonymous" address if the peer entity is not known to be an MMSE and address hiding has been requested by the originator MMS User Agent

- may override the address provided by the originator MMS User Agent in the MM (subject to MMS service provider’s preferences)

- is responsible for resolving the MM recipient’s address(es),

- is responsible to route the MM towards the MM recipients.

- should pass the indication whether or not a delivery report is requested unaltered when routing the MM towards the MM recipient(s)

- shall pass the indication whether or not a read-reply report is requested unaltered when routing the MM towards the MM recipient(s)

- shall pass the indication about MIME content type of the message and message qualifications (e.g. priority, message class, subject) unaltered when routing the MM towards the MM recipient(s)

- shall generate a delivery report indicating “indeterminate” status of the MM’s delivery if a delivery report was requested by the originator MMS User Agent and if the peer entity the MM is routed forward to is not known by the originator MMS Relay/Server.

Upon reception of an MM which contains a recall command the originator MMSE

- may be able to delete both the MM which contains the recall command and the MM to be recalled if this is still under control of the originator MMSE.

- may be able to route forward the MM which contains the recall command unaltered to the recipient MMSE.

A special case is where the recipient MMSE is also the originator MMSE. In this case the MM does not have to be routed forward.

### 6.1.2	Reception of a Multimedia Message in the recipient MMSE

Upon reception of an MM the recipient MMSE

- may verify the MM recipient’s user profile(s)

- shall store the MM at least until

- the associated time of expiry is reached,

- the MM is delivered,

- the recipient MMS User Agent requests the MM to be routed forward or

- the MM is rejected.

The term “associated time of expiry ” refers to either the desired time of expiry set by the originator MMS User Agent or an MMSE time of expiry setting.

- shall generate a notification to the recipient MMS User Agent.

Upon reception of an MM which contains a recall command the recipient MMSE

- may be able to delete both the MM which contains the recall command and the MM to be recalled if this is still under control of the recipient MMSE.

- may be able to notify the recipient MMS User Agent about an MM not being available for retrieval anymore if this MM has been deleted due to a recall command.

Incoming messages from legacy systems may be expected to be converted to MMs.

#### 6.1.2.1	Multimedia Message Notification

With the MM notification the recipient MMS User Agent shall receive a message reference that can be used for retrieving the MM from the recipient MMSE. The message reference that is conveyed in a notification shall at least be valid throughout the message expiry period, till the successful retrieval of the MM or until the MM was rejected.

With the MM notification the recipient MMS User Agent may receive additional information on the MM.

If the originator MMS User Agent has requested address hiding the recipient MMSE shall not include the originator address into the MM notification.

- In a response to the notification the MMS User Agent shall be able to

- reject the MM or

- retrieve the MM, either immediately or at a later time, either manually or automatically, as possibly determined by the operator configuration and user profile.

### 6.1.3	Retrieval of a Multimedia Message in the recipient MMSE

…

### 6.1.5	Delivery Report

The MMSE shall support the delivery reporting service. Delivery reports shall only be generated for MMs.

The originator MMS User Agent may be able to request a delivery report for a specific MM.

Within an MM notification or upon MM retrieval the recipient MMS User Agent may receive an indication that a delivery report is requested for the MM.

Within either a response to a notification or a response to an MM’s delivery, the recipient MMS User Agent may request a delivery report not to be generated by the MMSE.

The originator MMSE shall generate a delivery report if a delivery report has been requested by the originator MMS User Agent

- upon routing forward the MM, in case the peer entity is not known by the MMSE

The recipient MMSE shall generate a delivery report if a delivery report has been requested by the originator MMS User Agent and if the recipient MMS User Agent did not request a delivery report not to be generated

- upon receipt of a response to a notification, in case the MM is rejected by the recipient MMS User Agent

- upon receipt of a forwarding request, in case the MM is forwarded  by the recipient MMS User Agent to other MM recipient(s), without prior retrieval.

- upon receipt of a response to an MM’s delivery, in case the MM is retrieved by the MM recipient

- upon expiry of the MM, in case the MM is not rejected and not retrieved by the MM recipient before the expiry

The originator MMS User Agent, i.e. the MMS User Agent receiving the delivery report, may match the delivery report to the sent MM by retaining the message identification of the sent MM and comparing it to the received delivery report, which shall contain the message identification of the original MM. In case of multiple MM recipients, it is necessary for the originator MMS User Agent to retain the MM recipient addresses as well, to match the delivery report to the sent MM.

If a delivery report has been requested by the originator MMS User Agent and if the recipient MMS User Agent did not request a delivery report not to be generated, the recipient MMSE

- shall generate the delivery report

- shall deliver the delivery report to the originator MMSE.

- shall be responsible for the storage of delivery reports in the network until the originator MMSE becomes reachable or until the delivery report expires

Within the delivery report the recipient MMSE

- shall provide the MM originator address to the originator MMSE.

- shall provide the MM recipient address to the originator MMSE.

- shall provide the identification of the original MM for which the delivery report has been generated to the originator MMSE.

- shall provide status information how the MM was handled (e.g. expired, rejected, delivered, forwarded or indeterminate) to the originator MMSE

- shall provide a time stamp when the MM was handled to the originator MMSE

- may inform the originator MMSE about the execution of a recall request if the originator MMS User Agent has requested to recall an MM sent before.

For each MM recipient of the original MM for which the delivery report has been generated and becomes available at the originator MMSE, the originator MMSE

- shall deliver the delivery report to the originator MMS User Agent (i.e. the recipient MMS User Agent of the delivery report).

Within the delivery report the originator MMSE

- shall provide the MM recipient’s address to the originator MMS User Agent (the recipient MMS User Agent of the delivery report).

- shall provide the identification of the original MM for which the delivery report has been generated to the originator MMS User Agent (the recipient MMS User Agent of the delivery report).

- shall be responsible for the storage of delivery reports in the network until the originator MMS User Agent becomes reachable (e.g. user moves back into coverage, switches MMS User Agent on) or until the delivery report expires

- may inform the originator MMS User Agent about the execution of a recall request if the originator MMS User Agent has requested to recall an MM sent before.

### 6.1.6	Read-Reply Report

…

### 6.1.11	Support for Recall Functionality in MMS

The MMS User Agent may support recall functionality. If the MMS User Agent supports this feature it is expected that the MMS User Agent supports the following behaviour.

The MMS Relay/Server may support recall functionality. If the MMS Relay/Server supports this feature it is expected that the MMS Relay/Server supports the following behaviour.

A User of the MMS may be able to request the recall of an MM sent before. Therefore the originator of an MM should be able to submit a new MM to the same recipient without any additional content. This MM should contain the message  ID of the MM to be deleted. Upon reception of an MM an MMSE should be able to check if the MM is containing a recall request and should be able to delete the MM specified in the recall request if this MM is still under the control of  the MMSE as well as the MM which contains the recall command. If the recipient MMS User Agent has already been notified about the MM that the MM originator requests to be deleted and the MM has not been retrieved by the recipient  MMS User Agent the recipient MMSE should be able to notify the recipient MMS User Agent about this MM not being  available for retrieval anymore.

## MMSE Addressing responsibilities

…

## 8.1	Technical realisation of MMS on reference point MM1

### 8.1.1	Submission of Multimedia Message

This part of MMS service covers the submission of an MM. For sending purposes a terminal-originated MM shall always be submitted from the originator MMS User Agent to the corresponding MMS Relay/Server. Involved abstract messages are outlined in Table 1 from type and direction points of view.

Table 1: Abstract messages for submission of MM in MMS

| Abstract messages | Type | Direction |
| --- | --- | --- |
| MM1_submit.REQ | Request | MMS UA -> MMS Relay/Server |
| MM1_submit.RES | Response | MMS Relay/Server -> MMS UA |

#### 8.1.1.1	Normal operation

The originator MMS User Agent shall submit a terminal-originated MM to the originator MMS Relay/Server using the MM1_submit.REQ, which contains MMS control information and the MM content. The MMS Relay/Server shall respond with an MM1_submit.RES, which provides the status of the request. The MM1_submit.RES shall unambiguously refer to the corresponding MM1_submit.REQ.

Support for MM1_submit.REQ is optional for the MMS UA, support for MM1_submit.RES is mandatory for the MMS Relay/Server.

#### 8.1.1.2	Abnormal Operation

In this case the originator MMS Relay/Server shall respond with a MM1_submit.RES encapsulating a status which indicates the reason the multimedia message was not accepted, e.g. no subscription, corrupt message structure, service not available.

If the MMS Relay/Server does not provide the MM1_submit.RES the MMS User Agent should be able to recover.

#### 8.1.1.3	Features

Addressing: One or several MM recipients of a submitted MM shall be indicated in the addressing-relevant information field(s) of the MM1_submit.REQ. The originator of a submitted MM may be indicated in addressing-relevant information field(s) of the MM1_submit.REQ. The originator MMS User Agent may request to hide its identity from the MM recipient.

Time stamping: The originator MMS User Agent may time stamp the MM.

Time constraints: The originator MMS User Agent may also request an earliest desired time of delivery of the MM. The originator MMS User Agent may request a time of expiry for the MM. In case of reply-charging the originator MMS User Agent may also request a deadline for the latest time of submission of reply-MMs granted to the recipient(s).

Reply-Charging: The originator MMS User Agent may indicate that the sender wants to pay for a reply-MM in the MM1_submit.REQ.

Message class, priority and subject: The MM may be qualified further by adding a message class, priority and/or subject to the MM in the MM1_submit.REQ. Additional qualifiers may be added.

Reporting: The originator MMS User Agent may request a delivery report for the MM. In addition, the originator MMS User Agent may request a read-reply report when the user has viewed the MM.

Identification: The originator MMS Relay/Server shall always provide a message identification for an MM, which it has accepted for submission in the MM1_submit.RES. In case of reply-charging the MMS User Agent which submits a reply-MM (i.e. the MMS User Agent that received the original MM) shall provide the message-ID of the original MM which it replies to in the MM1_submit.REQ.

Content Type: The MIME type of the multimedia content shall always be identified in the MM1_submit.REQ.

Content: The originator MMS User Agent may add content in the MM1_submit.REQ.

Request Status: The originator MMS Relay/Server shall indicate the status of the MM1_submit.REQ in the associated MM1_submit.RES. The reason code given in the status information element of the MM1_submit.RES may be supported with an explanatory text further qualifying the status. If this text is available in the status text information element the MMS User Agent should bring it to the user’s attention. The choice of the language used in the status text information element is at the discretion of the MMS service provider.

Recall Feature: The originator MMS User Agent may indicate that the sender wants to recall an MM sent before in the MM1_submit.REQ by using the message ID of the MM to be recalled.

#### 8.1.1.4	Information Elements

Table 2: Information elements in the MM1_submit.REQ.

| Information element | Presence | Description |
| --- | --- | --- |
| Recipient address | Mandatory | The address of the recipient MMS User Agent. Multiple addresses are possible. |
| Content type | Mandatory | The content type of the MM’s content. |
| Sender address | Optional | The address of the MM originator. |
| Message class | Optional | The class of the MM (e.g., personal, advertisement, information service) |
| Date and time | Optional | The time and date of the submission of the MM (time stamp). |
| Time of Expiry | Optional | The desired time of expiry for the MM or reply-MM. |
| Earliest delivery time | Optional | The earliest desired time of delivery of the MM to the recipient. |
| Delivery report | Optional | A request for delivery report. |
| Reply-Charging | Optional | A request for reply-charging. |
| Reply-Deadline | Optional | In case of reply-charging the latest time of submission of replies granted to the recipient(s). |
| Priority | Optional | The priority (importance) of the message. |
| Sender visibility | Optional | A request to show or hide the sender's identity when the message is delivered to the recipient. |
| Read reply | Optional | A request for read reply report. |
| Subject | Optional | The title of the whole multimedia message. |
| Reply-Charging-ID | Optional | In case of reply-charging when the reply-MM is submitted within the MM1_submit.REQ this is the identification of the original MM that is replied to. |
| Content | Optional | The content of the multimedia message |
| Recall-ID | Optional | The identification of the MM the originator requests to be deleted. |

Table 3: Information elements in the MM1_submit.RES.

| Information element | Presence | Description |
| --- | --- | --- |
| Request Status | Mandatory | The status of the MM submit request. |
| Request Status Text | Optional | Description which qualifies the status of the MM submit request. |
| Message ID | Mandatory | The identification of the MM given to an accepted MM. |

### 8.1.2	Multimedia Message Notification

This part of the MMS service covers the notification about MM from the recipient MMS Relay/Server to the corresponding recipient MMS User Agent and involving abstract messages  are outlined in Table 4 from type, and direction points of view.

Table 4: abstract messages for notification of MM in MMS

| Abstract message | Type | Direction |
| --- | --- | --- |
| MM1_notification.REQ | Request | MMS Relay/Server -> MMS UA |
| MM1_notification.RES | Response | MMS UA -> MMS Relay/Server |

#### 8.1.2.1	Normal Operation

Upon receiving the MM1_notification.REQ, the recipient MMS User Agent shall respond with the MM1_notification.RES to the recipient MMS Relay/Server to acknowledge the successful reception of the MM1_notification.REQ.

The MM1_notification.RES shall unambiguously refer to the corresponding MM1_notification.REQ.

#### 8.1.2.2	Abnormal Operation

In this case the MMS UA shall respond with a MM1_notification.RES encapsulating a status which indicates the reason the notification could not be processed. If the MMS UA does not provide the MM1_notification.RES the MMS Relay/Server should be able to retransmit the notification at a later state.

#### 8.1.2.3	Features

Addressing: The MM originator address may be provided to recipient MMS User Agent in the MM1_notification.REQ.

Time constraints: The recipient MMS User Agent shall be provided a time of expiry of the MM. In case of reply-charging the deadline for the latest time of submission of a reply-MM should be conveyed within the MM1_notification.REQ.

Reply-Charging: In case of reply-charging the MMS Relay/Server may indicate in the MM1_notification.REQ that a reply to the notified original MM is free of charge.

Message class, message size and subject: The MM shall be qualified further by adding a message class and an approximate size to the MM in the MM1_notification.REQ. The MM may be qualified further by adding a subject to the MM. Additional qualifiers may be added.

Reporting: If the originator MMS User Agent has requested to have a delivery report, the recipient MMS Relay/Server may convey this information to the recipient MMS User Agent in the MM1_notification.REQ. The recipient MMS User Agent may indicate in the MM1_notification.RES that it would not wish a delivery report to be created.

Identification: In case of reply-charging when a reply-MM is notified within the MM1_notification.REQ the MMS Relay/Server should convey the identification of the original MM replied to within the same MM1_notification.REQ.

Message Reference: The recipient MMS Relay/Server shall always provide a reference, e.g., URI, for the MM in the MM1_notification.REQ.

MM Status: The recipient MMS User Agent may indicate in the MM1_notification.RES how it intends the MM to be handled, e.g. the immediate rejection of the MM.

Recall Feature: The MMS Relay/Server may convey the reference of an MM that has been deleted to the recipient in the MM1_notification.REQ if the sender’s recall request could be executed by the MMS Relay/Server.

Date of Execution: If a recall request could be executed by the MMS Relay/Server the date of the execution may be conveyed to the recipient in the MM1_notification.REQ.

#### 8.1.2.4	Information Elements

Table 5: Information elements in the MM1_notification.REQ.

| Information element | Presence | Description |
| --- | --- | --- |
| Message class | Mandatory | The class of the MM (e.g., personal, advertisement, information service; default = personal) |
| Message size | Mandatory | The approximate size of the MM |
| Time of expiry | Mandatory | The time of expiry for the MM. |
| Message Reference | Mandatory | a reference, e.g., URI, for the MM |
| Subject | Optional | The title of the whole MM. |
| Sender address | Optional | The address of the MM originator. |
| Delivery report | Optional | Request for delivery report |
| Reply-Charging | Optional | Information that a reply to this particular original MM is free of charge. |
| Reply-Deadline | Optional | In case of reply-charging the latest time of submission of a reply granted to the recipient. |
| Reply-Charging-ID | Optional | The identification of the original MM replied to if this notification indicates a reply-MM. |
| Reference of recalled Message | Optional | The reference (e.g. URI) of an MM that has been deleted by the MMS Relay/Server due to the sender’s recall request. |
| Date of Execution | Optional | Indicates the date of the execution of a recall request. |

Table 6: Information elements in the MM1_notification.RES.

| Information element | Presence | Description |
| --- | --- | --- |
| MM Status | Optional | The status of the MM’s retrieval |
| Report allowed | Optional | Request to allow or disallow the sending of a delivery report to the MM originator |

### Retrieval of Multimedia Message

…

### 8.1.5	Delivery Report

This part of MMS service covers the sending of delivery report from originator MMS Relay/Server to the originator MMS User Agent. The involved abstract message is outlined in Table 14 from type and direction points of view.

Table 14: abstract message for sending delivery reports in MMS

| Abstract Message | Type | Direction |
| --- | --- | --- |
| MM1_delivery_report.REQ | Request | MMS Relay/Server -> MMS UA |

#### 8.1.5.1	Normal Operation

The originator MMS Relay/Server shall (subject to user, MMS service provider and/or operator preferences) create the MM1_delivery_report.REQ and send it to the originator MMS User Agent when the appropriate information for the creation of a delivery report is available. Support for MM1_delivery_report.REQ is optional for the MMS User Agent but mandatory for the MMS Relay/Server.

#### 8.1.5.2	Abnormal Operation

The MMS protocol framework does not provide mechanisms to cover and handle the unsuccessful delivery of MM1_delivery_report.REQ. The underlying protocols shall provide reliable transport of MM1_delivery_report.REQ.

#### 8.1.5.3	Features

Identification: In the MM1_delivery_report.REQ the MMS Relay/Server shall always provide the original message identification of the MM that the delivery report corresponds to.

Addressing: The MM recipient address shall be provided to the originator MMS User Agent in the addressing-relevant information field of MM1_delivery_report.REQ.

Time stamping: The MM1_delivery_report.REQ shall carry the time and date of handling of the MM (e.g. retrieval, expiry, rejection).

MM Status: The MM1_delivery_report.REQ shall carry the status of the MM delivery, e.g. retrieved, forwarded, rejected, expired or indeterminate. If the originator MMS User Agent has requested the recall of an MM sent before the originator MMS User Agent may be informed in the MM1_delivery_report.REQ about the execution of this request.

#### 8.1.5.4	Information Elements

Table 15: Information elements in the MM1_delivery_report.REQ.

| Information element | Presence | Description |
| --- | --- | --- |
| Message ID | Mandatory | The identification of the original MM. |
| Recipient address | Mandatory | The address of the MM recipient of the original MM. |
| Event Date | Mandatory | Date and time the MM was handled (retrieved, expired, rejected, etc.) (time stamp) |
| MM Status | Mandatory | Status of the MM, e.g. retrieved, forwarded, expired, rejected, recall successful, recall failed |

### Read-Reply Report

…

## 8.4	Technical realisation of MMS on reference point MM4

An MMSE may be able to discover a peer MMSE. This subclause defines the interworking between MMSE once the peer systems are aware of each other being an MMSE.

Future releases may elaborate how peer MMSEs discover each other. In the mean time, it is expected that MMS service providers or network operators will develop solutions for their particular needs which may include static tables or other look-up methods.

### 8.4.1	Routing Forward of a Multimedia Message

This part of MMS service covers the routing forward of an MM from an originator MMS Relay/Server to a recipient MMS Relay/Server of different MMSEs. Involved abstract messages are outlined in Table 19 from type and direction points of view.

Table 19: Abstract messages for forwarding of MM in MMS

| Abstract messages | Type | Direction |
| --- | --- | --- |
| MM4_forward.REQ | Request | Originator MMS Relay/Server -> recipient MMS Relay/Server |
| MM4_forward.RES | Response | Recipient MMS Relay/Server -> originator MMS Relay/Server |

#### 8.4.1.1	Normal operation

After successful discovery of its peer entity the originator MMS Relay/Server shall route an MM forward to the recipient MMS Relay/Server using the MM4_forward.REQ, which contains MMS control information and the MM content. The recipient MMS Relay/Server shall respond with a MM4_forward.RES, which provides the status of the request if an MM4_forward.RES was requested.

Support for MM4_forward.REQ and MM4_forward.RES is mandatory for the MMS Relay/Server.

#### 8.4.1.2	Abnormal Operation

In this case the recipient MMS Relay/Server shall respond with a MM4_forward.RES, which includes a status that indicates the reason the multimedia message was not accepted, e.g. no subscription, bad address, network not reachable, etc., if an MM4_forward.RES was requested.

#### 8.4.1.3	Features

Addressing: The recipient(s) of a routed forward MM shall be indicated in the addressing-relevant information field(s) of the MM4_forward.REQ. If the addresses of several MM recipients of the MM are associated with a single MMSE then more than one MM recipient may be indicated in the addressing-relevant information field(s) of the MM4_forward.REQ. Addresses of all MM recipients of the MM (including those that are not associated with the MMSE the MM is forwarded to) shall be conveyed in the MM4_forward.REQ for the MM recipient’s informational purposes.

The MM originator of a routed forward MM shall be indicated in addressing-relevant information field(s) of the MM4_forward.REQ. If the originator MMS User Agent requested to hide its identity from the MM recipient then the information about this request shall also be conveyed in the MM4_forward.REQ.

Time stamping: The MM4_forward.REQ shall carry the time-stamp associated with the MM.

Time constraints: If the originator MMS User Agent requested a time of expiry for the MM then this information shall be conveyed in the MM4_forward.REQ.

Message class, priority and subject: If the MM is qualified further by message class, priority, subject and/or additional qualifiers then this information shall be conveyed in the MM4_forward.REQ.

Reporting: If the originator MMS User Agent requested a delivery report for the MM then the information about this request shall be conveyed in the MM4_forward.REQ. If, in addition, the originator MMS User Agent requested a read-reply report then the information about this request shall be conveyed in the MM4_forward.REQ.

Identification: The originator MMS Relay/Server shall always provide a unique message identification for an MM, which it routed forward to a peer MMS Relay/Server in the MM4_forward.REQ.

Content Type: The type of the multimedia content shall always be identified in the MM4_forward.REQ.

Acknowledgement Request: The originator MMS Relay/Server may request a MM4_forward.RES from the recipient MMS Relay/Server acknowledging the successful reception of the MM.

Request Status: The recipient MMS Relay/Server shall indicate the status of the MM4_forward.REQ in the associated MM4_forward.RES if requested.

Message Type: The type of message used on reference point MM4 indicating MM4_forward.REQ and MM4_forward.RES as such.

Transaction Identification: If the originator MMS Relay/Server requests an MM4_forward.RES from the recipient MMS Relay/Server it shall provide a transaction identification within an MM4_forward.REQ. The MM4_forward.RES shall unambiguously refer to the corresponding MM4_forward.REQ using the same transaction identification.

Forward_Counter: A Counter indicating the number of times the particular MM was forwarded.

Forwarded_by: The address of the forwarding MMS User Agent. Multiple addresses are possible. In the multiple address case this is a Sequential list of the address(es) of the forwarding MMS User Agents who forwarded the same MM.

Version: The MMS protocol shall provide unique means to identify the current version in the particular protocol environment.

Recall Feature: The originator MMS Relay/Server may indicate that the sender wants to recall an MM sent before in the MM4_forward.REQ.

#### 8.4.1.4	Information Elements

Table 20: Information elements in the MM4_forward.REQ.

| Information element | Presence | Description |
| --- | --- | --- |
| 3GPP MMS Version | Mandatory | The MMS version of the originator MMS Relay/Server as defined by this specification. |
| Message Type | Mandatory | The type of message used on reference point MM4: “MM4_forward.REQ”. |
| Transaction ID | Mandatory | The identification of the MM4_forward.REQ/ MM4_forward.RES pair. |
| Message ID | Mandatory | The identification of the MM. |
| Recipient(s) address | Mandatory | The address(es) of the MM recipient(s). Multiple addresses are possible. |
| Sender address | Mandatory | The address of the MM originator. |
| Content type | Mandatory | The content type of the MM’s content. |
| Message class | Conditional | The class of the MM (e.g., personal, advertisement, information service) if specified by the originator MMS User Agent |
| Date and time | Mandatory | The time and date of the submission of the Mm (time stamp) or the time and date of the forwarding of the MM.. |
| Time of Expiry | Conditional | The desired time of expiry for the MM if specified by the originator MMS User Agent. |
| Delivery report | Conditional | A request for delivery report if the originator MMS User Agent has requested a delivery report for the MM. |
| Priority | Conditional | The priority (importance) of the message if specified by the originator MMS User Agent. |
| Sender visibility | Conditional | A request to show or hide the sender's identity when the message is delivered to the MM recipient if the originator MMS User Agent has requested her address to be hidden from the recipient. |
| Read reply | Conditional | A request for read reply report if the originator MMS User Agent has requested a read-reply report for the MM.. |
| Subject | Conditional | The title of the whole MM if specified by the originator MMS User Agent. |
| Acknowledgement Request | Optional | Request for MM4_forward.RES |
| Forward_counter | Conditional | A counter indicating the number of times the particular MM was forwarded. |
| Forwarded_by | Conditional | The address of the forwarding MMS User Agent. Multiple addresses are possible. In the multiple address case this is a Sequential list of the address(es) of the forwarding MMS User Agents who forwarded the same MM. |
| Content | Conditional | The unaltered content of the multimedia message if specified by the originator MMS User Agent. |
| Recall-ID | Optional | The identification of the MM the originator requests to be deleted. |

Table 21: Information elements in the MM4_forward.RES.

| Information element | Presence | Description |
| --- | --- | --- |
| 3GPP MMS Version | Mandatory | The MMS version of the recipient MMS Relay/Server as defined by this specification. |
| Message Type | Mandatory | The type of message used on reference point MM4: “MM4_forward.RES”. |
| Transaction ID | Mandatory | The identification of the MM4_forward.REQ/ MM4_forward.RES pair. |
| Message ID | Mandatory | The Message ID of the MM which has been forwarded within the corresponding MM4_forward.REQ |
| Request Status Code | Mandatory | The status of the request to route forward the MM. |
| Status text | Optional | Status text corresponding to the code |

### 8.4.2	Routing Forward of a Delivery Report

This part of MMS service covers the routing forward of a delivery report from recipient MMS Relay/Server to originator MMS Relay/Server. The involved abstract messages are outlined in Table 22 from type and direction points of view.

Table 22: Abstract messages for routing delivery reports forward in MMS

| Abstract Message | Type | Direction |
| --- | --- | --- |
| MM4_delivery_report.REQ | Request | Recipient MMS Relay/Server -> originator MMS Relay/Server |
| MM4_delivery_report.RES | Response | Originator MMS Relay/Server -> recipient MMS Relay/Server |

#### 8.4.2.1	Normal Operation

After successful discovery of its peer entity the recipient MMS Relay/Server shall route a previously created delivery report forward to the originator MMS Relay/Server using the MM4_delivery_report.REQ which contains MMS control information only. The recipient MMS Relay/Server shall respond with a MM4_delivery_report.RES, which provides the status of the MM4_delivery_report.REQ if an MM4_delivery_report.RES was requested.

Support for MM4_delivery_report.REQ and MM4_delivery_report.RES is mandatory for the MMS Relay/Server.

#### 8.4.2.2	Abnormal Operation

In this case the originator MMS Relay/Server shall respond with a MM4_delivery_report.RES encapsulating a status which indicates the reason the delivery report was not accepted, if an MM4_delivery_report.RES was requested.

#### 8.4.2.3	Features

Addressing: Both the address of the recipient (which is the MM originator) and the address of the originator (which is the MM recipient) of a routed forward delivery report shall be provided to the originator MMS Relay/Server in the addressing-relevant information field of MM4_delivery_report.REQ.

Identification: In the MM4_delivery_report.REQ the recipient MMS Relay/Server shall always provide the original message identification of the MM that the delivery report corresponds to as obtained from the associated MM4_forward.req.

MM Time stamping: The MM4_delivery_report.REQ shall carry the time and date of handling of the MM (e.g. retrieval, expiry, rejection).

MM Status: The MM4_delivery_report.REQ shall carry the status of the MM delivery, e.g. retrieved, rejected, expired or indeterminate. If the originator MMS User Agent has requested the recall of an MM sent before the originator MMSE may be informed in the MM4_delivery_report.REQ about the execution of this request in the recipient MMSE.

Acknowledgement Request: The recipient MMS Relay/Server may request a MM4_delivery_report.RES from the originator MMS Relay/Server acknowledging the successful reception of the delivery report.

Request Status: The originator MMS Relay/Server shall indicate the status of the MM4_delivery_report.REQ in the associated MM4_delivery_report.RES if requested.

Version: The MMS protocol shall provide unique means to identify the current version in the particular protocol environment.

Message Type: The type of message used on reference point MM4 indicating MM4_delivery_report.REQ and MM4_delivery_report.RES as such.

Transaction Identification: If the originator MMS Relay/Server requests an MM4_delivery_report.RES from the recipient MMS Relay/Server it shall provide a transaction identification within an MM4_delivery_report.REQ. The MM4_delivery_report.RES shall unambiguously refer to the corresponding MM4_delivery_report.REQ using the same transaction identification.

#### 8.4.2.4	Information Elements

Table 23: Information elements in the MM4_delivery_report.REQ.

| Information element |  |  | Presence |  |  | Description |
| --- | --- | --- | --- | --- | --- | --- |
| 3GPP MMS Version |  | Mandatory |  |  | The MMS version of the recipient MMS Relay/Server as defined by this specification. |  |
| Message Type |  | Mandatory |  |  | The type of message used on reference point MM4: “ MM4_delivery_report.REQ”. |  |
| Transaction ID | Mandatory |  |  | The identification of the MM4_delivery_report.REQ/ MM4_delivery_report.RES pair. |  |  |
| MM Message ID |  |  | Mandatory |  |  | The identification of the original MM. |
| Recipient address |  |  | Mandatory |  |  | The address of the MM recipient of the original MM. |
| Sender address |  |  | Mandatory |  |  | The address of the MM originator of the original MM. |
| MM Date and time |  |  | Mandatory |  |  | Date and time the MM was handled (retrieved, expired, rejected, etc.) |
| Acknowledgement Request |  |  | Optional |  |  | Request for MM4_delivery_report.RES |
| MM Status Code |  |  | Mandatory |  |  | Status of the MM, e.g. retrieved, expired, rejected, recall successful, recall failed |
| Status text |  |  | Optional |  |  | Status text corresponding to the Status code |

Table 24: Information elements in the MM4_delivery_report.RES.

| Information element |  | Presence |  | Description |
| --- | --- | --- | --- | --- |
| 3GPP MMS Version |  | Mandatory |  | The MMS version of the recipient MMS Relay/Server as defined by this specification. |
| Message Type |  | Mandatory |  | The type of message used on reference point MM4: “ MM4_delivery_report.RES”. |
| Transaction ID | Mandatory |  | The identification of the MM4_delivery_report.REQ/ MM4_delivery_report.RES pair. |  |
| Message ID |  | Mandatory |  | The Message ID of the MM which caused the delivery report |
| Request Status Code |  | Mandatory |  | The status of the associated MM4_delivery_report.REQ. |
| Status text | Optional |  | The text explanation corresponding to the Status code |  |

### 8.4.3	Routing Forward of a Read-Reply Report

...

### 8.4.4 	Message format on MM4

All elements of an MM shall be included within a single SMTP “mail” message which shall be organised as MIME type application/multipart. All MM elements shall be of standard MIME content types. In addition to the MM elements this SMTP “mail” message should reflect all MMS information elements according to the definitions in subclauses 6 and 8.4.

All other MMS-related messages, such as delivery reports, read-reply reports, transfer acknowledgements shall each be transferred as a single SMTP “mail” message which shall be organised as MIME type text/plain. This SMTP “mail” message should reflect all MMS information elements as defined above.

#### Message header fields

MMS information elements should be reflected as “header fields” according to STD 11 in the SMTP “mail” message. See RFC 1327 [53] for a detailed description of the X.400 header to STD 11 headers mappings. Some of the mappings are context dependent.

For those information elements that cannot be mapped to standard STD 11 “header fields” the “X-“ extensions mechanism shall be used with an “X-MMS-” prefix.

The mapping of information elements to commonly used (RFC 1327) [53] or standard STD 11 “header fields” is shown in following tables.

#### MM4_Forward.REQ Header Mappings

The MM4 Forward request header mappings are detailed below.

Table 28: MM4_Forward.REQ Information Elements to
 STD 11 Header Mappings

| Information element | STD 11 Headers |
| --- | --- |
| 3GPP MMS Version | X-Mms-3GPP-MMS-Version: |
| Message Type | X-Mms-Message-Type: |
| Transaction ID | X-Mms-Transaction-ID: |
| Message ID | X-Mms-Message-ID: |
| Recipient(s) address | To:, CC: |
| Sender address | From: |
| Content type | Content-Type: |
| Message class | X-Mms-Message-Class: |
| Date and time | Date: |
| Time of Expiry | X-Mms-Expiry: |
| Delivery report | X-Mms-Delivery-Report: |
| Priority | X-Mms-Priority: |
| Sender visibility | X-Mms-Sender-Visibility: |
| Read reply | X-Mms-Read-Reply: |
| Subject | Subject: |
| Acknowledgement Request | X-Mms-Ack-Request: |
| Content | <message body> |
| Recall-ID | X-Mms-Recall-ID |
| - | Sender: |
| - | Message-ID: |

The table above indicates the mappings from MM4_Forward.REQ information elements to the corresponding STD 11 headers.

The MM Message-ID is not directly mapped to a corresponding STD 11 [5] “Message-ID:” header. Each STD 11 message must have a unique message id, which is carried in the “Message-ID:” header.

Content-type maps directly since both are defined as being MIME content types as specified in RFC 2046 [6].

The STD 11 “From:” header is determined by the mail user agent, or, in this case, the MMS User Agent.  This corresponds to the MM “Sender address”, as set by the MMS User Agent or MMS Relay/Server.

STD 11 messages are required to have a Sender: header that indicates the originator address (as determined by the SMTP “MAIL From” command).

#### MM4_Forward.RES Header Mappings

The MM4 Forward response information element mappings are detailed in the table below.

The transmission of the Forward Response from the recipient MMS Relay/Server requires a properly addressed STD 11 message.  While the addressing of the MM4_Forward.REQ is clearly that of the intended recipients and originator, the MM4_Forward.RES addressing is related to neither the recipients nor the originator of the original MM.  Instead, the MM4_Forward.RES addressing is based on special systems addresses. MMS Service Provider should configure appropriate system addresses which will be used as both the recipient and originator of these administrative messages.  It is suggested that the administrative addressing be based on the pattern:

system-user@mms-relay-host.mmse-domain.

Table 29: MM4_Forward.RES Information Elements to 
STD 11 Header Mappings

| Information element | STD Header |
| --- | --- |
| 3GPP MMS Version | X-Mms-3GPP-MMS-Version: |
| Message Type | X-Mms-Message-Type: |
| Transaction ID | X-Mms-Transaction-ID: |
| Message ID | X-Mms-Message-ID: |
| Request Status Code | X-Mms-Request-Status-Code: |
| Status text | X-Mms-Status-Text: |
| - | Sender: |
| - | To: |
| - | Message-ID: |
| - | Date: |

The Sender: and To: headers contain system addresses as described above, and do not map to MM4_Forward.RES information elements.  The STD 11 message requires a Date: header, but there currently is no corresponding MM4_Forward.RES information element.

#### MM4_Delivery_report.REQ Header Mappings

The mappings of the MM4_Delivery_report.REQ information elements to STD 11 headers is detailed in the table below.

Table 30: MM4_Delivery_report.REQ Information Elements to 
STD 11 Header Mappings

| Information element | STD 11 Header |
| --- | --- |
| 3GPP MMS Version | X-Mms-3GPP-MMS-Version: |
| Message Type | X-Mms-Message-Type: |
| Transaction ID | X-Mms-Transaction-ID: |
| MM Message ID | X-Mms-Message-ID: |
| Recipient address | From: |
| Sender address | To: |
| MM Date and time | Date: |
| Acknowledgement Request | X-Mms-Ack-Request: |
| MM Status Code | X-Mms-MM-Status-Code: |
| Status Text | X-Mms-Status-text: |
| - | Sender: |
| - | Message-ID: |

The meaning of Recipient address is that of the original MM, from whose MMS User Agent this Delivery-report is being generated.  The meaning of Sender address is that of the original MM, to whom the Delivery-report is being sent.

The value of the STD 11 Sender: header is a system administration address, to which the corresponding response will be sent.

The Sender: header value is automatically set to the system address of the MMS Relay/Server.

The Message-ID: value is automatically generated by the MMS Relay/Server, in conformance to STD 11 [5].

The other header mappings from information elements are similar to those already described above.

#### MM4_Delivery_report.RES Header Mappings

The mappings of the M4_Delivery_report.RES information elements to STD 11 headers is detailed in the table below.

Table 31: MM4_Delivery_report.RES Information Elements 
to STD 11 Header Mappings

| Information element | STD 11 Header |
| --- | --- |
| 3GPP MMS Version | X-Mms-3GPP-MMS-Version: |
| MM Message Type | X-Mms-Message-Type: |
| Transaction ID | X-Mms-Transaction-ID: |
| Message ID | X-Mms-Message-ID: |
| Request Status Code | X-Mms-Request-Status-Code: |
| Status text | X-Mms-Status-Text: |
| - | Sender: |
| - | To: |
| - | Message-ID: |
| - | Date: |

The Sender: header value is automatically set to the system address of the MMS Relay/Server that is replying to the MM4_Delivery_report.REQ.

The To: header value of the MM4_Delivery_report.RES abstract message is obtained from the Sender: header value of the corresponding MM4_Delivery_report.REQ.

The Date and Message-ID headers, which have no corresponding MM4_Forward.RES information attributes, are automatically provided values by the MMS Relay/Server.

#### MM4_Read_reply_report.REQ Header Mappings

The mappings of the MM4_Read_reply_report.REQ information elements to STD 11 headers is detailed in the table below.

Table 32: MM4_Read_reply_report.REQ Information Elements 
to STD 11 Header Mappings

| Information element | STD 11 Header |
| --- | --- |
| 3GPP MMS Version | X-Mms-3GPP-MMS-Version: |
| Message Type | X-Mms-Message-Type: |
| Transaction ID | X-Mms-Transaction-ID: |
| Recipient address | From: |
| Sender address | To: |
| Message-ID | X-Mms-Message-ID: |
| Date and time | Date: |
| Acknowledgement Request | X-Mms-Ack-Request: |
| MM Status Code | X-Mms-MM-Status-Code: |
| Status text | X-Mms-Status-Text: |
| - | Sender: |
| - | Message-ID: |
| - | Date: |

The meaning of Recipient address is that of the original MM, from whose MMS User Agent this Read-reply-report is being generated.  The meaning of Sender address is that of the original MM, to whom the Read-reply-report is being sent.

The value of the Sender: header is a system address, to which the corresponding MM4_Read_reply_report.RES shall be sent.

The Message-ID:, and Date: headers, which have no corresponding information attribute in the MM4_Read_reply_report.REQ, are automatically provided appropriate values by the MMS Relay/Server.

#### MM4_Read_reply_report.RES Header Mappings

The mappings of the MM4_Read_reply_report.RES information elements to STD 11 headers is detailed in the table below.

Table 33: MM4_Read_reply_report.RES Information Elements 
to STD 11 Header Mappings

| Information element | STD 11 Header |  |
| --- | --- | --- |
| 3GPP MMS Version | X-Mms-3GPP-MMS-Version: |  |
| MM Message Type | X-Mms-Message-Type: |  |
| Transaction ID |  | X-Mms-Transaction-ID: |
| Request Status Code | X-Mms-Request-Status-Code: |  |
| Status text |  | X-Mms-Status-Text: |
| - |  | Sender: |
| - |  | To: |
| - |  | Message-ID: |
| - |  | Date: |

The Sender: header value shall be the system address of the MMS Relay/Server that is replying to the MM4_Delivery_report.REQ.

The To: header value of the MM4_Delivery_report.RES abstract message shall be obtained from the corresponding MM4_Delivery_report.REQ Sender: header value.

The Date: and Message-ID: headers, which do not have corresponding information elements, shall be provided appropriate values automatically by the MMS Server/Relay.

#### Header Field Value Range

MMS information elements that are mapped to standard STD 11 “header fields”, i.e. which do not have an “X-MMS-” prefix, should be used according to [5].

The rest of the header definitions used in this section, including the mechanisms and pre-defined tokens, are described in an augmented Backus-Naur Form (BNF) defined in [48], similar to that used by RFC 822 [5]. Implementors will need to be familiar with the notation in order to understand these definitions.

For the residual MMS information elements the following applies:

X-Mms-3GPP-MMS-Version:

3GPP-MMS-Version = "X-Mms-3GPP-MMS-Version" ":"  1*DIGIT "." 1*DIGIT "." 1*DIGIT

Note that the numbers MUST be treated as separate integers and that each may be incremented higher than a single digit. Thus, 2.1.4 is a lower version than 2.1.13, which in turn is lower than 2.3.0 Leading zeros shall be ignored by recipient MMSE and shall NOT be sent. The version is according to the version of this specification (see also subclause “Foreword”).

X-Mms-Message-Type:

Message-type = "X-Mms-Message-Type" ":" ( "MM4_forward.REQ" | "MM4_forward.RES" | "MM4_delivery_report.REQ" | "MM4_delivery_report.RES" | "MM4_read_reply_report.REQ" | "MM4_read_reply_report.RES" )

X-Mms-Transaction-Id :

Transaction-id =  "X-Mms-Transaction-ID" ":" quoted-string

X-Mms-Message-Id :

Message-id =  "X-Mms-Message-ID" ":" quoted-string

X-Mms-Message-Class:

Message-class = "X-Mms-Message-Class" ":" ( Class-identifier | quoted-string )

Class-identifier = "Personal" | "Advertisement" | "Informational" | "Auto"

X-Mms-Expiry:

Expiry-value = "X-Mms-Expiry" ":" ( HTTP-date | delta-seconds )

X-Mms-Delivery-Report:

Delivery-report =  "X-Mms-Delivery-Report" ":" ( "Yes" | "No" )

X-Mms-Priority:

Priority = "X-Mms-Priority" ":" ( "Low" | "Normal" | "High" )

X-Mms-Sender-Visibility:

Sender-visibility = "X-Mms-Sender-Visibility" ":" ( "Hide" | "Show" )

X-Mms-Read-Reply:

Read-reply = "X-Mms-Read-Reply" ":" ( "Yes" | "No" )

X-Mms-Ack-Request:

Ack-Request = "X-Mms-Ack-Request" ":" ( "Yes" | "No" )

X-Mms-Request-Status-Code:

Request-status-Code =  "X-Mms-Request-Status-Code" ":" ( "Ok" | "Error-unspecified" | "Error-service-denied" | "Error-message-format-corrupt" | "Error-sending-address-unresolved" | "Error-message-not-found" | "Error-network-problem" | 	"Error-content-not-accepted" | "Error-unsupported-message" )

X-Mms-MM-Status-Code:

MM-Status-Code = "X-Mms-MM-Status-Code" ":" ( "Expired" | "Retrieved" | "Rejected" | "Deferred" | "Intermediate" | "Forwarded" | "Unrecognised" | “Recall successful” | “Recall failed” )

#### Message Encoding on MM4

The SMTP “mail” message shall be encoded according to STD 11  [5].

### 8.4.5 	Message Transfer Protocol on MM4

…