| 3GPP TSG-T2 SWG3#09 MMS ad hoc Paris, France 14-18 January, 2002 |  |  |  |  |  |  |  |  | T2M020075 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CR-Form-v5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 23.140 |  |  |  | CR | CRNum |  |  |  rev |  |  |  | - |  Current version: |  |  |  |  | 5.1.0 |  |  |  |
| For HELP on using this form, see bottom of this page or look at the pop-up text over the  symbols. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed change affects:  |  |  |  |  | (U)SIM |  |  | ME/UE |  |  |  | Radio Access Network |  |  |  |  |  |  |  | Core Network |  | X |
| Title:  |  | Functional Description and Abstract Messages for MM7 realization |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source:  |  | Comverse |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Work item code:  |  | MESS5-MMS |  |  |  |  |  |  |  |  |  |  |  |  | Date:  |  | January 5, 2002 |  |  |  |  |  |
| Category:  |  | B |  |  |  |  |  |  |  |  |  |  |  |  | Release:  |  | REL-5 |  |  |  |  |  |
|  |  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  |  |  |  |  |  |  | Use one of the following releases: 2 (GSM Phase 2) R96 (Release 1996) R97 (Release 1997) R98 (Release 1998) R99 (Release 1999) REL-4 (Release 4) REL-5 (Release 5) |  |  |  |  |  |  |
| Reason for change:  |  |  | MM7 reference point defines the interworking between a VASP and the MMS Relay/Server. Before defining an actual protocol or API, it is required to define the actual functionality and abstract messages of MM7. This CR deals with these issues |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Summary of change:  |  |  | - Text change in section 6.9 defines the functionality of MM7 in general - Changes in sections 7.1.5 and 7.1.6 to include VASP into delivery-report and read-reply considerations - New section 7.1.11 on the full functionality of VASP support - Completion of section 8.7 functionality of MM7 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  |  | Lack of functionality definition of MM7 that is a work item for Release 5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected:  |  |  | Sections 6.9, 7.1.5, 7.1.6, 7.1.11, 8.7 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other specs  |  |  |  | Other core specifications  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| affected: |  |  |  | Test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | O&M Specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other comments:  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

How to create CRs using this form:

Comprehensive information and tips about how to create CRs can be found at: http://www.3gpp.org/3G_Specs/CRs.htm.  Below is a brief summary:

1)	Fill out the above form. The symbols above marked  contain pop-up help information about the field that they are closest to.

2)	Obtain the latest version for the release of the specification to which the change is proposed. Use the MS Word "revision marks"  feature (also known as "track changes") when making the changes. All 3GPP specifications can be downloaded from the 3GPP server under ftp://ftp.3gpp.org/specs/ For the latest version, look for the directory name with the latest date e.g. 2001-03 contains the specifications resulting from the March 2001 TSG meetings.

3)	With "track changes" disabled, paste the entire CR form (use CTRL-A to select it) into the specification just in front of the clause containing the first piece of changed text.  Delete those parts of the specification which are not relevant to the change request.

## 6.9	MM7: MMS Relay/Server – MMS VAS Applications

Reference point MM7 is used to transfer MMs from MMS Relay/Server to MMS VAS applications and to transfer MMs from MMS VAS applications to MMS Relay/Server. This functionality is further elaborated in section 7.1.11. This reference point shall be based on existing protocols e.g. SMTP or HTTP for this release of the specification. Future releases may propose a mandatory protocol and encoding schemes. The service provider may decide to use an encoding format in this reference point, which uses the encoding implementation used in the MM1 reference point.

7.1.5	Delivery Report

The MMS Relay/Server shall support the delivery reporting service. Delivery reports shall only be generated for MMs.

The originator MMS User Agent or VASP may be able to request a delivery report for a specific MM.

Within an MM notification or upon MM retrieval the recipient MMS User Agent may receive an indication that a delivery report is requested for the MM.

Within either a response to a notification or a response to an MM’s delivery, the recipient MMS User Agent may request a delivery report not to be generated by the MMS Relay/Server.  When a VASP has requested the delivery report the MMS Relay/Server must send the delivery report regardless of the MMS User Agent’s request.

The originator MMS Relay/Server shall generate a delivery report if a delivery report has been requested by the originator MMS User Agent or VASP

- upon routing forward the MM, in case the peer entity is not known by the MMS Relay/Server

The recipient MMS Relay/Server shall generate a delivery report if a delivery report has been requested by the originator MMS User Agent and if the recipient MMS User Agent did not request a delivery report not to be generated or in any case that a VASP has requested a delivery report

- upon receipt of a response to a notification, in case the MM is rejected by the recipient MMS User Agent

- upon receipt of a forwarding request, in case the MM is forwarded  by the recipient MMS User Agent to other MM recipient(s), without prior retrieval.

- upon receipt of a response to an MM’s delivery, in case the MM is retrieved by the MM recipient

- upon expiry of the MM, in case the MM is not rejected and not retrieved by the MM recipient before the expiry

The originator MMS User Agent or VASP, i.e. the MMS User Agent or VASP receiving the delivery report, may match the delivery report to the sent MM by retaining the message identification of the sent MM and comparing it to the received delivery report, which shall contain the message identification of the original MM. In case of multiple MM recipients, it is necessary for the originator MMS User Agent or VASP to retain the MM recipient addresses as well, to match the delivery report to the sent MM.

If a delivery report has been requested by the originator MMS User Agent and if the recipient MMS User Agent did not request a delivery report not to be generated, or in any case that the request for the delivery report comes from a VASP, the recipient MMS Relay/Server

- shall generate the delivery report

- shall deliver the delivery report to the originator MMS Relay/Server.

- shall be responsible for the storage of delivery reports in the network until the originator MMS Relay/Server becomes reachable or until the delivery report expires

Within the delivery report the recipient MMS Relay/Server

- shall provide the MM originator address to the originator MMS Relay/Server.

- shall provide the MM recipient address to the originator MMS Relay/Server.

- shall provide the identification of the original MM for which the delivery report has been generated to the originator MMS Relay/Server.

- shall provide status information how the MM was handled (e.g. expired, rejected, delivered, forwarded or indeterminate) to the originator MMS Relay/Server

- shall provide a time stamp when the MM was handled to the originator MMS Relay/Server

For each MM recipient of the original MM for which the delivery report has been generated and becomes available at the originator MMS Relay/Server, the originator MMS Relay/Server

- shall deliver the delivery report to the originator MMS User Agent (i.e. the recipient MMS User Agent of the delivery report) or VASP.

Within the delivery report the originator MMS Relay/Server

- shall provide the MM recipient’s address to the originator MMS User Agent (the recipient MMS User Agent of the delivery report) or VASP.

- shall provide the identification of the original MM for which the delivery report has been generated to the originator MMS User Agent (the recipient MMS User Agent of the delivery report) or VASP.

- shall be responsible for the storage of delivery reports in the network until the originator MMS User Agent or VASP becomes reachable (e.g. user moves back into coverage, switches MMS User Agent on) or until the delivery report expires

7.1.6	Read-Reply Report

The MMS Relay/Server shall support the read-reply reporting service. Read-reply reports shall only be generated for MMs.

Upon MM submission the originator MMS User Agent or VASP may be able to request a read-reply report for a specific MM.

Upon MM retrieval the recipient MMS User Agent may receive an indication that a read-reply report is requested for the MM.

After having handled/rendered the MM the recipient MMS User Agent may generate a read-reply report if requested by the originator MMS User Agent and if the originator MMS User Agent address is available.  If the MM that was handled/rendered originated from a VASP, the recipient MMS User Agent may generate a read-reply report if requested by the VASP using the originating VAS address.

The originator MMS User Agent, i.e. the MMS User Agent receiving the read-reply report, or VASP may match the read-reply report to the sent MM by retaining the message identification of the sent MM and comparing it to the received read-reply report, which shall contain the message identification of the original MM. In case of multiple MM recipients, it is necessary for the originator MMS User Agent or VASP to retain the MM recipient addresses as well as to match the read-reply report to the sent MM.

If a read-reply report has been requested by the originator MMS User Agent or VASP and if the recipient MMS User Agent supports the read-reply feature and if the recipient allows its creation the recipient MMS User Agent shall submit the read-reply report to the recipient MMS Relay/Server at the earliest opportunity.

NOTE:	Since the MM recipient has the right to deny this service not receiving a read-reply report does not mean the message has not been rendered.

A read-reply report:

- shall contain the MM originator’s address

- shall contain the MM recipient’s address

- shall contain the message identification of the original MM for which the read-reply report has been generated.

- shall provide status information how the MM was rendered (e.g. read, deleted without being read)

- shall provide a time stamp for when the MM was rendered

The recipient MMS User Agent shall be responsible for the storage of read-reply reports in the UE until the recipient MMS Relay/Server becomes reachable (subject to support of the read-reply reporting service by the recipient MMS User Agent and storage place being available).

- Upon reception of a read-reply report from a recipient MMS User Agent the recipient MMS Relay/Server

- may provide a time stamp for the read-reply report, i.e. it may also override the MMS User Agent’s time stamp,

- shall pass the MM originator address unaltered when routing the read-reply report towards the originator MMS User Agent (the recipient MMS User Agent of the read reply report)

- shall insert the MM recipient’s address into the read-reply report if not yet provided

- may override the address provided by the recipient MMS User Agent in the read-reply report (subject to MMS service provider’s preferences)

- is responsible for resolving the MM originator’s address,

- is responsible to route the read-reply report towards the originator MMS User Agent of the original MM.

A special case is where the recipient MMS Relay/Server is also the originator MMS Relay/Server. In this case the MM does not have to be routed forward.

    - Support for Value Added Services (VAS) in MMS

The MMS Relay/Server may support services, in addition to user-to-user messaging, that are either provided by the MMS operator or by third-party Value Added Service Providers (VASP).  Examples of services that may be provided as:

- Messages that originate from the VASP to a single or mass-distribution of recipients.

- Messages that originate from a MMS Relay/Server to the VASP that may generate a VASP reply or a new MM submission.

Note: The above functionality will be specified within a single MMSE for this release.  Future releases may expand this support across multiple MMSE.

If the MMS Relay/Server supports connections to VASP then it must provide the appropriate functionality specified in 3GPP TS 22.140 [1].

      - Security

A major concern when connecting with third-party services is security of the information passed between the external service and the messaging server.  Therefore, optionally (e.g. dependent on the service level), the VASP interface may be carried over an encrypted and secure bearer, e.g. HTTP over SSL or TLS, or by use of application-layer encryption.  This may be further elaborated in future releases.

      - Charging Information

The MMS Relay/Server should generate charging events for all operations invoked, directly or indirectly, by VASP operations.  In addition, a VASP may produce service codes that contain billing information that may be transferred to the MMS Relay/Server and passed directly to the billing system without intervention.

## 8.7	Technical realisation of MMS on reference point MM7

The MMSE may support Value Added Services in addition to the basic messaging services defined for MMS.  These VAS may be provided by the network operator of the MMSE or by third-party Value Added Service Providers (VASP).  This clause defines the interworking between the MMS Relay/Server and the VASP.

The following figure illustrates an example data-flow of the message exchange involved in a VAS distribution of a MM as outlined by the abstract messages specified here:

Figure 1. Sample data flow of MM7 message distribution

Subsequent sub-clauses will specify the abstract messages that will define the MM7 protocol.

### Submitting a VAS MM

This section addresses the operations necessary for a VASP to provide the service by sending a multimedia message to one or more subscribers or to a distribution list.  The involved abstract messages are outlined in Table 36 from type and direction points of view.

Table 36: Abstract messages for submitting VAS message

| Abstract messages | Type | Direction |
| --- | --- | --- |
| MM7_submit.REQ | Request | VASP -> MMS Relay/Server |
| MM7_submit.RES | Response | MMS Relay/Server -> VASP |

#### Normal Operation

The VASP submits a message to the MMS Relay/Server by sending the MM7_submit.REQ supplying the multimedia message (MM) as the payload of the message.  The message may be directed to one or more subscribers or to a distribution list.  If the MMS Relay/Server accepts the submission, the MMS Relay/Server must send a MM7_submit.RES with a “success” status.  This in no way indicates that the MM was actually delivered to the destinations but states that the request has been accepted.

Support for MM7_submit.REQ and MM7_submit.RES is mandatory for all MMS Relay/Servers that support MM7.

#### Abnormal Operation

The MMS Relay/Server should reject the MM7_submit.REQ if the VASP cannot be authenticated and authorized or if the parameters of the request exceed the service level for the service being employed.  Similarly, if none of the destinations can be resolved then the response status should indicate an error.  If one or several  (but not all) addresses can be resolved, the MMS Relay/Server should deliver the message to those addresses and respond to the VAS using the MM7_submit.RES with a partial success to the VASP.  Partial success does not indicate that the MM was actually delivered to the destinations but states that the request has been at least partially accepted.

#### Features

Authentication: The VASP must supply its own identifier, the VAS identifier, or its address as part of the request.

Addressing: The VASP may direct the MM to a one or more subscribers or to a distribution list. The originator of a submitted MM may be indicated in addressing-relevant information field(s) of the MM7_submit.REQ

Version: The MM7 protocol shall provide unique means to identify the version supported by both the MMS Relay/Server and VASP.

Message Type: The type of message used on reference point MM7 indicating MM7_submit.REQ and MM7_submit.RES as such.

Transaction Identification: The VASP shall provide an unambiguous transaction identification within an MM7_submit.REQ. The MM7_submit.RES shall unambiguously refer to the corresponding MM7_submit.REQ using the same transaction identification.

Linked message identification: The VASP will supply a message identifier when submitting a message, that defines a correspondence to a previous message that was delivered by the MMS Relay/Server to the VASP

Message class,  priority, and subject: The VASP may qualify the MM further by adding a message class, a priority and/or subject to the MM7_submit.REQ.

Service code:  The VASP may mark the content of the message with a service code that may be transferred by the MMS Relay/Server in the form of charging information for use by the billing system to properly bill the user for the service being supplied.

Time stamping: The VASP may time stamp the MM.

Time constraints: The VASP may request an earliest desired time of delivery of the MM. The VASP may request a time of expiry for the MM

Delivery reporting: The VASP may request a delivery report for the MM

Read reporting: The VASP may request a read-reply report when the user has viewed the MM.

Content adaptation restriction: The VASP may request that the content of the MM will not be subjected to content adaptation.

Content type: The MIME type of the multimedia content shall always be identified in the MM7_submit.REQ.

Content: The VASP may add content in the MM7_submit.REQ.

Message identification: The MMS Relay/Server shall always provide a message identification for an MM, which it has accepted for submission in the MM7_submit.RES.

Request status: The MMS Relay/Server shall indicate the status of the MM7_submit.REQ in the associated MM7_submit.RES. The reason code given in the status information element of the MM7_submit.RES may be supported with an explanatory text further qualifying the status.

#### Information Elements

Table 37: Information elements in the MM7_submit.REQ .

| Information element |  | Presence |  | Description |
| --- | --- | --- | --- | --- |
| Transaction ID |  | Mandatory |  | The identification of the MM7_submit.REQ/ MM7_submit.RES pair. |
| Message type |  | Mandatory |  | Identifies this message as a MM7_submit request. |
| MM7 version |  | Mandatory |  | Identifies the version of the interface supported by the VASP |
| VASP ID |  | Optional |  | Identifier of the VASP for this MMS Relay/Server. |
| VAS ID |  | Optional |  | Identifier of the originating application. |
| Sender address | Optional |  | The address of the MM originator. |  |
| Recipient address | Mandatory |  | The address of the recipient MM. Multiple addresses are possible or the use of the alias that indicates the use of a distribution list. |  |
| Service code | Optional |  | Information supplied by the VASP which may be included in charging information. The syntax and semantics of the content of this information are out of the scope of this specification. |  |
| Linked ID | Optional |  | This identifies a correspondence to a previous valid message delivered to the VASP. |  |
| Message class | Optional |  | Class of the MM (e.g. advertisement, information service, accounting) |  |
| Date and time | Optional |  | The time and date of the submission of the MM (time stamp). |  |
| Time of Expiry | Optional |  | The desired time of expiry for the MM. |  |
| Earliest delivery time | Optional |  | The earliest desired time of delivery of the MM to the recipient. |  |
| Delivery report | Optional |  | A request for delivery report. |  |
| Read reply | Optional |  | A request for confirmation via a read report to be delivered as described in section 8.1 |  |
| Priority | Optional |  | The priority (importance) of the message. |  |
| Subject | Optional |  | The title of the whole multimedia message. |  |
| Adaptations | Optional |  | Indicates if VASP allows adaptation of the content (default True) |  |
| Content type | Mandatory |  | The content type of the MM’s content. |  |
| Content | Optional |  | The content of the multimedia message |  |

Table 38: Information elements in the MM7_submit.RES .

| Information element | Presence | Description |
| --- | --- | --- |
| Transaction ID | Mandatory | The identification of the MM7_submit.REQ/ MM7_submit.RES pair. |
| Message type | Mandatory | Identifies this message as a MM7_submit response. |
| MM7 version | Mandatory | Identifies the version of the interface supported by the MMS Relay/Server |
| Message ID | Conditional | If status indicates success then this contains the MMS Relay/Server generated identification of the submitted message. This ID may be used in subsequent requests and reports relating to this message. |
| Request Status | Mandatory | Status of the completion of the submission, no indication of delivery status is implied. |
| Request Status text | Optional | Text description of the status for display purposes, should qualify the Request Status code. |

### Delivery Request

This section addresses cases where a message that is passed by the MMS Relay/Server to a VASP for processing.  For example, this may include cases where the message originated from the MMS User-Agent.

The involved abstract messages are outlined in Table 44 from type and direction points of view.

Table 44: Abstract messages for demanding a service from a VASP

| Abstract messages | Type | Direction |
| --- | --- | --- |
| MM7_deliver.REQ | Request | MMS Relay/Server -> VASP |
| MM7_deliver.RES | Response | VASP -> MMS Relay/Server |

#### Normal Operation

The MMS Relay/Server will deliver messages to the VASP by supplying the MM as the payload of the MM7_deliver.REQ.  The message originates, for example, from a MMS User Agent, an external application, or from outside the MMSE.  This delivery may include an identification of the request that may be used by the VASP to correlate a response to the message.  The VASP should reply with a MM7_deliver.RES message indicating that the message has been successfully received and will be processed.

The following figure illustrates the data flow of a use case where a MMS User Agent requesting a service from a VAS that requires a response.

Figure 2. Use of MM7_deliver and subsequent response

Support for MM7_deliver.REQ and MM7_deliver.RES is mandatory for a MMS Relay/Server that supports MM7

#### Abnormal Operation

If the VASP cannot identify the requested content then it should indicate the failure in the MM7_deliver.RES status fields.

#### Features

Authentication: The MMS Relay/Server may supply its own identifier as part of the request.

Addressing: All relevant address information for the delivery of the message to the VASP – including the addressing information from the original message and the MMS Relay/Server and VASP addresses should be included in the relevant information elements of MM7_deliver.REQ.

Version: The MM7 protocol shall provide unique means to identify the version supported by both the MMS Relay/Server and VASP.

Message Type: The type of message used on reference point MM7 indicating MM7_deliver.REQ and MM7_deliver.RES as such.

Transaction Identification: The VASP shall provide an unambiguous transaction identification within a request. The response shall unambiguously refer to the corresponding request using the same transaction identification.

Message priority and subject: The MMS Relay/Server may qualify the MM further by adding a priority and/or subject to the MM7_deliver.REQ.  This information will originate from the end-user’s original request.

Linked message identification: The MMS Relay/Server will supply an identifier for the request that may be used by the VASP.

Service code:  The VASP may mark the response to the message with a service code that will be transferred to the charging information for use by the billing system to properly bill the user for the service being supplied.

Time stamping: The MM may include a time stamp indicating the time of original submission.

Content type: The MIME type of the multimedia content shall always be identified in the MM7_deliver.REQ.

Content: The originator of the MM may supply content that is delivered to the VASP in the MM7_deliver.REQ.

Request status: The MMS Relay/Server shall indicate the status of the request in the associated response. The reason code given in the status information element of the response may be supported with an explanatory text further qualifying the status.

#### Information Elements

Table 45: Information elements in the MM7_deliver.REQ .

| Information element |  |  | Presence |  |  | Description |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Transaction ID |  |  | Mandatory |  |  | The identification of the MM7_deliver.REQ/ MM7_deliver.RES pair. |  |
| Message type |  |  | Mandatory |  |  | Identifies this message as a MM7_deliver request. |  |
| MM7 version |  |  | Mandatory |  |  | Identifies the version of the interface supported by the MMS Relay/Server |  |
| MMS Relay/Server ID |  |  | Optional |  |  | Identifier of the MMS Relay/Server |  |
| VAS address |  | Mandatory |  |  | Address of the recipient VAS |  |  |
| Linked ID |  | Optional |  |  | Identifier that may be used by the VASP in a subsequent MM7_submit.REQ |  |  |
| Sender address |  | Mandatory |  |  | The address of the MM originator. |  |  |
| Recipient address |  | Optional |  |  | The address(es) of the intended recipients of the subsequent processing by the VASP or the original recipient address(es). |  |  |
| Date and time | Optional |  |  | The time and date of the submission of the MM (time stamp). |  |  |  |
| Priority | Optional |  |  | The priority (importance) of the message. |  |  |  |
| Subject |  | Optional |  |  | The title of the whole MM. |  |  |
| Content type |  | Mandatory |  |  | The content type of the MM’s content. |  |  |
| Content |  | Optional |  |  | The content of the multimedia message |  |  |

Table 46: Information elements in the MM7_deliver.RES .

| Information element |  | Presence | Description |  |
| --- | --- | --- | --- | --- |
| Transaction ID |  | Mandatory | The identification of the MM7_deliver.REQ/ MM7_deliver.RES pair. |  |
| Message type |  | Mandatory | Identifies this message as a MM7_deliver response. |  |
| MM7 version | Mandatory |  |  | Identifies the version of the interface supported by the VASP |
| Service code | Optional |  |  | Information supplied by the VASP which may be included in charging information. The syntax and semantics of the content of this information are out of the scope of this specification. |
| Request Status |  | Mandatory | Status of the completion of the request. |  |
| Request Status text |  | Optional | Text description of the status for display purposes, should qualify the Request Status code |  |

### Cancel and replace of MM

This section details the requests that should be supported in MM7 to allow a VASP to control or change the distribution of a message.  These operations will allow the VASP to cancel a submitted message prior to delivery or replace a submitted message with a new message.

The involved abstract messages are outlined in Table 39 from type and direction points of view.

Table 39: Abstract messages for controlling Distribution MM

| Abstract messages | Type | Direction |
| --- | --- | --- |
| MM7_cancel.REQ | Request | VASP -> MMS Relay/Server |
| MM7_cancel.RES | Response | MMS Relay/Server -> VASP |
| MM7_replace.REQ | Request | VASP -> MMS Relay/Server |
| MM7_replace.RES | Response | MMS Relay/Server -> VASP |

The following figure illustrates the interaction between the different MMS entities in canceling a VASP message.

Figure 3.  Data flow of VASP canceling a submitted message

#### Normal Operation

If the VASP has decided to cancel the delivery of a MM that it has already submitted, then the VASP should indicate this by sending the MM7_cancel.REQ message to the MMS Relay/Server.  The MMS Relay/Server should check the status of the message indicated by the Message ID and cancel delivery to all destinations that have not received notification of the MM or have not retrieved or forwarded the message as yet.  The MMS Relay/Server should respond to the request with a MM7_cancel.RES indicating that the request was processed.

If the VASP has new content that it wishes to submit in place of the content that was originally submitted it should submit the new replacement content using the MM7_replace.REQ message.  The MMS Relay/Server should check the status of the message indicated by the Message ID and replace the message content for all destinations that have not received notification of the MM or have not retrieved or forwarded the message as yet.  The MMS Relay/Server should redistribute the new content to the destination list from the original MM7_submit.REQ.  Optional information elements that appear in the MM7_replace.REQ message shall replace the corresponding information elements of the original submission, information elements that do not appear in the MM7_replace.REQ message shall retain the original submission values.  Replacement of messages that have been retrieved may be specified in future releases.

Support for MM7_cancel.REQ, MM7_cancel.RES, MM7_replace.REQ, and MM7_replace.RES is optional for all MMS Relay/Server that support MM7

#### Abnormal Operation

The MMS Relay/Server should reject a request to cancel or replace a message if it is unable to authenticate the VASP or find the Message ID indicated in the request, or cannot determine that the indicated message was originally submitted by the VASP.

#### Features

Authentication: The VASP must supply its own identifier, the VAS identifier, or its address as part of the request.

Addressing: When replacing a previously sent message the replacement is assumed to use the same recipients as the original being replaced.  The originator of a submitted MM may be indicated in addressing-relevant information field(s) of the MM7_replace.REQ

Version: The MM7 protocol shall provide unique means to identify the version supported by both the MMS Relay/Server and VASP.

Message type: The type of message used on reference point MM7 indicating MM7_cancel.REQ, MM7_cancel.RES, MM7_replace.REQ, and MM7_replace.RES as such.

Transaction identification: The VASP shall provide an unambiguous transaction identification within a request. The response shall unambiguously refer to the corresponding request using the same transaction identification.

Message class, priority, and subject: The MM shall be qualified further by adding a message class, a priority, and/or subject to the MM7_replace.REQ.

Service code:  The VASP may mark the content of the message with a service code that may be transferred by the MMS Relay/Server in the form of charging information for use by the billing system to properly bill the user for the service being supplied.

Time stamping: The VASP may time stamp the MM.

Time constraints: The VASP may also request an earliest desired time of delivery of the MM. The VASP may request a time of expiry for the MM

Delivery reporting: The VASP may request a delivery report for the MM.

Read reporting: The VASP may request a read-reply report when the user has viewed the MM.

Content adaptation restriction: The VASP may request that the content of the MM will not be subjected to content adaptation.

Content type: The MIME type of the multimedia content shall always be identified in the MM7_replace.REQ.

Content: The VASP may add content in the MM7_replace.REQ.

Message identification: The MMS Relay/Server shall always provide a message identification for an MM, which it has accepted for submission in the MM7_replace.RES. The VASP must supply this message identification when requesting to cancel or replace a previously submitted message.  When replacing a MM the updated message retains the identification of the original (replaced) message.

Request status: The MMS Relay/Server shall indicate the status of the request in the associated response. The reason code given in the status information element of the response may be supported with an explanatory text further qualifying the status.

#### Information Elements

Table 40: Information elements in the MM7_cancel.REQ .

| Information element | Presence | Description |
| --- | --- | --- |
| Transaction ID | Mandatory | The identification of the MM7_cancel.REQ/ MM7_cancel.RES pair. |
| Message type | Mandatory | Identifies this message as a MM7_cancel request. |
| MM7 version | Mandatory | Identifies the version of the interface supported by the VASP |
| VASP ID | Optional | Identifier of the VASP for this MMS Relay/Server. |
| VAS ID | Optional | Identifier of the originating application. |
| Sender address | Optional | The address of the MM originator. |
| Message ID | Mandatory | Identifier of the message to cancel. |

Table 41: Information elements in the MM7_cancel.RES .

| Information element | Presence | Description |
| --- | --- | --- |
| Transaction ID | Mandatory | The identification of the MM7_cancel.REQ/ MM7_cancel.RES pair. |
| Message type | Mandatory | Identifies this message as a MM7_cancel response. |
| MM7 version | Mandatory | Identifies the version of the interface supported by the MMS Relay/Server |
| Request Status | Mandatory | Status of the completion of the request. |
| Request Status text | Optional | Text description of the status for display purposes, should qualify the Request Status code |

Table 42: Information elements in the MM7_replace.REQ .

| Information element |  | Presence |  | Description |
| --- | --- | --- | --- | --- |
| Transaction ID |  | Mandatory |  | The identification of the MM7_replace.REQ/ MM7_replace.RES pair. |
| Message type |  | Mandatory |  | Identifies this message as a MM7_replace request. |
| MM7 version |  | Mandatory |  | Identifies the version of the interface supported by the VASP |
| VASP id |  | Optional |  | Identifier of the VASP for this MMS Relay/Server. |
| VAS ID |  | Optional |  | Identifier of the originating application. |
| Sender address |  | Optional |  | The address of the MM originator. |
| Message ID |  | Mandatory |  | Identifier of the message that current message replaces. |
| Service code | Optional |  | Information supplied by the VASP which may be included in charging information. The syntax and semantics of the content of this information are out of the scope of this specification. |  |
| Message class | Optional |  | Class of the MM (e.g. advertisement, information service, accounting) |  |
| Date and time | Optional |  | The time and date of the submission of the MM (time stamp). |  |
| Time of Expiry | Optional |  | The desired time of expiry for the MM or reply-MM. |  |
| Earliest delivery time | Optional |  | The earliest desired time of delivery of the MM to the recipient. |  |
| Delivery report | Optional |  | A request for delivery report. |  |
| Read reply | Optional |  | A request for confirmation via a read report to be delivered as described in section 8.1 |  |
| Priority | Optional |  | The priority (importance) of the message. |  |
| Subject | Optional |  | The title of the whole multimedia message. |  |
| Adaptations | Optional |  | Indicates if VASP allows adaptation of the content (default True) |  |
| Content type | Conditional |  | The content type of the MM’s content. If the Content IE appears, then the Content type IE must appear. |  |
| Content | Optional |  | The content of the multimedia message |  |

Table 43: Information elements in the MM7_replace.RES .

| Information element | Presence | Description |
| --- | --- | --- |
| Transaction ID | Mandatory | The identification of the MM7_replace.REQ/ MM7_replace.RES pair. |
| Message type | Mandatory | Identifies this message as a MM7_replace response. |
| MM7 version | Mandatory | Identifies the version of the interface supported by the MMS Relay/Server |
| Request Status | Mandatory | Status of the completion of the request. |
| Request Status text | Optional | Text description of the status for display purposes, should qualify the Request Status code |

### Delivery reporting to VASP

This part of MMS service covers the generation of a delivery report from the MMS Relay/Server to the VASP. The involved abstract messages are outlined in Table 47 from type and direction points of view.

Table 47: Abstract messages for delivery reports to VASP

| Abstract Message | Type | Direction |
| --- | --- | --- |
| MM7_delivery_report.REQ | Request | MMS Relay/Server -> VASP |
| MM7_delivery_report.RES | Response | VASP -> MMS Relay/Server |

#### Normal Operation

The MMS Relay/Server shall create the MM7_delivery_report.REQ and send it to the VASP when the appropriate information is available.

Support for MM7_delivery_report.REQ and MM7_delivery_report.RES is mandatory for a MMS Relay/Server that supports MM7.

#### Abnormal Operation

In case the VASP cannot authenticate the MMS Relay/Server or the Message ID is not known to it, then the VASP shall respond with a MM7_delivery_report.RES including a status which indicates the reason the delivery report was not accepted.

#### Features

Authentication: The MMS Relay/Server may supply its own identifier as part of the request.

Addressing: Both the address of the VAS (which is the original MM originator) and the address of the recipient of the original MM shall be provided in the addressing-relevant information fields of MM7_delivery_report.REQ.

Version: The MM7 protocol shall provide unique means to identify the version supported by both the MMS Relay/Server and VASP.

Message Type: The type of message used on reference point MM7 indicating MM7_delivery_report.REQ and MM7_delivery_report.RES as such.

Transaction Identification: The VASP shall provide an unambiguous transaction identification within a request. The response shall unambiguously refer to the corresponding request using the same transaction identification.

Time stamping: The MM7_delivery_report.REQ shall carry the time and date of handling of the MM (e.g. retrieval, expiry, rejection).

Message identification: In the MM7_delivery_report.REQ the MMS Relay/Server shall always provide the original message identification of the MM that the delivery report corresponds to as generated in response to the associated MM7_submit.REQ.

MM Status: The MM7_delivery_report.REQ shall carry the status of the MM delivery, e.g. retrieved, rejected, expired or indeterminate.

Request Status: The VASP shall indicate the status of the MM7_delivery_report.REQ in the associated MM7_delivery_report.RES. The reason code given in the status information element of the response may be supported with an explanatory text further qualifying the status.

#### Information Elements

Table 48: Information elements in the MM7_delivery_report.REQ.

| Information element |  | Presence |  | Description |
| --- | --- | --- | --- | --- |
| Transaction ID | Mandatory |  | The identification of the MM7_delivery_report.REQ/ MM7_delivery_report.RES pair. |  |
| Message Type |  | Mandatory |  | The type of message used on reference point MM7 “ MM7_delivery_report.REQ”. |
| MM7 Version |  | Mandatory |  | The version of MM7 supported by the MMS Relay/Server |
| MMS Relay/Server ID |  | Optional |  | Identifier of the MMS Relay/Server |
| Message ID |  | Mandatory |  | The identification of the original MM. |
| Recipient address |  | Mandatory |  | The address of the recipient of the original MM. |
| Sender address |  | Mandatory |  | The address of the VAS that submitted the original MM. |
| Date and time |  | Mandatory |  | Date and time the MM was handled (retrieved, expired, rejected, etc.) |
| MM Status |  | Mandatory |  | Status of the MM, e.g. retrieved, expired, rejected |
| Status text |  | Optional |  | Text description of the status for display purposes, should qualify the MM Status code |

Table 49: Information elements in the MM7_delivery_report.RES.

| Information element | Presence |  | Description |  |
| --- | --- | --- | --- | --- |
| Transaction ID |  | Mandatory |  | The identification of the MM7_delivery_report.REQ/ MM7_delivery_report.RES pair. |
| Message Type | Mandatory |  | The type of message used on reference point MM7: “MM7_delivery_report.RES”. |  |
| MM7 Version | Mandatory |  | The version of MM7 supported by the VASP |  |
| Request Status | Mandatory |  | The status of the associated MM7_delivery_report.REQ. |  |
| Request Status text |  | Optional |  | Text description of the status for display purposes, should qualify the Request Status code |

### Read-Reply Report for VASP

This part of MMS service covers the delivery of a read-reply report from the MMS Relay/Server to the VASP. The involved abstract messages are outlined in Table 50 from type and direction points of view.

Table 50: Abstract messages for sending and receiving read-reply reports in MM7

| Abstract messages | Type | Direction |
| --- | --- | --- |
| MM7_read_reply.REQ | Request | MMS Relay/Server -> VASP |
| MM7_read_reply.RES | Response | VASP -> MMS Relay/Server |

#### Normal Operation

If the VASP requested a read-reply report then the recipient MMS User Agent may create and send a read-reply to the MMS Relay/Server. The MMS Relay/Server must identify that this read-reply report is associated with a MM originating from the MM7 reference point and must create the MM7_read_reply.REQ and send it to the VASP.  The VASP shall return a MM7_read_reply.RES that reflects the successful reception of the read-reply report.

Support for MM7_read_reply_report.REQ and MM7_read_reply_report.RES is optional for a MMS Relay/Server that supports MM7.

#### Abnormal Operation

In case the VASP cannot authenticate the MMS Relay/Server or the Message ID is not known to it, then the VASP shall respond with a MM7_read_reply.RES including a status which indicates the reason the read reply report was not accepted.

#### Features

Authentication: The MMS Relay/Server may supply its own identifier as part of the request.

Addressing: Both, the address of the VASP (which is the MM originator), and the address of the originator (which is the MM recipient) of a read-reply report shall be provided in the addressing-relevant information fields of MM7_read_reply_report.REQ.

Version: The MM7 protocol shall provide unique means to identify the version supported by both the MMS Relay/Server and VASP.

Message Type: The type of message used on reference point MM7 indicating MM7_read_reply.REQ and MM7_read_reply.RES as such.

Transaction Identification: The VASP shall provide an unambiguous transaction identification within a request. The response shall unambiguously refer to the corresponding request using the same transaction identification.

Message identification: In the MM7_read_reply_report.REQ the MMS Relay/Server shall always provide the original message identification of the MM that the read-reply report corresponds to as generated for the MM7_submit.RES.

Time Stamping: The MM7_read_reply_report.REQ shall carry the time-stamp associated with the read-reply report.

Read Status: The MM7_read_reply_report.REQ shall carry the status of the MM retrieval, e.g. read or deleted without being read.

Request Status: The VASP shall indicate the status of the MM7_read_reply.REQ in the associated MM7_read_reply.RES.  The reason code given in the status information element of the response may be supported with an explanatory text further qualifying the status.

#### Information Elements

Table 51: Information elements in the MM7_read_reply_report.REQ.

| Information element | Presence | Description |
| --- | --- | --- |
| Transaction ID | Mandatory | The identification of the MM7_read_reply_report.REQ/ MM7_read_reply_report.RES pair. |
| Message Type | Mandatory | Identifies this message as a MM7_read_reply_report request. |
| MM7 Version | Mandatory | The version of MM7 supported by the MMS Relay/Server. |
| MMS Relay/Server ID | Optional | Identifier of the MMS Relay/Server |
| Recipient address | Mandatory | The address of the MM recipient of the original MM, i.e. the originator of the read-reply report. |
| VASP address | Mandatory | The address of the VASP (originator of the original MM) i.e. the recipient of the read-reply report. |
| Message-ID | Mandatory | The message ID of the original MM. |
| Date and time | Mandatory | Date and time the MM was handled (read, deleted without being read, etc.) (time stamp) |
| Read Status | Mandatory | Status of the MM, e.g. Read, Deleted without being read |
| Status text | Optional | Text description of the status for display purposes, should qualify the Read Status code |

Table 52: Information elements in the MM7_read_reply_report.RES.

| Information element | Presence | Description |
| --- | --- | --- |
| Transaction ID | Mandatory | The identification of the MM7_read_reply_report.REQ/ MM7_read_reply_report.RES pair. |
| Message Type | Mandatory | Identifies this message as a MM7_read_reply_report response. |
| MM7 Version | Mandatory | The version of MM7 supported by the VASP. |
| Request Status | Mandatory | The status of the associated MM7_read_reply_report.REQ. |
| Request Status text | Optional | Text description of the status for display purposes, should qualify the Request Status code. |

### Administrating the Distribution List

After a Value Added Service becomes available users may subscribe to the service using direct contact to the VASP (e.g. by sending a MM via MM1_submit.REQ to the service provider including registration information).  The distribution list may be maintained by the MMS Relay/Server.  The full definition of the administration of the distribution list may be specified in future releases of this specification.

### Generic Error Handling

When the MMS Relay/Server or VASP receives a MM7 abstract message that cannot be replied to with the specific response it shall reply using a generic error message as described here. The involved abstract messages are outlined in Table 53 from type and direction points of view.

Table 53: Abstract message for generic error notification

| Abstract message | Type | Direction |
| --- | --- | --- |
| MM7_RS_error.RES | Response | MMS Relay/Server -> VASP |
| MM7_VASP_error.RES | Response | VASP->MMS Relay/Server |

Normal Operation

If the MMS Relay/Server has received a message over the MM7 interface and does not recognize the Message Type, or the requested feature is not supported and the normal response message is not supported, then the MMS Relay/Server must generate a MM7_RS_error.RES message to reply to the VASP.

If the VASP has received a message over the MM7 interface and does not recognize the Message Type, or the requested feature is not supported and the normal response message is not supported, then the VASP must generate a MM7_VASP_error.RES message to reply to the MMS Relay/Server.

Support for the MM7_RS_error.RES and MM7_VASP_error.RES is Mandatory for a MMS Relay/Server that supports MM7

#### Features

Version: The MM7 protocol shall provide unique means to identify the version supported by both the MMS Relay/Server and VASP.

Message Type: The type of message used on reference point MM7 indicating MM7_RS_error.RES or MM7_VASP_error.RES as such.

Transaction Identification: The response shall unambiguously refer to the corresponding request using the same transaction identification.

Error Status: The MMS Relay/Server or VASP shall indicate the error condition that caused the generation of the error response.  The reason code given in the status information element of the response may be supported with an explanatory text further qualifying the status.

#### Information Elements

Table 54: Information elements in the MM7_RS_error.RES .

| Information element | Presence | Description |
| --- | --- | --- |
| Transaction ID | Mandatory | Identifier that corresponds to the Transaction ID of the incoming message. |
| Message type | Mandatory | Identifies this message as a MM7_RS_error response. |
| MM7 version | Mandatory | Identifies the version of the interface supported by the MMS Relay/Server |
| Error Status | Mandatory | Error code (e.g. not-supported). |
| Error Status text | Optional | Text description of the status for display purposes, should qualify the Error Status code. |

Table 55: Information elements in the MM7_VASP_error.RES .

| Information element | Presence | Description |
| --- | --- | --- |
| Transaction ID | Mandatory | Identifier that corresponds to the Transaction ID of the incoming message. |
| Message type | Mandatory | Identifies this message as a MM7_VASP_error response. |
| MM7 version | Mandatory | Identifies the version of the interface supported by the VASP |
| Error Status | Mandatory | Error code (e.g. not-supported). |
| Error Status text | Optional | Text description of the status for display purposes, should qualify the Error Status code. |