Source:	CN3

Title:		Report of CN3 workshop on CS Data Services in UMTS

TSG CN3 has discussed the CS Data Services in UMTS. This report summarises the result of the discussion, highlights the basic principles and lists the identified issue.

Input documents:

| N3-99083 | Liaison to TSG-S2, S4, N1, N2, N3, T2 regarding CS bearer services | TSG-SA1 |
| --- | --- | --- |
| N3-99099 | UMTS 25.321 version 2.0.0 | TSG-RAN2 |
| N3-99100 | UMTS 25.322 version 1.0.0 | TSG-RAN2 |
| N3-99112 | Discussion Paper on CS data and IU user plane and RLC requirements | Ericsson |
| N3-99117 | WI on CS Bearer Services | Alcatel, Siemens |
| N3-99123 | UMTS 25.415 | TSG-RAN3 |

CN3 has reviewed the requirements contained in Tdoc N3-99083 received from S1 (TSGS1#3(99)367).

In contradiction to the requirements we propose to support the following user rates:

- transparent services with user rates 28.8kbit/s (video telephony) and 64kbit/s (ISDN), other rates are FFS

- non-transparent services with user rates up to and including 64kbit/s

Output documents:

| N3-99152 | Report of CN3 workshop on CS Data Services in UMTS | TSG-CN3 |
| --- | --- | --- |
| N3-99151 | WI on CS Bearer Services | TSG-CN3 |
| N3-99147 | LS on CS Data Services | TSG-CN3 |

Proposed architecture:

The use of the GSM RLP for non-transparent services was agreed, however the RLP frame size needs further study.

The use of the Iu CS UP was questioned as the added value of it seems to be minor. However, if it is used it eases the standardization of CS data services as there’s no need to standardize a ‘new’ relay function for data services in RNC.

CN3 has to identify the requirements for the Iu CS UP for CS data services.

Requirements for the Iu CS UP (this list is not exhaustive and needs to be completed):

- octet alignment has to be ensured for transparent services

- RLP frame alignment has to be ensured for non-transparent services

- no CRC is needed from the Iu UP

Issues studied for the next meeting:

- requirements on Iu-UP

- used Data Bearer (AAL2/AAL5)

- used RLP frames size (fixed/variable, currently 576bits are proposed)

- used RLC application mode (transparent or unacknowledged mode, acknowledged mode seems to be not applicable)

- user rates to be supported for transparent services

- the need for a frame type identifier

- how is following control information transported (in the Iu UP Payload or in the Iu UP Control Field)

- end-to-end synchronization for transparent services

- NIC (is it required at all?)