| Meeting Number | GPRS 3 | GPRS Doc 16/99rev1 |
| --- | --- | --- |
| Meeting Date | 9 March 1999 |  |
| Meeting Location | Rome, Italy |  |

| Title | LS to SMG1 re: Proposed Modification to GSM 03.03 to include formally humanly-readable APN Operator Identifiers |
| --- | --- |
| Source | Chairman GPRS |
| Date | 12 May 1999 |

| Read Access*: |  |  |
| --- | --- | --- |
| Restricted - Members |  |  |

| Information Category | Roaming - Technical |
| --- | --- |

Restricted - Confidential Information

Access to and distribution of this document is restricted to the persons listed under the heading Read Access*. This document is confidential to the Association and is subject to copyright protection.  This document is to be used only for the purposes for which it has been supplied and information contained in it must not be disclosed or in any other way made available, in whole or in part, to persons other than those listed under Read Access* without the prior written approval of the Association.

The GSM MoU Association (“Association”) makes no representation, warranty or undertaking (express or implied) with respect to and does not accept any responsibility for, and hereby disclaims liability for the accuracy or completeness or timelinesss of the information contained in this document. The information contained in this document may be subject to change without prior notice.

© Copyright of the GSM MoU Association 1998

Document History

| Revision | Date | Brief Description |
| --- | --- | --- |
| 0 | 9-10 March 99 | Draft version of the liaison statement |
| 1 | 14 May 99 | Updated: 3GPP groups added |

## GPRS 16/99

GPRS 3

Sofia, Bulgaria

9-10 March 1999

Source Vodafone

# Liaison Statement

To:	SMG1

Copied:	SMG3, SMG12, SMG4, SERG, 3GPP TSG-S1, 3GPP TSG-S2,

3GPP TSG-CN

From:	GSM Association – IREG

Title:	Proposed Modification to GSM 03.03 to include formally humanly-readable APN Operator Identifiers.

Section 9 of GSM 03.03 provides a Definition of Access Point Name (APN), a definition of the Structure of APN; and the format of the APN Network Identifier and of the APN Operator Identifier.

IREG believes that these will be situations where a context is established and where an APN comprising both the Network and Operator identifiers will be supplied by the GPRS terminal, and hence previously will have been entered by the user.

Consequently IREG believes that the format of the APN Operator identifier should be “human-friendly”.

IREG notes that GSM 03.03 V6.1.0 (With approved CR A0016 rev2) Section 9.2.2 contains the following text:-

“The PLMN may provide DNS translations for other more human-readable APN Operator Identifiers……”.  However, IREG believes that GSM 03.03 should be amended to give addresses of the form:-

“<network name>.<country>.gprs” e.g. “vodafone.uk.gprs”

equal standing and prominence with the form:-

“mnc <MNC>.MCC<MCC>.gprs”. e.g.”mnc15.mcc234.gprs”

This will result in the appropriate functionality being universally supported throughout all GSM PLMNs, thereby aiding roaming.

IREG recognises that the <network name> and <country> labels will require registering, recording and dissemination, and believes that this will be readily undertaken by the GSM association.

IREG request that SMG1 consider this issue, and if in agreement, request the appropriate SMG STC(s) to modify 03.03 so that the use of a humanly readable form of Operator Identifiers within an APN is considered to be of equal significance and standing to the numeric/IMSI-derived form.

# Attached Section 9 from GSM 03.03 V6.1.0 (With approved CR A0016 rev2)

# 9  Definition of Access Point Name

In the GPRS backbone, an Access Point Name (APN) is a reference to a GGSN. To support inter-PLMN roaming, the internal GPRS DNS functionality is used to translate the APN into the IP address of the GGSN.

## 9.1	Structure of APN

The APN is composed of two parts as follows:

- The APN Network Identifier which defines to which external network the GGSN is connected to. This part of the APN is mandatory.

- The APN Operator Identifier which defines in which PLMN GPRS backbone the GGSN is located. This part of the APN is optional.

The APN Operator Identifier is placed after the APN Network Identifier. An APN consisting of both the Network Identifier and Operator Identifier corresponds to a DNS name of a GGSN and has a maximum length of 100 octets.

The syntax of the APN shall follow the Name Syntax defined in RFC 2181 [14] and RFC 1035 [15]. The APN consists of one or more labels. Each label is coded as one octet length field followed by that number of octets coded as 8 bit ASCII characters. Following RFC 1035 [15] the labels should consist only of the alphabetic characters (A-Z and a-z), digits (0-9) and the dash (-). The case of alphabetic characters is not significant. The APN is not terminated by a length byte of zero.

Note: A length byte of zero is added by the SGSN at the end of the APN before interrogating a DNS server.

For the purpose of presentation, an APN is usually displayed as a string in which the labels are separated by dots (e.g. “Label1.Label2.Label3”).

### 9.1.1	Format of APN Network Identifier

The APN Network Identifier shall contain at least one label and shall have a maximum length of 63 octets. An APN Network Identifier shall not start with the strings “rac”, “lac” or “sgsn” and it shall not end in “.gprs”. It shall also not take the value “*”.

In order to guarantee uniqueness of APN Network Identifier within the GPRS PLMN(s), an APN Network Identifier containing more than one label corresponds to an Internet domain name. This name should only be allocated by the PLMN to an organisation that has officially reserved this name in the Internet domain. Other types of APN Network Identifiers are not guaranteed to be unique within the GPRS PLMN(s).

### 9.1.2	Format of APN Operator Identifier

The APN Operator Identifier is composed of three labels. The last label shall be “gprs”. The first and second labels together shall uniquely identify the GPRS PLMN (e.g. “<operator-name>.<operator-group>.gprs”).

For each operator, there is a default APN Operator Identifier (i.e. domain name). This default APN Operator Identifier is derived from the IMSI as follows:

“mnc<MNC>.mcc<MCC>.gprs”

where:

“mnc” and“mcc” serve as invariable identifiers for the following digits.

<MNC> and <MCC> are the components of the IMSI defined in section 2.2.

This default APN Operator Identifier is used in inter-PLMN roaming situations when attempting to translate an APN consisting of Network Identifier only into the IP address of the GGSN residing in the HPLMN. The PLMN may provide DNS translations for other, more human-readable, APN Operator Identifiers in addition to the default Operator Identifier described above.

## 9.2	Definition of the Wild Card APN

The APN field in the HLR may contain a wild card APN if the HPLMN operator allows the subscriber to access any network of a given PDP Type. If an SGSN has received such a wild card APN, it may either choose the APN Network Identifier received from the Mobile Station or a default APN Network Identifier for addressing the GGSN when activating a PDP context.

### 9.2.1	Coding of the Wild Card APN

The wild card APN is coded as an APN with “*” as its single label, (i.e. a length octet with value one, followed by the ASCII code for the asterisks).