| 3GPP TSG-T2 #5 Helsinki, FINLAND, 6-9 September 1999 | TSGT2#5(99)679 |
| --- | --- |

Agenda Item:	Cell Broadcast Service

Source: 	Mannesmann Mobilfunk

Title: 	Cell Broadcast Enhancements 23.041

Document for:	Information

Mannesmann Mobilfunk has already launched successful a few cell broadcast services and expects an increasing number of other services based on cell broadcast information. Also the number of other network operators providing cell broadcast services is increasing.

To guarantee the continuity of this services cell broadcast was defined as a UMTS Phase 1 Release 99 requirement [1]. It shall be provided seamlessly (as far as the user or the users terminal equipment is concerned) across the UMTS and GSM network.

As already planed the GSM 03.41 is going to be moved to 3GPP as 23.041 version 3.0.0. In addition in the following necessary modification to adapt the service to the new radio interface are pointed out:

- The GSM page concept with fixed message size is a result of the GSM radio interface. With UMTS the radio interface changes totally. In UMTS it is required that the lower layers of the interfaces transmit the complete CB message.

- SA2 has decided (see LS provided with T2-99641) to integrate the CBC into the core network and to standardise the CBC-RNC interface and to introduce a common communication channel over IP. SA2 requests comments.

- The old GSM 03.41 contains primitives describing the operations between CBC and RNC as an informative part of the specification. These operations should be transformed into a protocol specification. A protocol architecture is given by T2-99641 (LS form SA regarding CBC architecture).

- The control of CB discontinuous reception by CBC on the radio interface should be revised. The mandatory usage of CB discontinuous reception in the UE is under discussion of RAN2.

- In GSM the reception of CBS messages was restricted to the GSM idle mode state. In 3GPP the requirement is that the CB message should be received whenever it is possible. This includes UMTS cell connected states.

- All parts of GSM 03.41 chapter 9.3 on message formats on GSM BTS – MS interface should be removed. The specification of the radio interface is a RAN2 item. An appropriate protocol architecture is approved by RAN2 with CR 011 on TS25.301 „Radio Interface Protocol Architecture“ (see R2-99977). For further information a clean version of TR25.925 „Radio interface for broadcast/multicast services“ is available in a LS from RAN2 to RAN3 (see R2-99990) there the new chapter 6 about CBS is incorporated.

Mannesmann Mobilfunk will provide a first draft of a CR on TS 23.041 after this meeting.

# References

| [1] | 3G TS 22.100. UMTS phase 1. Version 3.3.0. (Former UMTS 22.00) |
| --- | --- |