| 3GPP TSG-T2 #7 / ETSI SMG4 Ystad, SWEDEN, 22-26 November 1999 | TSGT2#7(99)1077 |
| --- | --- |

| Meeting Number | SERG #36 | SERG Doc 172/99 |
| --- | --- | --- |
| Meeting Date | 22-24th November, 1999 |  |
| Meeting Location | Capetown, South Africa |  |

Title:			Laison Statement on WIM and SAT OTA Administration

Source:		GSM Association SERG

To:	SMG 9, SMG 10

Cc:	WAP Forum CEG, WAP Forum SCEG, WAP Provisioning Group, GSM A Security Group, GSM A SCAG, 3GPP S1, T2, T3

SERG recognizes that the Wireless Identity Module, WIM, plays an important role in the WAP world. The WIM holds data like keys for authentication and ciphering, default gateway and administration information.

In order to ensure best security and easy handling of data SERG demands the storage of WAP specific parameters on the SIM Card.

SAT applications are becoming more and more important and allow offering of operator specific services.

Currently the main problem of SAT is that SAT applications can not efficiently be changed during the lifetime of a SIM. Therefore flexible Over The Air (OTA) administration possibilities are required to allow an easy exchange and upgrade of SAT applications.

SERG has studied the issue and concluded that using capabilities of WAP/MExE can speed up the standardisation process in this area.

As an example in the following a possible solution by using the WAP/MExE capabilities is described:

- SAT applications are transferred (by user download or server push services) via WAP/MExE to the terminal and stored in a cache.

- Now the integrity and security of the applications are checked with appropriate security algorithms stored and handled in a Wireless Identification Module (WIM) on the SIM card. If the security checks are successful, the SIM / SAT is notified with an appropriate event that new SAT application data is available.

- Now the SIM / SAT is able to store the data on the SIM. Additional security during this process is recommended and can be implemented optionally by the owner of the SIM Card (e.g. as described in 03.48 / 23.048 for SMS SAT transfer).

GSM Association SERG asks the addressed groups to discuss these issues and support the proposals. This is a matter of some urgency due to the imminent launch of WAP services by several operators.