| 3GPP TSG-T WG2 meeting #7 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Document |  |  |  |  |  |  |  |  |  |  | T2-99955 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ystad, Sweden, 22-26 November 1999 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | e.g. for 3GPP use the format TP-99xxx or for SMG, use the format P-99-xxx |  |  |  |  |  |  |  |  |  |  |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Please see embedded help file at the bottom of this page for instructions on how to fill in this form correctly. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | 07.07 |  |  |  |  |  | CR |  |  |  |  |  |  |  |  |  | Current Version: |  |  |  |  |  |  |  |  | 6.4.0 |  |  |  |  |
| GSM (AA.BB) or 3G (AA.BBB) specification number  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  CR number as allocated by MCC support team |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| For submission to: |  |  |  | T#6 |  |  |  |  | for approval |  |  |  |  |  |  | X |  |  | strategic |  |  |  |  |  |  |  |  |  |  |  |  |  | (for SMG |  |  |  |
| list expected approval meeting # here  |  |  |  |  |  | for information |  |  |  |  |  |  |  |  |  |  |  |  | non-strategic |  |  |  |  |  |  |  |  |  |  |  |  | X | use only) |  |  |  |
| Form: CR cover sheet, version 2 for 3GPP and SMG The latest version of this form is available from: ftp://ftp.3gpp.org/Information/CR-Form-v2.doc |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed change affects: |  |  |  |  |  |  | (U)SIM |  |  |  |  | ME |  |  |  | X | UTRAN / Radio |  |  |  |  |  |  |  |  |  |  | Core Network |  |  |  |  |  |  |  |  |
| (at least one should be marked with an X) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source: |  |  | Ericsson |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Date: |  |  |  |  |  |  | 1999-11-19 |  |  |  |  |  |
| Subject: |  |  | GPRS ATD command syntax |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Work item: |  |  | GPRS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Category: | F Correction |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release: |  |  |  |  | Phase 2 |  |  |  |  |  |  |
|  | A Corresponds to a correction in an earlier release |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release 96 |  |  |  |  |  |  |
| (only one category | B Addition of feature |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release 97 |  |  |  | X |  |  |
| shall be marked | C Functional modification of feature |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |  |  |  | Release 98 |  |  |  |  |  |  |
| with an X) | D Editorial modification |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release 99 |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release 00 |  |  |  |  |  |  |
| Reason for change: |  |  | Adds a new ' GPRS Service Code ' for the 'D' command that is intended for IP based services only (PDP types 'IP' and 'PPP'). The new service code allows for a more user friendly dialling string. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  | New section 10.2.1.2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other specs |  | Other 3G core specifications |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| affected: |  | Other GSM core specifications |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | MS test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | BSS test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | O&M specifications |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other comments: |  | There are two accompanying CRs for R'98 and R'99 too. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

<--------- double-click here for help and instructions on how to create a CR.

#### 10.2.1.2	Request GPRS IP service 'D'

- Table 1: D command syntax

| Command | Possible Response(s) |
| --- | --- |
| D*<GPRS_SC_IP>[*<cid>]# | CONNECT ERROR |

- Description

This command causes the MT to perform whatever actions are necessary to establish communication between the TE and the external PDN.

The V.25ter 'D' (Dial) command causes the MT to enter the V.25ter online data state and, with the TE, to start the specified layer 2 protocol. The MT shall return CONNECT to confirm acceptance of the command prior to entering the V.25ter online data state. No further commands may follow on the AT command line.

The detailed behaviour after the online data state has been entered is described briefly in clause 9, for IP, of GSM 07.60. GPRS attachment and PDP context activation procedures may take place prior to or during the PDP startup if they have not already been performed using the +CGATT and +CGACT commands.

When the layer 2 protocol has terminated, either as a result of an orderly shut down of the PDP or an error, the MT shall enter V.25ter command state and return the NO CARRIER final result code.

If <cid> is supported, its usage shall be the same as in the +CGDATA command. The +CGDCONT, +CGQREQ, etc. commands may then be used in the modem initialisation AT command string to set values for APN, QoS etc.

If <cid> is not supported or is supported but omitted, the MT shall attempt to activate the context using:

(a) any information provided by the TE during the PDP startup procedure, e.g. the TE may provide a PDP type and/or PDP address to the MT,

or, (b) a priori knowledge, e.g. the MT may implement only one PDP type,

or, (c) using the 'Empty PDP type' (GSM 04.08). (No PDP address or APN shall be sent in this case and only one PDP context subscription record shall be present in the HLR for this subscriber.)

This command may be used in both normal and modem compatibility modes.

NOTE.	The dial string conforms to the syntax specified in GSM 02.30.

- Defined Values

<GPRS_SC_IP>: (GPRS Service Code for IP) a digit string (value 98) which identifies a request to use the GPRS with IP (PDP types IP and PPP) .

<cid>: a digit string which specifies a particular PDP context definition (see +CGDCONT command).

- Implementation

Optional if the +CGDATA command is supported. If the D command is provided, then support for <cid> is optional. If it is not supported but a value is provided by the TE, the value shall be ignored and this shall not constitute an error.