| 3GPP TSG-T2 meeting #6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Document |  |  |  |  |  |  |  |  |  |  | T2-99958 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ystad, Sweden, 22-26 November 1999 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | e.g. for 3GPP use the format TP-99xxx or for SMG, use the format P-99-xxx |  |  |  |  |  |  |  |  |  |  |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Please see embedded help file at the bottom of this page for instructions on how to fill in this form correctly. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | 27.007 |  |  |  |  |  | CR |  |  |  |  |  |  |  |  |  | Current Version: |  |  |  |  |  |  |  |  | 3.2.0 |  |  |  |  |
| GSM (AA.BB) or 3G (AA.BBB) specification number  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  CR number as allocated by MCC support team |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| For submission to: |  |  |  | T2 |  |  |  |  | for approval |  |  |  |  |  |  | x |  |  | strategic |  |  |  |  |  |  |  |  |  |  |  |  |  | (for SMG |  |  |  |
| list expected approval meeting # here  |  |  |  |  |  | for information |  |  |  |  |  |  |  |  |  |  |  |  | non-strategic |  |  |  |  |  |  |  |  |  |  |  |  |  | use only) |  |  |  |
| Form: CR cover sheet, version 2 for 3GPP and SMG The latest version of this form is available from: ftp://ftp.3gpp.org/Information/CR-Form-v2.doc |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed change affects: |  |  |  |  |  |  | (U)SIM |  |  |  |  | ME |  |  |  | x | UTRAN / Radio |  |  |  |  |  |  |  |  |  |  | Core Network |  |  |  |  |  |  |  |  |
| (at least one should be marked with an X) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source: |  |  | NTT DoCoMo |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Date: |  |  |  |  |  |  | XX/09/1999 |  |  |  |  |  |
| Subject: |  |  | Additional parameter for +CBST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Work item: |  |  | Technical Enahncements |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Category: | F Correction |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release: |  |  |  |  | Phase 2 |  |  |  |  |  |  |
|  | A Corresponds to a correction in an earlier release |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release 96 |  |  |  |  |  |  |
| (only one category | B Addition of feature |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | x |  |  |  |  |  |  |  |  |  | Release 97 |  |  |  |  |  |  |
| shall be marked | C Functional modification of feature |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release 98 |  |  |  |  |  |  |
| with an X) | D Editorial modification |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release 99 |  |  |  | x |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release 00 |  |  |  |  |  |  |
| Reason for change: |  |  | To select the bearer services, 33600 bps (V.34),PIAFS32K and PIAFS64K. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  | 6.7 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other specs |  | Other 3G core specifications |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| affected: |  | Other GSM core specifications |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | MS test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | BSS test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | O&M specifications |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other comments: |  | T2 99721(Presentation to PHS/PIAFS – slides) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

<--------- double-click here for help and instructions on how to create a CR.

## 6.7	Select bearer service type +CBST

Table 1: +CBST parameter command syntax

| Command | Possible response(s) |
| --- | --- |
| +CBST=[<speed>[,<name>[,<ce>]]] |  |
| +CBST? | +CBST: <speed>,<name>,<ce> |
| +CBST=? | +CBST: (list of supported <speed>s),(list of supported <name>s),(list of supported <ce>s) |

Description

Set command selects the bearer service <name> with data rate <speed>, and the connection element <ce> to be used when data calls are originated (refer GSM 02.02 [1]). Values may also be used during mobile terminated data call setup, especially in case of single numbering scheme calls (refer +CSNS).

Test command returns values supported by the TA as compound values.

Defined values

NOTE:	The default values of the subparameters are manufacturer specific since they depend on the purpose of the device and data services provided by it. Not all combinations of these subparameters are supported by GSM (refer GSM 02.02 [1]).

<speed>:

0	autobauding (automatic selection of the speed; this setting is possible in case of 3.1 kHz modem and non-transparent service)

1	300 bps (V.21)

2	1200 bps (V.22)

3	1200/75 bps (V.23)

4	2400 bps (V.22bis)

5	2400 bps (V.26ter)

6	4800 bps (V.32)

7	9600 bps (V.32)

12	9600 bps (V.34)

14	14400 bps (V.34)

15	19200 bps (V.34)

16	28800 bps (V.34)

17	33600 bps (V.34)

34	1200 bps (V.120)

36	2400 bps (V.120)

38	4800 bps (V.120)

39	9600 bps (V.120)

43	14400 bps (V.120)

47	19200 bps (V.120)

48	28800 bps (V.120)

49	38400 bps (V.120)

50	48000 bps (V.120)

51	56000 bps (V.120)

65	300 bps (V.110)

66	1200 bps (V.110)

68	2400 bps (V.110 or X.31 flag stuffing)

70	4800 bps (V.110 or X.31 flag stuffing)

71	9600 bps (V.110 or X.31 flag stuffing)

75	14400 bps (V.110 or X.31 flag stuffing)

79	19200 bps (V.110 or X.31 flag stuffing)

80	28800 bps (V.110 or X.31 flag stuffing)

81	38400 bps (V.110 or X.31 flag stuffing)

82	48000 bps (V.110 or X.31 flag stuffing)

83	56000 bps (V.110 or X.31 flag stuffing)

115	56000 bps (bit transparent)

116	64000 bps (bit transparent)

120	32000 bps(PIAFS32k)

121	64000 bps(PIAFS64k)

also all other values below 128 are reserved by this TS

<name>:

0	data circuit asynchronous (UDI or 3.1 kHz modem)

1	data circuit synchronous (UDI or 3.1 kHz modem)

2	PAD Access (asynchronous) (UDI)

3	Packet Access (synchronous) (UDI)

4	data circuit asynchronous (RDI)

5	data circuit synchronous (RDI)

6	PAD Access (asynchronous) (RDI)

7	Packet Access (synchronous) (RDI)

also all other values below 128 are reserved by this TS

<ce>:

0	transparent

1	non-transparent

2	both, transparent preferred

3	both, non-transparent preferred

Implementation

Mandatory when data calls implemented.