TSG-T3WG3#3						Tdoc T3-99089 Helsinki; 16-18/3/99

Source:	Cellnet

## Title:	Directory Structure of USIM

# Introduction

This document builds upon discussions held at TSG T WG3#2, and considers service capabilities offered depending on the directory strutures in the USIM.

# Considerations

Five options of directory structure are shown; this document does not claim to be exhaustive.

Three attributes are considered, which are directly affected by choice of directory structure.

These are:

|  | Attribute | Comment |
| --- | --- | --- |
| 1 | Supports Roaming/Handover between UMTS networks | Assuming appropriate network architectures |
| 2 | Supports UMTS/GSM Handover on same network | Assuming a Dual Mode UMTS/GSM Terminal |
| 3 | Supports plastic Roaming to GSM Phase 2 terminal |  |

Although this document refers to GSM, this could be substituted by PDC.

# Assumptions

1) That rules for moving between files, and their selection by a GSM Phase 2 terminal remains as defined in TS GSM 11.11

2) That UMTS directories will be called from the MF level by application id's

# Conclusions

1) Options 5a & 5b provide functionality supporting the above service criteria

2) The Filters to provide GSM backward compatibility need to be specified as part of the USIM

|  | # Attribute |  |
| --- | --- | --- |
| 1 | Supports Roaming/Handover between UMTS networks | NO |
| 2 | Supports UMTS/GSM Handover on same network | NO |
| 3 | Supports plastic Roaming to GSM Phase 2 terminal | NO |

|  | # Attribute |  |
| --- | --- | --- |
| 1 | Supports Roaming/Handover between UMTS networks | # NO |
| 2 | Supports UMTS/GSM Handover | YES |
| 3 | Supports plastic Roaming to GSM Phase 2 terminal | YES |

|  | # Attribute |  |
| --- | --- | --- |
| 1 | Supports Roaming/Handover between UMTS networks | # YES |
| 2 | Supports UMTS/GSM Handover | Only on Network A |
| 3 | Supports plastic Roaming to GSM Phase 2 terminal | Only for network A |

| 1 | Supports Roaming/Handover between UMTS networks | YES |
| --- | --- | --- |
| 2 | Supports UMTS/GSM Handover | YES |
| 3 | Supports plastic Roaming to GSM Phase 2 terminal | NO |

Filter 1: 	acts as GSM directory, 7F20, and manages PIN;  PIN is used to identify whether GSM (A) or (B) is required;  filter then provides path to GSM under appropriate UMTS directory.

Filter 2 & 3:	acts as telecom directory, 7F10, and provides path to telecom directory

Filter 4:		allows a GSM terminal to return directly to the GSM directory from the telecom directory;  it therefore must remember the directory used previously to the telecom access

| 1 | Supports Roaming/Handover between UMTS networks | # YES |
| --- | --- | --- |
| 2 | Supports UMTS/GSM Handover | YES |
| 3 | Supports plastic Roaming to GSM Phase 2 terminal | YES |

Filter 1: 	acts as GSM directory, 7F20, and manages PIN;  PIN is used to identify whether GSM (A) or (B) is required;  filter then provides path to appropriate GSM directory.

Filter 2:		allows a GSM terminal to return directly to the GSM directory from the telecom directory;  it therefore must remember the directory used previously to the telecom access

| 1 | Supports Roaming/Handover between UMTS networks | # YES |
| --- | --- | --- |
| 2 | Supports UMTS/GSM Handover | YES |
| 3 | Supports plastic Roaming to GSM Phase 2 terminal | YES |