# Proposal for General Authentication Command to be incorporated in 31.101

Source: Nokia

### EXTERNAL AUTHENTICATE

| Code | Value |
| --- | --- |
| CLA | As specified in 12.2.1 |
| INS | As specified in 12.2.2 |
| P1 | Record number |
| P2 | Mode, see table 12.18 |
| Lc | Not present |
| Data | Not present |
| Le | Number of bytes to be read |

Parameter P1 specifies the authentication type:

| Coding | Meaning |
| --- | --- |
| '00' | Default Authentication Algorithm (specified by the application) |
| '01' | Authentication algorithm 1 (specified by the application) |
| '02' | Authentication algorithm 2 (specified by the application) |
| '03 | Authentication algorithm 3 (specified by the application) |
| '04' | Authentication algorithm 4 (specified by the application) |

All other values of P1 are RFU

P2<> '00' specifies the mode as follows:

| Coding | Meaning |
| --- | --- |
| '01' | Normal processing of the command |
| '02' | Resynchronisation Required |

All other values of P2 are RFU

If P1, P2= '00',  The default application authentication algorithm is indicated. The format, type and parameters of the command data field are proprietary to the application. The command data field is interpreted correctly by a terminal supporting the application.

If P1<>'00' The following tags are defined for the TLV object(s) in the command data field

| Tag | Meaning |
| --- | --- |
| 'XX' | RAND |
| 'XY' | AUTN |
| 'XZ' | Counter ID |

Command parameters/data:

In case P1, P2='00':

| Byte(s) | Description | Length |
| --- | --- | --- |
| 1 - X | Proprietary | X |

In case P1<>'00':

| Byte(s) | Description | Length |
| --- | --- | --- |
| 1-X | TLV Object(s) using Tags as defined in | X |

### Command Specific Errors

| SW1 | SW2 | Error description |
| --- | --- | --- |
| '98' | '62' | - Authentication failure, integrity problem |
| '98' | '64' | - Authentication failure, reference data mismatch |