| 3GPP TSG-T3 (USIM) Meeting #8 Bonn, Germany, August 23 - 25, 1999 | Tdoc T3-99232 |
| --- | --- |

## Disposal of EFs to be inherited from 2G SIM

Source: 	 31.102 Rappourteur

Agenda Item:

-----------------

Abstract

This document re-discusses disposals of the EFs which have been decided to be removed from 31.102 according to the previous discussion in T3#2.

1.Introduction

Document T3-99052 mentions disposals of EFs which are unclear if they are to be inherited to 3G environment.

2.Discussion

Following table lists up the EFs which have been removed from 31.102, and its cause of deletion.

Table1: EFs removed from 31.102

| EFs removed from 31.102 | Cause | Consideration | Remarks |
| --- | --- | --- | --- |
| LP, ELP | Simply replaced by PL | - |  |
| Phase | Simply replaced by APPI | - |  |
| VGCS,VGCSS | It is not clear if there is a requirement for this service in 3G. | As 22.100 states that " The standard shall support GSM Release '99 supplementary services. The control of such supplementary services shall be the same as for GSM, from the user's perspective.", this EFs should be re-installed. |  |
| VBS,VBSS | So as the VGCS/VGCSS. | So as the VGCS/VGCSS. |  |
| eMLPP,AAeM | So as the VGCS/VGCSS. | So as the VGCS/VGCSS. |  |
| DCK | So as the VGCS/VGCSS. | So as the VGCS/VGCSS. |  |
| CNL | So as the VGCS/VGCSS. | So as the VGCS/VGCSS. |  |
| NIA | So as the VGCS/VGCSS. | So as the VGCS/VGCSS. |  |
| LND | Simply replaced by OCI | - |  |
| BDN, EXT4 | … | So as the VGCS/VGCSS. |  |

3.Conclusion and Proposal

According to 22.100 statements, this document proposes to re-install following EFs.

-EF_VGCS

-EF_VGCSS

-EF_VBS

-EF_VBSS

-EF_eMLPP

-EF_AAeM

-EF_DCK

-EF_CNL

-EF_NIA

-EF_BDN

-EF_EXT4