3GPP TSG-T3#8 (USIM)							Tdoc T3-99240

23-25th of August, Bonn

Germany

## List of inconsistencies between current 11.14 and 3G documents

Source: 	 NTT DoCoMo, Japan Telecom, Mitsubishi Electric, Nippon Ericsson, Dai-Nippon Printing

Agenda Item:

Document for: Discussion

-----------------

1.Introduction

Our companion contribution titled "How to Upgrade GSM11.14 to 3GTS31.014/114" tries to show some possible roadmaps to transfer of SIMATK to 3G environment. This contribution complement its logic by showing how much impacts are foreseen in order to align with related 3G documents (i.e. 31.101/102 and 3G requirement documents). The list is based on our investigation and may need to be further studied.

2.List Of Inconsistencies between 11.14 and 3G documents

2.1.Newly-added 3G Features that should be taken into account

Above all, we reviewed whole stage1 documents (21 and 22 series) in order to pick up newly-added 3G services or service features which have any relationship with SIM application toolkit. As a result of the review, we reached a conclusion that following features should be taken into account.

- Multi-USIM environment (21.111)

- Multi-call capability(22.135)

Addition to that, following USIM specific issue should also be taken into account.

- Split of specification into generic part(31.101) and USIM application specific part(31.102)

- Refer latest ISO/IEC7816 (1995/1997 version) than 11.11/11.14 (11.11 and 11.14 are mainly referring 198x version)

More specifically, above 4) involves followings;

4-1) Introduction of T=1 transport protocol (as optional for USIM)

4-2) Introduction of APDU message structure

2.2.List of inconsistencies between 11.14v7.2.0 and 3G documents

| ID | Items | Impacting 3G features | Description | Remarks |
| --- | --- | --- | --- | --- |
| 1 | General | 3) | In order to align with 3G document structure, 11.14 may also needs to be separated into generic and USIM- specific parts in line with 31.101/102. But this is just an editorial issue, and no problem on actual implementation is foreseen even if it is not separated. |  |
| 2 | General | 1) | In multi-USIM environment, multiple USIMs are possible to be activated simultaneously in a UICC. And in order for UICC to identify each USIM, logical channel toward each USIM are assigned. Current 11.14 does not support this kind of features. This item will not be a problem for the operator who does not apply multi-USIM in his 3G network. |  |
| 3 | Proactive SIM (in general) | 4-1), 4-2) | Proactive commands in 11.14 is designed based on the T=0 TPDU format. Therefore difference between T=0 and T=1 like 4-1,2) cause inconsistencies if current 11.14 is used under 3G environment. However, 4-1) and 4-2) will not be a problem for the operator who does not apply T=1 UICC in his 3G network. |  |
| 4 | SET UP CALL | 2) | SET UP CALL proactive command does not support to set up multi (2nd or 3rd or further) call. However, this will not provably be a problem if single-voice-call will only be permitted in R99. |  |
| 5 |  |  |  |  |

3. Conclusion and Proposal

This document shows the list of inconsistencies between 11.14v7.2.0 and 3G documents.