3GPP TSG-T3#8 (USIM)							Tdoc T3-99255

23-25th of August, Bonn

Germany

## Discussion Document On The User Parameters For  ADN Implementation

Source: 	 Vodafone

The parameters listed are intended to guide T3 as to the performance of any solution for the 3rd generation ADN service.  The parameters listed have two levels of operation specified:

- Basic – the basic level of service acceptable for this parameter.

- Optimal – the preferred level of service that for this parameter.

| #### Parameter | Optimal | Basic |
| --- | --- | --- |
| # User Visible Parameters |  |  |
| Time for Phone book (whatever size) to be available to user following ME switch on. | Better than 5 seconds | Better than 30 seconds |
| Time for next entry to appear when the user scans through the address book. | < 100ms | Better than 500ms |
| Time for user to be able to view additional information on a selected phone book entry. | < 100ms | Better than 1 second |
| Time to match a label to an incoming number / ring style. | < 100ms | Better than 500ms |
| Time to match a label to an outgoing number | < 100ms | Better than 1 second |
| All address book timings when the USIM is in a 2G Terminal | Faster than a basic SIM | Same as a basic SIM |
| Address book functionality when the USIM is in a 2G Terminal | All items selectable accessible and changeable within GSM11.11 limits | Some items not accessible or changeable |
| ### Operator Visible Parameters |  |  |
| EEPROM memory required to implement that 3G ADN function | In less than 16KBytes | No more than 50Kbytes |
| ADN management overhead in EEPROM (total size of memory used – size of raw information) | Less than 2% | Less than 10% |
| ROM memory used for ADN management routines | Less than 2KBytes | Less than 10KBytes |
| FDN service implementation | FDN separate file to ADN file | FDN service within ADN service |
| SDN service | SDN separate file to ADN file | SDN service within ADN service |
| Email storage in ADN service | Only present if used | Optionally filled – always present |
| Second name storage in ADN service | Only present if used | Optionally filled – always present |
| Second number storage in ADN service | Only present if used | Optionally filled – always present |
| Scaling of services: FDN, SDN, Email, Second name and Second number. | Service individually scaleable by operator | Total allocation of all services scaleable by operator |
| Phone book deleted memory recovery | Automatic memory recovery | User can initiate recovery |
| USIM size required to implement an ADN service equivalent to a GSM ADN service with 100 records | 8K USIM | 16K USIM |
| OTA update of ADN service | Fully available | Not available |