| 3GPP T3 Meeting #15 San Diego, USA, 16 - 18 August, 2000 | Tdoc T3-000365 |
| --- | --- |

# KID and KIC algorithms to be used in 03.48 for Open Platform

# Date:	8th August 2000

Source:	Gemplus

In the 03.48 command packet structure the KIC and KID fields are used to specify the key set as defined by the Open Platform specification (coded on the 4 msb) and the algorithm to be used with the corresponding key of the key set (coded on the 4 lsb). Yet in the Open Platform specification each key of a key set is implicitly associated with a specific algorithm, so that specifying both the key set and the algorithm to be used with the key in the KID and KIC fields can result in an ambiguity : which algorithm shall be used : the algorithm implicitly associated with the Key or the algorithm specified in the 4 least significant bits of the KIC and KID parameters ?