| 3GPP TSG-T3 (USIM) Meeting #11 Sophia Antipolis, France, 8 - 10 December, 1999 | Tdoc T3-99401 |
| --- | --- |

# Liaison Statement

From: 	3GPP TSG-T3 (USIM)

To: 	3GPP TSG-T2/SWG1 (MExE)

Cc: 	S1, S2, N2, CN OSA ad hoc, WAP UAProf

Subject: 	Response to liaison statement on the interaction between MMS, SAT, MExE, non-MExE terminals and Camel/Open Service Architecture

___________________________________________________________________________

The T3 (USIM) group thanks T2/SWG1 for the copy of the Liaison Statement (Tdoc T3-99374, T2#7(99)1030), concerning the notion of indicating to the server that a terminal supports USAT.

T3 has considered this issue, and has come to the following conclusions:

USAT has an existing mechanism for capability negotiation between the ME and UICC. The UICC is told the ME’s capabilities through the PROFILE DOWNLOAD procedure. If a server needs to know the ME’s capabilities, an application in the UICC could inform the server.

Gaining knowledge of the UICC’s capabilities is more complex, as USAT is a “toolkit” and the UICC only needs to support the components that are used by resident applications. The SIM API (an open architecture for SAT, and USAT for 3G, based on JavaCard) does define a standard capability set that applets loaded onto a UICC can rely on. Negotiation and capability checking is built into the existing applet loading protocol for this architecture. Therefore this issue has been solved for UICCs complying to the SIM API architecture, but not for the general USAT case; T3 plan to study this issue as part of Release 2000.