| 3GPP TSG-T3 (USIM) Meeting #12 Rome, Italy, 18 - 21 January, 2000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Tdoc T3-000028 (superseeds T3-000020) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3G CHANGE REQUEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Please see embedded help file at the bottom of this page for instructions on how to fill in this form correctly. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | TS 31.110 |  |  |  |  |  | CR |  |  |  | 001 |  |  |  |  | Current Version: |  |  |  |  |  |  |  |  |  | V3.0.0 |  |  |  |  |  |
| 3G specification number  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  CR number as allocated by 3G support team |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| For submission to TSG |  |  |  |  | T # 7 |  |  | for approval |  |  |  |  |  | x |  |  | (only one box should |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| list TSG meeting no. here  |  |  |  |  |  | For information |  |  |  |  |  |  |  |  |  |  | be marked with an X) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Form: 3G CR cover sheet, version 1.0 The latest version of this form is available from: ftp://ftp.3gpp.org/Information/3GCRF-xx.rtf |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed change affects: |  |  |  |  |  |  | USIM |  |  | x | ME |  |  |  |  |  |  |  |  | UTRAN |  |  |  |  |  |  |  | Core Network |  |  |  |  |  |  |  |  |
| (at least one should be marked with an X) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source: |  |  | T3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Date: |  |  |  |  | 19/01/00 |  |  |  |  |  |  |
| Subject: |  |  | Definition USIM and UICC Application Identifier coding for release management |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3G Work item: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Category: | F Correction |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | A Corresponds to a correction in a 2G specification |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (only one category | B Addition of feature |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| shall be marked | C Functional modification of feature |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| with an X) | D Editorial modification |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reason for change: |  |  | Change the USIM and UICC Application identifier coding to allow release management. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  | Annex A, Annex B, Annex C (new) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other specs |  | Other 3G core specifications |  |  |  |  |  |  |  |  |  |  | X |  |  List of CRs: |  |  |  |  |  |  |  |  | TS 31.102 - CR 001 |  |  |  |  |  |  |  |  |  |  |  |  |
| affected: |  | Other 2G core specifications |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | MS test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | BSS test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | O&M specifications |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Annex A: (Normative)
Allocated 3G PIX numbers

Table A.1 below is shown for information. The original table can be found in EG 201 220 [12].

| - |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |

Table A.1: Allocated 3G PIX numbers

| 3G Application Identifiers |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Application | AID |  |  |  | 3G document |
|  |  | RID (note 1) | 3G App Code | PIX | (note 2) |
| 3G UICC |  | 'A000000087' | '1001' | See annex B for further coding details | 3G TS 31.101 [8] |
| 3G USIM |  | 'A000000087' | '1002' | See annex B for further coding details | 3G TS 31.102 [9] |
| 3G USIM toolkit |  | 'A000000087' | '1003' | See annex C for further coding details | 3G TS 31.111 [10] |
| NOTE 1: The 3GPP RID, as registered by ISO/IEC according to ISO/IEC 7816-5 [2], is 'A000000087'. NOTE 2: It is the responsibility of the 3GPP technical body, in charge of the application standardization, to inform the ETSI Secretariat when the respective 3G document is withdrawn or renumbered. |  |  |  |  |  |

Annex B: (Normative) 
Coding of the PIX for 3G UICC and USIM Applications

The following codings apply for the structure of the PIX when the application is a 3G telecommunication Integrated Circuits (IC) card application.

Digit 1-4	3G application code

Coding:	As specified in clause 4.2 of this document, and as shown in  Annex A.

Digits 5-8	Country code

Coding:	As specified in clause 4.2 of this document

Digits 9-14	Application provider code

Coding:	As defined below.

| 9 |  | 10 |  | 11 |  | 12 |  | 13 |  | 14 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  |  | Industry Code '89' for Telecom |
|  |  |  |  |  |  |  |  |  |  |  |  | Card issuer Code. Coded in BCD and right justified. Unused digits to be padded with 'F' on the left. |

Card issuer code and Industry code are coded in line with ITU-T recommendation E.118 [3].

Digits 15 up to 22	Application provider field. 8 digits

Coding:	
Digit 15 to 20, coded in BCD, refer to the specification version xx.yy.zz.
Digit 21 to 22 are coded in hexadecimal.
The application provider field format is as defined below:

| 15 |  | 16 |  | 17 |  | 18 |  | 19 |  | 20 |  | 21 |  | 22 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Application Provider specific data |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Specification version xx.yy.zz |

Application Provider specific data: For application administration purposes.

Annex C (Normative):
Coding of the PIX for 3G USIM Toolkit applications

The following codings apply for the structure of the PIX when the application is a 3G USIM Toolkit Application.

Digit 1-4	3G application code

Coding:	As specified in clause 4.2 of this document, and as shown in Annex A.

Digits 5-8	Country code

Coding:	As specified in clause 4.2 of this document

Digits 9-14	Application provider code

Coding:	As defined below.

| 9 |  | 10 |  | 11 |  | 12 |  | 13 |  | 14 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  |  | Industry Code ‘89’ for Telecom |
|  |  |  |  |  |  |  |  |  |  |  |  | Card issuer Code. Coded in BCD and right justified. Unused digits to be padded with ‘F’ on the left. |

Card issuer code and Industry code are coded in line with ITU-T recommendation E.118 [3].

Digits 15 up to 22	Application provider field. 8 digits

Coding:	Hexadecimal, as defined below.

| 15 |  | 16 |  | 17 |  | 18 |  | 19 |  | 20 |  | 21 |  | 22 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Application Provider specific data |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Toolkit Application Reference (TAR) |

Toolkit Application Reference as specified in 3G TS 31.xxx [TBD transfered GSM 03.48] [11], is managed by the application provider (i.e. operator in that case)

Application Provider specific data: For application administration purposes.

Annex D: (Informative)
Allocated ETSI PIX numbers

Table D.1: Allocated ETSI PIX numbers

Table D.1 below is shown for information. The original table can be found in EG 201 220 [12].

| ETSI Application Identifiers |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Application | AID |  |  |  | ETSI document |
|  |  | RID (note 1) | ETSI App Code | PIX |  |
| Reserved |  | 'A000000009' | '0000' | Reserved for ETSI |  |
| GSM |  | 'A000000009' | '0001' | See EG 201 220 [12] for further coding details | GSM 11.11 [5] |
| GSM SIM toolkit |  | 'A000000009' | '0002' | See EG 201 220 [12] for further coding details | GSM 11.14 [6] |
| GSM SIM API for Java™ Card |  | 'A000000009' | '0003' | See EG 201 220 [12] for further coding details | GSM 03.19 [7] |
| NOTE 1: The ETSI RID, as registered by ISO/IEC according to ISO/IEC 7816-5 [2], is 'A000000009'. |  |  |  |  |  |