AID used for API test package:

RID : A0 00 00 00 09 PIX :

- ETSI application code : 00 02

- Country code : FF FF

- Application provider code : FF FF 89

- Application provider field :

- TAR (3 bytes/ 24 bits):

- test package (3 bits):

- 000 [reserved](TAR= 00.00.00 is reserved for Card Manager)

- 001 API

- 010 Framework

- 011 Loader

- 111 org.t3.gsm0319.tests.util

- other values are RFU

- Specific applet test name

for org.t3.gsm0319.tests.util test package:

everything set to 0 rest is RFU

for API test package :

Package (3 bits):

001 sim.access

010 sim.toolkit

other are RFU

Class (5 bits):

need to be assigned specification order

Method (6 bits):

need to be assigned specification order

Applet Class number ( 5 bits):

linked to Test Area

Applet Instance number (2 bits)

defined in the test procedure

- Application Provider specific field (1 byte):

- Type of element :

- Package : 	[00]

- Applet class :  	[01]

- Applet Instance : 	[02]

AID of Package org.t3.gsm0319.tests.util :

A0 00 00 00 09