Comprehension Required Flags

GSM 03.19 version 7.1.0 Release 1998

Classes	Methods 						Comprehension required

flags

ViewHandler

findTLV	(byte tag,

byte occurence)				independent

findAndCopyValue	(byte tag,

byte[] dstBuffer,

short dstOffset)			 independent

findAndCopyValue	(byte tag,

byte occurence,

short valueOffset,

byte[] dstBuffer,

short dstOffset,

short dstLength)			 independent

findAndCompareValue	(byte tag,

byte[] compareBuffer,

short compareOffset)		independent

findAndCompareValue	(byte tag,

byte occurence,

short valueOffset,

byte[] compareBuffer,

short compareOffset

short compareLength)	independent

ProactiveHandler

initDisplayText	(byte qualifier,

byte dcs,

byte[] buffer,

short offset,

short length)				all set to 1

initGetInkey	(byte qualifier,

byte dcs,

byte[] buffer,

short offset,

short length)					all set to 1

initGetInput	(byte qualifier,

byte dcs,

byte[] buffer,

short offset,

short length,

short minRespLength,

short maxRespLength)			all set to 1

init		(byte type,

byte qualifier,

byte dstDevice)				all set to 1

GSM 11.14 version 8.2.0 Release 1999

Set up menu

If the SIM provides an Items Next Action Indicator data object, the comprehension required flag shall be set to '0'.

Select Item

The Comprehension Required flag in the Item Identifier data object shall be set to 0, indicating that it is not mandatory for the ME to support indication of the default item.

If the SIM provides an Items Next Action Indicator data object, the comprehension required flag shall be set to '0'.

Envelope Call Control

Command data

Location information: this data object contains the identification (MCC, MNC, LAC, Cell Identity) of the current serving cell of the MS. The comprehension required flag of this data object in this command shall be set to '0'.

Response data

Alpha identifier: this data object is only required if the SIM requests a particular indication to be given to the user. The comprehension required flag of this data object shall be set to '0'.

Terminal Response

Command details: this data object shall be identical to the command details data object (including the comprehension required flag) given by the SIM in the proactive command to which the ME is giving the result.

If the ME has not received a valid Command number, all Command Details object values shall be set to '00' and the Result shall indicate an error.

Unknown tag value :

If the comprehension required flag in an unknown tag is set to '1', and the ME either does not recognize or is not expecting one or more of the SIMPLE-TLV objects in the message, then it shall respond with "Command data not understood by ME".

If the comprehension required flag is set to '0', then the ME shall read the length field that follows and ignore that object. In this case the ME will be able to carry out the command without the SIMPLE-TLV components that it cannot understand. It shall respond with "command performed with partial comprehension ".