Class ToolkitException

Exception Constants

##### Test Area Reference: API_1_TKE_CONST

##### Conformance Requirement:

There is no API, only constants.

Normal Execution

CR1 : The Exception Constants must all have the same name and value than in the 03.19 normalization.

##### Test suite files:

No additional requirements for the GSM personalisation.

Test Script : 			API_1_TKE_CONST_0.scr

Test case trigger:	1- applet check every constant

Test Applet: 			API_1_TKE_CONST_0.java

Installation parameter:	Same as default applet

Load Script : 			API_1_TKE_CONST.ldr

At the end of the script the applet is loaded but not instantiated.

Conversion parameter:	API_1_TKE_CONST.cnv

##### Test Procedure

First applet triggered :

| Test Case |  |
| --- | --- |
| Id | Test purpose |
| 01 | Check constant HANDLER_OVERFLOW=1 |
| 02 | Check constant HANDLER_NOT_AVAILABLE=2 |
| 03 | Check constant UNAVAILABLE_ELEMENT=3 |
| 04 | Check constant MENU_ENTRY_NOT_FOUND=4 |
| 05 | Check constant REGISTRY_ERROR=5 |
| 06 | Check constant EVENT_NOT_SUPPORTED=6 |
| 07 | Check constant EVENT_ALREADY_REGISTERED=7 |
| 08 | Check constant OUT_OF_TLV_BOUDARIES=8 |
| 09 | Check constant ME_PROFILE_NOT_AVAILABLE=9 |
| 10 | Check constant ALLOWED_LENGTH_EXCEEDED=10 |
| 11 | Check constant NO_TIMER_AVAILABLE=11 |
| 12 | Check constant INVALID_TIMER_ID=12 |
| 13 | Check constant EVENT_NOT_ALLOWED=13 |
| 14 | Check constant BAD_INPUT_PARAMETER=14 |

We expect for each case the comparison is true

##### Test Coverage

| CR number | Test case number |
| --- | --- |
| 1 | each case |